3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240099

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3341 | rev | - | Current version: | 17.9.0 |  |
| For HELPon using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC testcase 8.1.1.2.4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Starpoint |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-01-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | v_NR_CellInfo.Sysinfo.BcchInfo.SIB1.message_.c1.systemInformationBlockType1.nonCriticalExtension and v_NR_CellInfo.Sysinfo.BcchInfo.SIB1.message_.c1.systemInformationBlockType1.nonCriticalExtension.nonCriticalExtension are not initialized, which will lead to ttcn error then using fucntion f_NR_CellInfo_SetSIB1_v1700_IEs |  |  |  |  |  |  |  |  |
| Summary of change: |  | Initialize related IEs and send the configuration to SS. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the testcase |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.1.2.4.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections required	2

2.1	fl_TC_8_1_1_2_4_TestBody	2

- Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D23wk49 related to the title of this CR.

Contact:	Ting Gao

gaoting@starpointcomm.com

- Corrections required

  - fl_TC_8_1_1_2_4_TestBody

| Functionname | fl_TC_8_1_1_2_4_TestBody |
| --- | --- |
| Reason for change | v_NR_CellInfo.Sysinfo.BcchInfo.SIB1.message_.c1.systemInformationBlockType1.nonCriticalExtension and v_NR_CellInfo.Sysinfo.BcchInfo.SIB1.message_.c1.systemInformationBlockType1.nonCriticalExtension.nonCriticalExtension are not initialized, which will lead to ttcn error then using fucntion f_NR_CellInfo_SetSIB1_v1700_IEs |
| Summary of change | Initialize related IEs and send the configuration to SS. |
| TTCN module | RRC_ConnEst_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change:

function fl_TC_8_1_1_2_4_TestBody() runs on NR5GC_PTC { //@sic R5-211312 R5s210243 Ch. 2 R5s210595 sic@ var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var UE_NR_Capability v_NrCapability; var NR_CellInfo_Type v_NR_CellInfo := f_NR_CellInfo_Get(nr_Cell1); var GMM_MobilityInfo_Type v_GMM_MobilityInfo; //@siclog "Step 1" siclog@ //System information that includes extended and spare fields that the UE does not comprehend is broadcasted on BCCH. v_NR_CellInfo.Sysinfo.BcchInfo.MIB.message_.mib.spare := '1'B; // Table 8.1.1.2.4.3.3-1: MIB f_NR_CellInfo_Set(nr_Cell1, v_NR_CellInfo); //Add SIB21 with SIB2 in SI[0] as per Table 8.1.1.2.4.3.3-2: SystemInformation (step 1, Table 8.1.1.2.4.3.2-1) //Modify Sysinfo combination to NR_21 f_NR_SetSysinfoCombination_Specific8_1_1_2_4(nr_Cell1, cs_SIB21_r17); //@sic R5-223423 R5-226045 R5-230109 sic@ f_NR_CellInfo_SetSIB1_v1700_IEs(nr_Cell1, cs_SIB1_v1700_IEs_HSDN_Cell_r17); //@sic R5-237351 sic@ //@siclog "Step 2" siclog@ //The UE is switched on. f_UT_SwitchOnUE(UT); //@siclog "Step 3" siclog@ // Does the UE transmit an RRCSetupRequest message on NR Cell 1? f_NR_RRC_SetupRequest_Def(nr_Cell1); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 3"); //@siclog "Step 4" siclog@ // The SS transmits an RRCSetup message on NR Cell 1. f_NR_RRCSetup_Def (nr_Cell1); //@siclog "Step 5" siclog@ // Check: Does the UE transmit an RRCSetupComplete message to confirm the successful completion of the connection establishment and initiates the registration procedure by including the REGISTRATION REQUEST message? v_ReceivedMsg := f_NR_RRCSetupComplete_Def(nr_Cell1, cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def), (tsc_SHT_NoSecurityProtection, tsc_SHT_IntegrityProtected)); if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Initial_NoSecurity)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } //@siclog "Step 6 - 10a2" siclog@ // Steps 5 to 20a1 of the registration procedure described in TS 38.508-1 Table 4.5.2.2-2 are performed on NR Cell 1. f_NR5GC_RRC_Idle_Steps5_20(v_GMM_MobilityInfo, nr_Cell1, v_ReceivedMsg, TESTMode_OFF, -, Initial_NoSecurity); v_NrCapability := f_NR_MobileInfo_NRCap_Get(); if (not match (v_NrCapability.accessStratumRelease, (rel15, rel16))) //@sic R5-211412 R5-226045 sic@ { f_SetVerdictInconc(__FILE__, __LINE__, "Test Case applicable to Rel-15 or Rel-16 UE"); // Condition for Rel-15/16 UE only checked at the very end of the test case due to test flow as UE is only registered at the end and preamble is done with STATE_OFF_0A } }

After Change:

function fl_TC_8_1_1_2_4_TestBody() runs on NR5GC_PTC { //@sic R5-211312 R5s210243 Ch. 2 R5s210595 sic@ var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var UE_NR_Capability v_NrCapability; var NR_CellInfo_Type v_NR_CellInfo := f_NR_CellInfo_Get(nr_Cell1); var GMM_MobilityInfo_Type v_GMM_MobilityInfo; //@siclog "Step 1" siclog@ //System information that includes extended and spare fields that the UE does not comprehend is broadcasted on BCCH. v_NR_CellInfo.Sysinfo.BcchInfo.MIB.message_.mib.spare := '1'B; // Table 8.1.1.2.4.3.3-1: MIB v_NR_CellInfo.Sysinfo.BcchInfo.SIB1.message_.c1.systemInformationBlockType1.nonCriticalExtension := valueof(cs_SIB1_v1610_IEs_Def); v_NR_CellInfo.Sysinfo.BcchInfo.SIB1.message_.c1.systemInformationBlockType1.nonCriticalExtension.nonCriticalExtension := valueof(cs_SIB1_v1630_IEs_Def(omit)); f_NR_CellInfo_Set(nr_Cell1, v_NR_CellInfo); //Add SIB21 with SIB2 in SI[0] as per Table 8.1.1.2.4.3.3-2: SystemInformation (step 1, Table 8.1.1.2.4.3.2-1) //Modify Sysinfo combination to NR_21 f_NR_SetSysinfoCombination_Specific8_1_1_2_4(nr_Cell1, cs_SIB21_r17); //@sic R5-223423 R5-226045 R5-230109 sic@ f_NR_CellInfo_SetSIB1_v1700_IEs(nr_Cell1, cs_SIB1_v1700_IEs_HSDN_Cell_r17); //@sic R5-237351 sic@ f_NR_SS_ConfigureSysinfo(nr_Cell1, -, f_NR_InitialiseBcchToPdschConfig(nr_Cell1)); //@siclog "Step 2" siclog@ //The UE is switched on. f_UT_SwitchOnUE(UT); //@siclog "Step 3" siclog@ // Does the UE transmit an RRCSetupRequest message on NR Cell 1? f_NR_RRC_SetupRequest_Def(nr_Cell1); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 3"); //@siclog "Step 4" siclog@ // The SS transmits an RRCSetup message on NR Cell 1. f_NR_RRCSetup_Def (nr_Cell1); //@siclog "Step 5" siclog@ // Check: Does the UE transmit an RRCSetupComplete message to confirm the successful completion of the connection establishment and initiates the registration procedure by including the REGISTRATION REQUEST message? v_ReceivedMsg := f_NR_RRCSetupComplete_Def(nr_Cell1, cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def), (tsc_SHT_NoSecurityProtection, tsc_SHT_IntegrityProtected)); if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Initial_NoSecurity)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } //@siclog "Step 6 - 10a2" siclog@ // Steps 5 to 20a1 of the registration procedure described in TS 38.508-1 Table 4.5.2.2-2 are performed on NR Cell 1. f_NR5GC_RRC_Idle_Steps5_20(v_GMM_MobilityInfo, nr_Cell1, v_ReceivedMsg, TESTMode_OFF, -, Initial_NoSecurity); v_NrCapability := f_NR_MobileInfo_NRCap_Get(); if (not match (v_NrCapability.accessStratumRelease, (rel15, rel16))) //@sic R5-211412 R5-226045 sic@ { f_SetVerdictInconc(__FILE__, __LINE__, "Test Case applicable to Rel-15 or Rel-16 UE"); // Condition for Rel-15/16 UE only checked at the very end of the test case due to test flow as UE is only registered at the end and preamble is done with STATE_OFF_0A } }