3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240104

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3343 | rev | - | Current version: | 17.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC testcase 10.1.8.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNS_Ph2-UEConTest, TEI17_Test |  |  |  |  |  | Date: |  |  | 2024-01-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the current TTCN implementation, after Step 9 based on pc_noOf_PDUsNewConnection > 0, the PDU Session establishment steps are executed. This worked well until R5-236305 was agreed. After this, in order to get an AT command prompt for establishment of a PDU Session for a device that is non-IMS, both PICS pc_noOf_PDUsSameConnection and pc_noOf_PDUsNewConnection have to be set as 0. With this setting, the testcase fails at Step 9, because after Registration Complete, the user directly gets a AT command prompt for Step 10, and the UE sends a PDU Session Establishment REQ on SRB2, which is not yet established. Besides, this also results in Step 9 not being followed fully. This needs to be addressed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated the TTCN implementation to keep aligned with settings of PICS pc_noOf_PDUsSameConnection and pc_noOf_PDUsNewConnection as per R5-236305. Please see below. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.1.8.2.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Overview	4

2	Corrections required	5

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D23wk49 related to the title of this CR.

Contact:	Parikshit Bhise

parikshit.bhise@rohde-schwarz.com

# Corrections required

  - Correction to function fl_TC_10_1_8_2_Body

| Function name | fl_TC_10_1_8_2_Body |
| --- | --- |
| Reason for change | In the current TTCN implementation, after Step 9 based on pc_noOf_PDUsNewConnection > 0, the PDU Session establishment steps are executed. This worked well until R5-236305 was agreed. After this, in order to get an AT command prompt for establishment of a PDU Session for a device that is non-IMS, both PICS pc_noOf_PDUsSameConnection and pc_noOf_PDUsNewConnection have to be set as 0. With this setting, the testcase fails at Step 9, because after Registration Complete, the user directly gets a AT command prompt for Step 10, and the UE sends a PDU Session Establishment REQ on SRB2, which is not yet established. Besides, this also results in Step 9 not being followed fully. This needs to be addressed. |
| Summary of change | Updated the TTCN implementation to keep aligned with settings of PICS pc_noOf_PDUsSameConnection and pc_noOf_PDUsNewConnection as per R5-236305. Please see below. |
| TTCN module | GSM_NR5GC.ttcn |
| MCC160 Comment |  |

Before:

<<SKIPPED CODE>> f_NR5GC_Registration(v_NGC_NASCellA); // @sic R5s230782 sic@ if (pc_noOf_PDUsSameConnection > 0) // @sic R5s230782 sic@ { f_NR5GC_RRC_Idle_Steps16_20(v_NGC_NASCellA, -, TESTMode_OFF); f_NR5GC_TriggerSinglePDUSession(v_NGC_NASCellA, f_NR5GC_MobileInfo_GetNoOfSessions()+1, -, tsc_RandomAPN, cs_S_NSSAI_SST1eMBB); }else { if (pc_noOf_PDUsNewConnection > 0 ) { f_NR_RRCRelease(v_NGC_NASCellA); f_UT_RequestPDUSession(UT, f_NR5GC_MobileInfo_GetNoOfSessions()+1, tsc_RandomAPN, -, -, v_NSSAI); v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (v_NGC_NASCellA, tsc_NR_RRC_TI_Def, ?); // @sic R5s200489 sic@ if (not f_Check_NG_ServiceReqMsg (v_ReceivedMsg.Pdu, cs_NAS_KeySetIdentifier_lv(f_NR_SecurityKSIamf_Get (), tsc_NasKsi_NativeSecurityContext), ?, '0000'B)) { // @sic R5s210189 sic@ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Service Request Message Failed"); } f_NR_RRC_ActivateSecurity (v_NGC_NASCellA, valueof(v_ReceivedMsg.SecurityProtection.NasCount)); // Use NasCount returned from Service Req SRB.send (cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellA, tsc_NR_RbId_SRB1, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, f_Get_NG_ServiceAcceptMsg(v_ReceivedMsg.Pdu.Msg.service_Request.pduSessionStatus)))); f_NR5GC_PDUSessionEstablishment (v_NGC_NASCellA, pc_noOf_PDUsNewConnection, tsc_NR_RbId_SRB1, // @sic R5-206291 sic@ omit, // @sic R5s210134 sic@ *, *, crs_SSC_Mode(-, '001'B), omit, omit, cs_Session_AMBR ('06'O, '05'O, '0004'O, '05'O, '0004'O), cs_S_NSSAI_SST1eMBB_WithIEI, omit); } } <<SKIPPED CODE>>

After:

<<SKIPPED CODE>> f_NR5GC_Registration(v_NGC_NASCellA); // @sic R5s230782 sic@ if (pc_noOf_PDUsSameConnection > 0) // @sic R5s230782 sic@ { f_NR5GC_RRC_Idle_Steps16_20(v_NGC_NASCellA, -, TESTMode_OFF); f_NR5GC_TriggerSinglePDUSession(v_NGC_NASCellA, f_NR5GC_MobileInfo_GetNoOfSessions()+1, -, tsc_RandomAPN, cs_S_NSSAI_SST1eMBB); }else { //WA#10_1_8_2 COMMENTED OUT if (pc_noOf_PDUsNewConnection > 0 ) { if(pc_noOf_PDUsNewConnection > 0 or pc_noOf_PDUsSameConnection == 0) { //WA#10_1_8_2 if (pc_noOf_PDUsNewConnection == 0 and pc_noOf_PDUsSameConnection == 0) { //WA#10_1_8_2 f_NR_RRCRelease(v_NGC_NASCellA); f_UT_RequestPDUSession(UT, f_NR5GC_MobileInfo_GetNoOfSessions()+1, tsc_RandomAPN, -, -, v_NSSAI); v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (v_NGC_NASCellA, tsc_NR_RRC_TI_Def, ?); // @sic R5s200489 sic@ if (not f_Check_NG_ServiceReqMsg (v_ReceivedMsg.Pdu, cs_NAS_KeySetIdentifier_lv(f_NR_SecurityKSIamf_Get (), tsc_NasKsi_NativeSecurityContext), ?, '0000'B)) { // @sic R5s210189 sic@ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Service Request Message Failed"); } f_NR_RRC_ActivateSecurity (v_NGC_NASCellA, valueof(v_ReceivedMsg.SecurityProtection.NasCount)); // Use NasCount returned from Service Req SRB.send (cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellA, tsc_NR_RbId_SRB1, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, f_Get_NG_ServiceAcceptMsg(v_ReceivedMsg.Pdu.Msg.service_Request.pduSessionStatus)))); f_NR5GC_PDUSessionEstablishment (v_NGC_NASCellA, 1, //WA#110_1_8_2 pc_noOf_PDUsNewConnection, tsc_NR_RbId_SRB1, // @sic R5-206291 sic@ omit, // @sic R5s210134 sic@ *, *, crs_SSC_Mode(-, '001'B), omit, omit, cs_Session_AMBR ('06'O, '05'O, '0004'O, '05'O, '0004'O), cs_S_NSSAI_SST1eMBB_WithIEI, omit); } } } <<SKIPPED CODE>>