3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240107

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4781 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to LTE CA Measurement testcases 8.3.1.22.x |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI10_Test |  |  |  |  |  | Date: |  |  | 2024-01-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The current TTCN is not handling the Measurement Reports from the UE when the IEs “locationInfo_r10” and “measResultServFreqList_r10” are not inlcluded. These needs to be handled by TTCN. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added the handling of the arrival of the IEs “locationInfo_r10” and “measResultServFreqList_r10” for Measurement reports |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3.1.22.1, 8.3.1.22.2, 8.3.1.22.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2022-09_D23wk49 related to the title of this CR.

Contact:	Mohit Kanchan
	mohit.kanchan@keysight.com

# Corrections required

## Change 1

| Function name | f_TC_8_3_1_22_Common |
| --- | --- |
| Reason for change | The current TTCN is not handling the Measurement Reports from the UE when the IEs “locationInfo_r10” and “measResultServFreqList_r10” are not inlcluded. These needs to be handled by TTCN. |
| Summary of change | Added the handling of the arrival of the IEs “locationInfo_r10” and “measResultServFreqList_r10” for Measurement reports |
| TTCN module | EUTRA_Measurements_CA.ttcn |
| MCC160 Comment |  |

Before Change:

function f_TC_8_3_1_22_Common(EUTRA_CellId_Type p_PCellId, EUTRA_CellId_Type p_SCellId, CA_Tested_Type p_CA_Type) runs on EUTRA_PTC { /* CA / Measurement configuration control and reporting / Intra E-UTRAN measurements / Event A1 / Event A2 */ var RRC_TransactionIdentifier v_RRC_TI := tsc_RRC_TI_Def; var Frequency_IE_Type v_Frequency_IE_PCell; var Frequency_IE_Type v_Frequency_IE_Scell; var ChannelBandwidthDependency_Type v_ChBandDependency_PCell; var ChannelBandwidthDependency_Type v_ChBandDependency_Scell; . . . . //@siclog "Step 8" siclog@ //Set cell power level according to T2/3 f_EUTRA_SetCellPowerList(v_CellPowerList_AtT0_T3); //@siclog "Step 9" siclog@ // Allow 15 seconds for power level changes to settle down v_TimerValue_1 := f_EUTRA_SetTimerToleranceMax(p_PCellId, nonProtocolTimer, 15.0); f_EUTRA_WaitAndIgnoreMeasurementReport(p_PCellId, v_TimerValue_1, cr_MeasurementReport(1, ?, ?, *, *)); //@siclog "Step 10" siclog@ //No Further measurement reports for next 10 seconds v_TimerValue_1 := f_EUTRA_SetTimerToleranceMax(p_PCellId, nonProtocolTimer, 10.0); f_EUTRA_NoMeasurementReport(p_PCellId, v_TimerValue_1, cr_MeasurementReport_CheckAny); . . . . //@siclog "Step 18" siclog@ // Allow 15 seconds for power level changes to settle down v_TimerValue_1 := f_EUTRA_SetTimerToleranceMax(p_PCellId, nonProtocolTimer, 15.0); f_EUTRA_WaitAndIgnoreMeasurementReport(p_PCellId, v_TimerValue_1, cr_MeasurementReport(1, ?, ?, *, *)); //@siclog "Step 19" siclog@ //No Further measurement reports for next 10 seconds v_TimerValue_1 := f_EUTRA_SetTimerToleranceMax(p_PCellId, nonProtocolTimer, 10.0); f_EUTRA_NoMeasurementReport(p_PCellId, v_TimerValue_1, cr_MeasurementReport_CheckAny);

After Change:

function f_TC_8_3_1_22_Common(EUTRA_CellId_Type p_PCellId, EUTRA_CellId_Type p_SCellId, CA_Tested_Type p_CA_Type) runs on EUTRA_PTC { /* CA / Measurement configuration control and reporting / Intra E-UTRAN measurements / Event A1 / Event A2 */ var RRC_TransactionIdentifier v_RRC_TI := tsc_RRC_TI_Def; var Frequency_IE_Type v_Frequency_IE_PCell; var Frequency_IE_Type v_Frequency_IE_Scell; var ChannelBandwidthDependency_Type v_ChBandDependency_PCell; var ChannelBandwidthDependency_Type v_ChBandDependency_Scell; . . . . //@siclog "Step 8" siclog@ //Set cell power level according to T2/3 f_EUTRA_SetCellPowerList(v_CellPowerList_AtT0_T3); //@siclog "Step 9" siclog@ // Allow 15 seconds for power level changes to settle down v_TimerValue_1 := f_EUTRA_SetTimerToleranceMax(p_PCellId, nonProtocolTimer, 15.0); f_EUTRA_WaitAndIgnoreMeasurementReport(p_PCellId, v_TimerValue_1, cr_MeasurementReport(1, ?, ?, *, *, -, *)); //@siclog "Step 10" siclog@ //No Further measurement reports for next 10 seconds v_TimerValue_1 := f_EUTRA_SetTimerToleranceMax(p_PCellId, nonProtocolTimer, 10.0); f_EUTRA_NoMeasurementReport(p_PCellId, v_TimerValue_1, cr_MeasurementReport_CheckAny); . . . . //@siclog "Step 18" siclog@ // Allow 15 seconds for power level changes to settle down v_TimerValue_1 := f_EUTRA_SetTimerToleranceMax(p_PCellId, nonProtocolTimer, 15.0); f_EUTRA_WaitAndIgnoreMeasurementReport(p_PCellId, v_TimerValue_1, cr_MeasurementReport(1, ?, ?, *, *, -, *)); //@siclog "Step 19" siclog@ //No Further measurement reports for next 10 seconds v_TimerValue_1 := f_EUTRA_SetTimerToleranceMax(p_PCellId, nonProtocolTimer, 10.0); f_EUTRA_NoMeasurementReport(p_PCellId, v_TimerValue_1, cr_MeasurementReport_CheckAny);