3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240113

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3347 | rev | - | Current version: | 17.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to eNS NSAC test case 9.1.12.4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test, eNS_Ph2-UEConTest |  |  |  |  |  | Date: |  |  | 2024-01-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Test fails at STEP Step 6A due to SD check which was removed in the RAN5#101 meeting via prose CR R5-237368 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Removed the SD check from the rejected NSSAI when passing to the function f_MatchXtdRejectedNSSAI_WithATCommand() at both step6A and 8. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.12.4.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required.	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2022-09_D23wk49 related to the title of this CR.

Contact:	Mohit Kanchan

mohit.kanchan@keysight.com

# Corrections required.

## Change 1

| Function name | fl_TC_9_1_12_4_Body() |
| --- | --- |
| Reason for change | Test fails at STEP Step 6A due to SD check which was removed in the RAN5#101 meeting via prose CR R5-237368 |
| Summary of change | Removed the SD check from the rejected NSSAI when passing to the function f_MatchXtdRejectedNSSAI_WithATCommand() at both step6A and 8. |
| TTCN module | 9.1.12\ NSAC_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change:

function fl_TC_9_1_12_4_Body() runs on NR_BASE_PTC { var charstring v_AT_Response; var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_CellsOnDifferentPLMN); var NAS_PlmnId v_PLMN := f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (v_NGC_NASCellA)); var charstring v_NSSAI := f_ConvertS_NSSAI_ForATCommand (cs_S_NSSAI('01'O, '01'O, 'FFFFFF'O)); var template (value) PartialXtdRejectedNSSAI v_PartialXtdRejectedNSSAI1 := f_BuildNR5GC_PartialXtdRejectedNSSAIType1({cs_XtdRejectedS_NSSAI('0001'B, '0011'B, '01'O, omit, omit, omit)}, bit2oct('10100010'B));// @sic R5-231543 R5-237368 sic@ var template (value) ExtdRejectedNSSAI v_ExtdRejectedNSSAI := f_BuildNR5GC_ExtdRejectedNSSAI({v_PartialXtdRejectedNSSAI1}); timer t_T3526 := 120.0;// @sic R5-231543 sic@ //@siclog "Step 1" siclog@ // @sic R5-231543 sic@ //The SS transmits CONFIGURATION UPDATE COMMAND including S-NSSAI=1 in rejected NSSAI with back-off timer set to 120 seconds. SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellA, tsc_NR_RbId_SRB2, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, cs_NG_CONFIGURATION_UPDATE_COMMAND(cs_ConfigUpdateInd('0'B, '1'B), -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, v_ExtdRejectedNSSAI)))); // @siclog "Step 2" siclog@ // @sic R5-231543 sic@ //Check: Does UE transmit CONFIGURATION UPDATE COMPLETE? SRB.receive (car_NR_SRB_NasPdu_IND(v_NGC_NASCellA, tsc_NR_RbId_SRB2, cr_NG_NAS_Ind(tsc_SHT_IntegrityProtected_Ciphered, cr_NG_CONFIGURATION_UPDATE_COMPLETE))); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 2"); t_T3526.start; // @siclog "Step 3" siclog@ //The SS transmits an RRCRelease message. f_NR_RRCRelease(v_NGC_NASCellA, omit); //@siclog "Step 4" siclog@ // @sic R5-231543 sic@ //Void //@siclog "Step 5" siclog@ //Cause the UE to request establishment of PDU session with S-NSSAI=1 f_UT_RequestPDUSession(UT, 1, -, -, -, v_NSSAI, -, -, -, -, -, -, LOCAL_CNF_REQUIRED); // @sic R5s230789 sic@ //@siclog "Step 6" siclog@ //Check: Does the UE transmit an RRCSetupRequest message within T3526 associated with S-NSSAI=1 in the next 15 seconds? f_NR_CheckNoRRCSetupReq(v_NGC_NASCellA, 15.0); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 6"); //@siclog "Step 6A" siclog@ // @sic R5-231543 sic@ //Check:Is S-NSSAI=1 in the Rejected NSSAI list with cause "S-NSSAI not available due to maximum number of UEs reached" associated with current PLMN using AT/MMI command (+C5GNSSAIRDP)? v_AT_Response := f_UT_ReadXtdRejectedNSSAI(UT, f_ConvertPLMNtoString(v_PLMN)); // @sic R5s230702 sic@ if (f_MatchXtdRejectedNSSAI_WithATCommand({cs_XtdRejectedS_NSSAI('0001'B, '0011'B, '01'O, 'FFFFFF'O, omit, omit)},v_AT_Response)){ // @sic R5s230702 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 6A"); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 6A"); } //@siclog "Step 7" siclog@ //Wait for the expiry of T3526 associated with S-NSSAI=1. t_T3526.timeout; //@siclog "Step 8" siclog@ // @sic R5s230702 sic@ //Check: Using AT/MMI command (+C5GNSSAIRDP) verify that S-NSSAI=1 is not in the Rejected NSSAI list associated with current PLMN v_AT_Response := f_UT_ReadXtdRejectedNSSAI(UT, f_ConvertPLMNtoString(v_PLMN)); // @sic R5s230702 sic@ if (f_MatchXtdRejectedNSSAI_WithATCommand({cs_XtdRejectedS_NSSAI('0001'B, '0011'B, '01'O, 'FFFFFF'O, omit, omit)},v_AT_Response)){ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 8"); } else { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 8"); } //@siclog "Steps 9-27a1" siclog@ // @sic R5-231543 sic@ //Void }

After Change:

function fl_TC_9_1_12_4_Body() runs on NR_BASE_PTC { var charstring v_AT_Response; var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_CellsOnDifferentPLMN); var NAS_PlmnId v_PLMN := f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (v_NGC_NASCellA)); var charstring v_NSSAI := f_ConvertS_NSSAI_ForATCommand (cs_S_NSSAI('01'O, '01'O, 'FFFFFF'O)); var template (value) PartialXtdRejectedNSSAI v_PartialXtdRejectedNSSAI1 := f_BuildNR5GC_PartialXtdRejectedNSSAIType1({cs_XtdRejectedS_NSSAI('0001'B, '0011'B, '01'O, omit, omit, omit)}, bit2oct('10100010'B));// @sic R5-231543 R5-237368 sic@ var template (value) ExtdRejectedNSSAI v_ExtdRejectedNSSAI := f_BuildNR5GC_ExtdRejectedNSSAI({v_PartialXtdRejectedNSSAI1}); timer t_T3526 := 120.0;// @sic R5-231543 sic@ //@siclog "Step 1" siclog@ // @sic R5-231543 sic@ //The SS transmits CONFIGURATION UPDATE COMMAND including S-NSSAI=1 in rejected NSSAI with back-off timer set to 120 seconds. SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellA, tsc_NR_RbId_SRB2, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, cs_NG_CONFIGURATION_UPDATE_COMMAND(cs_ConfigUpdateInd('0'B, '1'B), -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, v_ExtdRejectedNSSAI)))); // @siclog "Step 2" siclog@ // @sic R5-231543 sic@ //Check: Does UE transmit CONFIGURATION UPDATE COMPLETE? SRB.receive (car_NR_SRB_NasPdu_IND(v_NGC_NASCellA, tsc_NR_RbId_SRB2, cr_NG_NAS_Ind(tsc_SHT_IntegrityProtected_Ciphered, cr_NG_CONFIGURATION_UPDATE_COMPLETE))); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 2"); t_T3526.start; // @siclog "Step 3" siclog@ //The SS transmits an RRCRelease message. f_NR_RRCRelease(v_NGC_NASCellA, omit); //@siclog "Step 4" siclog@ // @sic R5-231543 sic@ //Void //@siclog "Step 5" siclog@ //Cause the UE to request establishment of PDU session with S-NSSAI=1 f_UT_RequestPDUSession(UT, 1, -, -, -, v_NSSAI, -, -, -, -, -, -, LOCAL_CNF_REQUIRED); // @sic R5s230789 sic@ //@siclog "Step 6" siclog@ //Check: Does the UE transmit an RRCSetupRequest message within T3526 associated with S-NSSAI=1 in the next 15 seconds? f_NR_CheckNoRRCSetupReq(v_NGC_NASCellA, 15.0); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 6"); //@siclog "Step 6A" siclog@ // @sic R5-231543 sic@ //Check:Is S-NSSAI=1 in the Rejected NSSAI list with cause "S-NSSAI not available due to maximum number of UEs reached" associated with current PLMN using AT/MMI command (+C5GNSSAIRDP)? v_AT_Response := f_UT_ReadXtdRejectedNSSAI(UT, f_ConvertPLMNtoString(v_PLMN)); // @sic R5s230702 sic@ if (f_MatchXtdRejectedNSSAI_WithATCommand({cs_XtdRejectedS_NSSAI('0001'B, '0011'B, '01'O, omit, omit, omit)},v_AT_Response)){ // @sic R5s230702 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 6A"); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 6A"); } //@siclog "Step 7" siclog@ //Wait for the expiry of T3526 associated with S-NSSAI=1. t_T3526.timeout; //@siclog "Step 8" siclog@ // @sic R5s230702 sic@ //Check: Using AT/MMI command (+C5GNSSAIRDP) verify that S-NSSAI=1 is not in the Rejected NSSAI list associated with current PLMN v_AT_Response := f_UT_ReadXtdRejectedNSSAI(UT, f_ConvertPLMNtoString(v_PLMN)); // @sic R5s230702 sic@ if (f_MatchXtdRejectedNSSAI_WithATCommand({cs_XtdRejectedS_NSSAI('0001'B, '0011'B, '01'O, omit, omit, omit)},v_AT_Response)){ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 8"); } else { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 8"); } //@siclog "Steps 9-27a1" siclog@ // @sic R5-231543 sic@ //Void }