3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240118

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3350 | rev | - | Current version: | 17.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction in setting of the bwp_SwitchingDelay PHY parameter based on reported PICS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-01-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | As per 3GPP 38.306 Section 4.2.7.10 Phy-Parameters bwp-SwitchingDelay – defines whether the UE supports DCI and timer based active BWP switching delay type1 or type2 specified in clause 8.6.2 of TS 38.133 [5]. It is mandatory to report type 1 or type 2 when bwp-SameNumerology or bwp-DiffNumerology is supported on at least one band. The above reference does make it mandatory for bwp-switchingDelay to be reported if bwp-SameNumerology or bwp-DiffNumerology is reported but the other way around is not correct. However, the current TTCN implementation is not aligned to this interpretation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding the check for pc_bwp_SwitchingDelay_Type1 if both bwp-SameNumerology or bwp-DiffNumerology are not supported by UE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.5.1.1.NR5GC, 8.2.1.1.1.ENDC, 8.2.1.1.2.NEDC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required.	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2022-09_D23wk49 related to the title of this CR.

Contact:	Mohit Kanchan
	mohit.kanchan@keysight.com

# Corrections required.

## Change 1

| Function name | function f_NR_Derive_Phy_ParametersCommon_bwp_SwitchingDelay() return template Phy_ParametersCommon_bwp_SwitchingDelay |
| --- | --- |
| Reason for change | As per 3GPP 38.306 Section 4.2.7.10 Phy-Parameters bwp-SwitchingDelay – defines whether the UE supports DCI and timer based active BWP switching delay type1 or type2 specified in clause 8.6.2 of TS 38.133 [5]. It is mandatory to report type 1 or type 2 when bwp-SameNumerology or bwp-DiffNumerology is supported on at least one band. The above reference does make it mandatory for bwp-switchingDelay to be reported if bwp-SameNumerology or bwp-DiffNumerology is reported but the other way around is not correct. However, the current TTCN implementation is not aligned to this interpretation. |
| Summary of change | Adding the check for pc_bwp_SwitchingDelay_Type1 if both bwp-SameNumerology or bwp-DiffNumerology are not supported by UE. |
| TTCN module | NR_AuxiliaryCapCheckFunctions.ttcn |
| MCC160 Comment |  |

Before Change:

function f_NR_Derive_Phy_ParametersCommon_bwp_SwitchingDelay() return template Phy_ParametersCommon_bwp_SwitchingDelay { var template Phy_ParametersCommon_bwp_SwitchingDelay v_UE_NR_Phy_ParametersCommon_bwp_SwitchingDelay := omit; var boolean v_BWP_DiffNumerology_Set := pc_bwp_DiffNumerology_FR1_FDD or pc_bwp_DiffNumerology_FR1_TDD or pc_bwp_DiffNumerology_FR2; var boolean v_BWP_SameNumerology_Set := (pc_bwp_SameNumerology_upto2_FR1_FDD or pc_bwp_SameNumerology_upto2_FR1_TDD or pc_bwp_SameNumerology_upto2_FR2 or pc_bwp_SameNumerology_upto4_FR1_FDD or pc_bwp_SameNumerology_upto4_FR1_TDD or pc_bwp_SameNumerology_upto4_FR2); if (v_BWP_SameNumerology_Set or v_BWP_DiffNumerology_Set) //@sic R5s221089 R5-230775 sic@ { if (not (pc_bwp_SwitchingDelay_Type1 xor pc_bwp_SwitchingDelay_Type2)){ f_SetVerdict(fail, __FILE__, __LINE__, "Acc. to 38.306, 4.2.7.10, It is mandatory to report type 1 or type 2"); } if (pc_bwp_SwitchingDelay_Type1) { v_UE_NR_Phy_ParametersCommon_bwp_SwitchingDelay := type1; } else { v_UE_NR_Phy_ParametersCommon_bwp_SwitchingDelay := type2; } } return v_UE_NR_Phy_ParametersCommon_bwp_SwitchingDelay ifpresent; //It is mandatory to report type 1 or type 2 if bwp-SameNumerology or bwp-DiffNumerology is supported on at least one band. This capability is not applicable to IAB-MT acc. to 38.306, 4.2.7.10 }

After Change:

function f_NR_Derive_Phy_ParametersCommon_bwp_SwitchingDelay() return template Phy_ParametersCommon_bwp_SwitchingDelay { var template Phy_ParametersCommon_bwp_SwitchingDelay v_UE_NR_Phy_ParametersCommon_bwp_SwitchingDelay := omit; var boolean v_BWP_DiffNumerology_Set := pc_bwp_DiffNumerology_FR1_FDD or pc_bwp_DiffNumerology_FR1_TDD or pc_bwp_DiffNumerology_FR2; var boolean v_BWP_SameNumerology_Set := (pc_bwp_SameNumerology_upto2_FR1_FDD or pc_bwp_SameNumerology_upto2_FR1_TDD or pc_bwp_SameNumerology_upto2_FR2 or pc_bwp_SameNumerology_upto4_FR1_FDD or pc_bwp_SameNumerology_upto4_FR1_TDD or pc_bwp_SameNumerology_upto4_FR2); if (v_BWP_SameNumerology_Set or v_BWP_DiffNumerology_Set) //@sic R5s221089 R5-230775 sic@ { if (not (pc_bwp_SwitchingDelay_Type1 xor pc_bwp_SwitchingDelay_Type2)){ f_SetVerdict(fail, __FILE__, __LINE__, "Acc. to 38.306, 4.2.7.10, It is mandatory to report type 1 or type 2"); } if (pc_bwp_SwitchingDelay_Type1) { v_UE_NR_Phy_ParametersCommon_bwp_SwitchingDelay := type1; } else { v_UE_NR_Phy_ParametersCommon_bwp_SwitchingDelay := type2; } } else { if (pc_bwp_SwitchingDelay_Type1) { v_UE_NR_Phy_ParametersCommon_bwp_SwitchingDelay := type1; } else { v_UE_NR_Phy_ParametersCommon_bwp_SwitchingDelay := type2; } } return v_UE_NR_Phy_ParametersCommon_bwp_SwitchingDelay ifpresent; //It is mandatory to report type 1 or type 2 if bwp-SameNumerology or bwp-DiffNumerology is supported on at least one band. This capability is not applicable to IAB-MT acc. to 38.306, 4.2.7.10 }