3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240119

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3351 | rev | - | Current version: | 17.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC test case 9.1.7.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-01-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the current TTCN implementation of TC 9.1.7.1, the SS transmits Close UE Test Loop Mode B message twice in preamble, first time with IP PDU Delay is set to 0s, and second time it is set to 5s. Based on the implemention of loopback mode, some UEs may ignore the second Close UE test loop message as and as a result the IP PDU Delay will be set to 0s, whereas the testcase requirement is 5s. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The function call f_NR5GC_Preamble(v_NGC_NASCellA, STATE_IDLE_1A, TEST_LOOPModeB_ON) isreplaced with f_NR5GC_Preamble(v_NGC_NASCellA, STATE_IDLE_1A, TESTModeB_ON). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.7.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required.	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2022-09_D23wk49 related to the title of this CR.

Contact:	Mohit Kanchan
	mohit.kanchan@keysight.com

# Corrections required.

## Change 1

| Function name | f_TC_9_1_7_1_NR5GC |
| --- | --- |
| Reason for change | In the current TTCN implementation of TC 9.1.7.1, the SS transmits Close UE Test Loop Mode B message twice in preamble, first time with IP PDU Delay is set to 0s, and second time it is set to 5s. Based on the implemention of loopback mode, some UEs may ignore the second Close UE test loop message as and as a result the IP PDU Delay will be set to 0s, whereas the testcase requirement is 5s. |
| Summary of change | The function call f_NR5GC_Preamble(v_NGC_NASCellA, STATE_IDLE_1A, TEST_LOOPModeB_ON) isreplaced with f_NR5GC_Preamble(v_NGC_NASCellA, STATE_IDLE_1A, TESTModeB_ON). |
| TTCN module | Service_Request_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change:

function f_TC_9_1_7_1_NR5GC() runs on NR5GC_PTC { //Service request / IDLE mode uplink user data transport / Rejected / Restricted service area var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_AllCellsOnSamePLMN); //@sic R5-196055 sic@ //var NR_CellId_Type v_NGC_NASCellB := f_NR5GC_MapNASCell(ngc_CellB, NG_AllCellsOnSamePLMN); f_NR5GC_Init_NAS(NR_1); //Create and configure cells f_NR_CellConfig_Def(v_NGC_NASCellA); //f_NR_CellConfig_Def(v_NGC_NASCellB); //Preamble f_NR5GC_Preamble(v_NGC_NASCellA, STATE_IDLE_1A, TEST_LOOPModeB_ON); // @sic R5s200087 sic@ f_NR5GC_RRC_ConnectedState3N_Def(v_NGC_NASCellA, TEST_LOOPModeB_ON, -, '05'O); //Main test body fl_TC_9_1_7_1_Body(); // Finish f_NR_Postamble(v_NGC_NASCellA, STATE_IDLE_1A); //@sic R5s200087 sic@ }

After Change:

function f_TC_9_1_7_1_NR5GC() runs on NR5GC_PTC { //Service request / IDLE mode uplink user data transport / Rejected / Restricted service area var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_AllCellsOnSamePLMN); //@sic R5-196055 sic@ //var NR_CellId_Type v_NGC_NASCellB := f_NR5GC_MapNASCell(ngc_CellB, NG_AllCellsOnSamePLMN); f_NR5GC_Init_NAS(NR_1); //Create and configure cells f_NR_CellConfig_Def(v_NGC_NASCellA); //f_NR_CellConfig_Def(v_NGC_NASCellB); //Preamble f_NR5GC_Preamble(v_NGC_NASCellA, STATE_IDLE_1A, TESTModeB_ON); // @sic R5s200087 sic@ f_NR5GC_RRC_ConnectedState3N_Def(v_NGC_NASCellA, TEST_LOOPModeB_ON, -, '05'O); //Main test body fl_TC_9_1_7_1_Body(); // Finish f_NR_Postamble(v_NGC_NASCellA, STATE_IDLE_1A); //@sic R5s200087 sic@ }