3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240120

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3352 | rev | - | Current version: | 17.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC test case 6.4.1.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-01-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Due to changes in TTCN 23WK49 delivery with CR R5-236305, PDU session is not getting established at step 19Aa1-20 when pc_noOf_PDUsSameConnection is set to 0. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Modified the condition to call function f_NR5GC_RRC_Idle_Extension if pc_noOf_PDUsSameConnection = 0 as well. Note: A draft prose CR is associated with this change |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4.1.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.508-1, 38.523-1 CR 3018, 4194 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iWD-TTCN3-B2022-09_D23wk49 related to the title of this CR.

Contact:	Mohit Kanchan
	mohit.kanchan@keysight.com

# Corrections required

## Change 1

| Function name | f_TC_6_4_1_1_NR5GC_TestBody |
| --- | --- |
| Reason for change | Due to changes in TTCN 23WK49 delivery with CR R5-236305, PDU session is not getting established at step 19Aa1-20 when pc_noOf_PDUsSameConnection is set to 0. |
| Summary of change | Modified the condition to call function f_NR5GC_RRC_Idle_Extension if pc_noOf_PDUsSameConnection = 0 as well. Note: A draft prose CR is associated with this change. |
| TTCN module | Inactive_PLMNSelection_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change:

function f_TC_6_4_1_1_NR5GC_TestBody() runs on NR5GC_PTC {// @sic R5s201047, R5-203382, R5s210094, R5-210062, R5s210454, R5-216995, R5-220543, R5s220481, R5-233462 sic@ ……………………………..SKIPPED CODE………………… // @siclog "Step 0A" siclog@ // SS adjusts cell levels according to row T0 of table 6.4.1.1.3.2-1/2 f_NR_SetCellPowerList(v_CellPowerList_T0); // @siclog "Step 0B" siclog@ // The UE is switched on f_UT_SwitchOnUE(UT); // @siclog "Step 0C1-0C19a1" siclog@ // The steps 1-19a1 specified in generic procedure TS 38.508-1 Table 4.5.3.2-1 are performed v_ASN_PLMN := f_NR_CellInfo_GetPLMN (nr_Cell12); v_PLMN := f_NR_Asn2Nas_PlmnId(v_ASN_PLMN); v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (nr_Cell12, tsc_NR_RRC_TI_Def, ?, (tsc_SHT_NoSecurityProtection, tsc_SHT_IntegrityProtected)); if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Initial_NoSecurity)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } f_NR5GC_RRC_Idle_Steps5_13(v_GMM_MobilityInfo, nr_Cell12, v_ReceivedMsg, v_PLMN, TESTMode_OFF, Initial_NoSecurity); v_MsgToSend := f_Get_NG_RegistrationAcceptMsg (Initial_Secure, v_GMM_MobilityInfo, f_NR5GC_GetMobileIdGUTI (nr_Cell12), cs_NG_TAIListNonConsecutive(f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (nr_Cell12)), {bit2oct(f_NR_CellInfo_GetTAC(nr_Cell12))}), cs_PLMN_List_1PLMN(f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (nr_Cell13)))); // Table 6.4.1.1.3.3-3 SRB.send(cas_NR_SRB_NasPdu_REQ(nr_Cell12, tsc_NR_RbId_SRB1, // No SRB2 so must be on SRB1 cs_TimingInfo_Now, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, v_MsgToSend))); f_NR5GC_Registration_Complete(nr_Cell12); if (pc_noOf_PDUsSameConnection < 0 ) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "3GPP RAN5 5GS test specifications do not support UEs configured with pc_noOf_PDUsSameConnection < 0"); } f_NR5GC_PDUSessionEstablishment(nr_Cell12, -, tsc_NR_RbId_SRB1); // @siclog "Step 0D" siclog@ // SS starts timers of tmin = 2min and tmax = (6 minutes + 5min) t_Tmin.start(f_NR_SetTimerToleranceMin(nr_Cell12, nonProtocolTimer, 120.0)); t_Tmax.start(f_NR_SetTimerToleranceMax(nr_Cell12, nonProtocolTimer, 660.0)); // @siclog "Step 0Ea1-0F" siclog@ // steps 19Aa1-20 specified in generic procedure TS 38.508-1 Table 4.5.3.2-1 are performed if (pc_noOf_PDUsNewConnection >0 ){ f_NR_RRCRelease(nr_Cell12); f_NR5GC_RRC_Idle_Extension(nr_Cell12); // @sic R5-236305 sic@ } f_NR_RRC_InactiveDef(nr_Cell12, -, false); v_ExistingDRBs := f_NR5GC_MobileInfo_GetDRBInfoList(); v_NR_RLC_BearerToAddModList[0] := cs_NR_BearerConfigDef(f_NR_LogicalChannelId_SRB(tsc_SRB2), omit, omit, cs_38508_LogicalChannelConfig_SRB(tsc_SRB2), true_); for (i:= 0; i < lengthof(v_ExistingDRBs); i := i+1) { v_DRB_ToAddModList[i] := cs_NR_DRB_ToAddMod(omit, v_ExistingDRBs[i].DRB_Id, true_, -, omit); v_NR_RLC_BearerToAddModList[i+1] := cs_NR_BearerConfigDef(f_NR_LogicalChannelId_DRB(v_ExistingDRBs[i].DRB_Id), omit, omit, omit, true_); } v_RadioBearerConfig := cs_38508_RadioBearerConfigDef( {cs_NR_SRB_ToAddMod_ReestablishPDCP(tsc_SRB2)}, v_DRB_ToAddModList); v_NR_UplinkBWP := f_NR_CellInfo_GetUplinkBWP (nr_Cell13, tsc_NR_BWP_Id); v_NR_UplinkBWP.Dedicated.R15.pusch_Config := omit; v_SpCellConfig := f_NR_GetSpCellConfigDef(nr_Cell13, cs_NR_ServingCellConfigDef(omit, 0, omit, cs_NR_UplinkConfigDef(v_NR_UplinkBWP.Dedicated.R15, omit))); v_SCGConfig := cs_38508_CellGroupConfig(f_NR_CellInfo_Get_CellGroupId(nr_Cell13), v_NR_RLC_BearerToAddModList, -, -, -, v_SpCellConfig); f_NR5GC_MobileInfo_SetSessionInfoList({}); // @sic R5s210407 sic@ // @siclog "Step 1" siclog@ // SS adjusts cell levels according to row T1 of table 6.4.1.1.3.2-1/2 f_NR_SetCellPowerList(v_CellPowerList_T1); f_NR_SS_AS_ActivateSecurity_Current(nr_Cell13 ,tsc_NR_38508_NextHopChainingCount); // @sic R5s200657, R5s220486 sic@ // @siclog "Step 2" siclog@ // Check: Does the UE transmit an RRCResumeRequest message on NR Cell 13? f_NR_RRC_ResumeRequest_Def(nr_Cell13, -, mo_Signalling); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 2"); ……………………………..SKIPPED CODE………………… }

After Change:

function f_TC_6_4_1_1_NR5GC_TestBody() runs on NR5GC_PTC {// @sic R5s201047, R5-203382, R5s210094, R5-210062, R5s ……………………………..SKIPPED CODE………………… // @siclog "Step 0A" siclog@ // SS adjusts cell levels according to row T0 of table 6.4.1.1.3.2-1/2 f_NR_SetCellPowerList(v_CellPowerList_T0); // @siclog "Step 0B" siclog@ // The UE is switched on f_UT_SwitchOnUE(UT); // @siclog "Step 0C1-0C19a1" siclog@ // The steps 1-19a1 specified in generic procedure TS 38.508-1 Table 4.5.3.2-1 are performed v_ASN_PLMN := f_NR_CellInfo_GetPLMN (nr_Cell12); v_PLMN := f_NR_Asn2Nas_PlmnId(v_ASN_PLMN); v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (nr_Cell12, tsc_NR_RRC_TI_Def, ?, (tsc_SHT_NoSecurityProtection, tsc_SHT_IntegrityProtected)); if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Initial_NoSecurity)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } f_NR5GC_RRC_Idle_Steps5_13(v_GMM_MobilityInfo, nr_Cell12, v_ReceivedMsg, v_PLMN, TESTMode_OFF, Initial_NoSecurity); v_MsgToSend := f_Get_NG_RegistrationAcceptMsg (Initial_Secure, v_GMM_MobilityInfo, f_NR5GC_GetMobileIdGUTI (nr_Cell12), cs_NG_TAIListNonConsecutive(f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (nr_Cell12)), {bit2oct(f_NR_CellInfo_GetTAC(nr_Cell12))}), cs_PLMN_List_1PLMN(f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (nr_Cell13)))); // Table 6.4.1.1.3.3-3 SRB.send(cas_NR_SRB_NasPdu_REQ(nr_Cell12, tsc_NR_RbId_SRB1, // No SRB2 so must be on SRB1 cs_TimingInfo_Now, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, v_MsgToSend))); f_NR5GC_Registration_Complete(nr_Cell12); if (pc_noOf_PDUsSameConnection < 0 ) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "3GPP RAN5 5GS test specifications do not support UEs configured with pc_noOf_PDUsSameConnection < 0"); } f_NR5GC_PDUSessionEstablishment(nr_Cell12, -, tsc_NR_RbId_SRB1); // @siclog "Step 0D" siclog@ // SS starts timers of tmin = 2min and tmax = (6 minutes + 5min) t_Tmin.start(f_NR_SetTimerToleranceMin(nr_Cell12, nonProtocolTimer, 120.0)); t_Tmax.start(f_NR_SetTimerToleranceMax(nr_Cell12, nonProtocolTimer, 660.0)); // @siclog "Step 0Ea1-0F" siclog@ // steps 19Aa1-20 specified in generic procedure TS 38.508-1 Table 4.5.3.2-1 are performed if (pc_noOf_PDUsNewConnection >0 or pc_noOf_PDUsSameConnection == 0 ){ f_NR_RRCRelease(nr_Cell12); f_NR5GC_RRC_Idle_Extension(nr_Cell12); } f_NR_RRC_InactiveDef(nr_Cell12, -, false); v_ExistingDRBs := f_NR5GC_MobileInfo_GetDRBInfoList(); v_NR_RLC_BearerToAddModList[0] := cs_NR_BearerConfigDef(f_NR_LogicalChannelId_SRB(tsc_SRB2), omit, omit, cs_38508_LogicalChannelConfig_SRB(tsc_SRB2), true_); for (i:= 0; i < lengthof(v_ExistingDRBs); i := i+1) { v_DRB_ToAddModList[i] := cs_NR_DRB_ToAddMod(omit, v_ExistingDRBs[i].DRB_Id, true_, -, omit); v_NR_RLC_BearerToAddModList[i+1] := cs_NR_BearerConfigDef(f_NR_LogicalChannelId_DRB(v_ExistingDRBs[i].DRB_Id), omit, omit, omit, true_); } v_RadioBearerConfig := cs_38508_RadioBearerConfigDef( {cs_NR_SRB_ToAddMod_ReestablishPDCP(tsc_SRB2)}, v_DRB_ToAddModList); v_NR_UplinkBWP := f_NR_CellInfo_GetUplinkBWP (nr_Cell13, tsc_NR_BWP_Id); v_NR_UplinkBWP.Dedicated.R15.pusch_Config := omit; v_SpCellConfig := f_NR_GetSpCellConfigDef(nr_Cell13, cs_NR_ServingCellConfigDef(omit, 0, omit, cs_NR_UplinkConfigDef(v_NR_UplinkBWP.Dedicated.R15, omit))); v_SCGConfig := cs_38508_CellGroupConfig(f_NR_CellInfo_Get_CellGroupId(nr_Cell13), v_NR_RLC_BearerToAddModList, -, -, -, v_SpCellConfig); f_NR5GC_MobileInfo_SetSessionInfoList({}); // @sic R5s210407 sic@ // @siclog "Step 1" siclog@ // SS adjusts cell levels according to row T1 of table 6.4.1.1.3.2-1/2 f_NR_SetCellPowerList(v_CellPowerList_T1); f_NR_SS_AS_ActivateSecurity_Current(nr_Cell13 ,tsc_NR_38508_NextHopChainingCount); // @sic R5s200657, R5s220486 sic@ // @siclog "Step 2" siclog@ // Check: Does the UE transmit an RRCResumeRequest message on NR Cell 13? f_NR_RRC_ResumeRequest_Def(nr_Cell13, -, mo_Signalling); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 2"); ……………………………..SKIPPED CODE………………… }