3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240126

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4783 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to f_EUTRA_CellInfo_Init |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI8_Test |  |  |  |  |  | Date: |  |  | 2024-01-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The 36.508 table 4.4.2.-1 defines which cell should be assigned which frequency, either F1, F2 or F3. However there are few band exceptions listed in the 36.508 cl. 6.2.3 where only F1 is defined, namely bands 13, 18, 31 and 72, in which case there may be a conflict when TTCN attempts to assign F2 or F3 frequencies. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In case of bands 13, 18, 31 and 72 the F1 will be always assigned. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Should the frequencies of bands 13, 18, 31 or 72 be mapped to such eutra_cell which according to 36.508 table 4.4.2.-1 assigns F2 or F3 then such configuration causes runtime error. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | f_EUTRA_CellInfo_Init |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections required	2

2.1	Change 1	2

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D23wk49 related to the title of this CR.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required

## Change 1

| Function name | f_EUTRA_CellInfo_Init |
| --- | --- |
| Reason for change | - The 36.508 table 4.4.2.-1 defines which cell should be assigned which frequency, either F1, F2 or F3. However there are few band exceptions listed in the 36.508 cl. 6.2.3 where only F1 is defined, namely bands 13, 18, 31 and 72, in which case there may be a conflict when TTCN attempts to assign F2 or F3 frequencies. |
| Summary of change | - In case of bands 13, 18, 31 and 72 the F1 will be always assigned. |
| TTCN module | EUTRA_CellInfoInit.ttcn |
| MCC160 Comment |  |

Before Change:

function f_EUTRA_CellInfo_Init(EUTRA_CellId_Type p_CellId, integer p_FrequencyBand, Combination_Type p_SysinfoCombination, Dl_Bandwidth_Type p_DL_Bandwidth, Ul_Bandwidth_Type p_UL_Bandwidth) return EUTRA_CellInfo_Type { /* @sic R5-096641 sic@ */ //@sic R5-163085 p_FrequencyBand type changed to integer sic@ var CellIdentities_Type v_CellIds := fl_EUTRA_InitCellIdentities(p_CellId); var EUTRA_FDD_TDD_CellInfo_Type v_FDD_TDD_Info := f_EUTRA_InitFDD_TDD_FromBand(p_FrequencyBand); var EUTRA_FDD_TDD_Mode_Type v_FDD_TDD := v_FDD_TDD_Info.FDD_TDD; var CellTimingInfo_Type v_Celltiming := fl_EUTRA_InitialiseCellTiming(p_CellId, v_FDD_TDD, LTE_AllCellsOnSamePLMN); //the parameter "LTE_AllCellsOnSamePLMN" is dummy for RRC cells, it is used only for NAS test cases var Combination_Type v_SysinfoCombination := fl_EUTRA_CheckSysinfoCombination(p_CellId, p_SysinfoCombination); var RootSequenceIndex_Type v_RootSequenceIndex_Type := f_EUTRA_InitialisePRACH_RootSequenceIndex(p_CellId, v_FDD_TDD, LTE_AllCellsOnSamePLMN); var RACH_TimingAdvance_Type v_RAR_TA := fl_EUTRA_InitialiseRAR_TA(v_FDD_TDD, v_Celltiming); // @sic R5-121777 sic@ var CellCarrierFreqEUTRA_Type v_UL_DL_Earfcn; var NAS_Parameter_Type v_NAS_Param; var template (value) BcchInfo_Type v_BCCH_Info; var template (value) EUTRA_CellInfo_Type v_CellInfo; var Frequency_fList_Type v_Freq; var ChannelBandwidthDependency_Type v_ChannelBandwidthDependency := f_EUTRA_BandDependentParam(p_DL_Bandwidth, p_UL_Bandwidth); //@sic R5s090180: return CFI value sic@ var SysinfoType_Type v_SysinfoType := sysinfoOnly; //@sic R5-167569 sic@ var template (omit) Bcch_BRInfo_Type v_BCCH_BR_Info := omit; //@sic R5-167569 sic@ // Initialise Frequencies f1/f2/f3/f4 v_Freq := f_EUTRA_InitFrequency_f1Tof4(p_FrequencyBand, p_DL_Bandwidth, p_UL_Bandwidth); // Initialised Frequency per cell. See 36.508 Table 6.2.3.1-1 // Cell 1/2/4/11 -> Frequency f1 // Cell 3/12 -> Frequency f2 // Cell 6/13 -> Frequency f3 // Cell 14 -> Frequency f4 select (p_CellId) { case (eutra_Cell1, eutra_Cell2, eutra_Cell4, eutra_Cell11, eutra_Cell10, eutra_Cell30, eutra_Cell31) { v_UL_DL_Earfcn := v_Freq.f1; //f1 or f5 } case (eutra_Cell3, eutra_Cell12, eutra_Cell23, eutra_Cell28, eutra_Cell29) { // @sic R5-121888 sic@ v_UL_DL_Earfcn := v_Freq.f2; // f2 } case (eutra_Cell6, eutra_Cell13) { v_UL_DL_Earfcn := v_Freq.f3; // f3 } case (eutra_Cell14) { v_UL_DL_Earfcn := v_Freq.f4; // f4 } case else { FatalError(__FILE__, __LINE__, "invalid cell Idendtity"); } } // Initialise NAS parameters v_NAS_Param := fl_EUTRA_InitNAS_forRRC_Cells(p_CellId, p_FrequencyBand, p_SysinfoCombination); //@sic R5s130498 sic@ // Initialise sysinfoType v_SysinfoType := f_EUTRA_InitSysinfoType(); // Initialise SYSINFO v_BCCH_Info := f_EUTRA_InitSystemInformation(p_CellId, v_CellIds.CellIdentity, v_SysinfoCombination, p_FrequencyBand, p_DL_Bandwidth, p_UL_Bandwidth, cs_PlmnIdList_1Entry(v_NAS_Param.Guti_Parameters.PLMN_Identity), v_NAS_Param.TrackingAreaCode, v_FDD_TDD_Info, v_RootSequenceIndex_Type, v_SysinfoType, v_UL_DL_Earfcn); if (v_SysinfoType != sysinfoOnly) { v_BCCH_BR_Info := f_EUTRA_InitSystemInformation_BR(v_CellIds.CellIdentity, v_SysinfoCombination, p_FrequencyBand, p_DL_Bandwidth, p_UL_Bandwidth, cs_PlmnIdList_1Entry(v_NAS_Param.Guti_Parameters.PLMN_Identity), v_NAS_Param.TrackingAreaCode, v_FDD_TDD_Info); } v_CellInfo := cs_EUTRA_CellInfoDef(v_CellIds, v_SysinfoCombination, p_FrequencyBand, v_UL_DL_Earfcn, p_DL_Bandwidth, p_UL_Bandwidth, v_NAS_Param, v_BCCH_Info, v_Celltiming, v_FDD_TDD_Info, v_ChannelBandwidthDependency.Cfi, v_RAR_TA, v_ChannelBandwidthDependency.sr_PUCCH_ResourceIndex, // @sic R5s090312 sic@ v_SysinfoType, v_BCCH_BR_Info); // @sic R5s090180 sic@ return valueof(v_CellInfo); // Note: valueof cannot be avoided here }

After Change:

function f_EUTRA_CellInfo_Init(EUTRA_CellId_Type p_CellId, integer p_FrequencyBand, Combination_Type p_SysinfoCombination, Dl_Bandwidth_Type p_DL_Bandwidth, Ul_Bandwidth_Type p_UL_Bandwidth) return EUTRA_CellInfo_Type { /* @sic R5-096641 sic@ */ //@sic R5-163085 p_FrequencyBand type changed to integer sic@ var CellIdentities_Type v_CellIds := fl_EUTRA_InitCellIdentities(p_CellId); var EUTRA_FDD_TDD_CellInfo_Type v_FDD_TDD_Info := f_EUTRA_InitFDD_TDD_FromBand(p_FrequencyBand); var EUTRA_FDD_TDD_Mode_Type v_FDD_TDD := v_FDD_TDD_Info.FDD_TDD; var CellTimingInfo_Type v_Celltiming := fl_EUTRA_InitialiseCellTiming(p_CellId, v_FDD_TDD, LTE_AllCellsOnSamePLMN); //the parameter "LTE_AllCellsOnSamePLMN" is dummy for RRC cells, it is used only for NAS test cases var Combination_Type v_SysinfoCombination := fl_EUTRA_CheckSysinfoCombination(p_CellId, p_SysinfoCombination); var RootSequenceIndex_Type v_RootSequenceIndex_Type := f_EUTRA_InitialisePRACH_RootSequenceIndex(p_CellId, v_FDD_TDD, LTE_AllCellsOnSamePLMN); var RACH_TimingAdvance_Type v_RAR_TA := fl_EUTRA_InitialiseRAR_TA(v_FDD_TDD, v_Celltiming); // @sic R5-121777 sic@ var CellCarrierFreqEUTRA_Type v_UL_DL_Earfcn; var NAS_Parameter_Type v_NAS_Param; var template (value) BcchInfo_Type v_BCCH_Info; var template (value) EUTRA_CellInfo_Type v_CellInfo; var Frequency_fList_Type v_Freq; var ChannelBandwidthDependency_Type v_ChannelBandwidthDependency := f_EUTRA_BandDependentParam(p_DL_Bandwidth, p_UL_Bandwidth); //@sic R5s090180: return CFI value sic@ var SysinfoType_Type v_SysinfoType := sysinfoOnly; //@sic R5-167569 sic@ var template (omit) Bcch_BRInfo_Type v_BCCH_BR_Info := omit; //@sic R5-167569 sic@ // Initialise Frequencies f1/f2/f3/f4 v_Freq := f_EUTRA_InitFrequency_f1Tof4(p_FrequencyBand, p_DL_Bandwidth, p_UL_Bandwidth); // Initialised Frequency per cell. See 36.508 Table 6.2.3.1-1 // Cell 1/2/4/11 -> Frequency f1 // Cell 3/12 -> Frequency f2 // Cell 6/13 -> Frequency f3 // Cell 14 -> Frequency f4 select (p_CellId) { case (eutra_Cell1, eutra_Cell2, eutra_Cell4, eutra_Cell11, eutra_Cell10, eutra_Cell30, eutra_Cell31) { v_UL_DL_Earfcn := v_Freq.f1; //f1 or f5 } case (eutra_Cell3, eutra_Cell12, eutra_Cell23, eutra_Cell28, eutra_Cell29) { // @sic R5-121888 sic@ v_UL_DL_Earfcn := v_Freq.f2; // f2 } case (eutra_Cell6, eutra_Cell13) { v_UL_DL_Earfcn := v_Freq.f3; // f3 } case (eutra_Cell14) { v_UL_DL_Earfcn := v_Freq.f4; // f4 } case else { FatalError(__FILE__, __LINE__, "invalid cell Idendtity"); } } //The 36.508 table 4.4.2.-1 defines which cell to be assigned which frequency, however there are few band exceptions listed in the 36.508 cl. 6.2.3 where only F1 is defined: if ((p_FrequencyBand == 13) or (p_FrequencyBand == 18) or (p_FrequencyBand == 31) or (p_FrequencyBand == 72)) { v_UL_DL_Earfcn := v_Freq.f1; } // Initialise NAS parameters v_NAS_Param := fl_EUTRA_InitNAS_forRRC_Cells(p_CellId, p_FrequencyBand, p_SysinfoCombination); //@sic R5s130498 sic@ // Initialise sysinfoType v_SysinfoType := f_EUTRA_InitSysinfoType(); // Initialise SYSINFO v_BCCH_Info := f_EUTRA_InitSystemInformation(p_CellId, v_CellIds.CellIdentity, v_SysinfoCombination, p_FrequencyBand, p_DL_Bandwidth, p_UL_Bandwidth, cs_PlmnIdList_1Entry(v_NAS_Param.Guti_Parameters.PLMN_Identity), v_NAS_Param.TrackingAreaCode, v_FDD_TDD_Info, v_RootSequenceIndex_Type, v_SysinfoType, v_UL_DL_Earfcn); if (v_SysinfoType != sysinfoOnly) { v_BCCH_BR_Info := f_EUTRA_InitSystemInformation_BR(v_CellIds.CellIdentity, v_SysinfoCombination, p_FrequencyBand, p_DL_Bandwidth, p_UL_Bandwidth, cs_PlmnIdList_1Entry(v_NAS_Param.Guti_Parameters.PLMN_Identity), v_NAS_Param.TrackingAreaCode, v_FDD_TDD_Info); } v_CellInfo := cs_EUTRA_CellInfoDef(v_CellIds, v_SysinfoCombination, p_FrequencyBand, v_UL_DL_Earfcn, p_DL_Bandwidth, p_UL_Bandwidth, v_NAS_Param, v_BCCH_Info, v_Celltiming, v_FDD_TDD_Info, v_ChannelBandwidthDependency.Cfi, v_RAR_TA, v_ChannelBandwidthDependency.sr_PUCCH_ResourceIndex, // @sic R5s090312 sic@ v_SysinfoType, v_BCCH_BR_Info); // @sic R5s090180 sic@ return valueof(v_CellInfo); // Note: valueof cannot be avoided here }