3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240127

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3356 | rev | - | Current version: | 17.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of NR5GC testcase 9.1.13.2 in FR1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNS_Ph2-UEConTest, TEI17_Test |  |  |  |  |  | Date: |  |  | 2024-01-19 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To add NR5GC test case 9.1.13.2 to the IWD_23wk49 ATS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to test case 9.1.13.2 required for approval. See detailed change description for further information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.13.2.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	4

3	Branches executed	8

4	Execution Log Files	8

4.1	Hisilicon Balong 5000 UE	8

5	References	9

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D23wk49 related to the title of this CR.

Contact:	Parikshit Bhise

parikshit.bhise@rohde-schwarz.com

## Verification Test Summary

Test Case: 	9.1.13.2.NR5GC

ATS Version:	iwd-TTCN3-B2022-09_D23wk49

System Simulator used:	R&S® 5G Protocol Conformance Test platform

UE used:	Hisilicon Balong 5000 UE

Verification Status:	PASS

# Corrections required

  - Correction to function fl_ TC_9_1_13_2_Body

| Function name | fl_TC_9_1_13_2_Body |
| --- | --- |
| Reason for change | The iel value in the v_NSSRG_Info for step 14 and step 19 is not correct. |
| Summary of change | Change iel value to '000E' for step 14 and '0006' for step 19. |
| TTCN module | NSSRG_NR5GC.ttcn |
| MCC160 Comment |  |

Before

<<SKIPPED CODE>>

function fl_TC_9_1_13_2_Body() runs on NR5GC_PTC

{

……

v_NSSRG_Info :={

iei := '70'O,

iel := '0001'O,

valueList := {

cs_NSSRG_1entry(cs_S_NSSAI('01'O, '01'O, omit), '01'O),

cs_NSSRG_2entry(cs_S_NSSAI('01'O, '02'O, omit), '02'O, '03'O), //cs_NSSRG_2entries corrected to cs_NSSRG_2entry.

cs_NSSRG_2entry(cs_S_NSSAI('01'O, '03'O, omit), '02'O, '04'O)//cs_NSSRG_3entries corrected to cs_NSSRG_2entry.

}};

//@siclog "Step 1" siclog@

f_NR5GC_SwitchOnAndResetIMS();

……

//@siclog "Step 14" siclog@

//The SS transmits a REGISTRATION ACCEPT message including Configured NSSAI with NSSRG information included.

v_MsgToSend := f_Get_NG_RegistrationAcceptMsg (Initial_Secure,

v_GMM_MobilityInfo,

v_GUTIToSend,

v_TAIList,

omit,

v_AllowedNSSAI, -, v_ConfiguredNSSAI,

-, -, -, -, -, -, -, -, -, -,

-, -, -, -, -, -, -, -, -, -,

-, -, -, -, -, -, -, -, -, -,

-, -, -, -, -, -, -, -, -, -,

v_NSSRG_Info);

……

//@siclog "Step 15-19a1" siclog@

// Steps 15 to 19a1 of Table 4.5.2.2-2 of the generic procedure in TS 38.508-1

f_NR5GC_RRC_Idle_Steps15_20 (v_NGC_NASCellA, TESTMode_OFF, noRrcConnectionRelease);

//@siclog "Step 20" siclog@

v_AllowedNSSAI:= {

iei := '15'O,

lvPart := {

iel := '04'O,

s_nssai := {cs_S_NSSAI('01'O, '04'O, omit), cs_S_NSSAI('01'O, '05'O, omit)}

}};

v_NSSRG_Info :={

iei := '70'O,

iel := '0001'O,

valueList := {

cs_NSSRG_1entry(cs_S_NSSAI('01'O, '04'O, omit), '01'O)

}};

SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellA, tsc_NR_RbId_SRB2, -,

cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered,

cs_NG_CONFIGURATION_UPDATE_COMMAND(cs_ConfigUpdateInd('1'B, '1'B), -, -, v_AllowedNSSAI,-, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, v_NSSRG_Info))));

……

} //End of fl_TC_9_1_13_2_Body<<SKIPPED CODE>>

After

<<SKIPPED CODE>>

function fl_TC_9_1_13_2_Body() runs on NR5GC_PTC

{

……

v_NSSRG_Info :={

iei := '70'O,

iel := '000E'O,

valueList := {

cs_NSSRG_1entry(cs_S_NSSAI('01'O, '01'O, omit), '01'O),

cs_NSSRG_2entry(cs_S_NSSAI('01'O, '02'O, omit), '02'O, '03'O), //cs_NSSRG_2entries corrected to cs_NSSRG_2entry.

cs_NSSRG_2entry(cs_S_NSSAI('01'O, '03'O, omit), '02'O, '04'O)//cs_NSSRG_3entries corrected to cs_NSSRG_2entry.

}};

//@siclog "Step 1" siclog@

f_NR5GC_SwitchOnAndResetIMS();

……

//@siclog "Step 14" siclog@

//The SS transmits a REGISTRATION ACCEPT message including Configured NSSAI with NSSRG information included.

v_MsgToSend := f_Get_NG_RegistrationAcceptMsg (Initial_Secure,

v_GMM_MobilityInfo,

v_GUTIToSend,

v_TAIList,

omit,

v_AllowedNSSAI, -, v_ConfiguredNSSAI,

-, -, -, -, -, -, -, -, -, -,

-, -, -, -, -, -, -, -, -, -,

-, -, -, -, -, -, -, -, -, -,

-, -, -, -, -, -, -, -, -, -,

v_NSSRG_Info);

……

//@siclog "Step 15-19a1" siclog@

// Steps 15 to 19a1 of Table 4.5.2.2-2 of the generic procedure in TS 38.508-1

f_NR5GC_RRC_Idle_Steps15_20 (v_NGC_NASCellA, TESTMode_OFF, noRrcConnectionRelease);

//@siclog "Step 20" siclog@

v_AllowedNSSAI:= {

iei := '15'O,

lvPart := {

iel := '04'O,

s_nssai := {cs_S_NSSAI('01'O, '04'O, omit), cs_S_NSSAI('01'O, '05'O, omit)}

}};

v_NSSRG_Info :={

iei := '70'O,

iel := '0006'O,

valueList := {

cs_NSSRG_1entry(cs_S_NSSAI('01'O, '04'O, omit), '01'O)

}};

SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellA, tsc_NR_RbId_SRB2, -,

cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered,

cs_NG_CONFIGURATION_UPDATE_COMMAND(cs_ConfigUpdateInd('1'B, '1'B), -, -, v_AllowedNSSAI,-, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, v_NSSRG_Info))));

} //End of fl_TC_9_1_13_2_Body

<<SKIPPED CODE>>

# Branches executed

## This NR5GC TC was executed in 5GS Stand Alone mode (Option 2) on NR Primary Band “n41” using NIA1 Integrity and NEA1 ciphering algorithms

# Execution Log Files

## Hisilicon Balong 5000 UE

The Hisilicon Balong 5000 UE passed this test case on R&S® 5G Protocol Conformance Test platform. The documentation below is enclosed as evidence of the successful test case run [1]:

Test case execution log file:

R&S :
tc_9_1_13_2_Hisilicon.log
(Note: PICS/PIXIT settings are captured at the beginning of the TLI log)

In the log file the complete test case execution as well as the PICS/PIXIT parameter settings can be seen. All TLI events are presented and message contents are fully decoded and can be verified. Preliminary verdicts and the final test case verdict can be seen in the log file.

# References

| [1] | R5s240128: Supporting information for addition of NR5GC testcase 9.1.13.2 in FR1 |
| --- | --- |