3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240148

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-5 | CR | 0104 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of f_MCX_IMS_ServiceAuthorization |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test, MCImp-UEConTest |  |  |  |  |  | Date: |  |  | 2024-02-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - Timer t_WaitForSubscribe_GROUPKEY is never started - Wrong condition is used at step 2a3 |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Timer t_WaitForSubscribe_GROUPKEY started, when SUBSCRIBE for group document has been received without entry requesting the GMK - Condition corrected at step 2a3 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Implementation may fail a UE implementation which requests the GMK with stand-alone SUBSCRIBE |  |  |  |  |  |  |  |  |
| Clauses affected: |  | f_MCX_IMS_ServiceAuthorization |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2022-09_D23wk49 related to the title of this CR.

Contact:	Olivier Genoud
	olivier.genoud@etsi.org

# Corrections required

| Function name | f_MCX_IMS_ServiceAuthorization |
| --- | --- |
| Reason for change | - Timer t_WaitForSubscribe_GROUPKEY is never started - Wrong condition is used at step 2a3 |
| Summary of change | - Timer t_WaitForSubscribe_GROUPKEY started, when SUBSCRIBE for group document has been received without entry requesting the GMK - Condition corrected at step 2a3 |
| TTCN module | MCX_SIP_Registration |

function f_MCX_IMS_ServiceAuthorization(MCX_ServiceType_Type p_ServiceType) runs on MCX_IMS_PTC { /* @sic R5-215730: p_Procedure removed sic@ */ var InternetProtocol_Type v_Protocol := f_IMS_PTC_ImsInfo_GetTransportProtocol(); var charstring v_MCSUEID := f_SIP_InstanceId(); var charstring v_GroupId := f_MCX_MCService_GroupID_Group_A(p_ServiceType); var MCX_SecurityParameters_Type v_SecurityParameters; var MCX_Coordination_MSG v_Dummy; var MCX_DataRequestWithXmlBodies_Type v_DataRequestWithXmlBodies_ByRef; var MCX_DataRequestWithXmlBodies_Type v_DataRequestWithXmlBodies; var template (value) MessageHeader v_MessageHeader_NOTIFY_CONFIG; var template (value) MessageHeader v_MessageHeader_NOTIFY_GROUPCONFIG; var template (value) MCX_XcapDiff_Type v_XcapDiff; var template (value) MIKEY_Message v_MIKEY_Message; var ResourceLists_Type v_ResourceLists; var ResourceLists_ListSequence_Type v_ResourceLists_SequenceList; var boolean v_WaitForRegister := true; var boolean v_WaitForPublish := true; var boolean v_WaitForSubscribe_CONFIG := true; var boolean v_WaitForSubscribe_GROUPCONFIG := true; var boolean v_WaitForSubscribe_GROUPKEY := tsc_MCX_Security; // group key subscription happens only if security is enabled var boolean v_WaitFor200OK_CONFIG := false; var boolean v_WaitFor200OK_GROUPCONFIG := false; var IMS_PTC_DialogIndex_Type v_DialogIndex_GMS := xcapDiffDialogGMS1; var IMS_PTC_DialogIndex_Type v_DialogIndex_GROUPCONFIG; var template (omit) IMS_PTC_DialogIndex_Type v_DialogIndex_GROUPKEY := omit; var template (present) ResourceLists_ListSequence_Type v_ResourceListsSequence_GMS; var SipUrl v_RequestUri_CMS := f_SIP_ParseURI("sip:" & tsc_MCX_CMS_Hostname); // 36.579-1 Table 5.5.2.14-1 condition CONFIG @sic R5s220869 Change 6: f_SIP_ParseURI instead of cr_SipUri_HostPort sic@ var SipUrl v_ContactUri_CMS := v_RequestUri_CMS; // 36.579-1 Table 5.5.2.17.1.2-1 condition CONFIG var SipUrl v_RequestUri_GMS := f_SIP_ParseURI("sip:" & tsc_MCX_GMSURI); // 36.579-1 Table 5.5.2.14-1 condition GROUPCONFIG @sic R5s220869 Change 6: f_SIP_ParseURI instead of cr_SipUri_HostPort sic@ var SipUrl v_ContactUri_GMS := v_RequestUri_GMS; // 36.579-1 Table 5.5.2.17.1.2-1 condition GROUPCONFIG @sic R5w220205 proposal 2.3-7a, R5-223945 Change 5a sic@ timer t_WaitForSubscribe_GROUPKEY := 5.0; while (v_WaitForRegister or v_WaitForPublish or v_WaitForSubscribe_CONFIG or v_WaitForSubscribe_GROUPCONFIG or v_WaitForSubscribe_GROUPKEY or v_WaitFor200OK_CONFIG or v_WaitFor200OK_GROUPCONFIG) { // determine receive template for ResourceLists at the GMS: if (v_WaitForSubscribe_GROUPCONFIG and v_WaitForSubscribe_GROUPKEY) { v_ResourceListsSequence_GMS := ( f_MCX_ResourceLists_SequenceList_GroupDocGKTP(p_ServiceType), // URI of Group Configuration document and URI of GKTP document in any order f_MCX_ResourceLists_SequenceList_GroupDocOnly(p_ServiceType), // URI of Group Configuration document f_MCX_ResourceLists_SequenceList_GKTP_Only(p_ServiceType) ); // URI of GKTP document } else if (v_WaitForSubscribe_GROUPCONFIG) { v_ResourceListsSequence_GMS := f_MCX_ResourceLists_SequenceList_GroupDocOnly(p_ServiceType); } else if (v_WaitForSubscribe_GROUPKEY) { v_ResourceListsSequence_GMS := f_MCX_ResourceLists_SequenceList_GKTP_Only(p_ServiceType); } alt { // ---------------------------------------------------- Table 5.3.2.3-2 ---------------------------------------------------- // Table 5.3.2.3-2: step 1a1, 1a2 REGISTER, 200 OK [v_WaitForRegister] a_MCX_SIP_ServiceAuthorization_ReceiveRegisterSend200OK(p_ServiceType) { v_WaitForRegister := false; } // Table 5.3.2.3-2: step 1a3, 1a4 or 1b1, 1b2 PUBLISH, 200 OK [v_WaitForPublish] a_MCX_SIP_PocSettings_ReceivePublishSend200OK(p_ServiceType, v_WaitForRegister) { // IF (v_WaitForRegister) THEN PUBLISH is for service authorization (i.e. shall contain MIKEY message) v_WaitForRegister := false; v_WaitForPublish := false; } // ---------------------------------------------------- Table 5.3.2.3-2A ---------------------------------------------------- // Table 5.3.2.3-2A: step 1, 2 SUBSCRIBE, 200 OK [v_WaitForSubscribe_CONFIG] a_MCX_SIP_ServiceAuthorization_ReceiveSubscribeSend200OK(v_DataRequestWithXmlBodies_ByRef, p_ServiceType, -, xcapDiffDialogCMS, v_RequestUri_CMS, v_ContactUri_CMS, f_MCX_ResourceLists_SequenceList_CONFIG(p_ServiceType, v_MCSUEID)) { v_DataRequestWithXmlBodies := v_DataRequestWithXmlBodies_ByRef; v_WaitForSubscribe_CONFIG := false; f_MCX_SendTrigger(HTTP, CMS); // send trigger to tell HTTP PTC, that from now on the UE can ask for configuration documents from CMS } // Table 5.3.2.3-2A: step 3 NOTIFY [] a_MCX_WaitForCoOrdMsg(HTTP, cmr_MCX_Trigger(CMS), v_Dummy) { // wait for confirmation the HTTP PTC is ready to receive CMS HTTP GET requests (Table 5.3.2.3-2A: step 5, 7, 9) v_XcapDiff := f_MCX_XcapDiff_CONFIG(p_ServiceType, v_MCSUEID); v_MessageHeader_NOTIFY_CONFIG := f_MCX_SIP_ServiceAuthorization_SendNotify(p_ServiceType, xcapDiffDialogCMS, v_Protocol, v_XcapDiff); v_WaitFor200OK_CONFIG := true; } // Table 5.3.2.3-2A: step 4 200 OK for NOTIFY [v_WaitFor200OK_CONFIG] a_MCX_SIP_ServiceAuthorization_NotifyReceive200OK(v_MessageHeader_NOTIFY_CONFIG, v_Protocol) { v_WaitFor200OK_CONFIG := false; } // ---------------------------------------------------- Table 5.3.2.3-2B or Table 5.3.2.3-2C -------------------------------- // Table 5.3.2.3-2B/2C: step 1, 2 SUBSCRIBE, 200 OK // NOTE: SUBSCRIBEs of Table 5.3.2.3-2A and Table 5.3.2.3-2C cannot be distinquished in the first place as they have the same message headers and the same request URI [v_WaitForSubscribe_GROUPCONFIG or v_WaitForSubscribe_GROUPKEY] a_MCX_SIP_ServiceAuthorization_ReceiveSubscribeSend200OK(v_DataRequestWithXmlBodies_ByRef, p_ServiceType, -, v_DialogIndex_GMS, v_RequestUri_GMS, v_ContactUri_GMS, cr_MCX_ResourceLists(v_ResourceListsSequence_GMS)) { v_DataRequestWithXmlBodies := v_DataRequestWithXmlBodies_ByRef; v_ResourceLists := v_DataRequestWithXmlBodies.XmlBodies.ResourceLists; v_ResourceLists_SequenceList := v_ResourceLists.sequence_list[0].list.sequence_list; if (match(v_ResourceLists_SequenceList, {*, f_MCX_ResourceLists_List_Sequence_GroupDoc(p_ServiceType), *} )) { v_DialogIndex_GROUPCONFIG := v_DialogIndex_GMS; v_WaitForSubscribe_GROUPCONFIG := false; f_MCX_SendTrigger(HTTP, GMS); // send trigger to tell HTTP PTC, that from now on the UE can ask for group configuration documents from GMS } if (match(v_ResourceLists_SequenceList, {*, f_MCX_ResourceLists_List_Sequence_GKTP(p_ServiceType), *} )) { v_DialogIndex_GROUPKEY := v_DialogIndex_GMS; v_WaitForSubscribe_GROUPKEY := false; t_WaitForSubscribe_GROUPKEY.stop; } v_DialogIndex_GMS := fl_MCX_IMS_GetNextDialogIndex(v_DialogIndex_GMS); /* 'increment' dialog index @sic R5-212291: use of int2enum sic@ */ if (v_WaitForSubscribe_GROUPKEY) { t_WaitForSubscribe_GROUPKEY.start; } } // Table 5.3.2.3-2B: step 3 NOTIFY [] a_MCX_WaitForCoOrdMsg(HTTP, cmr_MCX_Trigger(GMS), v_Dummy) { // wait for confirmation the HTTP PTC is ready to receive GMS HTTP GET request (Table 5.3.2.3-2B: step 5) v_XcapDiff := f_MCX_XcapDiff_GROUPCONFIG(p_ServiceType, v_GroupId); v_MessageHeader_NOTIFY_GROUPCONFIG := f_MCX_SIP_ServiceAuthorization_SendNotify(p_ServiceType, v_DialogIndex_GROUPCONFIG, v_Protocol, v_XcapDiff); v_WaitFor200OK_GROUPCONFIG := true; } // Table 5.3.2.3-2B: step 4 200 OK for NOTIFY [v_WaitFor200OK_GROUPCONFIG] a_MCX_SIP_ServiceAuthorization_NotifyReceive200OK(v_MessageHeader_NOTIFY_GROUPCONFIG, v_Protocol) { v_WaitFor200OK_GROUPCONFIG := false; } // ---------------------------------------------------- Table 5.3.2.3-2C ---------------------------------------------------- // Table 5.3.2.3-2C: step 2b1 TIMEOUT [] t_WaitForSubscribe_GROUPKEY.timeout { v_WaitForSubscribe_GROUPKEY := false; } } } f_MCX_WaitForTrigger(HTTP); // Trigger - HTTP is completed if (isvalue(v_DialogIndex_GROUPKEY)) { // ---------------------------------------------------- Table 5.3.2.3-2C ---------------------------------------------------- // Table 5.3.2.3-2C: step 2a3 NOTIFY v_SecurityParameters := f_MCX_IMS_PTC_SecurityParameters_Get(); v_MIKEY_Message := f_MIKEY_SAKKE_MessageTX_GMK(p_ServiceType, v_SecurityParameters, v_GroupId); v_XcapDiff := f_MCX_XcapDiff_GROUPKEY(p_ServiceType, v_GroupId, encvalue(v_MIKEY_Message)); f_MCX_SIP_XcapDiffNotification(p_ServiceType, valueof(v_DialogIndex_GROUPKEY), v_XcapDiff); } f_MCX_IMS_PTC_GroupInfo_Add(v_GroupId, v_DialogIndex_GROUPCONFIG, v_DialogIndex_GROUPKEY); }