3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240156

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3372 | rev | - | Current version: | 17.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of ENDC testcase 7.1.2.3.5a in FR2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-02-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To add ENDC test case 7.1.2.3.5a to the IWD_23wk49 ATS (in FR2). |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to test case 7.1.2.3.5a required for approval. See detailed change description for further information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.2.3.5a.ENDC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required to test case 7.1.2.3.5a	4

2.1	TC_7_1_2_3_5a_ENDC	4

3	Branches executed	6

4	Execution Log Files	6

4.1	Qualcomm	6

5	References	6

# Overview

This document lists all the essencial changes needed to correct problems in the TTCN implementation of ENDC test case 7.1.2.3.5a which is part of the ENDC test suite in iWD_TTCN3-B2022-09_D23wk49.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log file(s) is (are) provided as evidence.

Contact:	Parikshit Bhise

parikshit.bhise@rohde-schwarz.com

## Verification Test Summary

Test Case: 	7.1.2.3.5a.ENDC

ATS Version:	iwd-TTCN3-B2022-09_D23wk49

System Simulator used:	R&S® 5G Protocol Conformance Test platform

UE used:	Qualcomm Snapdragon® X75 5G Modem-RF System , Qualcomm Technologies’ 6th generation modem-to-antenna 5G solution UE

Verification Status:	PASS

# Corrections required to test case 7.1.2.3.5a

## TC_7_1_2_3_5a_ENDC

| Function name | TC_7_1_2_3_5a_ENDC |
| --- | --- |
| Reason for change | In the current TTCN implementation, the guard timer is set to 1500s which is insufficient to achieve a loop back of 64 * 2048 = 131072 RLC SDUs. Based on runtime evidence, the TC took around 3000s to complete the TC. |
| Summary of change | Extended the guard timer to 3000s |
| TTCN module | ENDC_Testsuite.ttcn |
| MCC160 Comment |  |

Before Change:

testcase TC_7_1_2_3_5a_ENDC() runs on MTC_ENDC system SYSTEM_ENDC { // @purpose // AM RLC / 18-bit SN / Control of transmit window / Control of receive window var ENDC_NR_PTC v_ENDC_NR := null; var EUTRA_5GS_PTC v_EUTRA_5GS := null; var IMS_PTC v_IMS1 := null; var IMS_PTC v_IMS2 := null; timer t_GuardTimer := int2float(1500); v_ENDC_NR := ENDC_NR_PTC.create("ENDC_NR") alive; v_EUTRA_5GS := EUTRA_5GS_PTC.create("EUTRA_5GS") alive; f_MTC_ConnectPTCs_ENDC(system, v_ENDC_NR, v_EUTRA_5GS, v_IMS1, v_IMS2); v_ENDC_NR.start(f_TC_7_1_2_3_5a_ENDC_NR()); v_EUTRA_5GS.start(f_TC_7_1_2_3_5a_ENDC_EUTRA()); t_GuardTimer.start; f_MTC_MainLoop_ENDC(t_GuardTimer); }

After Change:

testcase TC_7_1_2_3_5a_ENDC() runs on MTC_ENDC system SYSTEM_ENDC { // @purpose // AM RLC / 18-bit SN / Control of transmit window / Control of receive window var ENDC_NR_PTC v_ENDC_NR := null; var EUTRA_5GS_PTC v_EUTRA_5GS := null; var IMS_PTC v_IMS1 := null; var IMS_PTC v_IMS2 := null; timer t_GuardTimer := int2float(3000); //WA v_ENDC_NR := ENDC_NR_PTC.create("ENDC_NR") alive; v_EUTRA_5GS := EUTRA_5GS_PTC.create("EUTRA_5GS") alive; f_MTC_ConnectPTCs_ENDC(system, v_ENDC_NR, v_EUTRA_5GS, v_IMS1, v_IMS2); v_ENDC_NR.start(f_TC_7_1_2_3_5a_ENDC_NR()); v_EUTRA_5GS.start(f_TC_7_1_2_3_5a_ENDC_EUTRA()); t_GuardTimer.start; f_MTC_MainLoop_ENDC(t_GuardTimer); }

# Branches executed

## This ENDC testcase was executed in Band combination “DC_5A_n261A” using EEA1/EIA1 algorithms on R&S 5G Conformance Test Platform.

# Execution Log Files

## Qualcomm

The Qualcomm Snapdragon® X75 5G Modem-RF System , Qualcomm Technologies’ 6th generation modem-to-antenna 5G solution UE  passed this test case on R&S® 5G Protocol Conformance Test platform. The documentation below is enclosed as evidence of the successful test case run [1]:

Test case execution log file:

tc_ 7_1_2_3_5a_Qualcomm.log
(Note: PICS/PIXIT settings are captured at the beginning of the TLI log)

In the log file the complete test case execution as well as the PICS/PIXIT parameter settings can be seen. All TLI events are presented and message contents are fully decoded and can be verified. Preliminary verdicts and the final test case verdict can be seen in the log file.

# References

| [1] | R5s240157: Supporting information for addition of ENDC testcase 7.1.2.3.5a in FR2 |
| --- | --- |