3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240167

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.571-4 | CR | 0199 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to POS test case 9.3.1.2.19s |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, NR_pos-UEConTest |  |  |  |  |  | Date: |  |  | 2024-02-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | UE enabled with GPS only may fail the case unnesesarily because after checking each item like gps, sbas, qzss, galileo, glonass and bds ‘if condition’ is checking that “any of the supported GNSS” must be supported by UE. But here in current TTCN ‘if condition’ checking with ‘and’ operation making which is making each item of gnss must be supported by UE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Modified if condition with or operation. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test may fail |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.1.2.19s |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR… |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1	Change 1	5

# Overview

This document lists all the essential changes needed to correct problems in the TTCN implementation of test case 9.3.1.2.19s which is part of the POS test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:	Kalahasti, Narendra

Narendra.Kalahasti@anritsu.com

# Corrections required

## Change 1

| Function name | function f_POS_NR_CheckGnssSupportList () |
| --- | --- |
| Reason for change | - UE enabled with GPS only may fail the case unnesesarily because after checking each item like gps, sbas, qzss, galileo, glonass and bds ‘if condition’ is checking that “any of the supported GNSS” must be supported by UE. But here in current TTCN ‘if condition’ checking with ‘and’ operation making which is making each item of gnss must be supported by UE. |
| Summary of change | - Modified if condition with or operation. |
| TTCN module | NR5GC_Positioning_Functions |
| MCC160 Comment |  |

Before Change

function f_POS_NR_CheckGnssSupportList(GNSS_SupportList p_GnssSupportList) runs on NR_BASE_PTC { var boolean v_GpsFound := false; var boolean v_SbasFound := false; var boolean v_QzssFound := false; var boolean v_GalileoFound := false; var boolean v_GlonassFound := false; var boolean v_BdsFound := false; var integer i; ---skipped code--- //Check if the UE did not report capabilities for any of the supported GNSS if (not(((pc_A_GPS_L1C_A == v_GpsFound) or (pc_A_GPS_L1C_A_MGPS == v_GpsFound)) and (pc_A_SBAS == v_SbasFound) and (pc_A_QZSS == v_QzssFound) and (pc_A_Galileo == v_GalileoFound) and (pc_A_GLONASS == v_GlonassFound) and (pc_A_BDS == v_BdsFound))) { f_NR_SetVerdictFailOrInconc (__FILE__, __LINE__, "Not all supported GNSS were reported by the UE"); } }

After Change

function f_POS_NR_CheckGnssSupportList(GNSS_SupportList p_GnssSupportList) runs on NR_BASE_PTC { var boolean v_GpsFound := false; var boolean v_SbasFound := false; var boolean v_QzssFound := false; var boolean v_GalileoFound := false; var boolean v_GlonassFound := false; var boolean v_BdsFound := false; var integer i; ---skipped code--- //Check if the UE did not report capabilities for any of the supported GNSS if (not(((pc_A_GPS_L1C_A == v_GpsFound) or (pc_A_GPS_L1C_A_MGPS == v_GpsFound)) or (pc_A_SBAS == v_SbasFound) or (pc_A_QZSS == v_QzssFound) or (pc_A_Galileo == v_GalileoFound) or (pc_A_GLONASS == v_GlonassFound) or (pc_A_BDS == v_BdsFound))) { f_NR_SetVerdictFailOrInconc (__FILE__, __LINE__, "Not all supported GNSS were reported by the UE"); } }