3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240175

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4788 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of Rel-16 LTE RACS test case 8.5.5.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, RACS-UEConTest |  |  |  |  |  | Date: |  |  | 2024-02-23 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To add LTE test case 8.5.5.1 to the IWD_23wk49 |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to test case 8.5.5.1 required for approval. See detailed change description for further information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant device will fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.5.5.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 36.523-1 CR xxxx |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	5

2.3	Change 3	6

2.4	Change 4	7

2.5	Change 5	8

3	Branches executed	9

4	Execution Log Files	9

4.1	Qualcomm Snapdragon® X75 5G Modem-RF System , Qualcomm Technologies’ 6th generation modem-to-antenna 5G solution UE	9

5	References	10

# Overview

This document lists all the essential changes needed to correct problems in the TTCN implementation of test case 8.5.5.1 which is part of the LTE test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:	Mohit Kanchan
	mohit.kanchan@keysight.com

## Verification Test Summary

Test Case: 	8.5.5.1

ATS Version:	iwd-TTCN3-B2022-09_D23wk49

System Simulator used:	Keysight S8704A Protocol Conformance Toolset

UE used:	Qualcomm Snapdragon® X75 5G Modem-RF System , Qualcomm Technologies’ 6th

generation modem-to-antenna 5G solution UE

Verification Status:	PASS

# Corrections required

## Change 1

| Function name | function f_TC_8_5_5_1_EUTRA() |
| --- | --- |
| Reason for change | EUTRA cell was not initlaised due to which v_PhysicalCellIdentity_Cell1 remains uninitialized which is causing test case crash |
| Summary of change | Moved var PhysCellId v_PhysicalCellIdentity_Cell1 := f_EUTRA_CellInfo_GetPhyCellId(eutra_Cell1); after cell has been initialized |
| TTCN module | RRC_Others_PRO.ttcn |
| MCC160 Comment |  |

Before change

function f_TC_8_5_5_1_EUTRA() runs on EUTRA_PTC

{ /* RACS / UL Message Segment transfer / UECapabilityInformation / Success */

var PhysCellId v_PhysicalCellIdentity_Cell1 := f_EUTRA_CellInfo_GetPhyCellId(eutra_Cell1);

var ShortMAC_I v_ShortMAC_I;

var EUTRA_SecurityParams_Type v_AuthParams;

var octetstring v_Rx_SegmentOct := ''O;

var boolean v_Completed_AllSegments := false;

var ULDedicatedMessageSegment_r16_IEs.segmentNumber_r16 v_MessageSegmentRx := 0; //INTEGER (0..15)

var SRB_COMMON_IND v_ReceivedAspForUeCapabilityInfo;

var SRB_COMMON_IND v_Received_DedicatedMessageSegment;

var bitstring v_SuppEutraBandsAccToPICS := f_DeriveSuppEutraBandsFromPics(); // The position on this list represents the band number

var UECapabilityEnquiry_v1180_IEs.requestedFrequencyBands_r11 v_RequestedFrequencyBands := {px_ePrimaryFrequencyBand}; // Initialise with the primary frequency band

var UECapabilityEnquiry_v1180_IEs.requestedFrequencyBands_r11 v_BandToBeAdded;

var integer v_Index := 0;

var SubFrameTiming_Type v_TimingInfoStep4;

var SubFrameTiming_Type v_TimingInfoStep5;

var SubFrameTiming_Type v_TimingInfoStep6;

var integer v_GrantPeriod;

var EUTRA_FDD_TDD_Mode_Type v_EUTRA_FDD_TDD_Mode;

timer t_Watchdog := 5.0;

f_EUTRA_Init(c1);

//Create and configure cell

f_EUTRA_CellConfig_Def(eutra_Cell1);

v_EUTRA_FDD_TDD_Mode := f_EUTRA_CellInfo_GetFDD_TDD(eutra_Cell1);

/* Preamble to enter UE in state Generic RB Established (state 3) */

f_EUTRA_Preamble (eutra_Cell1, STATE2_IDLEUPDATE);

f_EUTRA_RbEst_Def(eutra_Cell1);

f_EUTRA_TestBody_Set(true);

After change

function f_TC_8_5_5_1_EUTRA() runs on EUTRA_PTC

{ /* RACS / UL Message Segment transfer / UECapabilityInformation / Success */

var ShortMAC_I v_ShortMAC_I;

var EUTRA_SecurityParams_Type v_AuthParams;

var octetstring v_Rx_SegmentOct := ''O;

var boolean v_Completed_AllSegments := false;

var ULDedicatedMessageSegment_r16_IEs.segmentNumber_r16 v_MessageSegmentRx := 0; //INTEGER (0..15)

var SRB_COMMON_IND v_ReceivedAspForUeCapabilityInfo;

var SRB_COMMON_IND v_Received_DedicatedMessageSegment;

var bitstring v_SuppEutraBandsAccToPICS := f_DeriveSuppEutraBandsFromPics(); // The position on this list represents the band number

var UECapabilityEnquiry_v1180_IEs.requestedFrequencyBands_r11 v_RequestedFrequencyBands := {px_ePrimaryFrequencyBand}; // Initialise with the primary frequency band

var UECapabilityEnquiry_v1180_IEs.requestedFrequencyBands_r11 v_BandToBeAdded;

var integer v_Index := 0;

var SubFrameTiming_Type v_TimingInfoStep4;

var SubFrameTiming_Type v_TimingInfoStep5;

var SubFrameTiming_Type v_TimingInfoStep6;

var integer v_GrantPeriod;

var EUTRA_FDD_TDD_Mode_Type v_EUTRA_FDD_TDD_Mode;

timer t_Watchdog := 5.0;

f_EUTRA_Init(c1);

var PhysCellId v_PhysicalCellIdentity_Cell1 := f_EUTRA_CellInfo_GetPhyCellId(eutra_Cell1);

//Create and configure cell

f_EUTRA_CellConfig_Def(eutra_Cell1);

v_EUTRA_FDD_TDD_Mode := f_EUTRA_CellInfo_GetFDD_TDD(eutra_Cell1);

/* Preamble to enter UE in state Generic RB Established (state 3) */

f_EUTRA_Preamble (eutra_Cell1, STATE2_IDLEUPDATE);

f_EUTRA_RbEst_Def(eutra_Cell1);

f_EUTRA_TestBody_Set(true);

## Change 2

| Function name | function f_TC_8_5_5_1_EUTRA() |
| --- | --- |
| Reason for change | At step4 SS is Configuring UE capabilities Enquiry at imidiate frame and grant is configured 15 subframes after UE capabilities enquiry. With this there was an extended delay of around 120 subframes in giving grant. This leads to synch loss at UE and UE iniate RACH. |
| Summary of change | Configured the UE capability Enquire messag to be sent on a specifica TTI |
| TTCN module | RRC_Others_PRO.ttcn |
| MCC160 Comment |  |

Before change

// Decode the UE-CapabilityRAT-Container

if (v_Completed_AllSegments){ //UECapabilityInformation Rx. on segments

if (lengthof(v_Rx_SegmentOct) < tsc_Max_LTE_PDCP_SDU_size) { //Set to <8188 by default - @sic R5s220330 sic@

f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "UE segmented the UECapabilityInformation message being smaller than Max PDCP SDU size");

}

f_EUTRA_DecodeULDcchMsg(v_Rx_SegmentOct);

f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 2a1");

}

//@siclog "Step 3" siclog@

// SS stops regular grant transmission upon scheduling request

f_EUTRA_StopULGrantTransmission_PUCCH_Sync(eutra_Cell1, cs_TimingInfo_Now);

//@siclog "Step 4" siclog@

// The SS transmits a UECapabilityEnquiry message to enquire max capability and with rrc-segAllowed-r16 set to enabled

v_TimingInfoStep4 := f_EUTRA_GetNextSendOccasion(eutra_Cell1);

SRB.send(cas_SRB1_RrcPdu_REQ(eutra_Cell1, cs_TimingInfo_Now(v_TimingInfoStep4), cs_508_UeCapabilityEnquiry(tsc_RRC_TI_Def, -,-,-,-, enabled, v_RequestedFrequencyBands)));

//@siclog "Step 5" siclog@

// The SS allocates 17 UL grants periodically from 15(FDD)/18 (TDD) subframes after successful completion of step 4 to schedule PUSCH till the ULDedicatedMessageSegment at step 6 is received.

if (v_EUTRA_FDD_TDD_Mode == FDD) {

v_TimingInfoStep5 := f_EUTRA_TimingInfoAdd(v_TimingInfoStep4, 15); // 15 subframes = 15 ms

v_GrantPeriod := 1; // No period value especified in the prose. Value chosed arbitrarily.//1

}

else {

v_TimingInfoStep5 := f_EUTRA_TimingInfoAdd(v_TimingInfoStep4, 18); // 18 subframes = 18 ms

v_GrantPeriod := 5; // No period value especified in the prose. Value chosed arbitrarily.

}

After change

// Decode the UE-CapabilityRAT-Container

if (v_Completed_AllSegments){ //UECapabilityInformation Rx. on segments

if (lengthof(v_Rx_SegmentOct) < tsc_Max_LTE_PDCP_SDU_size) { //Set to <8188 by default - @sic R5s220330 sic@

f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "UE segmented the UECapabilityInformation message being smaller than Max PDCP SDU size");

}

f_EUTRA_DecodeULDcchMsg(v_Rx_SegmentOct);

f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 2a1");

}

//@siclog "Step 3" siclog@

// SS stops regular grant transmission upon scheduling request

f_EUTRA_StopULGrantTransmission_PUCCH_Sync(eutra_Cell1, cs_TimingInfo_Now);

//@siclog "Step 4" siclog@

// The SS transmits a UECapabilityEnquiry message to enquire max capability and with rrc-segAllowed-r16 set to enabled

v_TimingInfoStep4 := f_EUTRA_GetNextSendOccasion(eutra_Cell1);

SRB.send(cas_SRB1_RrcPdu_REQ(eutra_Cell1, cs_TimingInfo_EUTRA_NB(v_TimingInfoStep4), cs_508_UeCapabilityEnquiry(tsc_RRC_TI_Def, -,-,-,-, enabled, v_RequestedFrequencyBands)));

//@siclog "Step 5" siclog@

// The SS allocates 17 UL grants periodically from 15(FDD)/18 (TDD) subframes after successful completion of step 4 to schedule PUSCH till the ULDedicatedMessageSegment at step 6 is received.

if (v_EUTRA_FDD_TDD_Mode == FDD) {

v_TimingInfoStep5 := f_EUTRA_TimingInfoAdd(v_TimingInfoStep4, 15); // 15 subframes = 15 ms

v_GrantPeriod := 1; // No period value especified in the prose. Value chosed arbitrarily.//1

}

else {

v_TimingInfoStep5 := f_EUTRA_TimingInfoAdd(v_TimingInfoStep4, 18); // 18 subframes = 18 ms

v_GrantPeriod := 5; // No period value especified in the prose. Value chosed arbitrarily.

## Change 3

| Function name | function f_TC_8_5_5_1_EUTRA() |
| --- | --- |
| Reason for change | Added 16 UL grants of default size (501 bytes, as per 36.523-3[20] Table 7.2.0-1) and an additional UL grant of size 177 bytes (NPRB=20, IMCS=4, as per 36.213 Table 7.1.7.2.1) are chosen to enable UE to transmit only one ulDedicatedMessageSegment as maximum PDCP SDU size 8188 bytes |
| Summary of change | Modified grant size to received 1st segment from UE side. Note: A prose CR is associated with this change |
| TTCN module | RRC_Others_PRO.ttcn |
| MCC160 Comment |  |

Before change

<<SKIPPED CODE>>

//@siclog "Step 5" siclog@

// The SS allocates 17 UL grants periodically from 15(FDD)/18 (TDD) subframes after successful completion of step 4 to schedule PUSCH till the ULDedicatedMessageSegment at step 6 is received.

if (v_EUTRA_FDD_TDD_Mode == FDD) {

v_TimingInfoStep5 := f_EUTRA_TimingInfoAdd(v_TimingInfoStep4, 15); // 15 subframes = 15 ms

v_GrantPeriod := 1; // No period value especified in the prose. Value chosed arbitrarily.//1

}

else {

v_TimingInfoStep5 := f_EUTRA_TimingInfoAdd(v_TimingInfoStep4, 18); // 18 subframes = 18 ms

v_GrantPeriod := 5; // No period value especified in the prose. Value chosed arbitrarily.

}

//@sic R5-231397 sic@

//

f_EUTRA_PeriodicCyclicULGrantTransmission(eutra_Cell1,

cs_TimingInfo_EUTRA_NB(v_TimingInfoStep5),

9, // Imcs => Acc. 36.523-3, 4008 bits TBS//9

25,// Nprb => Acc. 36.523-3, 4008 bits TBS //25

v_GrantPeriod,

17);

//@siclog "Step 6" siclog@

// The UE sends 1st segment of UECapabilityInformation message contained within the rrc-MessageSegmentContainer-r16 IE of the ULDedicatedMessageSegment message

SRB.receive(car_SRB1_RrcPdu_IND(eutra_Cell1, cr_508_ULDedicatedMessageSegment_r16(cr_ULDedicatedMessageSegment_r16_enabled(0))));

f_EUTRA_StopULGrantTransmission_PUCCH_Sync(eutra_Cell1, cs_TimingInfo_Now);

<<SKIPPED CODE>>

After change

<<SKIPPED CODE>>

//@siclog "Step 5" siclog@

// The SS allocates 17 UL grants periodically from 15(FDD)/18 (TDD) subframes after successful completion of step 4 to schedule PUSCH till the ULDedicatedMessageSegment at step 6 is received.

if (v_EUTRA_FDD_TDD_Mode == FDD) {

v_TimingInfoStep5 := f_EUTRA_TimingInfoAdd(v_TimingInfoStep4, 15); // 15 subframes = 15 ms

v_GrantPeriod := 1; // No period value especified in the prose. Value chosed arbitrarily.//1

}

else {

v_TimingInfoStep5 := f_EUTRA_TimingInfoAdd(v_TimingInfoStep4, 18); // 18 subframes = 18 ms

v_GrantPeriod := 5; // No period value especified in the prose. Value chosed arbitrarily.

}

//@sic R5-231397 sic@

//

f_EUTRA_PeriodicCyclicULGrantTransmission(eutra_Cell1,

cs_TimingInfo_EUTRA_NB(v_TimingInfoStep5),

9, // Imcs => Acc. 36.523-3, 4008 bits TBS

25,// Nprb => Acc. 36.523-3, 4008 bits TBS

v_GrantPeriod,

16);//16 grant

v_TimingInfoStep6 := f_EUTRA_TimingInfoAdd(v_TimingInfoStep5,16);//JPS

f_EUTRA_OneULGrantTransmission(eutra_Cell1, cs_TimingInfo_EUTRA_NB(v_TimingInfoStep6), 20, 4); // Imcs  & Nprb => Acc. 36.523-3, 1416 bits TBS

//@siclog "Step 6" siclog@

// The UE sends 1st segment of UECapabilityInformation message contained within the rrc-MessageSegmentContainer-r16 IE of the ULDedicatedMessageSegment message

SRB.receive(car_SRB1_RrcPdu_IND(eutra_Cell1, cr_508_ULDedicatedMessageSegment_r16(cr_ULDedicatedMessageSegment_r16_enabled(0))));

f_EUTRA_StopULGrantTransmission_PUCCH_Sync(eutra_Cell1, cs_TimingInfo_Now);

<<SKIPPED CODE>>

## Change 4

| Function name | function f_TC_8_5_5_1_EUTRA() |
| --- | --- |
| Reason for change | - The SS side is not configured for the resumption of SRBs and DRBs - There is no TA command transmitted after Re-establishment procedure due to which UE goes out of sync and initate RACH |
| Summary of change | - f_EUTRA_RRC_RRCConnectionReconfiguration_Resume_SRB2_DRBs function is called - TA configured in uplink grant. |
| TTCN module | RRC_Others_PRO.ttcn |
| MCC160 Comment |  |

Before change

<<SKIPPED CODE>>

//@siclog "Step 12" siclog@
    // The UE transmits an RRCReestablishmentComplete message on Cell 1?
    SRB.receive( car_SRB1_RrcPdu_IND(eutra_Cell1, cr_508_RRCConnectionReestablishmentComplete(tsc_RRC_TI_Def)));

    //@siclog "Step 12a" siclog@
    f_EUTRA_RRC_RRCConnectionReconfiguration_Resume_SRB2_DRBs(eutra_Cell1, tsc_RRC_TI_Def);

//@siclog "Step 13" siclog@
    // SS resumes regular UL grant transmission.
    f_EUTRA_ULGrantConfiguration_Common(eutra_Cell1, cs_TimingInfo_Now, -, cs_UplinkTimeAlignment_Synch_None);

    //@siclog "Step 14" siclog@
    // Check: Upon successful completion of RRC re-establishment procedure, does the UE discards remaining segments of UECapabilityInformation message
    // and does not send any ULDedicatedMessageSegment message in the next 5 seconds?
    t_Watchdog.start;
    alt {
      [] t_Watchdog.timeout {
         f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 14");
       }
      [] SRB.receive(car_SRB1_RrcPdu_IND(eutra_Cell1, cr_508_ULDedicatedMessageSegment_r16(cr_ULDedicatedMessageSegment_r16_enabled(?)))) {
         t_Watchdog.stop;
         f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "Unexpected segment received");
       }
    }

<<SKIPPED CODE>>

After change

<<SKIPPED CODE>>

//@siclog "Step 12" siclog@
    // The UE transmits an RRCReestablishmentComplete message on Cell 1?
    SRB.receive( car_SRB1_RrcPdu_IND(eutra_Cell1, cr_508_RRCConnectionReestablishmentComplete(tsc_RRC_TI_Def)));

      f_EUTRA_RRC_RRCConnectionReconfiguration_Resume_SRB2_DRBs(eutra_Cell1, tsc_RRC_TI_Def);

//@siclog "Step 13" siclog@
    // SS resumes regular UL grant transmission.
f_EUTRA_ULGrantConfiguration_Common(eutra_Cell1, cs_TimingInfo_Now, -, cs_UplinkTimeAlignment_Synch_Auto_Def);

    //@siclog "Step 14" siclog@
    // Check: Upon successful completion of RRC re-establishment procedure, does the UE discards remaining segments of UECapabilityInformation message
    // and does not send any ULDedicatedMessageSegment message in the next 5 seconds?
    t_Watchdog.start;
    alt {
      [] t_Watchdog.timeout {
         f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 14");
       }
      [] SRB.receive(car_SRB1_RrcPdu_IND(eutra_Cell1, cr_508_ULDedicatedMessageSegment_r16(cr_ULDedicatedMessageSegment_r16_enabled(?)))) {
         t_Watchdog.stop;
         f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "Unexpected segment received");
       }
    }

<<SKIPPED CODE>>

## Change 5

| Function name | function f_TC_8_5_5_1_EUTRA() |
| --- | --- |
| Reason for change | This test case required early contention due to the Reestablishment procedure |
| Summary of change | Added test case in the function f_GetTestcaseAttrib_EarlyContentionResolution for early contention resolution |
| TTCN module | TestcaseProperties.ttcn |
| MCC160 Comment |  |

Before change

<<SKIPPED CODE>>

function f_GetTestcaseAttrib_EarlyContentionResolution(charstring p_Testcase) return boolean

{

select (p_Testcase) {

case ("TC_7_1_3_5a") { return true; }

case ("TC_8_1_1_1a") { return true; }

case ("TC_8_1_1_2a") { return true; }

}

return false;

}

<<SKIPPED CODE>>

After change

<<SKIPPED CODE>>

function f_GetTestcaseAttrib_EarlyContentionResolution(charstring p_Testcase) return boolean

{

select (p_Testcase) {

case ("TC_7_1_3_5a") { return true; }

case ("TC_8_1_1_1a") { return true; }

case ("TC_8_1_1_2a") { return true; }

case ("TC_8_5_5_1") { return true; }

}

return false;

}

<<SKIPPED CODE>>

# Branches executed

The test case was executed on EUTRA band 4 using ciphering algorithm eea1 and integrity algorithm eia1.

# Execution Log Files

## Qualcomm Snapdragon® X75 5G Modem-RF System , Qualcomm Technologies’ 6th generation modem-to-antenna 5G solution UE

The Qualcomm Snapdragon x70 Qualcomm Snapdragon® X75 5G Modem-RF System , Qualcomm Technologies’ 6th generation modem-to-antenna 5G solution5G Modem RF System UE passed this test case on Keysight S8704A Protocol Conformance Toolset. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

TC_8_5_5_1_LOG.html

- PICS/PIXIT parameter file:

TC_8_5_5_1_PIXIT.xml

# References

| [1] | R5s240176: Supporting information for addition of Rel-16 LTE RACS test case 8.5.5.1 |
| --- | --- |