3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240185

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3291 | rev | 1 | Current version: | 17.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to template cads_NR_ServingCellConfigPUCCH_Cell_REQ |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-12-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the current TTCN implementation of 23wk49, Template cads_NR_ServingCellConfigPUCCH_Cell_REQ is used when configuring Scell. This template is used mainly to provide information about pucch_Cell. But in this template, other IEs inside ServingCellConfig.R15 are left uninitialized. this needs to be corrected. |  |  |  |  |  |  |  |  |
| Summary of change: |  | All the IEs inside ServingCellConfig.R15 are initialized. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Runtime exception UninitializedValueException will be seen during execution |  |  |  |  |  |  |  |  |
| Clauses affected: |  | All the CA cases |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections required	2

2.1	Change to the template cads_NR_ServingCellConfigPUCCH_Cell_REQ	2

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D23wk49 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required

## Change to the template cads_NR_ServingCellConfigPUCCH_Cell_REQ

| Template name | cads_NR_ServingCellConfigPUCCH_Cell_REQ |
| --- | --- |
| Reason for change | In the current TTCN implementation of 23wk49, Template cads_NR_ServingCellConfigPUCCH_Cell_REQ is used when configuring Scell. This template is used mainly to provide information about pucch_Cell. But in this template, other IEs inside ServingCellConfig.R15 are left uninitialized. this needs to be corrected. |
| Summary of change | All the IEs inside ServingCellConfig.R15 are initialized. |
| TTCN module | ttcn\develop\Common\NR\NR_CellCfg_Templates.ttcn |
| MCC160 Comment |  |

Before Change:

template (value) NR_SYSTEM_CTRL_REQ cads_NR_ServingCellConfigPUCCH_Cell_REQ(NR_CellId_Type p_NR_CellId, template (value) TimingInfo_Type p_TimingInfo := cs_TimingInfo_Now, template (value) NR_ServingCellConfig_Type p_NR_ServingCellConfig, template (omit) NR_SS_StaticCellResourceConfig_Type p_NR_SS_StaticCellResourceConfig_Type := omit, ServCellIndex p_PUCCH_Cell := tsc_NR_PCellIndex, template (omit) boolean p_CnfFlag := omit, boolean p_FollowOnFlag := tsc_NoFollowOnFlag ) modifies cas_NR_CellConfigAllOmit_REQ := { /* @status APPROVED (ENDC, NR5GC) */ Common := cs_NR_ReqAspCommonPart_CellCfg(p_NR_CellId, p_TimingInfo, p_CnfFlag, p_FollowOnFlag), Request := { Cell := { AddOrReconfigure := { StaticResourceConfig := p_NR_SS_StaticCellResourceConfig_Type, PhysicalLayer := { Common := omit, Downlink := { FrequencyInfoDL := omit, SSPbchBlock := omit, PdschCellLevelConfig := { DMRS_TypeA_Position := omit, RateMatchPattern := omit, ServingCellConfig := { R15 := { pucch_Cell := p_PUCCH_Cell } } }, BWPs := omit, CsiConfig := omit }, Uplink := omit }, ServingCellConfig := p_NR_ServingCellConfig } } } };

After Change:

template (value) NR_SYSTEM_CTRL_REQ cads_NR_ServingCellConfigPUCCH_Cell_REQ(NR_CellId_Type p_NR_CellId, template (value) TimingInfo_Type p_TimingInfo := cs_TimingInfo_Now, template (value) NR_ServingCellConfig_Type p_NR_ServingCellConfig, template (omit) NR_SS_StaticCellResourceConfig_Type p_NR_SS_StaticCellResourceConfig_Type := omit, ServCellIndex p_PUCCH_Cell := tsc_NR_PCellIndex, template (omit) boolean p_CnfFlag := omit, boolean p_FollowOnFlag := tsc_NoFollowOnFlag ) modifies cas_NR_CellConfigAllOmit_REQ := { /* @status APPROVED (ENDC, NR5GC) */ Common := cs_NR_ReqAspCommonPart_CellCfg(p_NR_CellId, p_TimingInfo, p_CnfFlag, p_FollowOnFlag), Request := { Cell := { AddOrReconfigure := { StaticResourceConfig := p_NR_SS_StaticCellResourceConfig_Type, PhysicalLayer := { Common := omit, Downlink := { FrequencyInfoDL := omit, SSPbchBlock := omit, PdschCellLevelConfig := { DMRS_TypeA_Position := omit, RateMatchPattern := omit, ServingCellConfig := { R15 := { //WA#WI=1227476 codeBlockGroupTransmission := omit, xOverhead := omit, nrofHARQ_ProcessesForPDSCH := omit, pucch_Cell := p_PUCCH_Cell, maxMIMO_Layers := omit, processingType2Enabled := omit, pdsch_CodeBlockGroupTransmissionList_r16 := omit, downlinkHARQ_FeedbackDisabled_r17 := omit, nrofHARQ_ProcessesForPDSCH_v1700 := omit } } }, BWPs := omit, CsiConfig := omit }, Uplink := omit }, ServingCellConfig := p_NR_ServingCellConfig } } } };