3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240198

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3307 | rev | 1 | Current version: | 17.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC testcase 11.4.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-12-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the current TTCN implementation, the function f_TC_11_4_1_NR5GC_IMS1 is still being used which calls function f_IMS_NR5GC_MOSpeechCallAndRelease. This is not needed anymore ans the Prose does not have a speech call. As a result, the function f_TC_11_4_1_NR5GC_IMS1 itself is now not needed. The basic IMS registration/deregistration can be handled on default IMS PTC. This needs to be corrected. After Step 3EA, when the rrc connection is released, the IMS PTC also needs resetting to prepare it for the next IMS registration. This needs to be implemented. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Removed the function f_TC_11_4_1_NR5GC_IMS1, and updated TC_11_4_1_NR5GC() accordingy. Called function f_NR5GC_ResetIMS(PDN_1); after Step 3EA. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.4.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

1.1	Change 2	5

1.1	Change 3	7

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D23wk49 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required

## Change 1

| Function name | f_TC_11_4_1_NR5GC_IMS1 |
| --- | --- |
| Reason for change | In the current TTCN implementation, the function f_TC_11_4_1_NR5GC_IMS1 is still being used which calls function f_IMS_NR5GC_MOSpeechCallAndRelease. This is not needed anymore ans the Prose does not have a speech call. As a result, the function f_TC_11_4_1_NR5GC_IMS1 itself is now not needed. The basic IMS registration/deregistration can be handled on default IMS PTC. |
| Summary of change | Removed (voided) the function f_TC_11_4_1_NR5GC_IMS1 |
| TTCN module | Emergency_NR5GC_IMS.ttcn |
| MCC160 Comment |  |

Before Change:

function f_TC_11_4_1_NR5GC_IMS1()runs on IMS_PTC { //@sic R5-220560 sic@ //Initial PDU session - for e.g Registration and 'normal' calls var template (present) SipUrl v_OtherCalleeUri := cr_SipUrl_Common((c_sipScheme, c_telScheme), ?); // @sic R5s210603 sic@ f_IMS_PTC_Init(PDN_1); f_IMS_IPCAN_SendCoOrdMsg(IPCAN); f_IMS_Registration(); f_IMS_IPCAN_SendCoOrdMsg(IPCAN); //@sic R5s221039 sic@ f_IMS_GlobalRelease(); f_IMS_IPCAN_SendCoOrdMsg(IPCAN); f_IMS_Registration(); f_IMS_IPCAN_SendCoOrdMsg(IPCAN); f_IMS_NR5GC_MOSpeechCallAndRelease(v_OtherCalleeUri); f_IMS_IpcanReleaseWithOptionalDeregistration(); }

After Change:

/*WA#11_4_1 COMMENTED OUT function f_TC_11_4_1_NR5GC_IMS1()runs on IMS_PTC { //@sic R5-220560 sic@ //Initial PDU session - for e.g Registration and 'normal' calls var template (present) SipUrl v_OtherCalleeUri := cr_SipUrl_Common((c_sipScheme, c_telScheme), ?); // @sic R5s210603 sic@ f_IMS_PTC_Init(PDN_1); f_IMS_IPCAN_SendCoOrdMsg(IPCAN); f_IMS_Registration(); f_IMS_IPCAN_SendCoOrdMsg(IPCAN); //@sic R5s221039 sic@ f_IMS_GlobalRelease(); f_IMS_IPCAN_SendCoOrdMsg(IPCAN); f_IMS_Registration(); f_IMS_IPCAN_SendCoOrdMsg(IPCAN); f_IMS_NR5GC_MOSpeechCallAndRelease(v_OtherCalleeUri); f_IMS_IpcanReleaseWithOptionalDeregistration(); }*/

## Change 2

| Function name | testcase TC_11_4_1_NR5GC() |
| --- | --- |
| Reason for change | See change 1 above |
| Summary of change | Updated the function to remove calls for IMS1. |
| TTCN module | NR5GC_Testsuite.ttcn |
| MCC160 Comment |  |

Before Change:

testcase TC_11_4_1_NR5GC() runs on MTC_NR5GC system SYSTEM_NR5GC { // @purpose // 5GMM-REGISTERED.NORMAL-SERVICE / 5GMM-IDLE / Emergency call / Utilising emergency number stored on the USIM / New emergency PDU session / Network failing the authentication check (5G AKA) var NR5GC_PTC v_NR5GC := null; var IMS_PTC v_IMS1 := null; var IMS_PTC v_IMS2 := null; timer t_GuardTimer := int2float(300); v_NR5GC := NR5GC_PTC.create("NR5GC") alive; v_IMS1 := IMS_PTC.create("IMS1") alive; v_IMS2 := IMS_PTC.create("IMS2") alive; f_MTC_ConnectPTCs_NR5GC(system, v_NR5GC, v_IMS1, v_IMS2); v_NR5GC.start(f_TC_11_4_1_NR5GC()); v_IMS1.start(f_TC_11_4_1_NR5GC_IMS1()); v_IMS2.start(f_TC_11_4_1_NR5GC_IMS2()); t_GuardTimer.start; f_MTC_MainLoop_NR5GC(t_GuardTimer); }

After Change:

testcase TC_11_4_1_NR5GC() runs on MTC_NR5GC system SYSTEM_NR5GC { // @purpose // 5GMM-REGISTERED.NORMAL-SERVICE / 5GMM-IDLE / Emergency call / Utilising emergency number stored on the USIM / New emergency PDU session / Network failing the authentication check (5G AKA) var NR5GC_PTC v_NR5GC := null; var IMS_PTC v_IMS1 := null; var IMS_PTC v_IMS2 := null; timer t_GuardTimer := int2float(300); v_NR5GC := NR5GC_PTC.create("NR5GC") alive; //WA#11_4_1 COMMENTED OUT v_IMS1 := IMS_PTC.create("IMS1") alive; v_IMS2 := IMS_PTC.create("IMS2") alive; f_MTC_ConnectPTCs_NR5GC(system, v_NR5GC, v_IMS1, v_IMS2); v_NR5GC.start(f_TC_11_4_1_NR5GC()); //WA#11_4_1 COMMENTED OUT v_IMS1.start(f_TC_11_4_1_NR5GC_IMS1()); v_IMS2.start(f_TC_11_4_1_NR5GC_IMS2()); t_GuardTimer.start; f_MTC_MainLoop_NR5GC(t_GuardTimer); }

## Change 3

| Function name | f_NR_SS_BWP_List_UL |
| --- | --- |
| Reason for change | After Step 3EA, when the rrc connection is released, the IMS PTC also needs resetting to prepare it for the next IMS registration. This needs to be implemented. |
| Summary of change | Called function f_NR5GC_ResetIMS(PDN_1); after Step 3EA. |
| TTCN module | Emergency_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change:

<<SKIPPf_NR5GC_EmergencyCallRelease(nr_Cell1); f_Delay(0.1); //@siclog "Step 3EA" siclog@ //@sic R5-237482 sic@ //The SS transmits an RRCRelease message f_NR_RRCRelease(nr_Cell1); deactivate(v_MyDefaultVar); //@sic Rs230748 sic@ //@siclog "Step 3EB" siclog@ //@sic R5-237482 sic@ //SS configures the RA Response. f_NR_SS_RachProcedureConfig_Def(nr_Cell1);ED CODE>> <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> f_NR5GC_EmergencyCallRelease(nr_Cell1); f_Delay(0.1); //@siclog "Step 3EA" siclog@ //@sic R5-237482 sic@ //The SS transmits an RRCRelease message f_NR_RRCRelease(nr_Cell1); deactivate(v_MyDefaultVar); //@sic Rs230748 sic@ f_NR5GC_ResetIMS(PDN_1); //WA#11_4_1 : This is needed to handle the next IMS registration. //@siclog "Step 3EB" siclog@ //@sic R5-237482 sic@ //SS configures the RA Response. f_NR_SS_RachProcedureConfig_Def(nr_Cell1); <<SKIPPED CODE>>