3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240199

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.571-4 | CR | 0194 | rev | 1 | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to function f_POS_NR_CheckCapabilities_NR_Ecid |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh-UEConTest |  |  |  |  |  | Date: |  |  | 2023-12-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The IE nr-ECID-MeasSupported-r16 is a BIT STRING with (SIZE(1..8)). In the function f_POS_NR_CheckCapabilities_NR_Ecid each bit from nr-ECID-MeasSupported-r16 is compared one by one to the bits from v_Bitstring, a 4 bit bistring. As the size of nr-ECID-MeasSupported-r16 can go up to 8 bits, some UEs report more than 4 bits for the nr-ECID-MeasSupported-r16. When comparing it with v_Bitstring, if the nr-ECID-MeasSupported-r16 is longer than 4 bits, we may try to access a value from v_Bitstring that is not defined (from 5th up to 8th bit), causing TC o fail. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change f_POS_NR_CheckCapabilities_NR_Ecid in a way that the index of v_Bitstring is not exceeeded |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail this test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.1.2, 9.3.3.1, 9.3.4.1, 9.3.4.2, 9.3.4.3, 9.3.4.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	2

1	Corrections required	3

1.1	Change 1	3

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D23wk49 related to the title of this CR.

Contact:	Parikshit Bhise

parikshit.bhise@rohde-schwarz.com

# Corrections required

## Change 1

| Function name | f_POS_NR_CheckCapabilities_NR_Ecid () |
| --- | --- |
| Reason for change | The IE nr-ECID-MeasSupported-r16 is a BIT STRING with (SIZE(1..8)). In the function f_POS_NR_CheckCapabilities_NR_Ecid each bit from nr-ECID-MeasSupported-r16 is compared one by one to the bits from v_Bitstring, a 4 bit bistring. As the size of nr-ECID-MeasSupported-r16 can go up to 8 bits, some UEs report more than 4 bits for the nr-ECID-MeasSupported-r16. When comparing it with v_Bitstring, if the nr-ECID-MeasSupported-r16 is longer than 4 bits, we may try to access a value from v_Bitstring that is not defined (from 5th up to 8th bit), causing TC o fail. |
| Summary of change | Add check to avoid using values of v_Bistring that ae not defined |
| TTCN module | NR5GC_Positioning_Functions.ttcn |
| MCC160 Comment |  |

Before change

function f_POS_NR_CheckCapabilities_NR_Ecid (NR_ECID_ProvideCapabilities_r16 p_NrEcidProvideCapabilities) runs on NR_BASE_PTC

{ //@sic POS Rel-16 R5-213638 sic@

var integer i;

var bitstring v_Bitstring := f_ConvertBoolToBit(pc_NR_ECID_SSRsrp) &

f_ConvertBoolToBit(pc_NR_ECID_SSRsrq) &

f_ConvertBoolToBit(pc_NR_ECID_CSIRsrp) &

f_ConvertBoolToBit(pc_NR_ECID_CSIRsrq);

for (i := 0; i < lengthof(p_NrEcidProvideCapabilities.nr_ECID_MeasSupported_r16)-1; i := i + 1) { // @sic R5s220031 sic@

if (not (v_Bitstring[i] == p_NrEcidProvideCapabilities.nr_ECID_MeasSupported_r16[i])) {

f_NR_SetVerdictFailOrInconc (__FILE__, __LINE__, "PICS does not match reported capability for NR ECID measurements");

}

}

if (pc_NR_ECID_periodicalReporting != ispresent(p_NrEcidProvideCapabilities.periodicalReporting_r16)) {

f_NR_SetVerdictFailOrInconc (__FILE__, __LINE__, "PICS does not match reported capability for NR ECID periodicalReporting");

}

if (pc_NR_ECID_triggeredReporting != ispresent(p_NrEcidProvideCapabilities.triggeredReporting_r16)) {

f_NR_SetVerdictFailOrInconc (__FILE__, __LINE__, "PICS does not match reported capability for NR ECID triggeredReporting");

}

}

After change

function f_POS_NR_CheckCapabilities_NR_Ecid (NR_ECID_ProvideCapabilities_r16 p_NrEcidProvideCapabilities) runs on NR_BASE_PTC

{ //@sic POS Rel-16 R5-213638 sic@

var integer i;

var bitstring v_Bitstring := f_ConvertBoolToBit(pc_NR_ECID_SSRsrp) &

f_ConvertBoolToBit(pc_NR_ECID_SSRsrq) &

f_ConvertBoolToBit(pc_NR_ECID_CSIRsrp) &

f_ConvertBoolToBit(pc_NR_ECID_CSIRsrq);

for (i := 0; i < lengthof(p_NrEcidProvideCapabilities.nr_ECID_MeasSupported_r16)-1; i := i + 1) { // @sic R5s220031 sic@

if(i == lengthof(v_Bitstring)){

break;

}

if (not (v_Bitstring[i] == p_NrEcidProvideCapabilities.nr_ECID_MeasSupported_r16[i])) {

f_NR_SetVerdictFailOrInconc (__FILE__, __LINE__, "PICS does not match reported capability for NR ECID measurements");

}

}

if (pc_NR_ECID_periodicalReporting != ispresent(p_NrEcidProvideCapabilities.periodicalReporting_r16)) {

f_NR_SetVerdictFailOrInconc (__FILE__, __LINE__, "PICS does not match reported capability for NR ECID periodicalReporting");

}

if (pc_NR_ECID_triggeredReporting != ispresent(p_NrEcidProvideCapabilities.triggeredReporting_r16)) {

f_NR_SetVerdictFailOrInconc (__FILE__, __LINE__, "PICS does not match reported capability for NR ECID triggeredReporting");

}

}