3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240227

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3384 | rev | - | Current version: | 17.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to function f_EUTRA38_MultiPDN_EstAddPDNs_DefaultActivated |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-03-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the current TTCN implementation of this function, f_EUTRA38_MultiPDN_GetPDNType(v_PDNConnReq.accessPointName) returns PDN Type as IMS_DNN even if the PDN connectivity REQ indicates the request type as emergency, and if there is no APN provided in the PDN Connectivity REQ. This then results in the timer v_WaitForUserplaneSignalling being set to waitForIMS. For a testcase like 11.1.7, this introduces a delay of 10s between the UE transmitting ACK (34.229-1 Annex C.22 Step 5) and receiving Activate DedicatedEPS Bearer Context REQ because when the function fl_TC_11_1_7_NR5GC_EUTRA_TestBody --> f_EUTRA38_MultiPDN_EstablishAdditionalPDNs --> f_EUTRA38_MultiPDN_EstAddPDNs_DefaultActivated --> f_EUTRA_DelayForUserPlaneSignalling is called. Some UE implementations release the IMS call because of this delay (by sending a BYE), which results in testcase failure. This needs to be addressed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated the implementation of function f_EUTRA38_MultiPDN_EstAddPDNs_DefaultActivated. Please see below. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.1.7.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Overview	4

2	Corrections required	4

2.1	Function f_EUTRA38_MultiPDN_EstAddPDNs_DefaultActivated	4

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D23wk49 related to the title of this CR.

Contact:	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required

## Function f_EUTRA38_MultiPDN_EstAddPDNs_DefaultActivated

| Function name | f_EUTRA38_MultiPDN_EstAddPDNs_DefaultActivated |
| --- | --- |
| Reason for change | In the current TTCN implementation of this function, f_EUTRA38_MultiPDN_GetPDNType(v_PDNConnReq.accessPointName) returns PDN Type as IMS_DNN even if the PDN connectivity REQ indicates the request type as emergency, and if there is no APN provided in the PDN Connectivity REQ. This then results in the timer v_WaitForUserplaneSignalling being set to waitForIMS. For a testcase like 11.1.7, this introduces a delay of 10s between the UE transmitting ACK (34.229-1 Annex C.22 Step 5) and receiving Activate DedicatedEPS Bearer Context REQ because when the function fl_TC_11_1_7_NR5GC_EUTRA_TestBody --> f_EUTRA38_MultiPDN_EstablishAdditionalPDNs --> f_EUTRA38_MultiPDN_EstAddPDNs_DefaultActivated --> f_EUTRA_DelayForUserPlaneSignalling is called. Some UE implementations release the IMS call because of this delay (by sending a BYE), which results in testcase failure. This needs to be addressed. |
| Summary of change | Updated the TTCN implementation to 1) check if the IE AccessPointName is present in the PDN Connectivity REQ, and based on that and RequestType, assigned the value to IE AccessPointName. Please see below. |
| TTCN module | Eutra38_NR5GC_InitialRegistration.ttcn |
| MCC160 Comment |  |

Before Change:

<<SKIPPED CODE>> v_SRB := tsc_SRB2; // SRB2 will have been configured by now, even if not earlier a_EUTRA38_MultiPDN_ConnectToAdditionalPDN_Step4(p_CellId, v_EpsBearerId); // If the UE has just moved from NR, don't need to wait for IMS signalling if (not isvalue(p_PduInfoFromNR)) { // @sic R5s230015 sic@ select (f_EUTRA38_MultiPDN_GetPDNType(v_PDNConnReq.accessPointName)) { // @sic R5-206296 sic@ case (IMS_DNN) { v_WaitForUserplaneSignalling := waitForIMS; } case (Internet_DNN) { v_WaitForUserplaneSignalling := dontWaitForIMS; } } } } } // Now all PDNs on same connection have been established, wait for any signalling on user plane to finish before releasing connection f_EUTRA_DelayForUserPlaneSignalling (v_WaitForUserplaneSignalling); return v_SRB; <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> v_SRB := tsc_SRB2; // SRB2 will have been configured by now, even if not earlier a_EUTRA38_MultiPDN_ConnectToAdditionalPDN_Step4(p_CellId, v_EpsBearerId); if( not ispresent(v_PDNConnReq.accessPointName)) { //WA#11_1_7 if(v_PDNConnReq.requestType.typeValue == '100'B){ v_PDNConnReq.accessPointName := valueof(cs_AccessPointName(f_DomainName_EncodeLabels({"sos"}))); } } // If the UE has just moved from NR, don't need to wait for IMS signalling if (not isvalue(p_PduInfoFromNR)) { // @sic R5s230015 sic@ select (f_EUTRA38_MultiPDN_GetPDNType(v_PDNConnReq.accessPointName)) { // @sic R5-206296 sic@ case (IMS_DNN) { v_WaitForUserplaneSignalling := waitForIMS; } case (Internet_DNN) { v_WaitForUserplaneSignalling := dontWaitForIMS; } } } } } // Now all PDNs on same connection have been established, wait for any signalling on user plane to finish before releasing connection f_EUTRA_DelayForUserPlaneSignalling (v_WaitForUserplaneSignalling); return v_SRB; <<SKIPPED CODE>>