3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240233

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3387 | rev | - | Current version: | 17.9.0 |  |
| For HELPon using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC testcase 11.4.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Starpoint |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-03-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | At Step 2C, the PTI in PDU SESSION RELEASE COMMAND should match the value recieved in PDU SESSION RELEASE REQUEST |  |  |  |  |  |  |  |  |
| Summary of change: |  | Use PTI indicated in PDU SESSION RELEASE REQUEST message for step2C |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the testcase |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.4.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections required	2

2.1	fl_TC_11_4_1_TestBody	2

- Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D23wk49 related to the title of this CR.

Contact:	Ting Gao

gaoting@starpointcomm.com

- Corrections required

  - fl_TC_11_4_1_TestBody

| Functionname | fl_TC_11_4_1_TestBody |
| --- | --- |
| Reason for change | At Step 2C, the PTI in PDU SESSION RELEASE COMMAND should match the value recieved in PDU SESSION RELEASE REQUEST |
| Summary of change | Use PTI indicated in PDU SESSION RELEASE REQUEST message for step2C |
| TTCN module | Emergency_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change:

function fl_TC_11_4_1_TestBody() runs on NR5GC_PTC { var template (value) NG_NAS_DL_Message_Type v_MsgToSend; var PLMN_Identity v_ASN_PLMN := f_NR_CellInfo_GetPLMN (nr_Cell1); var NAS_PlmnId v_PLMN_Cell1 := f_NR_Asn2Nas_PlmnId(v_ASN_PLMN); var NG_NAS_SecurityParams_Type v_NAS_SecurityParams := f_NR5GC_Security_Get(); var B3_Type v_KSI_InUse := v_NAS_SecurityParams.KSIamf; var GSM_MobilityInfo_Type v_GSM_MobilityInfo; var PDUSessionInfoList_Type v_PDUSessionInfo; var O1_Type v_PDUSessionIdToRelease; var NR_SRB_COMMON_IND v_ReceivedAsp; var integer i; var float v_T3520 := f_NR_SetTimerToleranceMin(nr_Cell1, nasTimer, 15.0); //@sic R5s221039 sic@ var default v_MyDefaultVar := null;//@sic R5s230748 sic@ @sic R5-237482 sic@ //@siclog "Step 1" siclog@ //Make the UE attempt an IMS emergency call dialling the number 144 which is stored on the USIM. f_UT_RequestIMSEmergencyCall (UT, "144"); //@siclog "Step 2" siclog@ //Check: Does the UE performs Generic Test Procedure for IMS Emergency call establishment with IMS emergency registration? f_NR5GC_EmergencyCallWithRegistration(nr_Cell1); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 2"); //@siclog "Step 2A" siclog@ //The SS initiates the 5G AKA based primary authentication and key agreement procedure by sending an AUTHENTICATION REQUEST message; v_MsgToSend := f_Get_5GAKA_AuthenticationReqMsg(v_NAS_SecurityParams, v_PLMN_Cell1); v_MsgToSend.authentication_Request.ngNasKeySetId.nasKeySetId := v_KSI_InUse; SRB.send(cas_NR_SRB_NasPdu_REQ(nr_Cell1, tsc_NR_RbId_SRB2, //@sic R5s221039 sic@ -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, v_MsgToSend))); //@siclog "Step 2B" siclog@ //The UE sends an AUTHENTICATION FAILURE message with 5GMM cause #71 "ngKSI already in use" SRB.receive(car_NR_SRB_NasPdu_IND(nr_Cell1, tsc_NR_RbId_SRB2, cr_NG_NAS_Ind(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp; //@sic R5s221039 sic@ if (match (v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg, cr_NG_AUTHENTICATION_FAILURE(cr_GMM_GSM_Cause(omit, '01000111'B)))) { // ngKSI already in use f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 2B"); } //@siclog "Step 2BA" siclog@ //Wait for T3520 expires. f_Delay(v_T3520); //@sic R5s221039 sic@ //@siclog "Step 2C" siclog@ //PDU session release procedure to release non-IMS Emergency relevant PDUs. //Make the UE to transmit a PDU SESSION RELEASE REQ v_PDUSessionInfo := f_NR5GC_MobileInfo_GetSessionInfoList(); //@sic R5s230143 sic@ //Reset the stored messages, to capture the PDU Session Release REQs f_NR5GC_MsgInDefault_ResetStoreOfNASMessageReq(); for ( i := 0; i<(lengthof(v_PDUSessionInfo) - 1); i:=i+1) //Capture PDU Session Release REQs { SRB.receive(car_NR_SRB_NasPdu_IND(nr_Cell1, ?, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp; f_NR5GC_MsgInDefault_StoreNASMessageReq (v_ReceivedAsp); } for ( i := 0; i<(lengthof(v_PDUSessionInfo) - 1); i:=i+1) //Now release fully the captured PDU Session Release REQs { if (v_PDUSessionInfo[i].PDUType != Emergency_PDN) { v_PDUSessionIdToRelease := v_PDUSessionInfo[i].SessionId; v_ReceivedAsp := f_NR5GC_MsgInDefault_GetOneNASMessageReq(i+1); if (not f_Check_NG_PDUSessionReleaseReq (v_GSM_MobilityInfo, v_PDUSessionInfo, v_ReceivedAsp.Signalling.Nas[0].Pdu, v_PDUSessionIdToRelease)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "PDU Session Release Request Message Failed"); } else { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 2C-1"); } f_NR5GC_PDUSessionRelease(nr_Cell1, v_PDUSessionIdToRelease, cs_GMM_GSM_Cause (omit, '00100100'B)); } } v_MyDefaultVar := activate(a_NR5GC_Deregistration_REQ(nr_Cell1)); //@sic R5s230748 sic@ //@siclog "Step 2CA" siclog@ //@sic R5-237482 sic@ //The SS is configured to Stop RA response f_NR_SS_RachProcedureConfig(nr_Cell1, cs_NR_RachProcedureConfig_NoResponse); f_NR_ULGrantConfiguration_Start(nr_Cell1); //@sic R5s240136 sic@ //@siclog "Step 3-3E" siclog@ //@sic Rs230748 sic@ f_NR5GC_EmergencyCallRelease(nr_Cell1); f_Delay(0.1); //@siclog "Step 3EA" siclog@ //@sic R5-237482 sic@ //The SS transmits an RRCRelease message f_NR_RRCRelease(nr_Cell1); deactivate(v_MyDefaultVar); //@sic R5s230748 sic@ f_NR5GC_ResetIMS(PDN_1); //@sic R5s240026 sic@ //@siclog "Step 3EB" siclog@ //@sic R5-237482 sic@ //SS configures the RA Response. f_NR_SS_RachProcedureConfig_Def(nr_Cell1); //@siclog "Step 3F-9A2" siclog@ //Void //@sic R5-237482 sic@ //@siclog "Step 9A3 - 9A21a1" siclog@ //Steps 2-20a1 of Table 4.5.2.2- 2: Registration procedure for initial registration as specified in TS 38.508-1 [4] are performed on NR Cell 1. f_NR5GC_RRC_IdleState1N_Def(nr_Cell1, -, -, Initial_NoSecurity); //@sic R5s230748 sic@ //@siclog "Step 9A22-23" siclog@ //Void //@sic R5-237482 sic@ }

After Change:

function fl_TC_11_4_1_TestBody() runs on NR5GC_PTC { var template (value) NG_NAS_DL_Message_Type v_MsgToSend; var PLMN_Identity v_ASN_PLMN := f_NR_CellInfo_GetPLMN (nr_Cell1); var NAS_PlmnId v_PLMN_Cell1 := f_NR_Asn2Nas_PlmnId(v_ASN_PLMN); var NG_NAS_SecurityParams_Type v_NAS_SecurityParams := f_NR5GC_Security_Get(); var B3_Type v_KSI_InUse := v_NAS_SecurityParams.KSIamf; var GSM_MobilityInfo_Type v_GSM_MobilityInfo; var PDUSessionInfoList_Type v_PDUSessionInfo; var O1_Type v_PDUSessionIdToRelease; var NR_SRB_COMMON_IND v_ReceivedAsp; var integer i; var float v_T3520 := f_NR_SetTimerToleranceMin(nr_Cell1, nasTimer, 15.0); //@sic R5s221039 sic@ var default v_MyDefaultVar := null;//@sic R5s230748 sic@ @sic R5-237482 sic@ var ProcedureTransactionIdentifier v_PTI; //@siclog "Step 1" siclog@ //Make the UE attempt an IMS emergency call dialling the number 144 which is stored on the USIM. f_UT_RequestIMSEmergencyCall (UT, "144"); //@siclog "Step 2" siclog@ //Check: Does the UE performs Generic Test Procedure for IMS Emergency call establishment with IMS emergency registration? f_NR5GC_EmergencyCallWithRegistration(nr_Cell1); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 2"); //@siclog "Step 2A" siclog@ //The SS initiates the 5G AKA based primary authentication and key agreement procedure by sending an AUTHENTICATION REQUEST message; v_MsgToSend := f_Get_5GAKA_AuthenticationReqMsg(v_NAS_SecurityParams, v_PLMN_Cell1); v_MsgToSend.authentication_Request.ngNasKeySetId.nasKeySetId := v_KSI_InUse; SRB.send(cas_NR_SRB_NasPdu_REQ(nr_Cell1, tsc_NR_RbId_SRB2, //@sic R5s221039 sic@ -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, v_MsgToSend))); //@siclog "Step 2B" siclog@ //The UE sends an AUTHENTICATION FAILURE message with 5GMM cause #71 "ngKSI already in use" SRB.receive(car_NR_SRB_NasPdu_IND(nr_Cell1, tsc_NR_RbId_SRB2, cr_NG_NAS_Ind(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp; //@sic R5s221039 sic@ if (match (v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg, cr_NG_AUTHENTICATION_FAILURE(cr_GMM_GSM_Cause(omit, '01000111'B)))) { // ngKSI already in use f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 2B"); } //@siclog "Step 2BA" siclog@ //Wait for T3520 expires. f_Delay(v_T3520); //@sic R5s221039 sic@ //@siclog "Step 2C" siclog@ //PDU session release procedure to release non-IMS Emergency relevant PDUs. //Make the UE to transmit a PDU SESSION RELEASE REQ v_PDUSessionInfo := f_NR5GC_MobileInfo_GetSessionInfoList(); //@sic R5s230143 sic@ //Reset the stored messages, to capture the PDU Session Release REQs f_NR5GC_MsgInDefault_ResetStoreOfNASMessageReq(); for ( i := 0; i<(lengthof(v_PDUSessionInfo) - 1); i:=i+1) //Capture PDU Session Release REQs { SRB.receive(car_NR_SRB_NasPdu_IND(nr_Cell1, ?, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp; f_NR5GC_MsgInDefault_StoreNASMessageReq (v_ReceivedAsp); } v_PTI := v_ReceivedAsp.Signalling.Nas[0].Pdu.PiggybackedPduList[0].Msg.pdu_Session_Release_Request.procedureTransactionIdentifier; for ( i := 0; i<(lengthof(v_PDUSessionInfo) - 1); i:=i+1) //Now release fully the captured PDU Session Release REQs { if (v_PDUSessionInfo[i].PDUType != Emergency_PDN) { v_PDUSessionIdToRelease := v_PDUSessionInfo[i].SessionId; v_ReceivedAsp := f_NR5GC_MsgInDefault_GetOneNASMessageReq(i+1); if (not f_Check_NG_PDUSessionReleaseReq (v_GSM_MobilityInfo, v_PDUSessionInfo, v_ReceivedAsp.Signalling.Nas[0].Pdu, v_PDUSessionIdToRelease)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "PDU Session Release Request Message Failed"); } else { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 2C-1"); } f_NR5GC_PDUSessionRelease(nr_Cell1, v_PDUSessionIdToRelease, cs_GMM_GSM_Cause (omit, '00100100'B), -, -, -, -,v_PTI); } } v_MyDefaultVar := activate(a_NR5GC_Deregistration_REQ(nr_Cell1)); //@sic R5s230748 sic@ //@siclog "Step 2CA" siclog@ //@sic R5-237482 sic@ //The SS is configured to Stop RA response f_NR_SS_RachProcedureConfig(nr_Cell1, cs_NR_RachProcedureConfig_NoResponse); f_NR_ULGrantConfiguration_Start(nr_Cell1); //@sic R5s240136 sic@ //@siclog "Step 3-3E" siclog@ //@sic Rs230748 sic@ f_NR5GC_EmergencyCallRelease(nr_Cell1); f_Delay(0.1); //@siclog "Step 3EA" siclog@ //@sic R5-237482 sic@ //The SS transmits an RRCRelease message f_NR_RRCRelease(nr_Cell1); deactivate(v_MyDefaultVar); //@sic R5s230748 sic@ f_NR5GC_ResetIMS(PDN_1); //@sic R5s240026 sic@ //@siclog "Step 3EB" siclog@ //@sic R5-237482 sic@ //SS configures the RA Response. f_NR_SS_RachProcedureConfig_Def(nr_Cell1); //@siclog "Step 3F-9A2" siclog@ //Void //@sic R5-237482 sic@ //@siclog "Step 9A3 - 9A21a1" siclog@ //Steps 2-20a1 of Table 4.5.2.2- 2: Registration procedure for initial registration as specified in TS 38.508-1 [4] are performed on NR Cell 1. f_NR5GC_RRC_IdleState1N_Def(nr_Cell1, -, -, Initial_NoSecurity); //@sic R5s230748 sic@ //@siclog "Step 9A22-23" siclog@ //Void //@sic R5-237482 sic@ }