3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240244

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3391 | rev | - | Current version: | 17.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of NR5GC testcase 11.3.11 in FR1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test |  |  |  |  |  | Date: |  |  | 2024-03-13 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To add NR5GC test case 11.3.11 to the IWD_23wk49 ATS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to test case 11.3.11 required for approval. See detailed change description for further information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.3.11.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  |  | 38.523-1 (R5-240510 agreed in RAN5#102)) |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required to test case 11.3.11	4

2.1	fl_TC_11_3_11_TestBody	4

3	Branches executed	8

4	Execution Log Files	8

4.1	Hisilicon	8

5	References	8

# Overview

This document lists all the essencial changes needed to correct problems in the TTCN implementation of NR5GC test case 11.3.11 which is part of the NR5GC test suite in iWD_TTCN3-B2022-09_D23wk49.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log file(s) is (are) provided as evidence.

Contact:	Parikshit Bhise

parikshit.bhise@rohde-schwarz.com

## Verification Test Summary

Test Case: 	11.3.11.NR5GC

ATS Version:	iwd-TTCN3-B2022-09_D23wk49

System Simulator used:	R&S® 5G Protocol Conformance Test platform

UE used:	Hisilicon Balong 5000

Verification Status:	PASS

# Corrections required to test case 11.3.11

## fl_TC_11_3_11_TestBody

| Function name | fl_TC_11_3_11_TestBody |
| --- | --- |
| Reason for change | In the current TTCN implementation, at Step 3 in the suspend config the RAN notification area with NR Cell 1 is being sent. This is not inline with Prose, which mentions NR Cell 3. This needs to be changed. In the current Prose, after Step 10 the UE is in idle state, and both NR cells are at serving cell power, and both cells are also on same PLMN and have same TAC. The UE while performing postamble may initiate a de-registration procedure on either of them depending on which cell it has reselected to. This causes unstable testcase behaviour and needs to be addressed. At Step 10, instead of performing rrc release, its proposed to perform the De-registration in connected mode. Prose CR R5-240510 Is already raised which got agreed at RAN5#102. This needs to be implemented. |
| Summary of change | Updated the TTCN for Step 3, and Step 10. Please see below. |
| TTCN module | UAC_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change:

function fl_TC_11_3_11_TestBody() runs on NR5GC_PTC { var template (value) QosFlow_Identification_Type v_QosFlowId := f_NR_GetQosFlowIdentification_ForDRBId(f_NR_GetDefaultDRB_ForFirstPDUSession()); var SDAP_SDUList_Type v_SDAP_Data := { f_IPv4IPv6_IcmpEchoReply(f_LoopbackModeB_IP_Address_UE()) }; //Create the IP Data var template (value) RedirectedCarrierInfo v_RedirectedCarrierInfo; //@siclog "Step 1" siclog@ //The SS transmits one IP PDU. DRB.send(cas_NR_DRB_COMMON_REQ_SDAP_SDUList(nr_Cell1, v_QosFlowId, v_SDAP_Data)); //@siclog "Step 2" siclog@ //Wait for 1 second after the IP packet has been transmitted in step 1. f_Delay(1.0); //@siclog "Step 3" siclog@ //The SS transmits an RRCRelease message (IE redirectedCarrierInfo including NR Cell 3 and IE mpsPriorityIndication and suspendConfig). v_RedirectedCarrierInfo := cs_NR_redirectedCarrierInfo(f_NR_CellInfo_GetFrequencySSB(nr_Cell3), f_NR_CellInfo_GetSSB_SCS(nr_Cell3), f_NR_GetSSB_MTC_InterFreqOdd(nr_Cell1, nr_Cell3)); SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, cs_TimingInfo_Now, cs_38508_RRCRelease(tsc_NR_RRC_TI_Def, cs_38508_SuspendConfig(cs_RAN_NotificationAreaInfo_1Cell ( f_NR_CellInfo_GetCellIdentity(nr_Cell1), omit)), v_RedirectedCarrierInfo, -, -, cs_RRCConnectionRelease_All_IEs_v1540(-, cs_RRCRelease_v1610_IEs(cs_RRCRelease_v1650_IEs(true_)))))); //Activate security at SS side in advance so that SS is ready to send RRCResume message // Only SRBs are configured - so security is activated on SRBs f_NR_SS_AS_ActivateSecurity_Current(nr_Cell3,tsc_NR_38508_NextHopChainingCount); //@siclog "Step 4" siclog@ SRB.receive(car_NR_SRB0_RrcPdu_IND(nr_Cell3, cr_38508_RRCResumeRequest(tsc_NR_ShortI_RNTI_Value1, mps_PriorityAccess))); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 4"); //@siclog "Step 5" siclog@ //The SS transmits an RRCResume message. f_NR_SendRRCResume_Def(nr_Cell3); // EXCEPTION: Steps 6 and 7 can happen in any order //@siclog "Step 6" siclog@ //The UE transmits an RRCResumeComplete message. //@siclog "Step 7 siclog@ //The UE loop back the IP PDU interleave { [] SRB.receive(car_NR_SRB1_RrcPdu_IND(nr_Cell3, cr_38508_RRCResumeComplete)){} [] DRB.receive(car_NR_DRB_COMMON_IND_SDAP_SDUList(nr_Cell3, v_QosFlowId, v_SDAP_Data)){} } //@siclog "Steps 8 - 9 siclog@ //The SS transmits DEACTIVATE TEST MODE message. //The UE transmits DEACTIVATE TEST MODE COMPLETE message. f_NR5GC_Deactivate_TestMode(nr_Cell3); //@siclog "Step 10 siclog@ //The SS transmits an RRCRelease message. f_NR_RRCRelease(nr_Cell3); }

After Change:

function fl_TC_11_3_11_TestBody() runs on NR5GC_PTC { var template (value) QosFlow_Identification_Type v_QosFlowId := f_NR_GetQosFlowIdentification_ForDRBId(f_NR_GetDefaultDRB_ForFirstPDUSession()); var SDAP_SDUList_Type v_SDAP_Data := { f_IPv4IPv6_IcmpEchoReply(f_LoopbackModeB_IP_Address_UE()) }; //Create the IP Data var template (value) RedirectedCarrierInfo v_RedirectedCarrierInfo; log("Bytecode #15 for 11.3.11 on 23wk49"); //@siclog "Step 1" siclog@ //The SS transmits one IP PDU. DRB.send(cas_NR_DRB_COMMON_REQ_SDAP_SDUList(nr_Cell1, v_QosFlowId, v_SDAP_Data)); //@siclog "Step 2" siclog@ //Wait for 1 second after the IP packet has been transmitted in step 1. f_Delay(1.0); //@siclog "Step 3" siclog@ //The SS transmits an RRCRelease message (IE redirectedCarrierInfo including NR Cell 3 and IE mpsPriorityIndication and suspendConfig). v_RedirectedCarrierInfo := cs_NR_redirectedCarrierInfo(f_NR_CellInfo_GetFrequencySSB(nr_Cell3), f_NR_CellInfo_GetSSB_SCS(nr_Cell3), f_NR_GetSSB_MTC_InterFreqOdd(nr_Cell1, nr_Cell3)); SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, cs_TimingInfo_Now, cs_38508_RRCRelease(tsc_NR_RRC_TI_Def, cs_38508_SuspendConfig(cs_RAN_NotificationAreaInfo_1Cell ( f_NR_CellInfo_GetCellIdentity(nr_Cell3), omit)), //WA#11_3_11 nr_cell1 --> nr_Cell3 v_RedirectedCarrierInfo, -, -, cs_RRCConnectionRelease_All_IEs_v1540(-, cs_RRCRelease_v1610_IEs(cs_RRCRelease_v1650_IEs(true_)))))); //Activate security at SS side in advance so that SS is ready to send RRCResume message // Only SRBs are configured - so security is activated on SRBs f_NR_SS_AS_ActivateSecurity_Current(nr_Cell3,tsc_NR_38508_NextHopChainingCount); //@siclog "Step 4" siclog@ SRB.receive(car_NR_SRB0_RrcPdu_IND(nr_Cell3, cr_38508_RRCResumeRequest(tsc_NR_ShortI_RNTI_Value1, mps_PriorityAccess))); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 4"); //@siclog "Step 5" siclog@ //The SS transmits an RRCResume message. f_NR_SendRRCResume_Def(nr_Cell3); // EXCEPTION: Steps 6 and 7 can happen in any order //@siclog "Step 6" siclog@ //The UE transmits an RRCResumeComplete message. //@siclog "Step 7 siclog@ //The UE loop back the IP PDU interleave { [] SRB.receive(car_NR_SRB1_RrcPdu_IND(nr_Cell3, cr_38508_RRCResumeComplete)){} [] DRB.receive(car_NR_DRB_COMMON_IND_SDAP_SDUList(nr_Cell3, v_QosFlowId, v_SDAP_Data)){} } //@siclog "Steps 8 - 9 siclog@ //The SS transmits DEACTIVATE TEST MODE message. //The UE transmits DEACTIVATE TEST MODE COMPLETE message. f_NR5GC_Deactivate_TestMode(nr_Cell3); //@siclog "Step 10 siclog@ //The SS transmits an RRCRelease message. //WA#11_3_11 COMMENTED OUT f_NR_RRCRelease(nr_Cell3); f_NR_UE_DeRegisterOnSwitchOff (nr_Cell3, STATE_CONNECTED_3A, tsc_NR_RbId_SRB2); //WA#11_3_11 Prose CR needed }

# Branches executed

## This NR5GC TC was executed in 5GS Stand Alone mode (Option 2) on NR Primary Band “n28” using NIA1 Integrity and NEA1 ciphering algorithms.

# Execution Log Files

## Hisilicon

The Hisilicon Balong 5000 UE  passed this test case on R&S® 5G Protocol Conformance Test platform. The documentation below is enclosed as evidence of the successful test case run [1]:

Test case execution log file:

R&S :
tc_11_3_11_Hisilicon.log

(Note: PICS/PIXIT settings are captured at the beginning of the TLI log)

In the log file the complete test case execution as well as the PICS/PIXIT parameter settings can be seen. All TLI events are presented and message contents are fully decoded and can be verified. Preliminary verdicts and the final test case verdict can be seen in the log file

# References

| [1] | R5s240245: Supporting information for addition of NR5GC testcase 11.3.11 in FR1 |
| --- | --- |