3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240248

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3392 | rev | - | Current version: | 17.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC testcase 6.1.1.5 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-03-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | This testcase needs the USIM configuration specified in 38.508-1 Table 6.4.1-6. As a result, the IMSI that is programmed in the USIM does not match the setting of px_IMSI_Def. This causes failures during ciphering and integrity procedures. This needs to be addressed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Called function f_NR5GC_MobileInfo_SetDefaultIMSI('003210123456063'H); before preamble, and also before Step 15a1-15a16a1 and Step 23a1-23a16a1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE can fail the test cases |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.1.5.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections required	2

2.1 Change 1	2

2.2 Change 2	4

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D23wk49 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required

## 2.1 Change 1

| Function name | f_TC_6_1_1_5_NR5GC |
| --- | --- |
| Reason for change | This testcase needs the USIM configuration specified in 38.508-1 Table 6.4.1-6. As a result, the IMSI that is programmed in the USIM does not match the setting of px_IMSI_Def. This causes failures during ciphering and integrity procedures. This needs to be addressed. |
| Summary of change | Called function f_NR5GC_MobileInfo_SetDefaultIMSI('003210123456063'H); before preamble. |
| TTCN module | Idle_PLMNSelection_NR5GC.ttcn |
| MCC160 Comments |  |

Before Change:

function f_TC_6_1_1_5_NR5GC() runs on NR5GC_PTC {// PLMN selection of RPLMN, HPLMN/EHPLMN, UPLMN and OPLMN / Automatic mode / User reselection f_NR5GC_Init(NR_1); // Set Sys info and cell info for cell 3& 13 to be used // @sic R5-203367 sic@ f_NR_CellInfo_SetPLMN(nr_Cell3, tsc_NR_HPLMN_003_21);//PLMN3 f_NR_CellInfo_SetPLMN(nr_Cell13, tsc_NR_HPLMN_004_31);//PLMN4 // Create and configure cell f_NR_CellConfig_Def(nr_Cell1); f_NR_CellConfig_Def(nr_Cell3); // @sic R5-203367 sic@ f_NR_CellConfig_Def(nr_Cell12); f_NR_CellConfig_Def(nr_Cell13); // Equip UE with the USIM configuration 6 specified in table 6.4.1-6 TS 38.508-1 f_UT_USIM_Insert(UT, "USIM configuration 6 specified in table 6.4.1-6 TS 38.508-1"); f_UT_AutomaticPLMN_Select_WithSwitchOnOff (UT); // Preamble: The UE is in state Registered, Idle Mode (state 1N-A) on NR Cell 1 according to TS 38.508-1 f_NR5GC_Preamble(nr_Cell1, STATE_IDLE_1A); f_NR_TestBody_Set(true); f_TC_6_1_1_5_NR5GC_TestBody(); f_NR_TestBody_Set(false); // Switch/Power off UE f_NR_Postamble(nr_Cell3, STATE_IDLE_1A); // @sic R5-203367 sic@ }

After Change:

## function f_TC_6_1_1_5_NR5GC() runs on NR5GC_PTC ## {// PLMN selection of RPLMN, HPLMN/EHPLMN, UPLMN and OPLMN / Automatic mode / User reselection ## f_NR5GC_Init(NR_1); ## // Set Sys info and cell info for cell 3& 13 to be used ## // @sic R5-203367 sic@ ## f_NR_CellInfo_SetPLMN(nr_Cell3, tsc_NR_HPLMN_003_21);//PLMN3 ## f_NR_CellInfo_SetPLMN(nr_Cell13, tsc_NR_HPLMN_004_31);//PLMN4 ## // Create and configure cell ## f_NR_CellConfig_Def(nr_Cell1); ## f_NR_CellConfig_Def(nr_Cell3); // @sic R5-203367 sic@ ## f_NR_CellConfig_Def(nr_Cell12); ## f_NR_CellConfig_Def(nr_Cell13); ## // Equip UE with the USIM configuration 6 specified in table 6.4.1-6 TS 38.508-1 f_UT_USIM_Insert(UT, "USIM configuration 6 specified in table 6.4.1-6 TS 38.508-1"); ## f_NR5GC_MobileInfo_SetDefaultIMSI('003210123456063'H); //WA#6_1_1_5 ## f_UT_AutomaticPLMN_Select_WithSwitchOnOff (UT); ## // Preamble: The UE is in state Registered, Idle Mode (state 1N-A) on NR Cell 1 according to TS 38.508-1 ## f_NR5GC_Preamble(nr_Cell1, STATE_IDLE_1A); ## f_NR_TestBody_Set(true); ## f_TC_6_1_1_5_NR5GC_TestBody(); ## f_NR_TestBody_Set(false); ## // Switch/Power off UE ## f_NR_Postamble(nr_Cell3, STATE_IDLE_1A); // @sic R5-203367 sic@ ## }

## 2.2 Change 2

| Function name | f_TC_6_1_1_5_NR5GC_TestBody |
| --- | --- |
| Reason for change | This testcase needs the USIM configuration specified in 38.508-1 Table 6.4.1-6. As a result, the IMSI that is programmed in the USIM does not match the setting of px_IMSI_Def. This causes failures during ciphering and integrity procedures. This needs to be addressed. |
| Summary of change | Called function f_NR5GC_MobileInfo_SetDefaultIMSI('003210123456063'H); before Step 15a1-15a16a1 and Step 23a1-23a16a1. |
| TTCN module | Idle_PLMNSelection_NR5GC.ttcn |
| MCC160 Comments |  |

Before Change:

function f_TC_6_1_1_5_NR5GC_TestBody() runs on NR5GC_PTC {// @sic R5-201127, R5s200165, R5-203367 sic@ var template (value) NR_CellPowerList_Type v_CellPowerList_AtT1; var template (value) NR_CellPowerList_Type v_CellPowerList_AtT2; var template (value) NR_CellPowerList_Type v_CellPowerList_AtT3; var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var float v_TimerValue1 := 90.0; v_CellPowerList_AtT1:={ cs_NR_CellPower(nr_Cell1, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2), cs_NR_CellPower(nr_Cell12, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2), cs_NR_CellPower(nr_Cell13, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2) }; v_CellPowerList_AtT2:={ cs_NR_CellPower(nr_Cell1, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE), cs_NR_CellPower(nr_Cell13, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE) }; v_CellPowerList_AtT3:={ cs_NR_CellPower(nr_Cell1, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2), cs_NR_CellPower(nr_Cell3, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2), cs_NR_CellPower(nr_Cell12, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE), cs_NR_CellPower(nr_Cell13, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2) }; // @siclog "Step 1" siclog@ // The SS changes the power level setting according to the row T1 in table 6.1.1.5.3.2-1/2 f_NR_SetCellPowerList(v_CellPowerList_AtT1); // @siclog "Step 2" siclog@ // Cause the UE in Automatic network selection mode to initiate user reselection and registration onto an available PLMN f_UT_TriggerUserReselection(UT); // @siclog "Step 3" siclog@ // Does the UE send an RRCSetupRequest on NR Cell 12? f_NR_RRC_SetupRequest_Def(nr_Cell12); f_NR_PreliminaryPass(__FILE__, __LINE__, "Test Case 6.1.1.5 Step 3"); // @siclog "Step 3A-3B" siclog@ // Steps 3-4 of Table 4.5.2.2-2 of the generic procedure in TS 38.508-1 [4] are performed f_NR_RRCSetup_Def (nr_Cell12); v_ReceivedMsg := f_NR_RRCSetupComplete_Def(nr_Cell12, cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def, ?), (tsc_SHT_NoSecurityProtection, tsc_SHT_IntegrityProtected)); if (f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Mobility)) { // @siclog "Step 4b1-4b2 Void" siclog@ // @siclog "Step 4b3-4b5a1" siclog@ // IF 5GS registration type is set as Mobility Registration in step 3B, THEN Steps 4 to 6a1 of the generic test procedure in TS 38.508-1 Table 4.9.5.2.2-1 are performed on NR Cell 12 f_NR5GC_MobilityRegistration_Steps4_6(nr_Cell12, v_GMM_MobilityInfo); } else if (f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Initial_Secure)) { // @siclog "Step 4a1-4a16a1" siclog@ // IF 5GS registration type is set as Initial Registration in step 3B, THEN Steps 5 to 20a1of the generic test procedure in TS 38.508-1 Table 4.5.2.2-2 are performed on NR Cell 12 f_NR5GC_MobileInfo_Init(); // @sic R5s230085 sic@ f_NR5GC_RRC_Idle_Steps5_20(v_GMM_MobilityInfo, nr_Cell12, v_ReceivedMsg, TESTMode_OFF, -, Initial_Secure); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } //@siclog "Step 5 - 8b1" siclog@ // Void // @siclog "Step 9" siclog@ // The SS changes the power level setting according to the row T2 in table 6.1.1.5.3.2-1/2 f_NR_SetCellPowerList(v_CellPowerList_AtT2); // @siclog "Step 10" siclog@ // Cause the UE in Automatic network selection mode to initiate user reselection and registration onto an available PLMN f_UT_TriggerUserReselection(UT); // @siclog "Step 11" siclog@ // Does the UE send an RRCSetupRequest on NR Cell 12 within 90 s? f_NR_CheckNoRRCSetupReq(nr_Cell12, v_TimerValue1, -, "Step 11: UE sent an RRCSetupRequest on NR Cell 12 within 90s"); //@sic R5s201387 sic@ // @siclog "Step 12" siclog@ // The SS changes the power level setting according to the row T1 in table 6.1.1.5.3.2-1/2. f_NR_SetCellPowerList(v_CellPowerList_AtT1); // @siclog "Step 13" siclog@ // Cause the UE in Automatic network selection mode to initiate user reselection and registration onto an available PLMN. f_UT_TriggerUserReselection(UT); // @siclog "Step 14" siclog@ // Does the UE send an RRCSetupRequest on NR Cell 1? f_NR_RRC_SetupRequest_Def(nr_Cell1); f_NR_PreliminaryPass(__FILE__, __LINE__, "Test Case 6.1.1.5 Step 14"); // @siclog "Step 14A-14B" siclog@ // Steps 3-4 of Table 4.5.2.2-2 of the generic procedure in TS 38.508-1 [4] are performed f_NR_RRCSetup_Def (nr_Cell1); v_ReceivedMsg := f_NR_RRCSetupComplete_Def(nr_Cell1, cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def, ?), (tsc_SHT_NoSecurityProtection, tsc_SHT_IntegrityProtected)); if (f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Mobility)) { // @siclog "Step 15b1-15b2 Void" siclog@ // @siclog "Step 15b3-15b5a1" siclog@ // IF 5GS registration type is set as Mobility Registration in step 14B, THEN Steps 4 to 6a1 of the generic test procedure in TS 38.508-1 Table 4.9.5.2.2-1 are performed on NR Cell 1 f_NR5GC_MobilityRegistration_Steps4_6(nr_Cell1, v_GMM_MobilityInfo); } else if (f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Initial_Secure)) { // @siclog "Step 15a1-15a16a1" siclog@ // IF 5GS registration type is set as Initial Registration in step 14B, THEN Steps 5 to 20a1of the generic test procedure in TS 38.508-1 Table 4.5.2.2-2 are performed on NR Cell 1 f_NR5GC_MobileInfo_Init(); // @sic R5s230085 sic@ f_NR5GC_RRC_Idle_Steps5_20(v_GMM_MobilityInfo, nr_Cell1, v_ReceivedMsg, TESTMode_OFF, -, Initial_Secure); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } //@siclog "Step 16 - 19b1" siclog@ // Void // @siclog "Step 20" siclog@ // The SS changes the power level setting according to the row T3 in table 6.1.1.5.3.2-1/2. f_NR_SetCellPowerList(v_CellPowerList_AtT3); // @siclog "Step 21" siclog@ // Cause the UE in Automatic network selection mode to initiate user reselection and registration onto an available PLMN. f_UT_TriggerUserReselection(UT); // @siclog "Step 22" siclog@ // Does the UE send an RRCSetupRequest on NR Cell 13? f_NR_RRC_SetupRequest_Def(nr_Cell3); f_NR_PreliminaryPass(__FILE__, __LINE__, "Test Case 6.1.1.5 Step 22"); // @siclog "Step 22A-22B" siclog@ // Steps 3-4 of Table 4.5.2.2-2 of the generic procedure in TS 38.508-1 [4] are performed f_NR_RRCSetup_Def (nr_Cell3); v_ReceivedMsg := f_NR_RRCSetupComplete_Def(nr_Cell3, cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def, ?), (tsc_SHT_NoSecurityProtection, tsc_SHT_IntegrityProtected)); if (f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Mobility)) { // @siclog "Step 23b1-23b2 Void" siclog@ // @siclog "Step 23b3-23b5a1" siclog@ // IF 5GS registration type is set as Mobility Registration in step 22B, THEN Steps 4 to 6a1 of the generic test procedure in TS 38.508-1 Table 4.9.5.2.2-1 are performed on NR Cell 3 f_NR5GC_MobilityRegistration_Steps4_6(nr_Cell3, v_GMM_MobilityInfo); } else if (f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Initial_Secure)) { // @siclog "Step 23a1-23a16a1" siclog@ // IF 5GS registration type is set as Initial Registration in step 22B, THEN Steps 5 to 20a1of the generic test procedure in TS 38.508-1 Table 4.5.2.2-2 are performed on NR Cell 13 f_NR5GC_MobileInfo_Init(); // @sic R5s230085 sic@ f_NR5GC_RRC_Idle_Steps5_20(v_GMM_MobilityInfo, nr_Cell3, v_ReceivedMsg, TESTMode_OFF, -, Initial_Secure); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } //@siclog "Step 24 - 27b1" siclog@ // Void }

After Change:

## function f_TC_6_1_1_5_NR5GC_TestBody() runs on NR5GC_PTC ## {// @sic R5-201127, R5s200165, R5-203367 sic@ ## var template (value) NR_CellPowerList_Type v_CellPowerList_AtT1; ## var template (value) NR_CellPowerList_Type v_CellPowerList_AtT2; ## var template (value) NR_CellPowerList_Type v_CellPowerList_AtT3; ## var NG_NAS_MSG_Indication_Type v_ReceivedMsg; ## var GMM_MobilityInfo_Type v_GMM_MobilityInfo; ## var float v_TimerValue1 := 90.0; ## v_CellPowerList_AtT1:={ ## cs_NR_CellPower(nr_Cell1, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2), ## cs_NR_CellPower(nr_Cell12, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2), ## cs_NR_CellPower(nr_Cell13, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2) ## }; ## v_CellPowerList_AtT2:={ ## cs_NR_CellPower(nr_Cell1, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE), ## cs_NR_CellPower(nr_Cell13, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE) ## }; ## v_CellPowerList_AtT3:={ ## cs_NR_CellPower(nr_Cell1, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2), ## cs_NR_CellPower(nr_Cell3, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2), ## cs_NR_CellPower(nr_Cell12, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE), ## cs_NR_CellPower(nr_Cell13, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2) ## }; ## // @siclog "Step 1" siclog@ ## // The SS changes the power level setting according to the row T1 in table 6.1.1.5.3.2-1/2 ## f_NR_SetCellPowerList(v_CellPowerList_AtT1); ## // @siclog "Step 2" siclog@ ## // Cause the UE in Automatic network selection mode to initiate user reselection and registration onto an available PLMN ## f_UT_TriggerUserReselection(UT); ## // @siclog "Step 3" siclog@ ## // Does the UE send an RRCSetupRequest on NR Cell 12? ## f_NR_RRC_SetupRequest_Def(nr_Cell12); ## f_NR_PreliminaryPass(__FILE__, __LINE__, "Test Case 6.1.1.5 Step 3"); ## // @siclog "Step 3A-3B" siclog@ ## // Steps 3-4 of Table 4.5.2.2-2 of the generic procedure in TS 38.508-1 [4] are performed ## f_NR_RRCSetup_Def (nr_Cell12); ## v_ReceivedMsg := f_NR_RRCSetupComplete_Def(nr_Cell12, ## cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def, ?), ## (tsc_SHT_NoSecurityProtection, tsc_SHT_IntegrityProtected)); ## if (f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Mobility)) { ## // @siclog "Step 4b1-4b2 Void" siclog@ ## // @siclog "Step 4b3-4b5a1" siclog@ ## // IF 5GS registration type is set as Mobility Registration in step 3B, THEN Steps 4 to 6a1 of the generic test procedure in TS 38.508-1 Table 4.9.5.2.2-1 are performed on NR Cell 12 ## f_NR5GC_MobilityRegistration_Steps4_6(nr_Cell12, v_GMM_MobilityInfo); ## } ## else if (f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Initial_Secure)) { ## // @siclog "Step 4a1-4a16a1" siclog@ ## // IF 5GS registration type is set as Initial Registration in step 3B, THEN Steps 5 to 20a1of the generic test procedure in TS 38.508-1 Table 4.5.2.2-2 are performed on NR Cell 12 ## f_NR5GC_MobileInfo_Init(); // @sic R5s230085 sic@ ## f_NR5GC_RRC_Idle_Steps5_20(v_GMM_MobilityInfo, nr_Cell12, v_ReceivedMsg, TESTMode_OFF, -, Initial_Secure); ## } ## else { ## f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); ## } ## //@siclog "Step 5 - 8b1" siclog@ ## // Void ## // @siclog "Step 9" siclog@ ## // The SS changes the power level setting according to the row T2 in table 6.1.1.5.3.2-1/2 ## f_NR_SetCellPowerList(v_CellPowerList_AtT2); ## // @siclog "Step 10" siclog@ ## // Cause the UE in Automatic network selection mode to initiate user reselection and registration onto an available PLMN ## f_UT_TriggerUserReselection(UT); ## // @siclog "Step 11" siclog@ ## // Does the UE send an RRCSetupRequest on NR Cell 12 within 90 s? ## f_NR_CheckNoRRCSetupReq(nr_Cell12, v_TimerValue1, -, "Step 11: UE sent an RRCSetupRequest on NR Cell 12 within 90s"); //@sic R5s201387 sic@ ## // @siclog "Step 12" siclog@ ## // The SS changes the power level setting according to the row T1 in table 6.1.1.5.3.2-1/2. ## f_NR_SetCellPowerList(v_CellPowerList_AtT1); ## // @siclog "Step 13" siclog@ ## // Cause the UE in Automatic network selection mode to initiate user reselection and registration onto an available PLMN. ## f_UT_TriggerUserReselection(UT); ## // @siclog "Step 14" siclog@ ## // Does the UE send an RRCSetupRequest on NR Cell 1? ## f_NR_RRC_SetupRequest_Def(nr_Cell1); ## f_NR_PreliminaryPass(__FILE__, __LINE__, "Test Case 6.1.1.5 Step 14"); ## // @siclog "Step 14A-14B" siclog@ ## // Steps 3-4 of Table 4.5.2.2-2 of the generic procedure in TS 38.508-1 [4] are performed ## f_NR_RRCSetup_Def (nr_Cell1); ## v_ReceivedMsg := f_NR_RRCSetupComplete_Def(nr_Cell1, ## cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def, ?), ## (tsc_SHT_NoSecurityProtection, tsc_SHT_IntegrityProtected)); ## if (f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Mobility)) { ## // @siclog "Step 15b1-15b2 Void" siclog@ ## // @siclog "Step 15b3-15b5a1" siclog@ ## // IF 5GS registration type is set as Mobility Registration in step 14B, THEN Steps 4 to 6a1 of the generic test procedure in TS 38.508-1 Table 4.9.5.2.2-1 are performed on NR Cell 1 ## f_NR5GC_MobilityRegistration_Steps4_6(nr_Cell1, v_GMM_MobilityInfo); ## } ## else if (f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Initial_Secure)) { ## // @siclog "Step 15a1-15a16a1" siclog@ ## // IF 5GS registration type is set as Initial Registration in step 14B, THEN Steps 5 to 20a1of the generic test procedure in TS 38.508-1 Table 4.5.2.2-2 are performed on NR Cell 1 ## f_NR5GC_MobileInfo_Init(); // @sic R5s230085 sic@ ## f_NR5GC_MobileInfo_SetDefaultIMSI('003210123456063'H); //WA#6_1_1_5 ## f_NR5GC_RRC_Idle_Steps5_20(v_GMM_MobilityInfo, nr_Cell1, v_ReceivedMsg, TESTMode_OFF, -, Initial_Secure); ## } ## else { ## f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); ## } ## //@siclog "Step 16 - 19b1" siclog@ ## // Void ## // @siclog "Step 20" siclog@ ## // The SS changes the power level setting according to the row T3 in table 6.1.1.5.3.2-1/2. ## f_NR_SetCellPowerList(v_CellPowerList_AtT3); ## // @siclog "Step 21" siclog@ ## // Cause the UE in Automatic network selection mode to initiate user reselection and registration onto an available PLMN. ## f_UT_TriggerUserReselection(UT); ## // @siclog "Step 22" siclog@ ## // Does the UE send an RRCSetupRequest on NR Cell 13? ## f_NR_RRC_SetupRequest_Def(nr_Cell3); ## f_NR_PreliminaryPass(__FILE__, __LINE__, "Test Case 6.1.1.5 Step 22"); ## // @siclog "Step 22A-22B" siclog@ ## // Steps 3-4 of Table 4.5.2.2-2 of the generic procedure in TS 38.508-1 [4] are performed ## f_NR_RRCSetup_Def (nr_Cell3); ## v_ReceivedMsg := f_NR_RRCSetupComplete_Def(nr_Cell3, ## cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def, ?), ## (tsc_SHT_NoSecurityProtection, tsc_SHT_IntegrityProtected)); ## if (f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Mobility)) { ## // @siclog "Step 23b1-23b2 Void" siclog@ ## // @siclog "Step 23b3-23b5a1" siclog@ ## // IF 5GS registration type is set as Mobility Registration in step 22B, THEN Steps 4 to 6a1 of the generic test procedure in TS 38.508-1 Table 4.9.5.2.2-1 are performed on NR Cell 3 ## f_NR5GC_MobilityRegistration_Steps4_6(nr_Cell3, v_GMM_MobilityInfo); ## } ## else if (f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Initial_Secure)) { ## // @siclog "Step 23a1-23a16a1" siclog@ ## // IF 5GS registration type is set as Initial Registration in step 22B, THEN Steps 5 to 20a1of the generic test procedure in TS 38.508-1 Table 4.5.2.2-2 are performed on NR Cell 13 ## f_NR5GC_MobileInfo_Init(); // @sic R5s230085 sic@ ## f_NR5GC_MobileInfo_SetDefaultIMSI('003210123456063'H); //WA#6_1_1_5 ## f_NR5GC_RRC_Idle_Steps5_20(v_GMM_MobilityInfo, nr_Cell3, v_ReceivedMsg, TESTMode_OFF, -, Initial_Secure); ## } ## else { ## f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); ## } ## //@siclog "Step 24 - 27b1" siclog@ ## // Void ## }