3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240255

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3396 | rev | - | Current version: | 17.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC MDT testcases 8.1.6.1.2.x |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, NR_SON_MDT-UEConTest |  |  |  |  |  | Date: |  |  | 2023-03-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the current implementation of R5-237358 and R5-237359, the placement of ifpresent is incorrect and is referring to cr_RRCSetupComplete_v1610_IEs instead of cr_UE_MeasurementsAvailable_r16. This needs to be corrected. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Move ifpresent one level inside to correctly refer to cr_UE_MeasurementsAvailable_r16. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.6.1.2.4.NR5GC, 8.1.6.1.2.5.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR … CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	5

3	References	8

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2022-09_D23wk49 related to the title of this CR.

Contact:                      Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required

## Change 1

| Function name | function f_TC_8_1_6_1_2_4_TestBody |
| --- | --- |
| Reason for change | In the current implementation of R5-237358 and R5-237359, the placement of ifpresent is incorrect and is referring to cr_RRCSetupComplete_v1610_IEs instead of cr_UE_MeasurementsAvailable_r16. This needs to be corrected. |
| Summary of change | Move ifpresent one level inside to correctly refer to cr_UE_MeasurementsAvailable_r16. |
| TTCN module | RRC_MDT_IntraNR_Logged_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change

function f_TC_8_1_6_1_2_4_TestBody() runs on NR5GC_PTC { // @sic R5-216300, R5-220331, R5-222118, R5-223496 sic@ var template (value) NR_CellPowerList_Type v_CellPowerList_AtT0; var template (value) NR_CellPowerList_Type v_CellPowerList_AtT1; var PLMN_Identity v_PlmnID_NRCell1; var PLMN_Identity v_PlmnID_NRCell11; var TrackingAreaCode v_TAC_NRCell11; var B48_Type v_AbsoluteTimeInfo; var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var NG_NAS_GutiParameters_Type v_GutiParams ; var template(present) NG_MobileIdentity v_S_TMSI; var template(present) LogMeasInfo_r16 v_LogMeasInfo_r16; var CellIdentity v_CellIdentity_NRCell11; var PhysCellId v_PhysCellId_NRCell1; var ARFCN_ValueNR v_CarrierFreq_NRCell1; var NR_SRB_COMMON_IND v_ReceivedAsp; var template (present) MeasQuantityResults v_MeasQuantityResults; var integer v_FieldCnt; var integer i; v_CellPowerList_AtT0 := {cs_NR_CellPower(nr_Cell1, -78, -82), cs_NR_CellPower(nr_Cell11, -85, -91)}; v_CellPowerList_AtT1 := {cs_NR_CellPower(nr_Cell1, -85, -91), cs_NR_CellPower(nr_Cell11, -78, -82)}; f_NR_SetCellPowerList(v_CellPowerList_AtT0); //@siclog "Steps 1" siclog@ //SS transmits a LoggedMeasurementConfiguration message with reportType is set to 'periodical' and LoggingInterval is set to 'ms10240', to configure the UE to perform logging of measurement results while in RRC_IDLE v_PlmnID_NRCell1 := f_NR_CellInfo_GetPLMN(nr_Cell1); v_AbsoluteTimeInfo := f_MDT_GetAbsolute_TimeStamp(); SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, -, cs_38508_LoggedMeasurementConfiguration(v_PlmnID_NRCell1, v_AbsoluteTimeInfo, cs_LoggedMeasurementConfiguration_reportType_Periodical(ms10240)))); //@siclog "Steps 2" siclog@ //The SS transmits an RRCRelease message f_NR_RRCRelease(nr_Cell1); //@siclog "Steps 3" siclog@ //Wait 5s and the LoggingInterval will not be expired f_Delay(5.0); //@siclog "Steps 4" siclog@ //The SS changes NR Cell 1 and NR Cell 11 levels according to the row "T1" in table 8.1.6.1.2.4.3.2-1/2 f_NR_SetCellPowerList(v_CellPowerList_AtT1); //@siclog "Steps 5" siclog@ //The UE transmits an RRCSetupRequest message on NR Cell 11 to initiate a tracking area update procedure //@siclog "Steps 6" siclog@ //SS transmits an RRCSetup message //@siclog "Steps 7" siclog@ //Check: Does the UE include the IE logMeasAvailable in the RRCSetupComplete message? v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas(nr_Cell11,tsc_NR_RRC_TI_Def,?,-,-,-,-,cr_RRCSetupComplete_v1610_IEs(*,*,cr_UE_MeasurementsAvailable_r16(omit)) ifpresent); // @sic R5s220072, R5s220812, R5s230791, R5-237358 sic@ if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Mobility)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } *** SKIPPED CODE ***

After Change

function f_TC_8_1_6_1_2_4_TestBody() runs on NR5GC_PTC { // @sic R5-216300, R5-220331, R5-222118, R5-223496 sic@ var template (value) NR_CellPowerList_Type v_CellPowerList_AtT0; var template (value) NR_CellPowerList_Type v_CellPowerList_AtT1; var PLMN_Identity v_PlmnID_NRCell1; var PLMN_Identity v_PlmnID_NRCell11; var TrackingAreaCode v_TAC_NRCell11; var B48_Type v_AbsoluteTimeInfo; var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var NG_NAS_GutiParameters_Type v_GutiParams ; var template(present) NG_MobileIdentity v_S_TMSI; var template(present) LogMeasInfo_r16 v_LogMeasInfo_r16; var CellIdentity v_CellIdentity_NRCell11; var PhysCellId v_PhysCellId_NRCell1; var ARFCN_ValueNR v_CarrierFreq_NRCell1; var NR_SRB_COMMON_IND v_ReceivedAsp; var template (present) MeasQuantityResults v_MeasQuantityResults; var integer v_FieldCnt; var integer i; v_CellPowerList_AtT0 := {cs_NR_CellPower(nr_Cell1, -78, -82), cs_NR_CellPower(nr_Cell11, -85, -91)}; v_CellPowerList_AtT1 := {cs_NR_CellPower(nr_Cell1, -85, -91), cs_NR_CellPower(nr_Cell11, -78, -82)}; f_NR_SetCellPowerList(v_CellPowerList_AtT0); //@siclog "Steps 1" siclog@ //SS transmits a LoggedMeasurementConfiguration message with reportType is set to 'periodical' and LoggingInterval is set to 'ms10240', to configure the UE to perform logging of measurement results while in RRC_IDLE v_PlmnID_NRCell1 := f_NR_CellInfo_GetPLMN(nr_Cell1); v_AbsoluteTimeInfo := f_MDT_GetAbsolute_TimeStamp(); SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, -, cs_38508_LoggedMeasurementConfiguration(v_PlmnID_NRCell1, v_AbsoluteTimeInfo, cs_LoggedMeasurementConfiguration_reportType_Periodical(ms10240)))); //@siclog "Steps 2" siclog@ //The SS transmits an RRCRelease message f_NR_RRCRelease(nr_Cell1); //@siclog "Steps 3" siclog@ //Wait 5s and the LoggingInterval will not be expired f_Delay(5.0); //@siclog "Steps 4" siclog@ //The SS changes NR Cell 1 and NR Cell 11 levels according to the row "T1" in table 8.1.6.1.2.4.3.2-1/2 f_NR_SetCellPowerList(v_CellPowerList_AtT1); //@siclog "Steps 5" siclog@ //The UE transmits an RRCSetupRequest message on NR Cell 11 to initiate a tracking area update procedure //@siclog "Steps 6" siclog@ //SS transmits an RRCSetup message //@siclog "Steps 7" siclog@ //Check: Does the UE include the IE logMeasAvailable in the RRCSetupComplete message? v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas(nr_Cell11,tsc_NR_RRC_TI_Def,?,-,-,-,-,cr_RRCSetupComplete_v1610_IEs(*,*,cr_UE_MeasurementsAvailable_r16(omit) ifpresent)); // @sic R5s220072, R5s220812, R5s230791, R5-237358 sic@ if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Mobility)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } *** SKIPPED CODE ***

## Change 2

| Function name | function f_TC_8_1_6_1_2_5_TestBody |
| --- | --- |
| Reason for change | Same as Change 1 |
| Summary of change | Move ifpresent one level inside to correctly refer to cr_UE_MeasurementsAvailable_r16. |
| TTCN module | RRC_MDT_IntraNR_Logged_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change

function f_TC_8_1_6_1_2_5_TestBody() runs on NR5GC_PTC { var PLMN_Identity v_PLMN_Identity; var B48_Type v_AbsoluteTimeInfo; var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var NG_NAS_GutiParameters_Type v_GutiParams ; var template(present) NG_MobileIdentity v_S_TMSI; var template(present) LogMeasInfo_r16 v_LogMeasInfo_r16; var CellIdentity v_CellIdentity; var TrackingAreaCode v_TAC; var NR_SRB_COMMON_IND v_ReceivedAsp; var template (present) MeasQuantityResults v_MeasQuantityResults; var integer v_FieldCnt; var integer i; var RSRP_Range v_Thres; var DeltaValues_Type v_Deltas; v_Deltas := f_NR_MobileInfo_GetPrimaryDeltaValue(); if (f_NR_CellInfo_GetIsFR1(nr_Cell1)){ v_Thres := 69; // @sic R5-216301 sic@ } else { v_Thres := 65 + v_Deltas.DeltaNRf1; // @sic R5-232393 sic@ } //@siclog "Steps 1" siclog@ //SS transmits a LoggedMeasurementConfiguration message with reportType is set to eventTriggered and eventType is set to eventL1, to configure the UE to perform logging of measurement results while in RRC_IDLE v_PLMN_Identity := f_NR_CellInfo_GetPLMN(nr_Cell1); v_AbsoluteTimeInfo := f_MDT_GetAbsolute_TimeStamp(); SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, -, cs_38508_LoggedMeasurementConfiguration(v_PLMN_Identity, v_AbsoluteTimeInfo, cs_LoggedMeasurementConfiguration_reportType_EventL1(v_Thres)))); //@siclog "Steps 2" siclog@ //The SS transmits an RRCRelease message f_NR_RRCRelease(nr_Cell1); //@siclog "Steps 3" siclog@ //Wait 30s to allow UE to activate logging f_Delay(30.0); //@siclog "Steps 4-6" siclog@ //Steps 1 to 3 of the generic procedure in TS 38.508-1 Table 4.5.4.2-3 are executed to successfully complete the service request procedure //@siclog "Steps 7" siclog@ //Check: Does the UE include the IE logMeasAvailable in the RRCSetupComplete message? f_NR_Paging_Def(nr_Cell1, RRC_IDLE); v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas(nr_Cell1,tsc_NR_RRC_TI_Def,?,-,-,-,-,cr_RRCSetupComplete_v1610_IEs(*,*,cr_UE_MeasurementsAvailable_r16(omit)) ifpresent); // @sic R5s220073, R5-223496, R5s220812, R5s230791, R5-237359 sic@ *** SKIPPED CODE ***

After Change

function f_TC_8_1_6_1_2_5_NR5GC() runs on NR5GC_PTC { //Logged MDT/ RRC_IDLE / Logging and reporting / event-based trigger f_NR5GC_Init(NR_1); f_NR_CellInfo_InitMaxReferencePower(nr_Cell1, -85, tsc_NR_ServingCellSSS_EPRE_FR2); // @sic R5-232393 sic@ //Create and configure cells f_NR_CellConfig_Def(nr_Cell1); f_NR5GC_Preamble(nr_Cell1, STATE_CONNECTED_3A); f_NR_SetCellPower(nr_Cell1, -85, tsc_NR_ServingCellSSS_EPRE_FR2); // @sic R5-232393 sic@ f_NR_TestBody_Set(true); f_TC_8_1_6_1_2_5_TestBody(); f_NR_TestBody_Set(false); f_NR_Postamble(nr_Cell1, STATE_CONNECTED_3A); } // End of function f_TC_8_1_6_1_2_5_NR5GC //---------------------------------------------------------------------------- /* * @desc REFERENCE TS 38.523-1 clause 8.1.6.1.2.5 * @status APPROVED (NR5GC) */ function f_TC_8_1_6_1_2_5_TestBody() runs on NR5GC_PTC { var PLMN_Identity v_PLMN_Identity; var B48_Type v_AbsoluteTimeInfo; var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var NG_NAS_GutiParameters_Type v_GutiParams ; var template(present) NG_MobileIdentity v_S_TMSI; var template(present) LogMeasInfo_r16 v_LogMeasInfo_r16; var CellIdentity v_CellIdentity; var TrackingAreaCode v_TAC; var NR_SRB_COMMON_IND v_ReceivedAsp; var template (present) MeasQuantityResults v_MeasQuantityResults; var integer v_FieldCnt; var integer i; var RSRP_Range v_Thres; var DeltaValues_Type v_Deltas; v_Deltas := f_NR_MobileInfo_GetPrimaryDeltaValue(); if (f_NR_CellInfo_GetIsFR1(nr_Cell1)){ v_Thres := 69; // @sic R5-216301 sic@ } else { v_Thres := 65 + v_Deltas.DeltaNRf1; // @sic R5-232393 sic@ } //@siclog "Steps 1" siclog@ //SS transmits a LoggedMeasurementConfiguration message with reportType is set to eventTriggered and eventType is set to eventL1, to configure the UE to perform logging of measurement results while in RRC_IDLE v_PLMN_Identity := f_NR_CellInfo_GetPLMN(nr_Cell1); v_AbsoluteTimeInfo := f_MDT_GetAbsolute_TimeStamp(); SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, -, cs_38508_LoggedMeasurementConfiguration(v_PLMN_Identity, v_AbsoluteTimeInfo, cs_LoggedMeasurementConfiguration_reportType_EventL1(v_Thres)))); //@siclog "Steps 2" siclog@ //The SS transmits an RRCRelease message f_NR_RRCRelease(nr_Cell1); //@siclog "Steps 3" siclog@ //Wait 30s to allow UE to activate logging f_Delay(30.0); //@siclog "Steps 4-6" siclog@ //Steps 1 to 3 of the generic procedure in TS 38.508-1 Table 4.5.4.2-3 are executed to successfully complete the service request procedure //@siclog "Steps 7" siclog@ //Check: Does the UE include the IE logMeasAvailable in the RRCSetupComplete message? f_NR_Paging_Def(nr_Cell1, RRC_IDLE); v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas(nr_Cell1,tsc_NR_RRC_TI_Def,?,-,-,-,-,cr_RRCSetupComplete_v1610_IEs(*,*,cr_UE_MeasurementsAvailable_r16(omit) ifpresent)); // @sic R5s220073, R5-223496, R5s220812, R5s230791, R5-237359 sic@ *** SKIPPED CODE ***

# References

| [1] | R5-237358: Correction to NR5GC MDT testcase 8.1.6.1.2.4 |
| --- | --- |
| [2] | R5-237359: Correction to NR5GC MDT testcase 8.1.6.1.2.5 |