3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240263

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4800 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NBIOT testcase 22.5.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI13_Test, NB_IOT-UEConTest |  |  |  |  |  | Date: |  |  | 2024-03-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to the Chapter 5.4.2.4 in 24.301, the value of eKSI should be different for new authentication procedure. In preamble procedure, there is already do the first authentication procedure.So at step 1, the value of eKSI in AUTHENTICATION REQUEST should not be reused. |  |  |  |  |  |  |  |  |
| Summary of change: |  | At step1, increase the eKSI value in AUTHENTICATION REQUEST |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformant UE may failed the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 22.5.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  |  | 36.523-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | A Prose CR will be raised for this. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2022-09_D23wk49 related to the title of this CR.

Contact:  Bhise Parikshit
parikshit.bhise@rohde-schwarz.com

# Corrections required

## Change 1

| Function name | fl_TC_22_5_1_Body() |
| --- | --- |
| Reason for change | According to the Chapter 5.4.2.4 in 24.301, the value of eKSI should be different for new authentication procedure. In preamble procedure, there is already do the first authentication procedure.So at step 1, the value of eKSI in AUTHENTICATION REQUEST should not be reused. |
| Summary of change | At step1, increase the eKSI value in AUTHENTICATION REQUEST |
| TTCN module | NBIOT_NAS_Authentication_Security.ttcn |

function fl_TC_22_5_1_Body() runs on NBIOT_PTC { var float v_NoReactionTime := 30.0; var GutiParameters_Type v_GutiParams := f_NBIOT_CellInfo_GetGuti(nbiot_Cell1); var NAS_PlmnId v_PLMN := f_Asn2Nas_PlmnId(v_GutiParams.PLMN_Identity); var NBIOT_SecurityParams_Type v_SecurityParams := f_NBIOT_Security_Get(); var B32_128_Type v_ReceivedRES; var NB_SRB_COMMON_IND v_ReceivedAsp; var NBIOT_SecurityParams_Type v_SecurityParams_MACerror; var float v_T3418Val := f_NBIOT_SetTimerToleranceMax(nasTimer, 260.0 ); // @sic R5s180465 sic@ var template (value) NB_CellPowerList_Type v_CellPowerList; timer t_Wait; //@siclog "Step 1" siclog@ //The SS transmits an AUTHENTICATION REQUEST message. SRB.send(cas_NB_SRB1bis_NasPdu_REQ(nbiot_Cell1, cs_TimingInfo_Now, cs_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, cs_AUTHENTICATION_REQUEST(v_SecurityParams.KSIasme, v_SecurityParams.AuthParams.RandValue, v_SecurityParams.AuthParams.AUTN)))); //@siclog "Step 2" siclog@ …………………… }

After Change:

function fl_TC_22_5_1_Body() runs on NBIOT_PTC { var float v_NoReactionTime := 30.0; var GutiParameters_Type v_GutiParams := f_NBIOT_CellInfo_GetGuti(nbiot_Cell1); var NAS_PlmnId v_PLMN := f_Asn2Nas_PlmnId(v_GutiParams.PLMN_Identity); var NBIOT_SecurityParams_Type v_SecurityParams := f_NBIOT_Security_Get(); var B32_128_Type v_ReceivedRES; var NB_SRB_COMMON_IND v_ReceivedAsp; var NBIOT_SecurityParams_Type v_SecurityParams_MACerror; var float v_T3418Val := f_NBIOT_SetTimerToleranceMax(nasTimer, 260.0 ); // @sic R5s180465 sic@ var template (value) NB_CellPowerList_Type v_CellPowerList; timer t_Wait; v_SecurityParams := f_NBIOT_Authentication_InitNAS(v_SecurityParams, v_PLMN); //WI=1263632 //@siclog "Step 1" siclog@ //The SS transmits an AUTHENTICATION REQUEST message. SRB.send(cas_NB_SRB1bis_NasPdu_REQ(nbiot_Cell1, cs_TimingInfo_Now, cs_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, cs_AUTHENTICATION_REQUEST(v_SecurityParams.KSIasme, v_SecurityParams.AuthParams.RandValue, v_SecurityParams.AuthParams.AUTN)))); //@siclog "Step 2" siclog@ ………………………… }