3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240270

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4803 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NBIOT testcase 22.2.4 (for NTN) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI13_Test, NB_IOT-UEConTest |  |  |  |  |  | Date: |  |  | 2024-03-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - According to the Table 8.1.4.2-6 in the 36.508, the TAC of Ncell3 should be set to 3. - For the NB-IoT NTN test, the trackingAreaList-r17 in plmn-IdentityList_v1700 should be configured follow the configuration of trackingAreaCode-r13. The value of NtnCellType should be configured to identify the different NCell with index i, According to current implementation of TTCN the value of NtnCellType is 1 for all different Ncells |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Set TAC of Ncell3 to 3. - Add the configuration of trackingAreaList-r17 Configure the NtnCellType with index i. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformant UE may failed the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 22.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	4

2.3	Change 3	4

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2022-09_D24wk12 related to the title of this CR.

Contact:  Bhise Parikshit
parikshit.bhise@rohde-schwarz.com

# Corrections required

## Change 1

| Function name | Function f_TC_22_2_4_NBIOT () |
| --- | --- |
| Reason for change | According to the Table 8.1.4.2-6 in the 36.508, the TAC of Ncell3 should be set to 3. |
| Summary of change | Set TAC of Ncell3 to 3. |
| TTCN module | \NBIOT\22_2\NBIOT_Idle_CellSelection.ttcn |

Before Change:

function f_TC_22_1_1_Module3() runs on NBIOT_PTC { ……………………… //Initialise all Ncells, security and mobile parameters f_NBIOT_Init(c2,-,-,-,{nbiot_Cell2, nbiot_Cell3}); // @sic R5-237126 sic@ //Set TAC according to 36.508 Table 8.1.4.2-6: Tracking Area Code (TAC) for NBIOT cells */ f_NBIOT_CellInfo_SetTAC(nbiot_Cell2, tsc_Idle_TAC_Cell2); //Set maximum cell power level for NCell 1 to be used in creation f_NBIOT_CellInfo_InitMaxReferencePower (nbiot_Cell1, -65); //Create and configure NCell 1 & 2 & 3 f_NBIOT_CellConfig_Def(nbiot_Cell1, CONTROL_PLANE); f_NBIOT_CellConfig_Def(nbiot_Cell2, CONTROL_PLANE); f_NBIOT_CellConfig_Def(nbiot_Cell3, CONTROL_PLANE); f_NBIOT_ConfigureVNG (nbiot_Cell1, -75); f_NBIOT_ConfigureVNG (nbiot_Cell3, -75); …………………… }

After Change:

function f_TC_22_1_1_Module3() runs on NBIOT_PTC { ……………………… //Initialise all Ncells, security and mobile parameters f_NBIOT_Init(c2,-,-,-,{nbiot_Cell2, nbiot_Cell3}); // @sic R5-237126 sic@ //Set TAC according to 36.508 Table 8.1.4.2-6: Tracking Area Code (TAC) for NBIOT cells */ f_NBIOT_CellInfo_SetTAC(nbiot_Cell2, tsc_Idle_TAC_Cell2); //WI=1264987 f_NBIOT_CellInfo_SetTAC(nbiot_Cell3, tsc_Idle_TAC_Cell3); //Set maximum cell power level for NCell 1 to be used in creation f_NBIOT_CellInfo_InitMaxReferencePower (nbiot_Cell1, -65); //Create and configure NCell 1 & 2 & 3 f_NBIOT_CellConfig_Def(nbiot_Cell1, CONTROL_PLANE); f_NBIOT_CellConfig_Def(nbiot_Cell2, CONTROL_PLANE); f_NBIOT_CellConfig_Def(nbiot_Cell3, CONTROL_PLANE); f_NBIOT_ConfigureVNG (nbiot_Cell1, -75); f_NBIOT_ConfigureVNG (nbiot_Cell3, -75); ………………………………… }

## Change 2

| Function name | Function f_NBIOT_CellInfo_SetTAC () |
| --- | --- |
| Reason for change | For the NB-IoT NTN test, the trackingAreaList-r17 in plmn-IdentityList_v1700 should be configured follow the configuration of trackingAreaCode-r13 |
| Summary of change | Add the configuration of trackingAreaList-r17. |
| TTCN module | \NBIOT\common\NBIOT_CellInfo.ttcn |

Before Change:

function f_NBIOT_CellInfo_SetTAC(NBIOT_CellId_Type p_CellId, EUTRA_ASN1_TrackingAreaCode_Type p_TAC) runs on NBIOT_PTC { var NBIOT_CellInfo_Type v_CellInfo := f_NBIOT_CellInfo_Get(p_CellId); v_CellInfo.NAS_Parameters.TrackingAreaCode := p_TAC; v_CellInfo.Sysinfo.BCCH_Info.SIB1_NB.message_.c1.systemInformationBlockType1_r13.cellAccessRelatedInfo_r13.trackingAreaCode_r13 := p_TAC; f_NBIOT_CellInfo_Set(p_CellId, v_CellInfo); }

After Change:

function f_NBIOT_CellInfo_SetTAC(NBIOT_CellId_Type p_CellId, EUTRA_ASN1_TrackingAreaCode_Type p_TAC) runs on NBIOT_PTC { var NBIOT_CellInfo_Type v_CellInfo := f_NBIOT_CellInfo_Get(p_CellId); v_CellInfo.NAS_Parameters.TrackingAreaCode := p_TAC; v_CellInfo.Sysinfo.BCCH_Info.SIB1_NB.message_.c1.systemInformationBlockType1_r13.cellAccessRelatedInfo_r13.trackingAreaCode_r13 := p_TAC; if(pc_NB_ntn_Connectivity_EPC or pc_NB_ntn_only_Connectivity_EPC){ //WA#WI=1266115 update TAC in r17 path for NTN case) v_CellInfo.Sysinfo.BCCH_Info.SIB1_NB.message_.c1.systemInformationBlockType1_r13.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.cellAccessRelatedInfo_NTN_r17.plmn_IdentityList_v1700[0].trackingAreaList_r17:={p_TAC}; } f_NBIOT_CellInfo_Set(p_CellId, v_CellInfo); }

## Change 3

| Function name | Function fl_NBIOT_Common_Init () |
| --- | --- |
| Reason for change | The value of NtnCellType should be configured to identify the different NCell with index i, According to current implementation of TTCN the value of NtnCellType is 1 for all different Ncells. |
| Summary of change | Configure the NtnCellType with index i. |
| TTCN module | \NBIOT\common\NBIOT_CellInfo.ttcn |

Before Change:

function fl_NBIOT_Common_Init(NB_InitParams_Local_Type p_Params, boolean p_DoAttachWithoutPDN, template (omit)NBIOT_CellId_List_Type p_Neighbour_NTN_Cells := omit) runs on NBIOT_PTC { var integer i; var integer v_Index; vc_NBIOT_Global.Ctrl.Default := null; vc_NBIOT_Global.Ctrl.TestBody := false; vc_NBIOT_Global.Ctrl.VerdictCtrl := normal; vc_NBIOT_Global.Ctrl.HarqErrorHandling := ignore; vc_NBIOT_Global.Ctrl.LoopbackIsActivated := false; // Initialise security parameters vc_NBIOT_Global.Security := f_NBIOT_Security_Init(); // Initialise Cell array vc_NBIOT_Global.CellArray := f_NBIOT_CellArray_Init(p_Params.SysinfoCombination, p_DoAttachWithoutPDN); //@sic R5-235468, R5s230706 sic@ vc_NBIOT_Global.CellArray := f_NBIOT_NAS_CellArray_Init(vc_NBIOT_Global.CellArray, p_Params.CellsOnSamePLMN, p_Params.SysinfoCombination, p_DoAttachWithoutPDN, p_Params.CellSpecialFreq); if(ispresent(p_Neighbour_NTN_Cells) and f_NBIOT_MobileInfo_GetNTNSatelliteType() != ntn_None) {//@sic R5-235468, R5s230706, R5-237126 sic@ for(i:=0; i<lengthof(p_Neighbour_NTN_Cells); i:=i+1) { v_Index := f_NBIOT_CellInfo_GetIndex(valueof(p_Neighbour_NTN_Cells[i])); f_NBIOT_CellInfo_SetSIB31(valueof(p_Neighbour_NTN_Cells[i]), cs_SystemInformationBlockType31_NB_r17_GSO_Array[i+1]); vc_NBIOT_Global.CellArray[v_Index].NtnInfo.NtnCellType := vc_NBIOT_Global.CellArray[v_Index].NtnInfo.NtnCellType+1; //@sic R5-237126 sic@ } } //Initialise Mobile Info vc_NBIOT_Global.MobileInfo.Tmsi := '00000000'O; vc_NBIOT_Global.MobileInfo.P_Tmsi := '00000000'O; vc_NBIOT_Global.MobileInfo.PDNs := omit; f_NBIOT_ActivateDefault(StandardDefault); }

After Change:

function fl_NBIOT_Common_Init(NB_InitParams_Local_Type p_Params, boolean p_DoAttachWithoutPDN, template (omit)NBIOT_CellId_List_Type p_Neighbour_NTN_Cells := omit) runs on NBIOT_PTC { var integer i; var integer v_Index; vc_NBIOT_Global.Ctrl.Default := null; vc_NBIOT_Global.Ctrl.TestBody := false; vc_NBIOT_Global.Ctrl.VerdictCtrl := normal; vc_NBIOT_Global.Ctrl.HarqErrorHandling := ignore; vc_NBIOT_Global.Ctrl.LoopbackIsActivated := false; // Initialise security parameters vc_NBIOT_Global.Security := f_NBIOT_Security_Init(); // Initialise Cell array vc_NBIOT_Global.CellArray := f_NBIOT_CellArray_Init(p_Params.SysinfoCombination, p_DoAttachWithoutPDN); //@sic R5-235468, R5s230706 sic@ vc_NBIOT_Global.CellArray := f_NBIOT_NAS_CellArray_Init(vc_NBIOT_Global.CellArray, p_Params.CellsOnSamePLMN, p_Params.SysinfoCombination, p_DoAttachWithoutPDN, p_Params.CellSpecialFreq); if(ispresent(p_Neighbour_NTN_Cells) and f_NBIOT_MobileInfo_GetNTNSatelliteType() != ntn_None) {//@sic R5-235468, R5s230706, R5-237126 sic@ for(i:=0; i<lengthof(p_Neighbour_NTN_Cells); i:=i+1) { v_Index := f_NBIOT_CellInfo_GetIndex(valueof(p_Neighbour_NTN_Cells[i])); f_NBIOT_CellInfo_SetSIB31(valueof(p_Neighbour_NTN_Cells[i]), cs_SystemInformationBlockType31_NB_r17_GSO_Array[i+1]); vc_NBIOT_Global.CellArray[v_Index].NtnInfo.NtnCellType := vc_NBIOT_Global.CellArray[v_Index].NtnInfo.NtnCellType+i+1; //@sic R5-237126 sic@ //wi=1264987 } } //Initialise Mobile Info vc_NBIOT_Global.MobileInfo.Tmsi := '00000000'O; vc_NBIOT_Global.MobileInfo.P_Tmsi := '00000000'O; vc_NBIOT_Global.MobileInfo.PDNs := omit; f_NBIOT_ActivateDefault(StandardDefault); }