3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240279

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-5 | CR | 0108 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections for MCX registration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160, UPV/EHU |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test, MCImp-UEConTest |  |  |  |  |  | Date: |  |  | 2024-03-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Due to changes in TTCN following the prose CRs RAN5#102 errors have been introduced which fail the test cases. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Errors corrected |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test cases fail with runtime error |  |  |  |  |  |  |  |  |
| Clauses affected: |  | f_MCX_SIP_Registration, f_MCX_IMS_PTC_SecurityParameters_GetKey, f_MCX_SecurityParameters_GetKey, f_MCX_SecurityParameters_GetOptionalKey (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2022-09_D24wk12 related to the title of this CR.

Contact:	Olivier Genoud
	olivier.genoud@etsi.org

# Corrections required

## 2.1 Correction of f_MCX_SIP_Registration

| Function name | f_MCX_SIP_Registration |
| --- | --- |
| Reason for change | Message body of SIP REGISTER needs to be optional |
| Summary of change | Message body made optional when calling cr_REGISTER_Request |
| TTCN module | MCX_SIP_Registration |

function f_MCX_SIP_Registration(MCX_ServiceType_Type p_ServiceType) runs on MCX_IMS_PTC return IMS_DATA_REQ { var IMS_DATA_REQ v_IMS_DATA_REQ_ByRef; var IMS_DATA_REQ v_IMS_DATA_REQ; var REGISTER_Request v_RegisterReq; var MIME_Message v_MIME_Message; var template (present) SemicolonParam_List v_MandatoryContactParams := f_MCX_SIP_Register_ContactParams_RX(p_ServiceType); var template (present) MCX_ServiceInfo_Type v_MCS_Info; var MCX_MIME_XmlBodies_Type v_XmlBodies; // Step 1 v_IMS_DATA_REQ := f_IMS_REGISTER_InitialRequest(-, -, v_MandatoryContactParams); // check REGISTER and init GlobalInfo v_RegisterReq := v_IMS_DATA_REQ.Request.Register; f_IMS_Register_SecurityInit_ImsSecurity(v_RegisterReq); // Step 2 IMS_Server.send(cas_IMS_DATA_RSP(f_IMS_RoutingInfo_ULtoDL(v_IMS_DATA_REQ.RoutingInfo), cs_Response(c_statusLine401, f_IMS_RegisterResponse_401_MessageHeaderTX(v_RegisterReq)))); // Step 3a1, 3b1 a_IMS_ReceiveRequest(car_IMS_Register_Request(cr_REGISTER_Request(?, ?, *)), v_IMS_DATA_REQ_ByRef); v_IMS_DATA_REQ := f_IMS_REGISTER_SubsequentRequest(v_IMS_DATA_REQ_ByRef, v_MandatoryContactParams); v_RegisterReq := v_IMS_DATA_REQ.Request.Register; if (isvalue(v_IMS_DATA_REQ.Request.Register.messageBody.mimeMessageBody)) { /* @sic R5-240555: Step 3a1 with SIP REGISTER (access token, CSK) sic@ */ if (not match(v_IMS_DATA_REQ.Request.Register.msgHeader.contentType, cr_ContentTypeMIME)) { f_IMS_SetVerdictFailOrInconc(__FILE__, __LINE__, "Invalid Content-Type"); } v_MIME_Message := v_RegisterReq.messageBody.mimeMessageBody; f_MCX_SIP_RegistrationSubscription_DecodeMatchAndCheckMikeyMessage_SetCSK(p_ServiceType, v_MIME_Message, CSK); v_MCS_Info := fl_MCX_SIP_Registration_McsInfo(p_ServiceType, v_IMS_DATA_REQ.Request); v_XmlBodies := f_MCX_MIME_DecodeAndMatchXML(v_MIME_Message, p_ServiceType, -, v_MCS_Info.Mcptt, -, v_MCS_Info.Mcvideo, -, v_MCS_Info.Mcdata); // Store client-id (if any) fl_MCX_SIP_Registration_ClientId_Set(v_XmlBodies); } // Step 4 IMS_Server.send(cas_IMS_DATA_RSP(f_IMS_RoutingInfo_ULtoDL(v_IMS_DATA_REQ.RoutingInfo), cs_Response(c_statusLine200, f_MCX_RegisterResponse_200_MessageHeaderTX(v_RegisterReq, p_ServiceType)))); return v_IMS_DATA_REQ; }

## 2.2 Runtime with f_MCX_IMS_PTC_SecurityParameters_GetKey

| Function names | f_MCX_IMS_PTC_SecurityParameters_GetKey, f_MCX_SecurityParameters_GetKey, f_MCX_SecurityParameters_GetOptionalKey (new) |
| --- | --- |
| Reason for change | f_MCX_SecurityParameters_GetKey causes a runtime error when a key does not exist; nevertheless f_MCX_IMS_PTC_SecurityParameters_GetKey can cope with not existing keys |
| Summary of change | f_MCX_SecurityParameters_GetKey renamed to f_MCX_SecurityParameters_GetExistingKey and using new function f_MCX_SecurityParameters_GetOptionalKey |
| TTCN modules | MCX_IMS_Component, MCX_SecurityParameters |

function f_MCX_IMS_PTC_SecurityParameters_GetKey(MCX_KeyIndex_Type p_KeyIndex) runs on MCX_IMS_PTC return template (omit) MCX_SecurityKey_Type { var MCX_SecurityParameters_Type v_SecurityParameters; var template (omit) MCX_SecurityKey_Type v_Key := omit; v_SecurityParameters := f_MCX_IMS_PTC_SecurityParameters_Get(); v_Key := f_MCX_SecurityParameters_GetOptionalKey(v_SecurityParameters, p_KeyIndex); return v_Key; }

function f_MCX_SecurityParameters_GetExistingKey(MCX_SecurityParameters_Type p_SecurityParameters, MCX_KeyIndex_Type p_KeyIndex) return MCX_SecurityKey_Type { var template (omit) MCX_SecurityKey_Type v_Key := f_MCX_SecurityParameters_GetOptionalKey(p_SecurityParameters, p_KeyIndex); if (not isvalue(v_Key)) { FatalError(__FILE__, __LINE__, "key is not initialised"); } return valueof(v_Key); }

function f_MCX_SecurityParameters_GetOptionalKey(MCX_SecurityParameters_Type p_SecurityParameters, MCX_KeyIndex_Type p_KeyIndex) return template (omit) MCX_SecurityKey_Type { var integer v_Index := enum2int(p_KeyIndex); if (not f_MCX_SecurityParameters_KeyExists(p_SecurityParameters, p_KeyIndex)) { return omit; } return p_SecurityParameters.Keys[v_Index].Key; }