3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240282

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3410 | rev | - | Current version: | 17.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR CA test case 8.1.4.1.9.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-03-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | As per current TTCN msg4 is getting scheduled with coreset1 which may lead to fail the case at step 21 specially in scenario when second cell is FDD. From spec 38.523-3 Table 7.1.2.2.3.1-1: MSG4 over CSSType1 containing the DCCH message. It should use RbIndex without OffsetCarrierCoreset0 in case of DCCH over CSS. OffsetCarrierCoreset0 should not considered in case of DCCH over CSS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Changed the template cs_NR_SearchSpaceDlDciAssignment_CSSType1_Msg4 to use coreset 0 for msg4 . |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.4.1.9.2.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR… |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1	Change 1	5

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2022-09_D24wk12 related to the title of this CR.

Contact:	Kalahasti, Narendra

# Corrections required

## Change 1

| Function name | function f_NR5GC_RRCReestablishment_Steps0To2 () |
| --- | --- |
| Reason for change | From spec 38.523-3 Table 7.1.2.2.3.1-1: MSG4 over CSSType1 containing the DCCH message. It should use RbIndex without OffsetCarrierCoreset0 in case of DCCH over CSS. OffsetCarrierCoreset0 should not considered in case of DCCH over CSS. |
| Summary of change | Changed the template cs_NR_SearchSpaceDlDciAssignment_CSSType1_Msg4 to use coreset 0 for msg4 . |
| TTCN module | NR5GC_RRCSteps.ttcn |
| MCC160 Comment |  |

Before Change

function f_NR5GC_RRCReestablishment_Steps0To2(NR_CellId_Type p_NR_SourceCellId, NR_CellId_Type p_NR_TargetCellId, template(present) ReestablishmentCause p_ReestablishmentCause := handoverFailure, RNTI_Value p_RNTI_Value := tsc_C_RNTI_Value4, template (omit) charstring p_VerdictMsg := omit, template (omit) MacBearerRouting_Type p_MacBearerRoutingCA := omit, template (omit) NR_CellPowerList_Type p_CellPowerListBeforeStep2 := omit ) runs on NR_BASE_PTC { //@sic R5-211391 R5-213516: added p_MacBearerRoutingCA sic@ //@sic R5-221467 renamed function from f_NR5GC_RRCReestablishment_Steps1To2 to f_NR5GC_RRCReestablishment_Steps0To2 sic@ //@sic R5s230515: added p_CellPowerListBeforeStep2 sic@ var RNTI_Value v_RNTI_Value := f_NR_CellInfo_GetRNTI(p_NR_SourceCellId); //C-RNTI used by UE in RRCReestablismentRequest var PhysCellId v_PhysCellId := f_NR_CellInfo_GetPhysicalCellId(p_NR_SourceCellId); var NR_PhysicalParameters_Type v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(p_NR_TargetCellId); var ShortMAC_I v_ShortMAC_I:= f_NR_Calculate_ShortMAC(p_NR_SourceCellId, p_NR_TargetCellId); //@sic R5s211169 change 1.1 sic@ var IntegerList_Type v_DrbIdList := f_NR_GetActiveDRBs(p_NR_SourceCellId); // 0. Source Cell: If the Source Cell is the same as the Target Cell: Release SRBs and DRBs. (Re-)configure SRBs. //@sic R5-221467 sic@ if (p_NR_SourceCellId == p_NR_TargetCellId) { f_NR_SS_RRC_ReleaseSecurity(p_NR_SourceCellId); //@sic R5s220275 sic@ f_NR_SS_SRBs_DRBs_Release(p_NR_SourceCellId, cs_TimingInfo_Now, v_DrbIdList); //@sic R5-211417 R5s211169 R5s220176 sic@ f_NR_SS_SRBs_DRBs_Config(p_NR_SourceCellId, cs_TimingInfo_Now, omit); //@sic R5s220176 sic@ // @sic R5-217830 sic@ } // 1. Target Cell: Reconfigure DCCH/DTCH DCI on CSS and reconfigure RACH procedure contention-based with UE Contention Resolution Identity MAC CE and with a new C-RNTI. f_NR_CellInfo_SetRNTI(p_NR_TargetCellId, p_RNTI_Value); f_NR_SS_C_RNTI_Config(p_NR_TargetCellId, p_RNTI_Value, -, tsc_NoCnfReq, tsc_FollowOnFlag); // NOTE 1: The FollowOnFlag is set to true in the ASP reconfiguring C-RNTI. f_NR_SS_CommonCellConfig(p_NR_TargetCellId, cads_NR_RachProcedure_DcchDtchToCSS_Config_REQ(p_NR_TargetCellId, cs_TimingInfo_Now, tsc_CnfReq, cs_NR_RachProcedureConfig_RRCReestablishment(v_NR_PhysicalParameters, p_RNTI_Value), //@sic R5-225418 sic@ cs_NR_SearchSpaceDlDciAssignment_CSSType1(v_NR_PhysicalParameters), cs_NR_SearchSpaceUlDciAssignmentCSS(v_NR_PhysicalParameters, cs_NR_DciFormat_0_0_Params))); //@sic R5s230515 sic@ if ( isvalue(p_CellPowerListBeforeStep2)) { f_NR_SetCellPowerList (p_CellPowerListBeforeStep2); } // 2. Target Cell: Receive RRCReestablismentRequest. SRB.receive(car_NR_SRB0_RrcPdu_IND(p_NR_TargetCellId, cr_38508_RRCReestablishmentRequest(cr_NR_ReestabUE_Identity (v_RNTI_Value, v_PhysCellId, v_ShortMAC_I), p_ReestablishmentCause), p_MacBearerRoutingCA)); if (isvalue(p_VerdictMsg)) { f_NR_PreliminaryPass(__FILE__, __LINE__, valueof(p_VerdictMsg)); } }

After Change

function f_NR5GC_RRCReestablishment_Steps0To2(NR_CellId_Type p_NR_SourceCellId, NR_CellId_Type p_NR_TargetCellId, template(present) ReestablishmentCause p_ReestablishmentCause := handoverFailure, RNTI_Value p_RNTI_Value := tsc_C_RNTI_Value4, template (omit) charstring p_VerdictMsg := omit, template (omit) MacBearerRouting_Type p_MacBearerRoutingCA := omit, template (omit) NR_CellPowerList_Type p_CellPowerListBeforeStep2 := omit ) runs on NR_BASE_PTC { //@sic R5-211391 R5-213516: added p_MacBearerRoutingCA sic@ //@sic R5-221467 renamed function from f_NR5GC_RRCReestablishment_Steps1To2 to f_NR5GC_RRCReestablishment_Steps0To2 sic@ //@sic R5s230515: added p_CellPowerListBeforeStep2 sic@ var RNTI_Value v_RNTI_Value := f_NR_CellInfo_GetRNTI(p_NR_SourceCellId); //C-RNTI used by UE in RRCReestablismentRequest var PhysCellId v_PhysCellId := f_NR_CellInfo_GetPhysicalCellId(p_NR_SourceCellId); var NR_PhysicalParameters_Type v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(p_NR_TargetCellId); var ShortMAC_I v_ShortMAC_I:= f_NR_Calculate_ShortMAC(p_NR_SourceCellId, p_NR_TargetCellId); //@sic R5s211169 change 1.1 sic@ var IntegerList_Type v_DrbIdList := f_NR_GetActiveDRBs(p_NR_SourceCellId); // 0. Source Cell: If the Source Cell is the same as the Target Cell: Release SRBs and DRBs. (Re-)configure SRBs. //@sic R5-221467 sic@ if (p_NR_SourceCellId == p_NR_TargetCellId) { f_NR_SS_RRC_ReleaseSecurity(p_NR_SourceCellId); //@sic R5s220275 sic@ f_NR_SS_SRBs_DRBs_Release(p_NR_SourceCellId, cs_TimingInfo_Now, v_DrbIdList); //@sic R5-211417 R5s211169 R5s220176 sic@ f_NR_SS_SRBs_DRBs_Config(p_NR_SourceCellId, cs_TimingInfo_Now, omit); //@sic R5s220176 sic@ // @sic R5-217830 sic@ } // 1. Target Cell: Reconfigure DCCH/DTCH DCI on CSS and reconfigure RACH procedure contention-based with UE Contention Resolution Identity MAC CE and with a new C-RNTI. f_NR_CellInfo_SetRNTI(p_NR_TargetCellId, p_RNTI_Value); f_NR_SS_C_RNTI_Config(p_NR_TargetCellId, p_RNTI_Value, -, tsc_NoCnfReq, tsc_FollowOnFlag); // NOTE 1: The FollowOnFlag is set to true in the ASP reconfiguring C-RNTI. f_NR_SS_CommonCellConfig(p_NR_TargetCellId, cads_NR_RachProcedure_DcchDtchToCSS_Config_REQ(p_NR_TargetCellId, cs_TimingInfo_Now, tsc_CnfReq, cs_NR_RachProcedureConfig_RRCReestablishment(v_NR_PhysicalParameters, p_RNTI_Value), //@sic R5-225418 sic@ cs_NR_SearchSpaceDlDciAssignment_CSSType1_Msg4(v_NR_PhysicalParameters), cs_NR_SearchSpaceUlDciAssignmentCSS(v_NR_PhysicalParameters, cs_NR_DciFormat_0_0_Params))); //@sic R5s230515 sic@ if ( isvalue(p_CellPowerListBeforeStep2)) { f_NR_SetCellPowerList (p_CellPowerListBeforeStep2); } // 2. Target Cell: Receive RRCReestablismentRequest. SRB.receive(car_NR_SRB0_RrcPdu_IND(p_NR_TargetCellId, cr_38508_RRCReestablishmentRequest(cr_NR_ReestabUE_Identity (v_RNTI_Value, v_PhysCellId, v_ShortMAC_I), p_ReestablishmentCause), p_MacBearerRoutingCA)); if (isvalue(p_VerdictMsg)) { f_NR_PreliminaryPass(__FILE__, __LINE__, valueof(p_VerdictMsg)); } }