3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240283

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3411 | rev | - | Current version: | 17.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC test case 7.1.1.8.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-03-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The BWP switching at Steps 5, 9, 16 and 20 are being done with immediate activation time. There is not enough time for the RLC STATUS PDUs to be transmitted on the BWP where the loopback of data happened, before the BWP switch occurs, leading to a situation where the BWP has been switched and UE has not received the RLC STATUS PDU leading it to re-transmit the same data on the UL again. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added a delay of 100ms before the BWP switch at Steps 5, 9, 16 and 20 to ensure the RLC STATUS PDU is received by the UE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.8.1.ENDC, 7.1.1.8.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections required	2

2.1	Correction to function f_TC_7_1_1_8_1_NR_CommonTestBody	2

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D24wk12 related to the title of this CR.

Contact: 	Juan Garcia Martin

Juan.GarciaMartin@rohde-schwarz.com

# Corrections required

## Correction to function f_TC_7_1_1_8_1_NR_CommonTestBody

| Function name | f_TC_7_1_1_8_1_NR_CommonTestBody |
| --- | --- |
| Reason for change | The BWP switching at Steps 5, 9, 16 and 20 are being done with immediate activation time. There is not enough time for the RLC STATUS PDUs to be transmitted on the BWP where the loopback of data happened, before the BWP switch occurs, leading to a situation where the BWP has been switched and UE has not received the RLC STATUS PDU leading it to re-transmit the same data on the UL again |
| Summary of change | Added a delay of 100ms before the BWP switch at Steps 5, 9, 16 and 20 to ensure the RLC STATUS PDU is received by the UE. |
| TTCN module | MAC_BWP_Common_NR |

Before Change:

function f_TC_7_1_1_8_1_NR_CommonTestBody(NR_CellId_Type p_NR_CellId, DRB_Identity p_DrbId, template (value) NR_ResourceAllocation_Type p_ResourceAllocationUL) runs on NR_BASE_PTC { //@sic R5-213459 R5-227417 sic@ <<SKIPPED CODE>> // @siclog "Steps 3-3A" siclog@ fl_TC_7_1_1_8_1_ReceiveData(p_NR_CellId, p_DrbId, {v_RLC_SDU1}, true, v_SN); //@sic R5-231578 R5s230220 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 3"); v_SN := v_SN + 1; if (v_BWP_SameNumerology_upto4) { //@sic R5s230634 sic@ f_NR_SS_ConfigureBWPs(p_NR_CellId, -, tsc_NR_BWP_Id_FirstActive, tsc_NR_BWP_Id_2, tsc_NR_BWP_Id_2, tsc_NR_BWP_Id_2); } else { f_NR_SS_ConfigureBWPs(p_NR_CellId, -, tsc_NR_BWP_Id_FirstActive, tsc_NR_BWP_Id_Initial, tsc_NR_BWP_Id_Initial, tsc_NR_BWP_Id_Initial); } // @siclog "Step 5" siclog@ v_TimingInfoDL := f_TC_7_1_1_8_1_NR_GetNextSendOccasion_DL(p_NR_CellId); v_RLC_SDU1 := fl_TC_7_1_1_8_1_SendData(p_NR_CellId, p_DrbId, cs_TimingInfo_NR(v_TimingInfoDL), v_SN); <<SKIPPED CODE>> // @siclog "Steps 7-7A" siclog@ fl_TC_7_1_1_8_1_ReceiveData(p_NR_CellId, p_DrbId, {v_RLC_SDU1}, true, v_SN); //@sic R5-231578 R5s230220 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 7"); v_SN := v_SN + 1; //@sic R5s230220 change 9.2 sic@ if (v_BWP_SameNumerology_upto4) { //@sic R5-2134599 sic@ f_NR_SS_ConfigureBWPs_DL(p_NR_CellId, -, tsc_NR_BWP_Id_2); } else { f_NR_SS_ConfigureBWPs_DL(p_NR_CellId, -, tsc_NR_BWP_Id_Initial); } // @siclog "Step 9" siclog@ v_TimingInfoDL := f_TC_7_1_1_8_1_NR_GetNextSendOccasion_DL(p_NR_CellId, 300); v_RLC_SDU1 := fl_TC_7_1_1_8_1_SendData(p_NR_CellId, p_DrbId, cs_TimingInfo_NR(v_TimingInfoDL), v_SN); <<SKIPPED CODE>> // @siclog "Steps 13D-13E" siclog@ fl_TC_7_1_1_8_1_ReceiveData(p_NR_CellId, p_DrbId, {v_RLC_SDU1}, true, v_SN); //@sic R5-231578 R5s230220 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 13D"); f_NR_SS_SystemIndCtrlConfig(p_NR_CellId, cds_NR_SystemIndCtrl_RachPreamble(disable)); //@sic R5s230634 sic@ v_SN := v_SN + 1; //@sic R5s230634 sic@ f_NR_SS_ConfigureBWPs(p_NR_CellId, -, tsc_NR_BWP_Id_Initial, tsc_NR_BWP_Id_1, tsc_NR_BWP_Id_1, tsc_NR_BWP_Id_1); // @siclog "Step 16" siclog@ v_TimingInfoDL := f_TC_7_1_1_8_1_NR_GetNextSendOccasion_DL(p_NR_CellId); //@sic R5s230220 change 10 sic@ v_TimingInfoConfig := f_SubFrameTiming_AddMilliSeconds(v_TimingInfoDL, 100); // 100ms after step 16 v_RLC_SDU1 := fl_TC_7_1_1_8_1_SendData(p_NR_CellId, p_DrbId, cs_TimingInfo_NR(v_TimingInfoDL), v_SN); v_SN := v_SN + 1; //@sic R5s230220 change 10 sic@ <<SKIPPED CODE>> // @siclog "Steps 19-19A" siclog@ fl_TC_7_1_1_8_1_ReceiveData(p_NR_CellId, p_DrbId, {v_RLC_SDU1, v_RLC_SDU2}, true, v_SN); //@sic R5-231578 R5s230220 sic@ v_SN := v_SN + 1; //@sic R5-216169 R5s230220 R5s230634 sic@ if (v_BWP_SameNumerology_upto4 ) { f_NR_SS_ConfigureBWPs(p_NR_CellId, -, tsc_NR_BWP_Id_2, tsc_NR_BWP_Id_2, tsc_NR_BWP_Id_2, tsc_NR_BWP_Id_2); } else { f_NR_SS_ConfigureBWPs(p_NR_CellId, -, tsc_NR_BWP_Id_Initial, tsc_NR_BWP_Id_Initial, tsc_NR_BWP_Id_Initial, tsc_NR_BWP_Id_Initial); } // @siclog "Step 20" siclog@ v_TimingInfoDL := f_SubFrameTiming_AddMilliSeconds(v_TimingInfoDL, 1400); // 1400ms after step 17 => 1000ms after step 18 v_RLC_SDU1 := fl_TC_7_1_1_8_1_SendData(p_NR_CellId, p_DrbId, cs_TimingInfo_NR(v_TimingInfoDL), v_SN); <<SKIPPED CODE>>

After Change:

function f_TC_7_1_1_8_1_NR_CommonTestBody(NR_CellId_Type p_NR_CellId, DRB_Identity p_DrbId, template (value) NR_ResourceAllocation_Type p_ResourceAllocationUL) runs on NR_BASE_PTC { //@sic R5-213459 R5-227417 sic@ <<SKIPPED CODE>> // @siclog "Steps 3-3A" siclog@ fl_TC_7_1_1_8_1_ReceiveData(p_NR_CellId, p_DrbId, {v_RLC_SDU1}, true, v_SN); //@sic R5-231578 R5s230220 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 3"); v_SN := v_SN + 1; f_Delay (0.1); if (v_BWP_SameNumerology_upto4) { //@sic R5s230634 sic@ f_NR_SS_ConfigureBWPs(p_NR_CellId, -, tsc_NR_BWP_Id_FirstActive, tsc_NR_BWP_Id_2, tsc_NR_BWP_Id_2, tsc_NR_BWP_Id_2); } else { f_NR_SS_ConfigureBWPs(p_NR_CellId, -, tsc_NR_BWP_Id_FirstActive, tsc_NR_BWP_Id_Initial, tsc_NR_BWP_Id_Initial, tsc_NR_BWP_Id_Initial); } // @siclog "Step 5" siclog@ v_TimingInfoDL := f_TC_7_1_1_8_1_NR_GetNextSendOccasion_DL(p_NR_CellId); v_RLC_SDU1 := fl_TC_7_1_1_8_1_SendData(p_NR_CellId, p_DrbId, cs_TimingInfo_NR(v_TimingInfoDL), v_SN); <<SKIPPED CODE>> // @siclog "Steps 7-7A" siclog@ fl_TC_7_1_1_8_1_ReceiveData(p_NR_CellId, p_DrbId, {v_RLC_SDU1}, true, v_SN); //@sic R5-231578 R5s230220 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 7"); v_SN := v_SN + 1; f_Delay (0.1); //@sic R5s230220 change 9.2 sic@ if (v_BWP_SameNumerology_upto4) { //@sic R5-2134599 sic@ f_NR_SS_ConfigureBWPs_DL(p_NR_CellId, -, tsc_NR_BWP_Id_2); } else { f_NR_SS_ConfigureBWPs_DL(p_NR_CellId, -, tsc_NR_BWP_Id_Initial); } // @siclog "Step 9" siclog@ v_TimingInfoDL := f_TC_7_1_1_8_1_NR_GetNextSendOccasion_DL(p_NR_CellId, 300); v_RLC_SDU1 := fl_TC_7_1_1_8_1_SendData(p_NR_CellId, p_DrbId, cs_TimingInfo_NR(v_TimingInfoDL), v_SN); <<SKIPPED CODE>> // @siclog "Steps 13D-13E" siclog@ fl_TC_7_1_1_8_1_ReceiveData(p_NR_CellId, p_DrbId, {v_RLC_SDU1}, true, v_SN); //@sic R5-231578 R5s230220 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 13D"); f_NR_SS_SystemIndCtrlConfig(p_NR_CellId, cds_NR_SystemIndCtrl_RachPreamble(disable)); //@sic R5s230634 sic@ v_SN := v_SN + 1; f_Delay (0.1); //@sic R5s230634 sic@ f_NR_SS_ConfigureBWPs(p_NR_CellId, -, tsc_NR_BWP_Id_Initial, tsc_NR_BWP_Id_1, tsc_NR_BWP_Id_1, tsc_NR_BWP_Id_1); // @siclog "Step 16" siclog@ v_TimingInfoDL := f_TC_7_1_1_8_1_NR_GetNextSendOccasion_DL(p_NR_CellId); //@sic R5s230220 change 10 sic@ v_TimingInfoConfig := f_SubFrameTiming_AddMilliSeconds(v_TimingInfoDL, 100); // 100ms after step 16 v_RLC_SDU1 := fl_TC_7_1_1_8_1_SendData(p_NR_CellId, p_DrbId, cs_TimingInfo_NR(v_TimingInfoDL), v_SN); v_SN := v_SN + 1; //@sic R5s230220 change 10 sic@ <<SKIPPED CODE>> // @siclog "Steps 19-19A" siclog@ fl_TC_7_1_1_8_1_ReceiveData(p_NR_CellId, p_DrbId, {v_RLC_SDU1, v_RLC_SDU2}, true, v_SN); //@sic R5-231578 R5s230220 sic@ v_SN := v_SN + 1; f_Delay (0.1); //@sic R5-216169 R5s230220 R5s230634 sic@ if (v_BWP_SameNumerology_upto4 ) { f_NR_SS_ConfigureBWPs(p_NR_CellId, -, tsc_NR_BWP_Id_2, tsc_NR_BWP_Id_2, tsc_NR_BWP_Id_2, tsc_NR_BWP_Id_2); } else { f_NR_SS_ConfigureBWPs(p_NR_CellId, -, tsc_NR_BWP_Id_Initial, tsc_NR_BWP_Id_Initial, tsc_NR_BWP_Id_Initial, tsc_NR_BWP_Id_Initial); } // @siclog "Step 20" siclog@ v_TimingInfoDL := f_SubFrameTiming_AddMilliSeconds(v_TimingInfoDL, 1300); // 1300ms after step 17 => 1000ms after step 18 v_RLC_SDU1 := fl_TC_7_1_1_8_1_SendData(p_NR_CellId, p_DrbId, cs_TimingInfo_NR(v_TimingInfoDL), v_SN); <<SKIPPED CODE>>