3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240292

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3416 | rev | - | Current version: | 17.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of NR5GC test case 8.1.1.3.8 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test |  |  |  |  |  | Date: |  |  | 2024-04-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Sib4 of cell 1 should be set to interfrequency DL_ARFCN of cell 23, and function f_NR5GC_CellInfo_GetSIB4_InterFreqCarrierFreqList provides interfrequency DL_ARFCN of neighbor cell. To set SIB4 of cell1, TTCN should call f_NR5GC_CellInfo_GetSIB4_InterFreqCarrierFreqList for cell 1 itself to fetch interfrequency DL_ARFCN. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated argument for f_NR5GC_CellInfo_SetSIB4_InterFreqCarrierFreqList for each cell 1 and cell 23. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Confromant UE may fail |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.1.3.8.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR… |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1	Change 1	5

# Overview

This document lists all the essential changes needed to correct problems in the TTCN implementation of test case 8.1.1.3.8 which is part of the 5G Pos test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:	Kalahasti, Narendra

Narendra.Kalahasti@anritsu.com

# Corrections required

## Change 1

| Function name | function f_TC_8_1_1_3_8_NR5GC () |
| --- | --- |
| Reason for change | Sib4 of cell 1 should be set to interfrequency DL_ARFCN of cell 23, and function f_NR5GC_CellInfo_GetSIB4_InterFreqCarrierFreqList provides interfrequency DL_ARFCN of neighbor cell. To set SIB4 of cell1, TTCN should call f_NR5GC_CellInfo_GetSIB4_InterFreqCarrierFreqList for cell 1 itself to fetch interfrequency DL_ARFCN. |
| Summary of change | Updated argument for f_NR5GC_CellInfo_SetSIB4_InterFreqCarrierFreqList for each cell 1 and cell 23. |
| TTCN module | RRC_ConnRel_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change

function f_TC_8_1_1_3_8_NR5GC() runs on NR5GC_PTC { //RRC connection release / Redirection to another NR frequency / MPS Priority Indication var InterFreqCarrierFreqList v_InterFreqCarrierFreqList_Cell1; var InterFreqCarrierFreqList v_InterFreqCarrierFreqList_Cell23; f_NR5GC_Init(NR_4); v_InterFreqCarrierFreqList_Cell1 := f_NR5GC_CellInfo_GetSIB4_InterFreqCarrierFreqList(nr_Cell23); v_InterFreqCarrierFreqList_Cell23 := f_NR5GC_CellInfo_GetSIB4_InterFreqCarrierFreqList(nr_Cell1); f_NR5GC_CellInfo_SetSIB4_InterFreqCarrierFreqList(nr_Cell1, {v_InterFreqCarrierFreqList_Cell1[0]}); f_NR5GC_CellInfo_SetSIB4_InterFreqCarrierFreqList(nr_Cell23, {v_InterFreqCarrierFreqList_Cell23[0]}); f_NR5GC_CellInfo_SetSIB4_InterFreqCarrierFreq_CellReselectionPriority(nr_Cell1, 0, omit); f_NR5GC_CellInfo_SetSIB4_InterFreqCarrierFreq_CellReselectionPriority(nr_Cell23, 0, omit); f_NR_CellConfig_Def(nr_Cell1); f_NR_CellConfig_Def(nr_Cell23); //The UE is in test state 1N-A with IMS registered as defined in TS 38.508-1 [4], subclause 4.4A.2 on NR Cell 1. f_NR5GC_Preamble (nr_Cell1, STATE_IDLE_1A); f_NR_TestBody_Set(true); fl_TC_8_1_1_3_8_TestBody(); f_NR_TestBody_Set(false); f_NR_Postamble(nr_Cell23, STATE_IDLE_1A); }

After Change

function f_TC_8_1_1_3_8_NR5GC() runs on NR5GC_PTC { //RRC connection release / Redirection to another NR frequency / MPS Priority Indication var InterFreqCarrierFreqList v_InterFreqCarrierFreqList_Cell1; var InterFreqCarrierFreqList v_InterFreqCarrierFreqList_Cell23; f_NR5GC_Init(NR_4); v_InterFreqCarrierFreqList_Cell1 := f_NR5GC_CellInfo_GetSIB4_InterFreqCarrierFreqList(nr_Cell23); v_InterFreqCarrierFreqList_Cell23 := f_NR5GC_CellInfo_GetSIB4_InterFreqCarrierFreqList(nr_Cell1); f_NR5GC_CellInfo_SetSIB4_InterFreqCarrierFreqList(nr_Cell1, {v_InterFreqCarrierFreqList_Cell23[0]}); f_NR5GC_CellInfo_SetSIB4_InterFreqCarrierFreqList(nr_Cell23, {v_InterFreqCarrierFreqList_Cell1[0]}); f_NR5GC_CellInfo_SetSIB4_InterFreqCarrierFreq_CellReselectionPriority(nr_Cell1, 0, omit); f_NR5GC_CellInfo_SetSIB4_InterFreqCarrierFreq_CellReselectionPriority(nr_Cell23, 0, omit); f_NR_CellConfig_Def(nr_Cell1); f_NR_CellConfig_Def(nr_Cell23); //The UE is in test state 1N-A with IMS registered as defined in TS 38.508-1 [4], subclause 4.4A.2 on NR Cell 1. f_NR5GC_Preamble (nr_Cell1, STATE_IDLE_1A); f_NR_TestBody_Set(true); fl_TC_8_1_1_3_8_TestBody(); f_NR_TestBody_Set(false); f_NR_Postamble(nr_Cell23, STATE_IDLE_1A); } // 2. Target Cell: Receive RRCReestablismentRequest. SRB.receive(car_NR_SRB0_RrcPdu_IND(p_NR_TargetCellId, cr_38508_RRCReestablishmentRequest(cr_NR_ReestabUE_Identity (v_RNTI_Value, v_PhysCellId, v_ShortMAC_I), p_ReestablishmentCause), p_MacBearerRoutingCA)); if (isvalue(p_VerdictMsg)) { f_NR_PreliminaryPass(__FILE__, __LINE__, valueof(p_VerdictMsg)); } }