3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240303

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3421 | rev | - | Current version: | 17.10.0 |  |
| For HELPon using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC testcase 9.1.4.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Starpoint |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-04-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Testcase will not stop even error occurred when using function f_AT_ResponseCheck. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add self.kill at the end of the function |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Testcase will not stop even error occurred. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.4.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections required	2

2.1	f_AT_ResponseCheck	2

- Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D24wk12 related to the title of this CR.

Contact:	Ting Gao

gaoting@starpointcomm.com

- Corrections required

  - f_AT_ResponseCheck

| Functionname | f_AT_ResponseCheck |
| --- | --- |
| Reason for change | Testcase will not stop even error occurred when using function f_AT_ResponseCheck. |
| Summary of change | Add self.kill at the end of the function |
| TTCN module | NAS_AuxiliaryDefsAndFunctions.ttcn |
| MCC160 Comment |  |

Before Change:

function f_AT_ResponseCheck(charstring p_AT_Response, charstring p_ExpectedString, charstring p_Text := "", charstring p_Expression := "*[""]([^""]+)[""]*", integer p_Group := 0) { /* p_Expression := "*[""]([^""]+)[""]*"; TTCN-3 regular expression: anything + quote + no quote (at least one character) + quote + anything; * => the text between the most inner quotes is returned */ var charstring v_Expression := p_Expression; var charstring v_Result; var template charstring v_ExpectedString := pattern p_ExpectedString; var boolean v_MatchResult; var charstring v_CRLF := oct2char('0D'O) & oct2char('0A'O); v_Result := regexp(p_AT_Response, v_Expression, p_Group); v_MatchResult := match(v_Result, v_ExpectedString); if (v_MatchResult == true) { f_SetVerdict(pass,__FILE__, __LINE__, p_Text); } else { f_ErrorLog(__FILE__, __LINE__, v_CRLF & "Value expected: " & p_ExpectedString & v_CRLF & "Value received: " & v_Result); // if anything went wrong, show it f_SetVerdict(fail,__FILE__, __LINE__, p_Text); } }

After Change:

function f_AT_ResponseCheck(charstring p_AT_Response, charstring p_ExpectedString, charstring p_Text := "", charstring p_Expression := "*[""]([^""]+)[""]*", integer p_Group := 0) { /* p_Expression := "*[""]([^""]+)[""]*"; TTCN-3 regular expression: anything + quote + no quote (at least one character) + quote + anything; * => the text between the most inner quotes is returned */ var charstring v_Expression := p_Expression; var charstring v_Result; var template charstring v_ExpectedString := pattern p_ExpectedString; var boolean v_MatchResult; var charstring v_CRLF := oct2char('0D'O) & oct2char('0A'O); v_Result := regexp(p_AT_Response, v_Expression, p_Group); v_MatchResult := match(v_Result, v_ExpectedString); if (v_MatchResult == true) { f_SetVerdict(pass,__FILE__, __LINE__, p_Text); } else { f_ErrorLog(__FILE__, __LINE__, v_CRLF & "Value expected: " & p_ExpectedString & v_CRLF & "Value received: " & v_Result); // if anything went wrong, show it f_SetVerdict(fail,__FILE__, __LINE__, p_Text); self.kill; // => test case will be terminated immediately } }