3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240314

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4813 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to EUTRA EIEI test case 8.2.4.28 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test, EIEI-UEConTest |  |  |  |  |  | Date: |  |  | 2024-04-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | f_EUTRA_Init function is called twice and second time with wrong Cell combination C1 which initiates multiple EUTRA initialization process for the same Cell. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Removed repetition of EUTRA initialization. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A confomant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.4.28 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required.	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2022-09_D24wk12 related to the title of this CR.

Contact:	Mohit Kanchan
	mohit.kanchan@keysight.com

# Corrections required.

## Change 1

| Function name | f_TC_8_2_4_28_EUTRA () |
| --- | --- |
| Reason for change | f_EUTRA_Init function is called twice and second time with wrong Cell combination C1 which initiates multiple EUTRA initialization process for the same Cell. |
| Summary of change | Removed repetition of EUTRA initialization. |
| TTCN module | IMS_ECall_RRC_Handover.ttcn |
| MCC160 Comment |  |

Before change

function f_TC_8_2_4_28_EUTRA() runs on EUTRA_PTC

{

var template (value) CellPowerList_Type v_CellPowerList_AtT0 := { cs_CellPower(eutra_Cell3, -97) };

var template (value) CellPowerList_Type v_CellPowerList_AtT1 := { cs_CellPower(eutra_Cell3, -73) };

var Frequency_IE_Type v_Frequency_IE_Cell1;

var Frequency_IE_Type v_Frequency_IE_Cell3;

var ChannelBandwidthDependency_Type v_ChBandDependency_Cell1;

var ChannelBandwidthDependency_Type v_ChBandDependency_Cell3;

var PhysCellId v_PhysicalCellIdentity_Cell3;

var template (value) RadioBearerList_Type v_DrbConfigList := {cs_OneDRB_ConfigAM(tsc_DRB1), cs_OneDRB_ConfigAM(tsc_DRB2), cs_OneDRB_ConfigUM(tsc_DRB3)};

var default v_DefaultRef;

f_EUTRA_Init(c3, HANDLE_UM_DATA, USE_BIG_GRANTS); //@sic R5s240105 sic@

f_EUTRA_Init(c1);

f_EUTRA_CellInfo_InitMaxReferencePower(eutra_Cell3, -73);

// Get cell specific parameters

v_PhysicalCellIdentity_Cell3 := f_EUTRA_CellInfo_GetPhyCellId(eutra_Cell3);

v_Frequency_IE_Cell3 := f_EUTRA_CellInfo_GetFrequencyIEs(eutra_Cell3);

v_ChBandDependency_Cell3 := f_EUTRA_BandDependentParam(v_Frequency_IE_Cell3.DL_ChBandwidth,

v_Frequency_IE_Cell3.UL_ChBandwidth);

v_Frequency_IE_Cell1 := f_EUTRA_CellInfo_GetFrequencyIEs(eutra_Cell1);

v_ChBandDependency_Cell1 := f_EUTRA_BandDependentParam(v_Frequency_IE_Cell1.DL_ChBandwidth,

v_Frequency_IE_Cell1.UL_ChBandwidth);

// Set new C-RNTI to be used by UE in cell 3 at creation

f_EUTRA_CellInfo_SetC_RNTI(eutra_Cell3, tsc_C_RNTI_Def2);

// Create and configure all cells

f_EUTRA_CellConfig_Def(eutra_Cell1);

f_EUTRA_CellConfig_Def(eutra_Cell3);

f_EUTRA_SetCellPower(eutra_Cell1, tsc_ServingCellRS_EPRE); //@sic R5s240105 sic@

// Insert USIM for eCall

f_UT_USIM_Insert (UT, "36.508 Table 4.9.3.5-2");

//Preamble

//The UE has established an eCall over IMS according to [18] subclause 4.5A.27 by executing steps 1 to 25.

f_EUTRA_SwitchOnUEandStartIP (eutra_Cell1);  // @sic R5s240105 sic@

f_Delay (15.0);  // @sic R5s240105 sic@

f_UT_InitiateECall(UT, MANUAL);

f_EUTRA_508_IMSeCallOnly_Steps3_25(eutra_Cell1);

//IMS call has been established

f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS2]);

// Set initial cell power level according to T0

f_EUTRA_SetCellPowerList(v_CellPowerList_AtT0);

f_EUTRA_TestBody_Set(true);

//@siclog "Step 1-2" siclog@

f_EUTRA_RRCConnectionReconfiguration_MeasConfig(eutra_Cell1,

cs_508_RRCConnectionReconfiguration_ConditionMEAS(tsc_RRC_TI_Def,

cs_Measurement_Config_A3_InterFreq(v_Frequency_IE_Cell3.UL_DL_Earfcn.dl_CarrierFreq,

v_ChBandDependency_Cell3.AllowedMeasBandwidth,

v_Frequency_IE_Cell1.UL_DL_Earfcn.dl_CarrierFreq,

v_ChBandDependency_Cell1.AllowedMeasBandwidth,

cs_508_MeasGapConfig_GP1, //@sic R5s240105 sic@

f_ReturnMeasObjectToAddModList_2Entries_v9e0(v_Frequency_IE_Cell1.UL_DL_Earfcn.dl_CarrierFreq_v9e0, v_Frequency_IE_Cell3.UL_DL_Earfcn.dl_CarrierFreq_v9e0))));

//@siclog "Step 3" siclog@

//Set cell settings according to T1

f_EUTRA_SetCellPowerList(v_CellPowerList_AtT1);

//@siclog "Step 4" siclog@

//Receive MasurementReport

SRB.receive(car_SRB1_RrcPdu_IND(eutra_Cell1, cr_MeasurementReport(1,//Meas id

?,//RSRP result

?,//RSRQ result

cr_MeasResultNeighCells_measResultListEUTRA_1Entry(v_PhysicalCellIdentity_Cell3,

omit,//CGI info

?,//RSRP result

?))));//RSRQ result

// Activate the default to receive further measurement reports for cell 3 on cell 1

v_DefaultRef := activate (a_EUTRA_AddDefault_PeriodicalMeasurmentReport(eutra_Cell1,

cr_MeasurementReport(1, ?, ?,

cr_MeasResultNeighCells_measResultListEUTRA_1Entry(v_PhysicalCellIdentity_Cell3, omit, ?, ?))));

//@siclog "Step 5-6" siclog

//Reconfigure SS and send RRCConnectionReconfiguration for HO to cell

//Receive RRCConnectionReconfigurationComplete

f_EUTRA_508RRC_IntraLTE_HO_InterCell(eutra_Cell1, eutra_Cell3, v_DrbConfigList, -, -, -, -, -, -, cs_CarrierFreqEUTRA_v9e0(v_Frequency_IE_Cell3.UL_DL_Earfcn.dl_CarrierFreq_r9, omit)); //@sic R5s240105 sic@

//SS automatically checks that UE transmit an RRCConnectionReconfigurationComplete using the security key derived from the currently active KeNB

//* @verdict pass RRCConnectionReconfigurationComplete received on Cell 3

f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 6: Handover performed");

//Deactivate the default to stop receiving measurement reports for cell 3 on cell 1

deactivate(v_DefaultRef);

//@siclog "Step 7" siclog@

// Release eCall over IMS using the generic test procedure described in TS 34.229-1 [35] subclause C.33

f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS2]);     // @sic R5s240105 sic@

f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS2]);

// The SS deactivates the EPS bearer. [This is the rest of C.33]

f_EUTRA_DeactivateEPS_BearerContext (eutra_Cell3,

tsc_EpsDedicatedBearerId2); //@sic R5s240105, R5-235076 sic@

//@siclog "Step 8" siclog@

// The SS releases the RRC connection.

f_EUTRA_RRC_ConnectionRelease(eutra_Cell3); //@sic R5s240105, R5-235076 sic@

f_EUTRA_TestBody_Set(false);

//Switch/power off UE

f_EUTRA_Postamble(eutra_Cell3, E1_IDLE); //@sic R5s240105 sic@

}

After change

function f_TC_8_2_4_28_EUTRA() runs on EUTRA_PTC

{

var template (value) CellPowerList_Type v_CellPowerList_AtT0 := { cs_CellPower(eutra_Cell3, -97) };

var template (value) CellPowerList_Type v_CellPowerList_AtT1 := { cs_CellPower(eutra_Cell3, -73) };

var Frequency_IE_Type v_Frequency_IE_Cell1;

var Frequency_IE_Type v_Frequency_IE_Cell3;

var ChannelBandwidthDependency_Type v_ChBandDependency_Cell1;

var ChannelBandwidthDependency_Type v_ChBandDependency_Cell3;

var PhysCellId v_PhysicalCellIdentity_Cell3;

var template (value) RadioBearerList_Type v_DrbConfigList := {cs_OneDRB_ConfigAM(tsc_DRB1), cs_OneDRB_ConfigAM(tsc_DRB2), cs_OneDRB_ConfigUM(tsc_DRB3)};

var default v_DefaultRef;

f_EUTRA_Init(c3, HANDLE_UM_DATA, USE_BIG_GRANTS); //@sic R5s240105 sic@

f_EUTRA_CellInfo_InitMaxReferencePower(eutra_Cell3, -73);

// Get cell specific parameters

v_PhysicalCellIdentity_Cell3 := f_EUTRA_CellInfo_GetPhyCellId(eutra_Cell3);

v_Frequency_IE_Cell3 := f_EUTRA_CellInfo_GetFrequencyIEs(eutra_Cell3);

v_ChBandDependency_Cell3 := f_EUTRA_BandDependentParam(v_Frequency_IE_Cell3.DL_ChBandwidth,

v_Frequency_IE_Cell3.UL_ChBandwidth);

v_Frequency_IE_Cell1 := f_EUTRA_CellInfo_GetFrequencyIEs(eutra_Cell1);

v_ChBandDependency_Cell1 := f_EUTRA_BandDependentParam(v_Frequency_IE_Cell1.DL_ChBandwidth,

v_Frequency_IE_Cell1.UL_ChBandwidth);

// Set new C-RNTI to be used by UE in cell 3 at creation

f_EUTRA_CellInfo_SetC_RNTI(eutra_Cell3, tsc_C_RNTI_Def2);

// Create and configure all cells

f_EUTRA_CellConfig_Def(eutra_Cell1);

f_EUTRA_CellConfig_Def(eutra_Cell3);

f_EUTRA_SetCellPower(eutra_Cell1, tsc_ServingCellRS_EPRE); //@sic R5s240105 sic@

// Insert USIM for eCall

f_UT_USIM_Insert (UT, "36.508 Table 4.9.3.5-2");

//Preamble

//The UE has established an eCall over IMS according to [18] subclause 4.5A.27 by executing steps 1 to 25.

f_EUTRA_SwitchOnUEandStartIP (eutra_Cell1);  // @sic R5s240105 sic@

f_Delay (15.0);  // @sic R5s240105 sic@

f_UT_InitiateECall(UT, MANUAL);

f_EUTRA_508_IMSeCallOnly_Steps3_25(eutra_Cell1);

//IMS call has been established

f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS2]);

// Set initial cell power level according to T0

f_EUTRA_SetCellPowerList(v_CellPowerList_AtT0);

f_EUTRA_TestBody_Set(true);

//@siclog "Step 1-2" siclog@

f_EUTRA_RRCConnectionReconfiguration_MeasConfig(eutra_Cell1,

cs_508_RRCConnectionReconfiguration_ConditionMEAS(tsc_RRC_TI_Def,

cs_Measurement_Config_A3_InterFreq(v_Frequency_IE_Cell3.UL_DL_Earfcn.dl_CarrierFreq,

v_ChBandDependency_Cell3.AllowedMeasBandwidth,

v_Frequency_IE_Cell1.UL_DL_Earfcn.dl_CarrierFreq,

v_ChBandDependency_Cell1.AllowedMeasBandwidth,

cs_508_MeasGapConfig_GP1, //@sic R5s240105 sic@

f_ReturnMeasObjectToAddModList_2Entries_v9e0(v_Frequency_IE_Cell1.UL_DL_Earfcn.dl_CarrierFreq_v9e0, v_Frequency_IE_Cell3.UL_DL_Earfcn.dl_CarrierFreq_v9e0))));

//@siclog "Step 3" siclog@

//Set cell settings according to T1

f_EUTRA_SetCellPowerList(v_CellPowerList_AtT1);

//@siclog "Step 4" siclog@

//Receive MasurementReport

SRB.receive(car_SRB1_RrcPdu_IND(eutra_Cell1, cr_MeasurementReport(1,//Meas id

?,//RSRP result

?,//RSRQ result

cr_MeasResultNeighCells_measResultListEUTRA_1Entry(v_PhysicalCellIdentity_Cell3,

omit,//CGI info

?,//RSRP result

?))));//RSRQ result

// Activate the default to receive further measurement reports for cell 3 on cell 1

v_DefaultRef := activate (a_EUTRA_AddDefault_PeriodicalMeasurmentReport(eutra_Cell1,

cr_MeasurementReport(1, ?, ?,

cr_MeasResultNeighCells_measResultListEUTRA_1Entry(v_PhysicalCellIdentity_Cell3, omit, ?, ?))));

//@siclog "Step 5-6" siclog

//Reconfigure SS and send RRCConnectionReconfiguration for HO to cell

//Receive RRCConnectionReconfigurationComplete

f_EUTRA_508RRC_IntraLTE_HO_InterCell(eutra_Cell1, eutra_Cell3, v_DrbConfigList, -, -, -, -, -, -, cs_CarrierFreqEUTRA_v9e0(v_Frequency_IE_Cell3.UL_DL_Earfcn.dl_CarrierFreq_r9, omit)); //@sic R5s240105 sic@

//SS automatically checks that UE transmit an RRCConnectionReconfigurationComplete using the security key derived from the currently active KeNB

//* @verdict pass RRCConnectionReconfigurationComplete received on Cell 3

f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 6: Handover performed");

//Deactivate the default to stop receiving measurement reports for cell 3 on cell 1

deactivate(v_DefaultRef);

//@siclog "Step 7" siclog@

// Release eCall over IMS using the generic test procedure described in TS 34.229-1 [35] subclause C.33

f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS2]);     // @sic R5s240105 sic@

f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS2]);

// The SS deactivates the EPS bearer. [This is the rest of C.33]

f_EUTRA_DeactivateEPS_BearerContext (eutra_Cell3,

tsc_EpsDedicatedBearerId2); //@sic R5s240105, R5-235076 sic@

//@siclog "Step 8" siclog@

// The SS releases the RRC connection.

f_EUTRA_RRC_ConnectionRelease(eutra_Cell3); //@sic R5s240105, R5-235076 sic@

f_EUTRA_TestBody_Set(false);

//Switch/power off UE

f_EUTRA_Postamble(eutra_Cell3, E1_IDLE); //@sic R5s240105 sic@

}