3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240316

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3426 | rev | - | Current version: | 17.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC eCPSOR test case 6.3.2.4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test, eCPSOR_CON-UEConTest |  |  |  |  |  | Date: |  |  | 2024-04-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | As per the test purpose UE shall receive SOR Transparent container included in DL NAS TRANSPORT message with Tsor-cm timer value is other than infinity and is smaller than the current value of the running Tsor-cm timer. However, at step 25 in the test procedure, DL NAS TRANSPORT message with new shorter Tsor-cm value of 20sec is sent 49 secs after the 60sec (current Tsor-cm timer value) is started. That means, device will not restart the timer with shorter value (20sec), since current value of the running timer is only 11 secs (< 20sec). |  |  |  |  |  |  |  |  |
| Summary of change: |  | Function fl_TC_6_3_2_4_TestBody is modified to send DL NAS TRANSPORT message with a TSor-CM timer value of 10sec (‘05’O) rather than 20sec (‘0A’O) to make sure current value of the running Tsor-cm (11 Sec) is greater than the new shorter timer (10sec) Please note that a prose CR is associated with this TTCN change. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.2.4.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.523-1 CR 4344 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2022-09_D24wk12 related to the title of this CR.

Contact:	Mohit Kanchan
	mohit.kanchan@keysight.com

# Corrections required.

## Change 1

| Function name | fl_TC_6_3_2_4_TestBody |
| --- | --- |
| Reason for change | As per the test purpose UE shall receive SOR Transparent container included in DL NAS TRANSPORT message with Tsor-cm timer value is other than infinity and is smaller than the current value of the running Tsor-cm timer. However, at step 25 in the test procedure, DL NAS TRANSPORT message with new shorter Tsor-cm value of 20sec is sent 49 secs after the 60sec (current Tsor-cm timer value) is started. That means, device will not restart the timer with shorter value (20sec), since current value of the running timer is only 11 secs (< 20sec). |
| Summary of change | Function fl_TC_6_3_2_4_TestBody is modified to send DL NAS TRANSPORT message with a TSor-CM timer value of 10sec (‘05’O) rather than 20sec (‘0A’O) to make sure current value of the running Tsor-cm (11 Sec) is greater than the new shorter timer (10sec) Please note that a prose CR is associated with this TTCN change. |
| TTCN module | SOR_CMCI_NR5GC |
| MCC160 Comment |  |

Before Change:

function fl_TC_6_3_2_4_TestBody() runs on NR5GC_PTC { var octetstring v_Data; var NAS_PlmnId v_PlmnID := f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (nr_Cell11)); var O2_Type v_AccessTechId :='0800'O; var O1_Type v_SORInfo :='01'O;//SI/SSCMI/SSSI//@sic R5-233459 sic@ var octetstring v_NAS_TxMsgContainer_SOR_CMCI; var O16_Type v_Mac; var B128_Type v_Mac_A17; var NG_NAS_SecurityParams_Type v_NAS_SecurityParams; var octetstring v_CriterionType:= '01'O; var octetstring v_CriterionValue:= f_DomainName_Encode(pc_APN_ID_Internet); var O2_Type v_Length_SOR_CMCI_rule_contents :=int2oct(2+lengthof(v_CriterionValue),2);//@sic R5s230605 sic@ var O2_Type v_Length_SOR_CMCI_contents :=int2oct(4+lengthof(v_CriterionValue),2);//@sic R5s230605 sic@ var octetstring v_TsorcmTimer:= '21'O; var O2_Type v_CounterSOR :='0001'O; var template (value) SOR_Header v_Header := cs_NG_SOR_Header('1'B,'1'B,'1'B,'0'B,'1'B);//@sic R5-233459 sic@ var template (value) NG_NAS_DL_Pdu_Type v_PduToSend; var NR_SRB_COMMON_IND v_ReceivedAsp; var O1_Type v_Length_PlmnID_AccessTechId := int2oct(lengthof(v_PlmnID & v_AccessTechId),1);//@sic R5s230605 sic@ timer t_Sorcm:=60.0; timer t_Wait:=49.0; <<<Skip >>> //@siclog "Step 24" siclog@ //The UE transmits an ULInformationTransfer message carrying acknowledgement of successful reception of the steering of roaming information SRB.receive(car_NR_SRB_NasPdu_IND(nr_Cell12, tsc_NR_RbId_SRB2, cr_NG_NAS_Ind(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp; // @sic R5s201387 Baseline Moving sic@ @sic R5s210026 sic@ if (not f_Check_NG_ULNASTransport(v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg, cr_PayloadContainerType(tsc_PayloadContainerSORMsg), omit)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "UL NAS Transport Message Failed"); } t_Wait.timeout; //@siclog "Step 25-34b3a1" siclog@ v_CounterSOR :='0002'O;//@sic R5s230605 sic@ v_TsorcmTimer:='0A'O; v_Data := v_Length_PlmnID_AccessTechId & v_PlmnID & v_AccessTechId & v_SORInfo & v_Length_SOR_CMCI_contents & v_Length_SOR_CMCI_rule_contents & v_TsorcmTimer & v_CriterionType & v_CriterionValue;//@sic R5s230605 sic@ v_NAS_SecurityParams := f_NR5GC_Security_Get(); v_Mac_A17 :=f_NG_Authentication_A17(bit2oct(encvalue(v_Header)), v_NAS_SecurityParams.KAUSF, v_CounterSOR, v_Data, v_NAS_SecurityParams.KDF); v_Mac := bit2oct(v_Mac_A17); v_NAS_TxMsgContainer_SOR_CMCI := bit2oct(encvalue(cs_NG_SORTransparentContainerValue(v_Header, v_Mac, v_CounterSOR, v_Data, omit, omit)));//@sic R5s240027 sic@ v_PduToSend := f_Get_NG_DLNASTransport (cs_PayloadContainerType(tsc_PayloadContainerSORMsg), omit, cs_PayloadContainer(omit, v_NAS_TxMsgContainer_SOR_CMCI)); t_Sorcm.stop; f_NR5GC_TransportSorCmCheckfunction(nr_Cell12,nr_Cell11,v_PduToSend,-,"Step 29 PASS","Step 31 PASS",-,10.0); }//End of fl_TC_6_3_2_4_TestBody

After Change:

function fl_TC_6_3_2_4_TestBody() runs on NR5GC_PTC { var octetstring v_Data; var NAS_PlmnId v_PlmnID := f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (nr_Cell11)); var O2_Type v_AccessTechId :='0800'O; var O1_Type v_SORInfo :='01'O;//SI/SSCMI/SSSI//@sic R5-233459 sic@ var octetstring v_NAS_TxMsgContainer_SOR_CMCI; var O16_Type v_Mac; var B128_Type v_Mac_A17; var NG_NAS_SecurityParams_Type v_NAS_SecurityParams; var octetstring v_CriterionType:= '01'O; var octetstring v_CriterionValue:= f_DomainName_Encode(pc_APN_ID_Internet); var O2_Type v_Length_SOR_CMCI_rule_contents :=int2oct(2+lengthof(v_CriterionValue),2);//@sic R5s230605 sic@ var O2_Type v_Length_SOR_CMCI_contents :=int2oct(4+lengthof(v_CriterionValue),2);//@sic R5s230605 sic@ var octetstring v_TsorcmTimer:= '21'O; var O2_Type v_CounterSOR :='0001'O; var template (value) SOR_Header v_Header := cs_NG_SOR_Header('1'B,'1'B,'1'B,'0'B,'1'B);//@sic R5-233459 sic@ var template (value) NG_NAS_DL_Pdu_Type v_PduToSend; var NR_SRB_COMMON_IND v_ReceivedAsp; var O1_Type v_Length_PlmnID_AccessTechId := int2oct(lengthof(v_PlmnID & v_AccessTechId),1);//@sic R5s230605 sic@ timer t_Sorcm:=60.0; timer t_Wait:=49.0; <<<Skip >>> //@siclog "Step 24" siclog@ //The UE transmits an ULInformationTransfer message carrying acknowledgement of successful reception of the steering of roaming information SRB.receive(car_NR_SRB_NasPdu_IND(nr_Cell12, tsc_NR_RbId_SRB2, cr_NG_NAS_Ind(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp; // @sic R5s201387 Baseline Moving sic@ @sic R5s210026 sic@ if (not f_Check_NG_ULNASTransport(v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg, cr_PayloadContainerType(tsc_PayloadContainerSORMsg), omit)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "UL NAS Transport Message Failed"); } t_Wait.timeout; //@siclog "Step 25-34b3a1" siclog@ v_CounterSOR :='0002'O;//@sic R5s230605 sic@ v_TsorcmTimer:='05'O; v_Data := v_Length_PlmnID_AccessTechId & v_PlmnID & v_AccessTechId & v_SORInfo & v_Length_SOR_CMCI_contents & v_Length_SOR_CMCI_rule_contents & v_TsorcmTimer & v_CriterionType & v_CriterionValue;//@sic R5s230605 sic@ v_NAS_SecurityParams := f_NR5GC_Security_Get(); v_Mac_A17 :=f_NG_Authentication_A17(bit2oct(encvalue(v_Header)), v_NAS_SecurityParams.KAUSF, v_CounterSOR, v_Data, v_NAS_SecurityParams.KDF); v_Mac := bit2oct(v_Mac_A17); v_NAS_TxMsgContainer_SOR_CMCI := bit2oct(encvalue(cs_NG_SORTransparentContainerValue(v_Header, v_Mac, v_CounterSOR, v_Data, omit, omit)));//@sic R5s240027 sic@ v_PduToSend := f_Get_NG_DLNASTransport (cs_PayloadContainerType(tsc_PayloadContainerSORMsg), omit, cs_PayloadContainer(omit, v_NAS_TxMsgContainer_SOR_CMCI)); t_Sorcm.stop; f_NR5GC_TransportSorCmCheckfunction(nr_Cell12,nr_Cell11,v_PduToSend,-,"Step 29 PASS","Step 31 PASS",-,10.0); }//End of fl_TC_6_3_2_4_TestBody