3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240332

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-5 | CR | 0106 | rev | 1 | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Misc corrections for MCX |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test, MCImp-UEConTest |  |  |  |  |  | Date: |  |  | 2024-03-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - According to 36.579-1 Table 5.5.2.8-1 Max-Forwards shall be 67 but in f_IMS_NotifyRequest_MessageHeader_Common it is 69 per default - Use of domain name and port for f_IMS_SipRequest_ViaTX is confusing and can be simplified by using a single HostPort parameter. - As the host (domain) name of the INVITE's From and Contact header fields are the same, in f_MCX_InviteRequest_MessageHeaderTX the HostPort of v_Contact can be used as parameter for f_IMS_SipRequest_ViaTX. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Max-Forwards corrected in f_MCX_NotifyRequest_MessageHeaderTX - Implementation simplified using HostPort as parameter for f_IMS_SipRequest_ViaTX. - HostPort of v_Contact used as parameter for f_IMS_SipRequest_ViaTX |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent TTCN implementation |  |  |  |  |  |  |  |  |
| Clauses affected: |  | f_MCX_NotifyRequest_MessageHeaderTX, f_IMS_SipRequest_ViaTX, f_IMS_InviteRequest_MessageHeaderTX_Common, f_IMS_UpdateRequest_MessageHeaderTX, f_IMS_ByeRequest_MessageHeaderTX, f_MCX_InviteRequest_MessageHeaderTX, f_MCX_CallReleaseMT_WithoutRRCConnectionRelease, f_MCPTT_SendUPDATE_Receive200OK |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2022-09_D23wk49 related to the title of this CR.

Contact:	Olivier Genoud
	olivier.genoud@etsi.org

# Corrections required

## 2.1 Max-Forwards in SIP NOTIFY

| Function name | f_MCX_NotifyRequest_MessageHeaderTX |
| --- | --- |
| Reason for change | According to 36.579-1 Table 5.5.2.8-1 Max-Forwards shall be 67 but in f_IMS_NotifyRequest_MessageHeader_Common it is 69 per default |
| Summary of change | Max-Forwards corrected in f_MCX_NotifyRequest_MessageHeaderTX |
| TTCN module | MCX_SIP_Registration |

function f_MCX_NotifyRequest_MessageHeaderTX(MCX_ServiceType_Type p_ServiceType, IMS_PTC_DialogIndex_Type p_DialogIndex, SipUrl p_ContactURI, template (value) ContentType p_ContentType) runs on IMS_PTC return template (value) MessageHeader { var EventType v_EventType := f_SIP_SUBSCRIBE_DialogIndex2EventType(p_DialogIndex); var charstring v_PcscfAddress := f_IMS_PTC_Pcscf_Get(); var integer v_MaxForwards := 67; // 36.579-1 Table 5.5.2.8-1 var charstring v_ServerName; var charstring v_PublicServiceId; var CharStringList_Type v_ViaAddressList; var template (value) Via v_Via; var template (value) MessageHeader v_NotifyHeader; select (p_DialogIndex) { case (xcapDiffDialogCMS) { v_ServerName := tsc_MCX_CMS_Hostname; // 36.579-1 Table 5.5.2.8-1 } case (xcapDiffDialogGMS1, xcapDiffDialogGMS2, xcapDiffDialogGMS3, xcapDiffDialogGMS4) { v_ServerName := tsc_MCX_GMS_Hostname; // 36.579-1 Table 5.5.2.8-1 } case else { v_ServerName := f_SIP_SipUrl_GetDomainName(p_ContactURI); } } v_ViaAddressList := {v_PcscfAddress, tsc_IMS_Scscf, v_ServerName}; v_Via := f_IMS_NotifyRequest_ViaTX(v_ViaAddressList); v_NotifyHeader := f_IMS_NotifyRequest_MessageHeader_Common(p_DialogIndex, v_Via, cs_Contact(p_ContactURI), v_EventType, cs_SubscriptionState_Active("7200"), p_ContentType, v_MaxForwards); if (p_DialogIndex == confEventDialog) { // 36.579-2 Table 6.1.3.1.3.3-9 and Table 6.1.3.1.3.3-12 v_PublicServiceId := f_MCX_MCServer_PublicServiceId(p_ServiceType); v_NotifyHeader.pAssertedID := cs_PAssertedId(f_SIP_BuildSipUri_TX(v_PublicServiceId)); v_NotifyHeader.expires := cs_Expires("3600"); v_NotifyHeader.subscriptionState := cs_SubscriptionState_Active("3600"); v_NotifyHeader.pPreferredService := cs_PPreferredService(tsc_MCX_FeatureTag_3gpp_service_ims_icsi_mcptt); /* NOTE: 24.379 clause 6.3.3.4 specifies that the controlling function shall include a P-Preferred-Service header field; 24.379 clause 10.1.3.3 specifies that the participating function forwards the NOTIFY to the client; nevertheless looking at RFC 6050, it does not make much sense to send P-Preferred-Service header field to a client */ }

## 2.2 Via header fields in DL SIP requests

| Function names | f_IMS_SipRequest_ViaTX, f_IMS_InviteRequest_MessageHeaderTX_Common, f_IMS_UpdateRequest_MessageHeaderTX, f_IMS_ByeRequest_MessageHeaderTX, f_MCX_InviteRequest_MessageHeaderTX, f_MCX_CallReleaseMT_WithoutRRCConnectionRelease, f_MCPTT_SendUPDATE_Receive200OK |
| --- | --- |
| Reason for change | - Use of domain name and port for f_IMS_SipRequest_ViaTX is confusing and can be simplified by using a single HostPort parameter. - As the host (domain) name of the INVITE's From and Contact header fields are the same, in f_MCX_InviteRequest_MessageHeaderTX the HostPort of v_Contact can be used as parameter for f_IMS_SipRequest_ViaTX. |
| Summary of change | - Implementation simplified using HostPort as parameter for f_IMS_SipRequest_ViaTX. - HostPort of v_Contact used as parameter for f_IMS_SipRequest_ViaTX |
| TTCN modules | IMS_Procedures_Common, IMS_Procedures_CallControlCommon, MCX_SIP_CallControl_Common, MCPTT_ClientConfiguration_MCX_IMS |

function f_IMS_SipRequest_ViaTX(template (value) HostPort p_CallerHostPort := cs_HostPort(tsc_IMS_CallerDomain, tsc_IMS_CallerPort) ) runs on IMS_PTC return template (value) Via { /* @sic R5s190727 change 2.2: v_TransportProtocol1, v_TransportProtocol2 sic@ */ var template (value) ViaBody_List v_ViaBodyList; var template (omit) GenericParam v_SigCompParam := f_IMS_PTC_ImsInfo_GetSigCompParam(); // omit when sigcomp is not started var charstring v_Host := f_IMS_PTC_Pcscf_Get(); var integer v_Port_ps := f_IMS_PTC_GetPort_ps(); var InternetProtocol_Type v_InternetProtocol := f_IMS_PTC_ImsInfo_GetTransportProtocol(); var charstring v_TransportProtocol1 := f_IMS_ConvertTransportProtocolToString(v_InternetProtocol); /* @sic NOTE: acc. to discussions regarding rejection of R5-130765 at RAN WG5 Meeting #58 UDP is used for all requests in DL sic@ */ var charstring v_TransportProtocol2 := "UDP"; /* acc. to via parameter in TS 34.229 A.2.9 udp shall be used */ var boolean v_IsMCX := f_IMS_PTC_TestsuiteInfo_IsMCX(); if (v_IsMCX) { /* @sic R5-196983 sic@ !!!! PROSE ISSUE: via should correspond to the RecordRoute => at least for INVITE and BYE it should be the same as for IMS !!!! */ v_ViaBodyList := { cs_ViaBody(v_TransportProtocol1, cs_HostPort(v_Host, v_Port_ps), f_ViaParamsTX(f_IMS_PTC_ImsInfo_GetNextBranch(), v_SigCompParam)), cs_ViaBody(v_TransportProtocol2, p_CallerHostPort, f_ViaParamsTX(f_IMS_PTC_ImsInfo_GetNextBranch())) }; } else { v_ViaBodyList := { cs_ViaBody(v_TransportProtocol1, cs_HostPort(v_Host, v_Port_ps), f_ViaParamsTX(f_IMS_PTC_ImsInfo_GetNextBranch(), v_SigCompParam)), cs_ViaBody(v_TransportProtocol2, cs_HostPort("scscf1.3gpp.org"), f_ViaParamsTX(f_IMS_PTC_ImsInfo_GetNextBranch())), cs_ViaBody(v_TransportProtocol2, cs_HostPort("scscf2.3gpp.org"), f_ViaParamsTX(f_IMS_PTC_ImsInfo_GetNextBranch())), cs_ViaBody(v_TransportProtocol2, cs_HostPort("pcscf2.3gpp.org"), f_ViaParamsTX(f_IMS_PTC_ImsInfo_GetNextBranch())), cs_ViaBody(v_TransportProtocol2, p_CallerHostPort, f_ViaParamsTX(f_IMS_PTC_ImsInfo_GetNextBranch())) }; } return cs_Via(v_ViaBodyList); }

function f_IMS_InviteRequest_MessageHeaderTX_Common(OptionTag_List p_OptionTagsForSupported := tsc_OptionTagList_Invite_C11, boolean p_IsReInvite := false, charstring p_CallerSipAddrStr := tsc_IMS_CallerSipAddrStr, template (value) integer p_CallerHostPort := cs_HostPort(tsc_IMS_CallerDomain, tsc_IMS_CallerPort), integer p_CSeqValue := 4711) runs on IMS_PTC return template (value) MessageHeader { /* @sic R5s200210: p_CallerDomain sic@ */ var template (value) MessageHeader v_MessageHeader_Invite := cs_MessageHeader_Dummy; var template (value) SipUrl v_PublicUserId_SipUrl := f_IMS_PTC_ImsInfo_GetSipUriTX(defaultId); // @sic R5-133304: use default public user id sic@ var charstring v_CallIdString := f_IMS_PTC_GenerateCallId("INVITE"); var charstring v_FromTag := f_IMS_GenerateTag("INVITE", "From"); var template (value) RouteBody_List v_RouteBodyList; var integer v_CSeqValue; var template (value) From v_From; var template (value) To v_To; var template (value) CallId v_CallId; if (not p_IsReInvite) { v_CSeqValue := p_CSeqValue; v_From := cs_From(f_SIP_BuildSipUri_TX(p_CallerSipAddrStr), v_FromTag); v_To := cs_ToDef(v_PublicUserId_SipUrl); v_CallId := cs_CallId(v_CallIdString); v_RouteBodyList := f_IMS_RouteSet_MT_Call_TX(); /* @sic R5s140324 - MCC160 Implementation sic@ */ } else { v_CSeqValue := f_IMS_PTC_ImsInfo_CseqIncr(dialogLocal); v_From := f_IMS_RequestInDialog_FromHeaderTX(); v_To := f_IMS_RequestInDialog_ToHeaderTX(); v_CallId := f_IMS_PTC_ImsInfo_DialogGetCallId(); if (f_IMS_PTC_ImsInfo_DialogIsMO()) { /* @sic R5s140324 - MCC160 Implementation sic@ */ v_RouteBodyList := f_IMS_RouteSet_ReverseOrder(f_IMS_RouteSet_MO_Call_TX()); /* @sic R5-142320, R5-144601 sic@ */ } else { v_RouteBodyList := f_IMS_RouteSet_MT_Call_TX(); } } v_MessageHeader_Invite.via := f_IMS_SipRequest_ViaTX(p_CallerHostPort); /* @sic R5-134116: New function fl_IMS_InviteRequest_ViaTX sic@ */ v_MessageHeader_Invite.recordRoute := cs_RecordRoute(v_RouteBodyList); v_MessageHeader_Invite.fromField := v_From; v_MessageHeader_Invite.toField := v_To; v_MessageHeader_Invite.callId := v_CallId; v_MessageHeader_Invite.cSeq := cs_CseqDef(v_CSeqValue, "INVITE"); v_MessageHeader_Invite.supported := cs_Supported(p_OptionTagsForSupported); v_MessageHeader_Invite.pCalledPartyID := cs_PCalledPartyId(v_PublicUserId_SipUrl); v_MessageHeader_Invite.maxForwards := f_IMS_SipRequest_MaxForwardsTX(); /* @sic R5-203073: Max-Forwards for MCX sic@ */ if (not p_IsReInvite) { f_IMS_PTC_ImsInfo_DialogInit(dialogLocal, valueof(v_MessageHeader_Invite)); } f_IMS_PTC_ImsInfo_DialogSetRemoteRSeq(omit); /* reset RSeq for every (re-)INVITE @sic R5-181190 sic@ */ return v_MessageHeader_Invite; }

function f_IMS_UpdateRequest_MessageHeaderTX(template (value) INVITE_Request p_InviteRequest, IMS_PreconditionIndication_Type p_PreconditionIndication := NoPreconditionIndication, template (omit) Contact p_Contact := omit, template (value) integer p_CallerHostPort := cs_HostPort(tsc_IMS_CallerDomain, tsc_IMS_CallerPort) ) runs on IMS_PTC return template (value) MessageHeader { /* @sic R5-160790, R5-160795: p_PreconditionIndication sic@ */ /* @sic R5-174544: SessionId removed sic@ */ /* @sic R5s200206 change 2.1: p_Contact and p_CallerDomain; p_Contact shall be present for MO Call sic@ */ var template (value) MessageHeader v_MessageHeader_Invite := p_InviteRequest.msgHeader; var template (value) MessageHeader v_MessageHeader_Update := cs_MessageHeader_Dummy; var integer v_CSeqValue := f_IMS_PTC_ImsInfo_CseqIncr(dialogLocal); var template (omit) Contact v_Contact; var template (omit) Require v_Require := omit; var template (omit) Supported v_Supported := omit; select (p_PreconditionIndication) { case (PreconditionRequired) { v_Require := cs_Require(tsc_OptionTagList_precondition); } /* @sic R5-160790 sic@ */ case (PreconditionSupported) { v_Supported := cs_Supported(tsc_OptionTagList_precondition); } /* @sic R5-160795 sic@ */ } if (fl_IMS_InviteRequestIsMO(p_InviteRequest)) { /* @sic R5s200206 change 2.1 sic@ */ v_Contact := p_Contact; } else { v_Contact := v_MessageHeader_Invite.contact; } v_MessageHeader_Update.via := f_IMS_SipRequest_ViaTX(p_CallerHostPort); /* @sic R5-200457 sic@ @sic R5s210144 sic@ */ v_MessageHeader_Update.toField := f_IMS_RequestInDialog_ToHeaderTX(); v_MessageHeader_Update.fromField := f_IMS_RequestInDialog_FromHeaderTX(); v_MessageHeader_Update.callId := v_MessageHeader_Invite.callId; v_MessageHeader_Update.cSeq := cs_CseqDef(v_CSeqValue, "UPDATE"); v_MessageHeader_Update.maxForwards := f_IMS_SipRequest_MaxForwardsTX(); /* @sic R5-203073: Max-Forwards for MCX sic@ */ v_MessageHeader_Update.contentType := cs_ContentTypeSDP; v_MessageHeader_Update.contact := v_Contact; /* @sic R5-150692 sic@ */ v_MessageHeader_Update.require := v_Require; /* @sic R5-160790 sic@ */ v_MessageHeader_Update.supported := v_Supported; /* @sic R5-160795 sic@ */ return v_MessageHeader_Update; }

function f_IMS_ByeRequest_MessageHeaderTX(template (omit) ContentType p_ContentType := omit, template (value) integer p_CallerHostPort := cs_HostPort(tsc_IMS_CallerDomain, tsc_IMS_CallerPort) ) runs on IMS_PTC return template (value) MessageHeader { /* @sic R5s130756 additional changes: p_IsGIBA removed sic@ */ /* @sic R5-174544: SessionId removed sic@ */ /* @sic R5s201620: p_InviteRequest removed sic@ */ var template (value) MessageHeader v_MessageHeader_Bye := cs_MessageHeader_Dummy; var template (value) Via v_Via := f_IMS_SipRequest_ViaTX(p_CallerHostPort); /* @sic R5s190882, R5s200210: p_CallerDomain sic@ @sic R5-200457 sic@ */ var template (value) To v_To := f_IMS_RequestInDialog_ToHeaderTX(); var template (value) From v_From := f_IMS_RequestInDialog_FromHeaderTX(); var template (value) CallId v_CallId := f_IMS_PTC_ImsInfo_DialogGetCallId(); var integer v_CSeqValue := f_IMS_PTC_ImsInfo_CseqIncr(dialogLocal); v_MessageHeader_Bye.via := v_Via; v_MessageHeader_Bye.callId := v_CallId; /* @sic R5s201620 sic@ */ v_MessageHeader_Bye.cSeq := cs_CseqDef(v_CSeqValue, "BYE"); v_MessageHeader_Bye.maxForwards := f_IMS_SipRequest_MaxForwardsTX(); /* @sic R5-203073: Max-Forwards for MCX sic@ */ v_MessageHeader_Bye.toField := v_To; v_MessageHeader_Bye.fromField := v_From; v_MessageHeader_Bye.contentType := p_ContentType; return v_MessageHeader_Bye; }

function f_MCX_InviteRequest_MessageHeaderTX(MCX_ServiceType_Type p_ServiceType, MCX_CommencementMode_Type p_CommencementMode := NONE, template (omit) ContentType p_ContentType, boolean p_IsReInvite := false) runs on IMS_PTC return template (value) MessageHeader { /* @sic R5w220006, R5-223945: p_PriorityCallType removed; resourcePriority removed from MessageHeader sic@ */ /* @sic R5w220111, R5-223945: global variable ContactUrlSS => p_Contact removed sic@ */ var charstring v_PublicServiceId := f_MCX_MCServer_PublicServiceId(p_ServiceType); // SIP URI to be used in From header field var template (value) SipUrl v_PublicServiceIdSipUrl := f_SIP_BuildSipUri_TX(v_PublicServiceId); // @sic R5s220869 Change 7: f_SIP_BuildSipUri_TX instead of f_SIP_BuildSipUri_lr_TX sic@ var OptionTag_List v_OptionTagsForSupported := { "100rel", "timer", "tdialog", "norefersub" }; var template (value) Contact v_Contact := f_MCX_CallSetupMO_ContactSS(p_ServiceType); /* @sic R5w220111, R5-223945: Sesssion id instead of PSI sic@ NOTE: f_MCX_IMS_PTC_ImsInfo_SetContactUrlSS needs to be called beforehand */ var template (value) SipUrl v_ContactSipUrl := f_Contact_GetSipUrl(v_Contact); var template (value) HostPort v_HostPort := v_ContactSipUrl.components.sip.hostPort; var template (value) MessageHeader v_MessageHeader_Invite := f_IMS_InviteRequest_MessageHeaderTX_Common(v_OptionTagsForSupported, p_IsReInvite, v_PublicServiceId, v_HostPort); var template (value) GenericParamList_Type v_AcceptContactParams := { crs_MCX_FeatureParam_3gpp_icsi_ref(p_ServiceType), crs_MCX_FeatureTag_g_3gpp_mcs(p_ServiceType) }; v_MessageHeader_Invite.sessionExpires := cs_SessionExpires(1800); v_MessageHeader_Invite.pEarlyMedia := cs_PEarlyMedia({"inactive"}); v_MessageHeader_Invite.require := cs_Require(tsc_OptionTagList_sec_agree); v_MessageHeader_Invite.proxyRequire := cs_ProxyRequire(tsc_OptionTagList_sec_agree); v_MessageHeader_Invite.pAssertedID := cs_PAssertedId(v_PublicServiceIdSipUrl); v_MessageHeader_Invite.contact := v_Contact; v_MessageHeader_Invite.accept := cs_Accept({cs_AcceptBody(c_sdpApplication), cs_AcceptBody(f_MCX_MimeType_mcs_info(p_ServiceType))}); v_MessageHeader_Invite.acceptContact := cs_AcceptContact_Common(f_MCX_AcceptContact_AcValues_Common_TX(v_AcceptContactParams)); v_MessageHeader_Invite.contentType := p_ContentType; if (not p_IsReInvite) { select (p_CommencementMode) { case (AUTOMATIC) { v_MessageHeader_Invite.answerMode := cs_AnswerMode_Auto; } case (AUTOMATIC_FORCE, AMBIENT_LISTENING_LOCAL) { v_MessageHeader_Invite.privAnswerMode := cs_PrivAnswerMode_Auto; } case (AMBIENT_LISTENING_REMOTE, AMBIENT_VIEWING_REMOTE) { v_MessageHeader_Invite.privAnswerMode := cs_PrivAnswerMode_Auto; v_MessageHeader_Invite.alertInfo := cs_AlertInfo("<file:///dev/null>"); } case (MANUAL) { v_MessageHeader_Invite.answerMode := cs_AnswerMode_Manual; } case (PRIV_MANUAL) { v_MessageHeader_Invite.privAnswerMode := cs_PrivAnswerMode_Manual; } } } return v_MessageHeader_Invite; }

function f_MCX_CallReleaseMT_WithoutRRCConnectionRelease(template (omit) charstring p_RequestUri := omit, template (omit) Reason p_Reason := omit, template (omit) ContentType p_ContentType := omit, template (omit) MessageBody p_MessageBody := omit) runs on IMS_PTC { var template (value) SipUrl v_ContactSipUrlSS := f_IMS_PTC_ImsInfo_GetContactUrlSS(); var template (value) HostPort v_HostPortSS := v_ContactSipUrlSS.components.sip.hostPort; var template (value) MessageHeader v_MessageHeader_Bye := f_IMS_ByeRequest_MessageHeaderTX(p_ContentType, v_HostPortSS); var template (present) MessageHeader v_MessageHeader_200OK := f_MCX_CommonResponse_200_MessageHeaderRX(v_MessageHeader_Bye); var template (value) SipUrl v_RequestSipUri; v_MessageHeader_Bye.reason := p_Reason; if (isvalue(p_RequestUri)) { v_RequestSipUri := f_SIP_BuildSipUri_TX(valueof(p_RequestUri)); } else { v_RequestSipUri := f_IMS_PTC_ImsInfo_GetContactUrlUE(dialog); } /* @sic R5-216669: pAssertedID removed sic@ */ f_IMS_CallReleaseMT_Common(v_RequestSipUri, v_MessageHeader_Bye, v_MessageHeader_200OK, p_MessageBody); }

function f_MCPTT_SendUPDATE_Receive200OK(IMS_InviteRequestWithSdp_Type p_InviteRequestWithSdp, template (value) SDP_Message p_SdpOffer, template (present) MCPTT_MCVideo_SdpInfoRX_Type p_SdpInfoAnswer) runs on MCX_IMS_PTC { var charstring v_PublicServiceId := f_MCX_MCServer_PublicServiceId(MCPTT); var SipUrl v_PublicServiceIdSipUrl := f_SIP_ParseURI(v_PublicServiceId); var charstring v_DomainName := f_SIP_SipUrl_GetDomainName(v_PublicServiceIdSipUrl); // Host name of the SIP URI being used in the From header var SipUrl v_ContactUrlUE := f_IMS_PTC_ImsInfo_GetContactUrlUE(dialog); var template (value) Contact v_ContactSS := f_MCX_CallSetupMO_ContactSS(MCPTT); // same Contact as in the response for the INVITE creating the dialog (200 OK) var template (value) v_ContactSipUrlSS := f_Contact_GetSipUrl(v_ContactSS); var template (value) v_HostPortSS := v_ContactSipUrlSS.components.sip.hostPort; var InternetProtocol_Type v_Protocol := f_IMS_PTC_ImsInfo_GetTransportProtocol(); var template (value) MessageHeader v_MessageHeaderUpdate; var template (value) IMS_DATA_REQ v_IMS_DATA_REQ; v_MessageHeaderUpdate := f_IMS_UpdateRequest_MessageHeaderTX(p_InviteRequestWithSdp.Invite, -, v_ContactSS, v_HostPortSS); v_IMS_DATA_REQ := cas_IMS_Update_Request(cs_IMS_RoutingInfo(v_Protocol), cs_UPDATE_Request(v_ContactUrlUE, v_MessageHeaderUpdate, cs_MessageBody_SDP(p_SdpOffer))); IMS_Client.send(v_IMS_DATA_REQ); f_MCPTT_MCVideo_InviteUpdateMT_Receive200OK(MCPTT, -, v_MessageHeaderUpdate, v_Protocol, -, PRE_EST_SESSION_ESTAB_MOD, p_SdpInfoAnswer); }