3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240336

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-3 | CR | 0943 | rev | 1 | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to IMS 5GS Call Control test case 7.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-03-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | This test case allows different behaviors after the call has not been established due to SS sending 503 (Service Unavailable), there are three possibilities after waiting 30 seconds: 1. UE retries and a Call with preconditions is placed: 2. UE retries and a Call without preconditions is placed. 3. UE does not rety and test case ends after some time. Currently postamble is set as: f_IMS_CC_PostambleWithCallReleaseMT(IPCAN_MO_SpeechCall); Inside this function, first step will be to release the call but this will not work for point 3 as there is no call placed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added an alternative postamble that will be chosen in case UE does not retry IMS call after 503 is sent by SS. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE can fail this test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections required	2

2.1	Correction to function f_TC_7_1_NR5GC_IMS1	2

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D23wk49 related to the title of this CR.

Contact: 	Juan Garcia Martin

Juan.GarciaMartin@rohde-schwarz.com

# Corrections required

## Correction to function f_TC_7_1_NR5GC_IMS1

| Function name | f_TC_7_1_NR5GC_IMS1 |
| --- | --- |
| Reason for change | First step inside current postamble will be to release the call but this will not work when UE behaviour is not to retry the call after 503 (Service Unavailable) is sent. |
| Summary of change | Added an alternative postamble that will be chosen in case UE does not retry IMS call after 503 (Service Unavailable) is sent by SS. |
| TTCN module | IMS\NR5GC_7\IMS_NR5GC_CallControlTestcases.ttcn |
| MCC160 Comment |  |

Before Change:

<<SKIPPED CODE>> function f_TC_7_1_NR5GC_IMS1() runs on IMS_PTC { // MTSI MO Voice Call / 503 Service Unavailable / 5GS var template (present) SipUrl v_CalleeUri := f_IMS_INVITE_CalleeUriRX(); // as used per default in the INVITE and in this test case for the ACK as the UE has not gotten any callee's contact URI var IMS_InviteRequestWithSdp_Type v_InviteRequestWithSdp; var INVITE_Request v_InviteRequest; var template (value) MessageHeader v_MessageHeader503; var template (present) MessageHeader v_MessageHeaderACK; var template (value) IMS_RoutingInfo_Type v_RoutingInfo_DL; var integer v_RetryAfter := 20; // @sic R5-210882: 60s -> 20s sic@ var float v_RetryAfterMin := f_IMS_SetTimerToleranceMin(int2float(v_RetryAfter)); var boolean v_InviteWithPreconditions; timer t_Timer; f_IMS_CC_Preamble(IPCAN_SpeechCall, IMS_REGISTERED); f_IMS_TestBody_Set(true); // @siclog "Step 1" siclog@ f_IMS_CC_StartCall(IPCAN_MO_SpeechCall); // @siclog "Step 2 - 7" siclog@ v_InviteWithPreconditions := fl_IMS_MOCallSetup_CheckInviteWithPreconditions(); if (v_InviteWithPreconditions) { // @sic R5-213521 sic@ // @siclog "Step 8a" siclog@ INVITE v_InviteRequestWithSdp := f_IMS_MOCallSetup_NR5GC_A_4_1a_Step1(v_CalleeUri); // @sic R5-225925, R5-236507: A_4_1 -> A_4_1a sic@ } else { // @siclog "Step 8b" siclog@ INVITE v_InviteRequestWithSdp := f_IMS_MOCallSetup_NR5GC_A_4_2a_Step1(v_CalleeUri); // @sic R5-236507: A_4_2 -> A_4_2a sic@ } v_InviteRequest := v_InviteRequestWithSdp.Invite; // @siclog "Step 9" siclog@ 503 Service Unavailable v_MessageHeader503 := f_IMS_CommonResponse_503_MessageHeaderTX(v_InviteRequest.msgHeader, cs_RetryAfter(v_RetryAfter)); v_RoutingInfo_DL := f_IMS_RoutingInfo_ULtoDL(v_InviteRequestWithSdp.RoutingInfo); IMS_Server.send(cas_IMS_DATA_RSP(v_RoutingInfo_DL, cs_Response(c_statusLine503, v_MessageHeader503))); // @siclog "Step 10" siclog@ ACK v_MessageHeaderACK := f_IMS_AckRequest_MessageHeaderRX(v_InviteRequest, c_statusLine503); f_IMS_ReceiveRequest(car_IMS_Ack_Request(cr_ACK_Request(v_CalleeUri, v_MessageHeaderACK))); f_IMS_PTC_ImsInfo_DialogRelease(); // @siclog "Step 11 - 13" siclog@ Check: Does the UE (not) transmit INVITE request message f_Delay(v_RetryAfterMin); t_Timer.start(30.0); alt { // @sic R5-216210 sic@ [] IMS_Server.check(receive(car_IMS_Invite_Request)) { t_Timer.stop; if (v_InviteWithPreconditions) { // @sic R5-213521 sic@ // @siclog "Step 14a1" siclog@ INVITE v_InviteRequestWithSdp := f_IMS_MOCallSetup_NR5GC_A_4_1a_Step1(v_CalleeUri); // @sic R5-225925, R5-236507: A_4_1 -> A_4_1a sic@ f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 14a1"); // @siclog "Step 14a2-14a15" siclog@ Steps 2 to 12 of A.4.1a f_IMS_MOCallSetup_NR5GC_A_4_1a_Step2_12(v_InviteRequestWithSdp); // @sic R5-225925, R5-236507: A_4_1 -> A_4_1a sic@ } else { // @siclog "Step 14b1" siclog@ INVITE v_InviteRequestWithSdp := f_IMS_MOCallSetup_NR5GC_A_4_2a_Step1(v_CalleeUri); // @sic R5-236507: A_4_2 -> A_4_2a sic@ f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 14b1"); // @siclog "Step 14b2-14b8" siclog@ Steps 2 to 8 of subclause A.4.2a f_IMS_MOCallSetup_NR5GC_A_4_2a_Step2_8(v_InviteRequestWithSdp); // @sic R5-236507: A_4_2 -> A_4_2a sic@ } } [] t_Timer.timeout { f_IMS_CC_AbortCall(); // @sic R5s230086 sic@ } } f_IMS_TestBody_Set(false); // Call release and postamble: f_IMS_CC_PostambleWithCallReleaseMT(IPCAN_MO_SpeechCall); }<<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> function f_TC_7_1_NR5GC_IMS1() runs on IMS_PTC { // MTSI MO Voice Call / 503 Service Unavailable / 5GS var template (present) SipUrl v_CalleeUri := f_IMS_INVITE_CalleeUriRX(); // as used per default in the INVITE and in this test case for the ACK as the UE has not gotten any callee's contact URI var IMS_InviteRequestWithSdp_Type v_InviteRequestWithSdp; var INVITE_Request v_InviteRequest; var template (value) MessageHeader v_MessageHeader503; var template (present) MessageHeader v_MessageHeaderACK; var template (value) IMS_RoutingInfo_Type v_RoutingInfo_DL; var integer v_RetryAfter := 20; // @sic R5-210882: 60s -> 20s sic@ var float v_RetryAfterMin := f_IMS_SetTimerToleranceMin(int2float(v_RetryAfter)); var boolean v_InviteWithPreconditions; var boolean v_MOVoiceCallEstablished := true; timer t_Timer; f_IMS_CC_Preamble(IPCAN_SpeechCall, IMS_REGISTERED); f_IMS_TestBody_Set(true); // @siclog "Step 1" siclog@ f_IMS_CC_StartCall(IPCAN_MO_SpeechCall); // @siclog "Step 2 - 7" siclog@ v_InviteWithPreconditions := fl_IMS_MOCallSetup_CheckInviteWithPreconditions(); if (v_InviteWithPreconditions) { // @sic R5-213521 sic@ // @siclog "Step 8a" siclog@ INVITE v_InviteRequestWithSdp := f_IMS_MOCallSetup_NR5GC_A_4_1a_Step1(v_CalleeUri); // @sic R5-225925, R5-236507: A_4_1 -> A_4_1a sic@ } else { // @siclog "Step 8b" siclog@ INVITE v_InviteRequestWithSdp := f_IMS_MOCallSetup_NR5GC_A_4_2a_Step1(v_CalleeUri); // @sic R5-236507: A_4_2 -> A_4_2a sic@ } v_InviteRequest := v_InviteRequestWithSdp.Invite; // @siclog "Step 9" siclog@ 503 Service Unavailable v_MessageHeader503 := f_IMS_CommonResponse_503_MessageHeaderTX(v_InviteRequest.msgHeader, cs_RetryAfter(v_RetryAfter)); v_RoutingInfo_DL := f_IMS_RoutingInfo_ULtoDL(v_InviteRequestWithSdp.RoutingInfo); IMS_Server.send(cas_IMS_DATA_RSP(v_RoutingInfo_DL, cs_Response(c_statusLine503, v_MessageHeader503))); // @siclog "Step 10" siclog@ ACK v_MessageHeaderACK := f_IMS_AckRequest_MessageHeaderRX(v_InviteRequest, c_statusLine503); f_IMS_ReceiveRequest(car_IMS_Ack_Request(cr_ACK_Request(v_CalleeUri, v_MessageHeaderACK))); f_IMS_PTC_ImsInfo_DialogRelease(); // @siclog "Step 11 - 13" siclog@ Check: Does the UE (not) transmit INVITE request message f_Delay(v_RetryAfterMin); t_Timer.start(30.0); alt { // @sic R5-216210 sic@ [] IMS_Server.check(receive(car_IMS_Invite_Request)) { t_Timer.stop; if (v_InviteWithPreconditions) { // @sic R5-213521 sic@ // @siclog "Step 14a1" siclog@ INVITE v_InviteRequestWithSdp := f_IMS_MOCallSetup_NR5GC_A_4_1a_Step1(v_CalleeUri); // @sic R5-225925, R5-236507: A_4_1 -> A_4_1a sic@ f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 14a1"); // @siclog "Step 14a2-14a15" siclog@ Steps 2 to 12 of A.4.1a f_IMS_MOCallSetup_NR5GC_A_4_1a_Step2_12(v_InviteRequestWithSdp); // @sic R5-225925, R5-236507: A_4_1 -> A_4_1a sic@ } else { // @siclog "Step 14b1" siclog@ INVITE v_InviteRequestWithSdp := f_IMS_MOCallSetup_NR5GC_A_4_2a_Step1(v_CalleeUri); // @sic R5-236507: A_4_2 -> A_4_2a sic@ f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 14b1"); // @siclog "Step 14b2-14b8" siclog@ Steps 2 to 8 of subclause A.4.2a f_IMS_MOCallSetup_NR5GC_A_4_2a_Step2_8(v_InviteRequestWithSdp); // @sic R5-236507: A_4_2 -> A_4_2a sic@ } } [] t_Timer.timeout { f_IMS_CC_AbortCall(); // @sic R5s230086 sic@ v_MOVoiceCallEstablished := false; } } f_IMS_TestBody_Set(false); // Call release if established and postamble: if (v_MOVoiceCallEstablished){ f_IMS_CC_PostambleWithCallReleaseMT(IPCAN_MO_SpeechCall); } else{ f_IMS_CC_Postamble(IPCAN_MO_IMS_Signalling); } } <<SKIPPED CODE>>