3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240338

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4796 | rev | 1 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NBIOT testcase 22.4.24 (for NTN) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test, NB_IOTenh-UEConTest |  |  |  |  |  | Date: |  |  | 2024-03-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In f_TC_22_4_24_NBIOT, during the Init system information combination setup there is an if conditional which sets the sys info combination to c12. This is no longer needed due to sys info combinations for NTN are now changed within the init function itself if NTN is configured in PICS (see R5s240138 and R5s240168). |  |  |  |  |  |  |  |  |
| Summary of change: |  | Removed the if conditional branch for NTN configuration at the start of the test body. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 22.4.24 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Overview	4

2	Corrections required	5

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D23wk49 related to the title of this CR.

Contact:	Parikshit Bhise

Parikshit.Bhise@rohde-schwarz.com

# Corrections required

  - Correction to function f_TC_22_4_24_NBIOT

| Function name | f_TC_22_4_24_NBIOT |
| --- | --- |
| Reason for change | During the Init system information combination setup there is an if conditional which sets the sys info combination to c12. This is no longer needed due to sys info combinations for NTN are now changed within the init function itself if NTN is configured in PICS (see R5s240138 and R5s240168) and will cause an "Invalid sysinfo combination" "Invalid sysinfo combination" error |
| Summary of change | Removed if conditional statement and f_PreconfigureUE_ForNTN function that is handled in the preamble. This leaves the base Init(c6) which is changed to c12within the Init function itself. |
| TTCN module | NBIOT_RRC_Paging.ttcn |
| MCC160 Comment |  |

Before

function f_TC_22_4_24_NBIOT() runs on NBIOT_PTC

{// NB-IoT / RRC / Paging for connection in idle mode / Non-anchor carrier

var float v_WaitForConnection := f_NBIOT_SetTimerToleranceMax(rrcTimer, 10.0);

var boolean v_Rxd_RRCConnReq := false;

var template (value) SystemInformationBlockType22_NB_r14 v_SIB22;

var NB_CarrierFreq_Type v_NB_CarrierFreq;

var template (value) NB_NonAnchorCarrierCommonDL_List_Type v_NB_NonAnchorCarrierCommonDL_List;

var template (value) DL_ConfigCommon_NB_r14 v_DL_ConfigCommon;

var integer v_NonAnchorCarrierId := 1;

var integer v_AnchorCarrierId := 0;  //@sic R5s230349 sic@

if(pc_NB_ntn_only_Connectivity_EPC){ //@sic R5-235333 sic@

f_NBIOT_Init(c12);

f_PreconfigureUE_ForNTN(UT);

}

else{

f_NBIOT_Init(c6);

}

v_NB_CarrierFreq := f_NBIOT_CellInfo_GetEARFCN(nbiot_Cell1);

After

function f_TC_22_4_24_NBIOT() runs on NBIOT_PTC

{// NB-IoT / RRC / Paging for connection in idle mode / Non-anchor carrier

var float v_WaitForConnection := f_NBIOT_SetTimerToleranceMax(rrcTimer, 10.0);

var boolean v_Rxd_RRCConnReq := false;

var template (value) SystemInformationBlockType22_NB_r14 v_SIB22;

var NB_CarrierFreq_Type v_NB_CarrierFreq;

var template (value) NB_NonAnchorCarrierCommonDL_List_Type v_NB_NonAnchorCarrierCommonDL_List;

var template (value) DL_ConfigCommon_NB_r14 v_DL_ConfigCommon;

var integer v_NonAnchorCarrierId := 1;

var integer v_AnchorCarrierId := 0;  //@sic R5s230349 sic@

f_NBIOT_Init(c6);

v_NB_CarrierFreq := f_NBIOT_CellInfo_GetEARFCN(nbiot_Cell1);