3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240349

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4802 | rev | 1 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

s

| Title: | Correction to the NB-NTN testcases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI13_Test, NB_IOT-UEConTest |  |  |  |  |  | Date: |  |  | 2024-03-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - The function f_NBIOT_DelayForUserPlaneSignalling_CP checks if NTN is configured and sets the delay accordingly, the condition to set v_IsNTN is incorrect and should use !=. - f_NBIOT_GetNPDCCH_PeriodBundle is used to get the no of NPDCCH period and this is later passed into other functions like f_NBIOT_IncrementSearchSpace, the expectation here is that the parameter passed is no of NPDCCH period and not the NPDCCH period bundle. The GetNPDCCH_PeriodBundle currently returns the NPDCCH period bundle (640) and in function f_NBIOT_IncrementSearchSpace this is further multiplied by v_CycleLength and incremented for activation time calculation, this results in incorrect timings for PDUs to be sent. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Updated the condition to check != ntn_None. - Updated the function to return no of periods (10 for NTN and 1 for legacy) and this can be used in other functions like f_NBIOT_IncrementSearchSpace to get the bundle period (640msec). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | ALL NTN test cases |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections to NTN test case execution	2

2.1	Correction to function f_NBIOT_DelayForUserPlaneSignalling_CP	2

2.2	Correction to function f_NBIOT_GetNPDCCH_PeriodBundle	4

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D24wk12 related to the title of this CR.

Contact: 	Juan Garcia Martin

Juan.GarciaMartin@rohde-schwarz.com

# Corrections to NTN test case execution

## Correction to function f_NBIOT_DelayForUserPlaneSignalling_CP

| Function name | f_NBIOT_DelayForUserPlaneSignalling_CP |
| --- | --- |
| Reason for change | The function checks if NTN is configured and sets the delay accordingly, the condition to set v_IsNTN is incorrect and should use != |
| Summary of change | Updated the condition to check != ntn_None |
| TTCN module | NBIOT_NASSteps |
| MCC160 Comment |  |

Before Change:

function f_NBIOT_DelayForUserPlaneSignalling_CP(NBIOT_CellId_Type p_CellId) runs on NBIOT_PTC { var NB_SRB_COMMON_IND v_NB_SRB_COMMON_IND; var template (value) UserDataContainer v_DataContainer; var EPS_BearerIdentity v_BearerId; var ProcedureTransactionIdentifier v_PTI; var octetstring v_DataReply; var boolean v_IsNTN := f_NBIOT_MobileInfo_GetNTNSatelliteType() == ntn_None; timer t_WaitForIPSignalling; if (v_IsNTN) //@sic R5s240246, R5-234501 sic@ { t_WaitForIPSignalling.start(2.48); } else { t_WaitForIPSignalling.start(1.2); } alt { [] t_WaitForIPSignalling.timeout { // just continue } [] SRB.receive(car_NB_SRB_NasPdu_IND(p_CellId, tsc_SRB1bis, cr_NAS_Indication(tsc_SHT_IntegrityProtected_Ciphered, cr_ESM_DATA_TRANSPORT(?, cr_UserDataContainer))))-> value v_NB_SRB_COMMON_IND { v_BearerId := v_NB_SRB_COMMON_IND.Signalling.Nas[0].Pdu.Msg.eSM_DATA_TRANSPORT.epsBearerId; v_PTI := v_NB_SRB_COMMON_IND.Signalling.Nas[0].Pdu.Msg.eSM_DATA_TRANSPORT.procedureTransactionIdentifier; v_DataReply := f_NBIOT_ProcessIPv6Address(v_NB_SRB_COMMON_IND.Signalling.Nas[0].Pdu.Msg.eSM_DATA_TRANSPORT.userDatacontainer.dataContainer); v_DataContainer := cs_UserDataContainer(v_DataReply); SRB.send(cas_NB_SRB_NasPdu_REQ(p_CellId, tsc_SRB1bis, cs_TimingInfo_Now, cs_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, cs_ESM_DATA_TRANSPORT(v_BearerId, v_PTI, v_DataContainer)))); repeat; // @sic R5s190423 sic@ } } if (v_IsNTN) {//@sic R5s240246 sic@ f_Delay(2.0); //delay to handle RLC ACK } }

After Change:

function f_NBIOT_DelayForUserPlaneSignalling_CP(NBIOT_CellId_Type p_CellId) runs on NBIOT_PTC { var NB_SRB_COMMON_IND v_NB_SRB_COMMON_IND; var template (value) UserDataContainer v_DataContainer; var EPS_BearerIdentity v_BearerId; var ProcedureTransactionIdentifier v_PTI; var octetstring v_DataReply; var boolean v_IsNTN := f_NBIOT_MobileInfo_GetNTNSatelliteType() != ntn_None; // change to not equal to timer t_WaitForIPSignalling; if (v_IsNTN) //@sic R5s240246, R5-234501 sic@ { t_WaitForIPSignalling.start(2.48); } else { t_WaitForIPSignalling.start(1.2); } alt { [] t_WaitForIPSignalling.timeout { // just continue } [] SRB.receive(car_NB_SRB_NasPdu_IND(p_CellId, tsc_SRB1bis, cr_NAS_Indication(tsc_SHT_IntegrityProtected_Ciphered, cr_ESM_DATA_TRANSPORT(?, cr_UserDataContainer))))-> value v_NB_SRB_COMMON_IND { v_BearerId := v_NB_SRB_COMMON_IND.Signalling.Nas[0].Pdu.Msg.eSM_DATA_TRANSPORT.epsBearerId; v_PTI := v_NB_SRB_COMMON_IND.Signalling.Nas[0].Pdu.Msg.eSM_DATA_TRANSPORT.procedureTransactionIdentifier; v_DataReply := f_NBIOT_ProcessIPv6Address(v_NB_SRB_COMMON_IND.Signalling.Nas[0].Pdu.Msg.eSM_DATA_TRANSPORT.userDatacontainer.dataContainer); v_DataContainer := cs_UserDataContainer(v_DataReply); SRB.send(cas_NB_SRB_NasPdu_REQ(p_CellId, tsc_SRB1bis, cs_TimingInfo_Now, cs_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, cs_ESM_DATA_TRANSPORT(v_BearerId, v_PTI, v_DataContainer)))); repeat; // @sic R5s190423 sic@ } } if (v_IsNTN) {//@sic R5s240246 sic@ f_Delay(2.0); //delay to handle RLC ACK } }

## Correction to function f_NBIOT_GetNPDCCH_PeriodBundle

| Function name | f_NBIOT_GetNPDCCH_PeriodBundle |
| --- | --- |
| Reason for change | GetNPDCCH_PeriodBundle is used to get the no of NPDCCH period and this is later passed into other functions like f_NBIOT_IncrementSearchSpace, the expectation here is that the parameter passed is no of NPDCCH period and not the NPDCCH period bundle. The GetNPDCCH_PeriodBundle currently returns the NPDCCH period bundle (640) and in function f_NBIOT_IncrementSearchSpace this is further multiplied by v_CycleLength and incremented for activation time calculation, this results in incorrect timings for PDUs to be sent |
| Summary of change | Updated the function to return no of periods (10 for NTN and 1 for legacy) and this can be used in other functions like f_NBIOT_IncrementSearchSpace to get the bundle period (640msec) |
| TTCN module | NBIOT_Timing |
| MCC160 Comment |  |

Before Change:

function f_NBIOT_GetNPDCCH_PeriodBundle(NBIOT_CellId_Type p_CellId) runs on NBIOT_PTC return integer { var integer v_T_offset; var integer v_NPDCCH_Period; v_NPDCCH_Period := f_NBIOT_SearchSpace_GetPdcchPeriod(); if(f_NBIOT_MobileInfo_GetNTNSatelliteType()!= ntn_None) {//@sic R5-235443, R5s230478 cl 2.7, R5-237436 sic@ v_T_offset := f_NBIOT_NTN_Get_T_NTNoffset(p_CellId); return(f_RoundedUpDivision(v_T_offset,v_NPDCCH_Period)*v_NPDCCH_Period + v_NPDCCH_Period) } else { return v_NPDCCH_Period }

After Change:

function f_NBIOT_GetNPDCCH_PeriodBundle(NBIOT_CellId_Type p_CellId) runs on NBIOT_PTC return integer { var integer v_T_offset; var integer v_NPDCCH_Period; v_NPDCCH_Period := f_NBIOT_SearchSpace_GetPdcchPeriod(); if(f_NBIOT_MobileInfo_GetNTNSatelliteType()!= ntn_None) {//@sic R5-235443, R5s230478 cl 2.7, R5-237436 sic@ v_T_offset := f_NBIOT_NTN_Get_T_NTNoffset(p_CellId); return((f_RoundedUpDivision(v_T_offset,v_NPDCCH_Period)*v_NPDCCH_Period + v_NPDCCH_Period))/v_NPDCCH_Period // expect as no of bundle } else { return (1) }