3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240350

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3429 | rev | - | Current version: | 17.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC Test case 9.1.5.1.4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-04-23 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To give UE ample time to apply the AT Command associated with MICO Mode Activation prior to requesting a connection to the Network. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The current implementation of the function f_UT_TriggerMICO is done with NO_CNF_REQUIRED, which creates an issue wherein once AT Command for MICO is sent, TTCN immediately powers up the cell, and since SWITCH_ON was also sent prior to AT Command for MICO Mode, UE immediately initiates connection request while still applying the AT Command for MICO Mode, leading to TC failing due to MICO IE not present in Registration Request. By modifying the function f_UT_TriggerMICO to have “CNF_REQUIRED”, we can ensure that UE has applied the setting prior to powering up the cell. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.5.1.4.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR… |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2022-09_D24wk12 related to the title of this CR.

Contact:	Kalahasti, Narendra

Narendra.Kalahasti@anritsu.com

# Corrections required

## Change 1

| Function name | function f_UT_TriggerMICO() |
| --- | --- |
| Reason for change | To modify the implementation |
| Summary of change | Modified the function f_UT_TriggerMICO to have “CNF_REQUIRED” parameter. |
| TTCN module | UpperTesterFunctions.ttcn |
| MCC160 Comment |  |

Before Change

/* * @desc To trigger MICO (start or stop) * @param p_Port * @param p_StartMICO Set to true to start and false to stop MICO * @status */ function f_UT_TriggerMICO(UT_PTC_MTC_PORT p_Port, boolean p_StartMICO) { fl_UT_ApplyCommand(p_Port, cas_UT_Req(MICO, NO_CNF_REQUIRED, {cs_Ut_Parameter_Common("MICO", bit2str(f_ConvertBoolToBit(p_StartMICO)))})); };

After Change

/* * @desc To trigger MICO (start or stop) * @param p_Port * @param p_StartMICO Set to true to start and false to stop MICO * @status */ function f_UT_TriggerMICO(UT_PTC_MTC_PORT p_Port, boolean p_StartMICO) { fl_UT_ApplyCommand(p_Port, cas_UT_Req(MICO, CNF_REQUIRED, {cs_Ut_Parameter_Common("MICO", bit2str(f_ConvertBoolToBit(p_StartMICO)))})); };