3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240351

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4817 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Re-verification of LTE NAS RACS testcase 9.2.5.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, RACS-UEConTest |  |  |  |  |  | Date: |  |  | 2024-04-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To re-verify test case 9.2.5.1 from LTE_A_PRO ATS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to test case 9.2.5.1 required for approval. See detailed change description for further information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be verified |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.5.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS36.523-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	7

3	Branches executed	9

4	Execution Log Files	9

4.1	Samsung Shannon S5E9945ERD UE	9

5	References	9

# Overview

This document lists all the essential changes needed to correct problems in the TTCN implementation of test case 9.2.5.1 which is part of the LTE_A_PRO test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:	Kalahasti, Narendra

Narendra.Kalahasti@anritsu.com

## Verification Test Summary

Test Case: 	9.2.5.1

ATS Version:	iwd-TTCN3-B2022-09_D24wk12

System Simulator used:	Anritsu Protocol Conformance Test System ME7834NR

UE used:	Samsung Shannon S5E9945ERD UE

Verification Status:	PASS

# Corrections required

## Change 1

| Testcase name | fl_TC_9_2_5_1_Body |
| --- | --- |
| Reason for change | Incorrect EPS update type is expected in TAU request at Step18 and Step25Ab1 and incorrect EPS update type is provided in TAU complete at Step22 and Step27. |
| Summary of change | Derive the EPS update type using the function f_GetEPSTAUType(NORMAL) |
| TTCN module | LTE_A_PRO\9_2\NAS_RACS.ttcn |
| MCC160 Comment |  |

Before change:

function fl_TC_9_2_5_1_Body() runs on EUTRA_PTC

{

var NAS_MSG_Indication_Type v_NAS_Ind;

var template (omit) EPS_BearerContextStatus v_EPS_ContextStatus;

var NasCount_Type v_NasCountUL;

var EUTRA_SecurityParams_Type v_AuthParams;

var template (omit) UERadioCapId v_UERadioCapId;

//@siclog "Step 1" siclog@

//The UE is switched on.

f_EUTRA_SwitchOnUEandStartIP(eutra_CellA);

//@siclog "Step 2" siclog@

//Check: Does the UE include UE radio capability availability indication in ATTACH REQUEST message on Cell A?

v_NAS_Ind:= f_EUTRA_RRC_ConnEst_DefWithNas(eutra_CellA,

tsc_RRC_TI_Def,

cr_EstablishmentCause_Any,

cr_NAS_IndicationWithPiggybacking(tsc_SHT_IntegrityProtected,

cr_508_ATTACH_REQUEST(f_GetEAttachType(NORMAL), f_GetAdditionalUpdateType(NORMAL)),

cr_508_PDN_CONNECTIVITY_REQUEST(tsc_PdnRequest_InitialAttach,

f_GetPdnType())));

if (match (v_NAS_Ind.Pdu.Msg.aTTACH_REQUEST.ueRadioCapIdAvailability.availability, '001'B)) { //prose CR needed.

f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 2");

}

//@siclog "Step 3-4" siclog@

//Steps 5-6 of the generic procedure for UE registration specified in TS 36.508 subclause 4.5.2.3 are performed.

f_EUTRA_InitialRegistration_Step5_6(eutra_CellA, v_NAS_Ind);

v_AuthParams := f_EUTRA_Security_Get();

//@siclog "Step 5" siclog@

//The SS transmits a NAS SECURITY MODE COMMAND message requesting UE radio capability ID.

//siclog "Step 6" siclog@

//Check: Does the UE transmit a NAS SECURITY MODE COMPLETE message including UE radio capability ID 1?

if (ispresent(v_NAS_Ind.Pdu.Msg.aTTACH_REQUEST.ueAddSecurityCap)) {

v_AuthParams.NAS_SecurityCap5G := v_NAS_Ind.Pdu.Msg.aTTACH_REQUEST.ueAddSecurityCap;

}

v_NasCountUL:= f_EUTRA_NAS_ActivateSecurity(eutra_CellA,v_AuthParams,-,true,true, v_NAS_Ind.Pdu.Msg.aTTACH_REQUEST.ueRadioCapIdAvailability);

v_UERadioCapId := f_EUTRA_MobileInfo_GetRxdRACS_Id();

if (isvalue(v_UERadioCapId)) {

if (match(valueof(v_UERadioCapId), cs_UERadioCapId('66'O, '01000000000010'O))) {

f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 6");

} else {

f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 6");

}

} else {

f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 6");

}

//@siclog "Steps 7-11" siclog@

//Steps 9-13 of the generic procedure for UE registration specified in TS 36.508 subclause 4.5.2.3 are performed.

//@siclog "Step 12" siclog@

//SS transmits a ATTACH ACCEPT message with a new UE radio capability ID 2.

//@siclog "Steps 13-16" siclog@

//Steps 15-18 of the generic procedure for UE registration specified in TS 36.508 subclause 4.5.2.3 are performed.//step15 is done before this.

f_EUTRA_MobileInfo_SetRACS_IdToSend(valueof(cs_UERadioCapId('66'O, '01000000000020'O)));

f_EUTRA_InitialRegistration_Step9_18b1(eutra_CellA,v_NAS_Ind,v_NasCountUL);

//@siclog "Step 17" siclog@

//The SS configures:- Cell A as the "Non-Suitable cell".- Cell B as a "Serving cell".

f_EUTRA_SetCellPower(eutra_CellA, tsc_NonSuitableCellRS_EPRE);

f_EUTRA_SetCellPower(eutra_CellB, tsc_ServingCellRS_EPRE);

//@siclog "Step 18" siclog@

//Check: Does the UE include UE radio capability availability indication in TRACKING AREA UPDATE REQUEST message?

v_NAS_Ind := f_EUTRA_TrackingAreaUpdate_Step1_3(eutra_CellB,

tsc_RRC_TI_Def,

NORMAL,

?,

'000'B);

if (match (v_NAS_Ind.Pdu.Msg.tRACKING_AREA_UPDATE_REQUEST.ueRadioCapIdAvailability.availability, '001'B)) { //prose CR needed.

f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 18");

}

v_AuthParams := f_EUTRA_Security_Get();

//@siclog "Step 19" siclog@

//The SS transmits a NAS SECURITY MODE COMMAND message requesting UE radio capability ID.

//@siclog "Step 20" siclog@

//Check: Does the UE transmit a NAS SECURITY MODE COMPLETE message including UE radio capability ID 2?

v_NasCountUL:= f_EUTRA_NAS_ActivateSecurity(eutra_CellB,v_AuthParams, -, omit, false, v_NAS_Ind.Pdu.Msg.tRACKING_AREA_UPDATE_REQUEST.ueRadioCapIdAvailability);///@sic R5s240215 sic@

v_UERadioCapId := f_EUTRA_MobileInfo_GetRxdRACS_Id();

if (isvalue(v_UERadioCapId)) {

if (match(valueof(v_UERadioCapId), cs_UERadioCapId('66'O, '01000000000020'O))) {

f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 20");

} else {

f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 20");

}

} else {

f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 20");

}

//@siclog "Step 21" siclog@

//The SS transmits a TRACKING AREA UPDATE ACCEPT message.

//@siclog "Step 22" siclog@

//The UE transmits a TRACKING AREA UPDATE COMPLETE message.

v_EPS_ContextStatus := v_NAS_Ind.Pdu.Msg.tRACKING_AREA_UPDATE_REQUEST.epsBearerContextStatus;

f_EUTRA_TrackingAreaUpdate_Accept_Complete(eutra_CellB, f_EUTRA_CellInfo_GetGuti(eutra_CellB), '000'B, v_EPS_ContextStatus, v_NAS_Ind.Pdu.Msg.tRACKING_AREA_UPDATE_REQUEST.addUpdateType, NORMAL);

//@siclog "Step 23" siclog@

//The SS releases the RRC connection.

f_EUTRA_RRC_ConnectionRelease(eutra_CellB);

//@siclog "Step 24" siclog@

//The SS configures:- Cell B as the "Non-Suitable cell".- Cell G as a "Serving cell".

f_EUTRA_SetCellPower(eutra_CellB, tsc_NonSuitableCellRS_EPRE);

f_EUTRA_SetCellPower(eutra_CellG, tsc_ServingCellRS_EPRE);

// @siclog "Step 25" siclog@  //@sic R5-241690 R5s240215 sic@

// Void.

// EXCEPTION: Steps 25Aa1-25Ab1 describe behaviour that depends on UE configuration; the "lower case letter" identifies a step sequence that takes place depending on whether the UE has a manufacturer-assigned UE Radio Capability ID

// @siclog "Step 25Aa1" siclog@  //@sic R5-241690 R5s240215 sic@

//  IF the UE has a manufacturer-assigned UE Radio Capability ID THEN Check: Does the UE indicate UE radio Capability ID available in UE radio capability availability indication IE in TRACKING AREA UPDATE REQUEST message?

// @siclog "Step 25Aa2" siclog@  //@sic R5-241690 R5s240215 sic@

// The SS transmits a NAS SECURITY MODE COMMAND message requesting UE radio capability ID.

// @siclog "Step 25Aa3" siclog@  //@sic R5-241690 R5s240215 sic@

// Check: Does the UE transmit a NAS SECURITY MODE COMPLETE message including the manufacturer-assigned UE Radio Capability ID?

// @siclog "Step 25Ab1" siclog@  //@sic R5-241690 R5s240215 sic@

// ELSECheck: Does the UE not indicate a UE radio Capability ID is available in TRACKING AREA UPDATE REQUEST message?

//Receive the TAU

v_NAS_Ind := f_EUTRA_TrackingAreaUpdate_Step1_3(eutra_CellG,

tsc_RRC_TI_Def,

NORMAL,

?,

'000'B);

f_EUTRA_Check_TAU_SMC(eutra_CellG,v_NAS_Ind);  //@sic R5-241690 R5s240215 sic@

//@siclog "Step 26" siclog@

//The SS transmits a TRACKING AREA UPDATE ACCEPT message.

//@siclog "Step 27" siclog@

//The UE transmits a TRACKING AREA UPDATE COMPLETE message.

v_EPS_ContextStatus := v_NAS_Ind.Pdu.Msg.tRACKING_AREA_UPDATE_REQUEST.epsBearerContextStatus;

f_EUTRA_TrackingAreaUpdate_Accept_Complete(eutra_CellG, f_EUTRA_CellInfo_GetGuti(eutra_CellG), '000'B, v_EPS_ContextStatus, v_NAS_Ind.Pdu.Msg.tRACKING_AREA_UPDATE_REQUEST.addUpdateType, NORMAL);

//@siclog "Step 28" siclog@

//The SS releases the RRC connection.

f_EUTRA_RRC_ConnectionRelease(eutra_CellG);

} //End of fl_TC_9_2_5_1_Body

After change:

function fl_TC_9_2_5_1_Body() runs on EUTRA_PTC

{

var NAS_MSG_Indication_Type v_NAS_Ind;

var template (omit) EPS_BearerContextStatus v_EPS_ContextStatus;

var NasCount_Type v_NasCountUL;

var EUTRA_SecurityParams_Type v_AuthParams;

var template (omit) UERadioCapId v_UERadioCapId;

//@siclog "Step 1" siclog@

//The UE is switched on.

f_EUTRA_SwitchOnUEandStartIP(eutra_CellA);

//@siclog "Step 2" siclog@

//Check: Does the UE include UE radio capability availability indication in ATTACH REQUEST message on Cell A?

v_NAS_Ind:= f_EUTRA_RRC_ConnEst_DefWithNas(eutra_CellA,

tsc_RRC_TI_Def,

cr_EstablishmentCause_Any,

cr_NAS_IndicationWithPiggybacking(tsc_SHT_IntegrityProtected,

cr_508_ATTACH_REQUEST(f_GetEAttachType(NORMAL), f_GetAdditionalUpdateType(NORMAL)),

cr_508_PDN_CONNECTIVITY_REQUEST(tsc_PdnRequest_InitialAttach,

f_GetPdnType())));

if (match (v_NAS_Ind.Pdu.Msg.aTTACH_REQUEST.ueRadioCapIdAvailability.availability, '001'B)) { //prose CR needed.

f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 2");

}

//@siclog "Step 3-4" siclog@

//Steps 5-6 of the generic procedure for UE registration specified in TS 36.508 subclause 4.5.2.3 are performed.

f_EUTRA_InitialRegistration_Step5_6(eutra_CellA, v_NAS_Ind);

v_AuthParams := f_EUTRA_Security_Get();

//@siclog "Step 5" siclog@

//The SS transmits a NAS SECURITY MODE COMMAND message requesting UE radio capability ID.

//siclog "Step 6" siclog@

//Check: Does the UE transmit a NAS SECURITY MODE COMPLETE message including UE radio capability ID 1?

if (ispresent(v_NAS_Ind.Pdu.Msg.aTTACH_REQUEST.ueAddSecurityCap)) {

v_AuthParams.NAS_SecurityCap5G := v_NAS_Ind.Pdu.Msg.aTTACH_REQUEST.ueAddSecurityCap;

}

v_NasCountUL:= f_EUTRA_NAS_ActivateSecurity(eutra_CellA,v_AuthParams,-,true,true, v_NAS_Ind.Pdu.Msg.aTTACH_REQUEST.ueRadioCapIdAvailability);

v_UERadioCapId := f_EUTRA_MobileInfo_GetRxdRACS_Id();

if (isvalue(v_UERadioCapId)) {

if (match(valueof(v_UERadioCapId), cs_UERadioCapId('66'O, '01000000000010'O))) {

f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 6");

} else {

f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 6");

}

} else {

f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 6");

}

//@siclog "Steps 7-11" siclog@

//Steps 9-13 of the generic procedure for UE registration specified in TS 36.508 subclause 4.5.2.3 are performed.

//@siclog "Step 12" siclog@

//SS transmits a ATTACH ACCEPT message with a new UE radio capability ID 2.

//@siclog "Steps 13-16" siclog@

//Steps 15-18 of the generic procedure for UE registration specified in TS 36.508 subclause 4.5.2.3 are performed.//step15 is done before this.

f_EUTRA_MobileInfo_SetRACS_IdToSend(valueof(cs_UERadioCapId('66'O, '01000000000020'O)));

f_EUTRA_InitialRegistration_Step9_18b1(eutra_CellA,v_NAS_Ind,v_NasCountUL);

//@siclog "Step 17" siclog@

//The SS configures:- Cell A as the "Non-Suitable cell".- Cell B as a "Serving cell".

f_EUTRA_SetCellPower(eutra_CellA, tsc_NonSuitableCellRS_EPRE);

f_EUTRA_SetCellPower(eutra_CellB, tsc_ServingCellRS_EPRE);

//@siclog "Step 18" siclog@

//Check: Does the UE include UE radio capability availability indication in TRACKING AREA UPDATE REQUEST message?

v_NAS_Ind := f_EUTRA_TrackingAreaUpdate_Step1_3(eutra_CellB,

tsc_RRC_TI_Def,

NORMAL,

?,

f_GetEPSTAUType(NORMAL));

if (match (v_NAS_Ind.Pdu.Msg.tRACKING_AREA_UPDATE_REQUEST.ueRadioCapIdAvailability.availability, '001'B)) { //prose CR needed.

f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 18");

}

v_AuthParams := f_EUTRA_Security_Get();

//@siclog "Step 19" siclog@

//The SS transmits a NAS SECURITY MODE COMMAND message requesting UE radio capability ID.

//@siclog "Step 20" siclog@

//Check: Does the UE transmit a NAS SECURITY MODE COMPLETE message including UE radio capability ID 2?

v_NasCountUL:= f_EUTRA_NAS_ActivateSecurity(eutra_CellB,v_AuthParams, -, omit, false, v_NAS_Ind.Pdu.Msg.tRACKING_AREA_UPDATE_REQUEST.ueRadioCapIdAvailability);///@sic R5s240215 sic@

v_UERadioCapId := f_EUTRA_MobileInfo_GetRxdRACS_Id();

if (isvalue(v_UERadioCapId)) {

if (match(valueof(v_UERadioCapId), cs_UERadioCapId('66'O, '01000000000020'O))) {

f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 20");

} else {

f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 20");

}

} else {

f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 20");

}

//@siclog "Step 21" siclog@

//The SS transmits a TRACKING AREA UPDATE ACCEPT message.

//@siclog "Step 22" siclog@

//The UE transmits a TRACKING AREA UPDATE COMPLETE message.

v_EPS_ContextStatus := v_NAS_Ind.Pdu.Msg.tRACKING_AREA_UPDATE_REQUEST.epsBearerContextStatus;

f_EUTRA_TrackingAreaUpdate_Accept_Complete(eutra_CellB, f_EUTRA_CellInfo_GetGuti(eutra_CellB), f_GetEPSTAUType(NORMAL), v_EPS_ContextStatus, v_NAS_Ind.Pdu.Msg.tRACKING_AREA_UPDATE_REQUEST.addUpdateType, NORMAL);

//@siclog "Step 23" siclog@

//The SS releases the RRC connection.

f_EUTRA_RRC_ConnectionRelease(eutra_CellB);

//@siclog "Step 24" siclog@

//The SS configures:- Cell B as the "Non-Suitable cell".- Cell G as a "Serving cell".

f_EUTRA_SetCellPower(eutra_CellB, tsc_NonSuitableCellRS_EPRE);

f_EUTRA_SetCellPower(eutra_CellG, tsc_ServingCellRS_EPRE);

// @siclog "Step 25" siclog@  //@sic R5-241690 R5s240215 sic@

// Void.

// EXCEPTION: Steps 25Aa1-25Ab1 describe behaviour that depends on UE configuration; the "lower case letter" identifies a step sequence that takes place depending on whether the UE has a manufacturer-assigned UE Radio Capability ID

// @siclog "Step 25Aa1" siclog@  //@sic R5-241690 R5s240215 sic@

//  IF the UE has a manufacturer-assigned UE Radio Capability ID THEN Check: Does the UE indicate UE radio Capability ID available in UE radio capability availability indication IE in TRACKING AREA UPDATE REQUEST message?

// @siclog "Step 25Aa2" siclog@  //@sic R5-241690 R5s240215 sic@

// The SS transmits a NAS SECURITY MODE COMMAND message requesting UE radio capability ID.

// @siclog "Step 25Aa3" siclog@  //@sic R5-241690 R5s240215 sic@

// Check: Does the UE transmit a NAS SECURITY MODE COMPLETE message including the manufacturer-assigned UE Radio Capability ID?

// @siclog "Step 25Ab1" siclog@  //@sic R5-241690 R5s240215 sic@

// ELSECheck: Does the UE not indicate a UE radio Capability ID is available in TRACKING AREA UPDATE REQUEST message?

//Receive the TAU

v_NAS_Ind := f_EUTRA_TrackingAreaUpdate_Step1_3(eutra_CellG,

tsc_RRC_TI_Def,

NORMAL,

?,

f_GetEPSTAUType(NORMAL));

f_EUTRA_Check_TAU_SMC(eutra_CellG,v_NAS_Ind);  //@sic R5-241690 R5s240215 sic@

//@siclog "Step 26" siclog@

//The SS transmits a TRACKING AREA UPDATE ACCEPT message.

//@siclog "Step 27" siclog@

//The UE transmits a TRACKING AREA UPDATE COMPLETE message.

v_EPS_ContextStatus := v_NAS_Ind.Pdu.Msg.tRACKING_AREA_UPDATE_REQUEST.epsBearerContextStatus;

f_EUTRA_TrackingAreaUpdate_Accept_Complete(eutra_CellG, f_EUTRA_CellInfo_GetGuti(eutra_CellG), f_GetEPSTAUType(NORMAL), v_EPS_ContextStatus, v_NAS_Ind.Pdu.Msg.tRACKING_AREA_UPDATE_REQUEST.addUpdateType, NORMAL);

//@siclog "Step 28" siclog@

//The SS releases the RRC connection.

f_EUTRA_RRC_ConnectionRelease(eutra_CellG);

} //End of fl_TC_9_2_5_1_Body

## Change 2

| Testcase name | f_EUTRA_Check_TAU_SMC |
| --- | --- |
| Reason for change | Incorrect ID is used to match against manufacturer-assigned UE Radio Capability ID. According to Table 9.9.3.58.1, TS 24.301, values other than ‘001’ for availability IE is interpreted as UE radio capability ID not available, hence there is no need for else-if condition for the value ‘000’. The UERadioCapId check will not be performed when UE omits ueRadioCapIdAvailability IE in TAU request. |
| Summary of change | Modify the ID used for matching manufacturer-assigned UE Radio Capability ID, in the if case. Removed else-if condition used for checking the matching availability IE with the value ‘000’. Corrected if-else statement to pass the check, when UE omits ueRadioCapIdAvailability IE. |
| TTCN module | LTE_A_PRO\9_2\NAS_RACS.ttcn |
| MCC160 Comment |  |

Before change:

function f_EUTRA_Check_TAU_SMC(EUTRA_CellId_Type p_CellId,

NAS_MSG_Indication_Type p_NAS_Ind) runs on EUTRA_PTC

{

var NAS_MSG_Indication_Type v_NAS_Ind:=p_NAS_Ind;

var EUTRA_SecurityParams_Type v_AuthParams;

var template (omit) UERadioCapId v_UERadioCapId;

if(ispresent(v_NAS_Ind.Pdu.Msg.tRACKING_AREA_UPDATE_REQUEST.ueRadioCapIdAvailability.availability)){

if (match (v_NAS_Ind.Pdu.Msg.tRACKING_AREA_UPDATE_REQUEST.ueRadioCapIdAvailability.availability, '001'B)) {

// @siclog "Step n+0Aa1" siclog@

//IF the UE has a manufacturer-assigned UE Radio Capability ID THEN Check: Does the UE indicate UE radio Capability ID available in UE radio capability availability indication IE in TRACKING AREA UPDATE REQUEST message?

v_AuthParams := f_EUTRA_Security_Get();

// @siclog "Step n+0Aa2" siclog@

// The SS transmits a NAS SECURITY MODE COMMAND message requesting UE radio capability ID.

// @siclog "Step n+0Aa3" siclog@

// Check: Does the UE transmit a NAS SECURITY MODE COMPLETE message including the manufacturer-assigned UE Radio Capability ID?

f_EUTRA_NAS_ActivateSecurity(p_CellId,v_AuthParams,-,omit ,false, v_NAS_Ind.Pdu.Msg.tRACKING_AREA_UPDATE_REQUEST.ueRadioCapIdAvailability);

v_UERadioCapId := f_EUTRA_MobileInfo_GetRxdRACS_Id();

if (isvalue(v_UERadioCapId)) {

if (match(valueof(v_UERadioCapId), cs_UERadioCapId('66'O, '00000000000000'O))) {

f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "UERadioCapId check pass");

} else {

f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "UERadioCapId check fail");

}

} else {

f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "UERadioCapId check error");

}

}

else if(match (v_NAS_Ind.Pdu.Msg.tRACKING_AREA_UPDATE_REQUEST.ueRadioCapIdAvailability.availability, '000'B)){

// @siclog "Step n+0b1" siclog@

//Check: Does the UE not indicate a UE radio Capability ID is available in TRACKING AREA UPDATE REQUEST message?

f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "UERadioCapId check pass");

}

else {

f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "UERadioCapId check pass");

}

}

}

After change:

function f_EUTRA_Check_TAU_SMC(EUTRA_CellId_Type p_CellId,

NAS_MSG_Indication_Type p_NAS_Ind) runs on EUTRA_PTC

{

var NAS_MSG_Indication_Type v_NAS_Ind:=p_NAS_Ind;

var EUTRA_SecurityParams_Type v_AuthParams;

var template (omit) UERadioCapId v_UERadioCapId;

if(ispresent(v_NAS_Ind.Pdu.Msg.tRACKING_AREA_UPDATE_REQUEST.ueRadioCapIdAvailability.availability)){

if (match (v_NAS_Ind.Pdu.Msg.tRACKING_AREA_UPDATE_REQUEST.ueRadioCapIdAvailability.availability, '001'B)) {

// @siclog "Step n+0Aa1" siclog@

//IF the UE has a manufacturer-assigned UE Radio Capability ID THEN Check: Does the UE indicate UE radio Capability ID available in UE radio capability availability indication IE in TRACKING AREA UPDATE REQUEST message?

v_AuthParams := f_EUTRA_Security_Get();

// @siclog "Step n+0Aa2" siclog@

// The SS transmits a NAS SECURITY MODE COMMAND message requesting UE radio capability ID.

// @siclog "Step n+0Aa3" siclog@

// Check: Does the UE transmit a NAS SECURITY MODE COMPLETE message including the manufacturer-assigned UE Radio Capability ID?

f_EUTRA_NAS_ActivateSecurity(p_CellId,v_AuthParams,-,omit ,false, v_NAS_Ind.Pdu.Msg.tRACKING_AREA_UPDATE_REQUEST.ueRadioCapIdAvailability);

v_UERadioCapId := f_EUTRA_MobileInfo_GetRxdRACS_Id();

if (isvalue(v_UERadioCapId)) {

if (match(valueof(v_UERadioCapId), cs_UERadioCapId('66'O, '00?????????'O))) {

f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "UERadioCapId check pass");

} else {

f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "UERadioCapId check fail");

}

} else {

f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "UERadioCapId check error");

}

}

else {

// @siclog "Step n+0b1" siclog@

//Check: Does the UE not indicate a UE radio Capability ID is available in TRACKING AREA UPDATE REQUEST message?

f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "UERadioCapId check pass");

}

}

else {

f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "UERadioCapId check pass");

}

}

# Branches executed

The test case implementation was executed with Integrity and ciphering enabled using LTE FDD band 4.

# Execution Log Files

## Samsung Shannon S5E9945ERD UE

The Samsung Shannon S5E9945ERD UE passed this test case on Anritsu Protocol Conformance Test System ME7834NR. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

TC_9_2_5_1_LOG.xml2

- PICS/PIXIT parameter file:

TC_9_2_5_1 _PIXIT.txt

# References

| [1] | R5s240352: Supporting information for reverification of LTE NAS RACS testcase 9.2.5.1 |
| --- | --- |