3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240358

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3436 | rev | - | Current version: | 17.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of NR5GC testcase 8.1.5.9.3 in FR1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test |  |  |  |  |  | Date: |  |  | 2024-05-03 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | To add NR5GC test case 8.1.5.9.3 to the IWD_24wk12 ATS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to test case 11.3.11 required for approval. See detailed change description for further information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.5.9.3.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1	Correction to the Function f_TC_8_1_5_9_3_NR5GC_TestBody()	3

3	Branches executed	9

4	Execution Log Files	10

4.1	MediaTek MT6983	10

5	References	10

# Overview

This document lists all the essencial changes needed to correct problems in the TTCN implementation of NR5GC test case 8.1.5.9.3.NR5GC which is part of the NR5GC test suite in iwd-TTCN3-B2022-09_D24wk12 TTCN-3.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log file(s) is (are) provided as evidence.

Contact:	Parikshit Bhise

parikshit.bhise@rohde-schwarz.com

## Verification Test Summary

Test Case: 	8.1.5.9.3.NR5GC

ATS Version:	iwd-TTCN3-B2022-09_D24wk12 TTCN-3

System Simulator used:	R&S® 5G Protocol Conformance Test platform

UE used:	MediaTek MT6983

Verification Status:	PASS

# Corrections required

## Correction to the Function f_TC_8_1_5_9_3_NR5GC_TestBody()

| Function name | f_TC_8_1_5_9_3_NR5GC_TestBody |
| --- | --- |
| Reason for change | In the current TTCN Implementation of the function f_TC_8_1_5_9_3_NR5GC_TestBody in 24wk12 - DRB configuration and associated security configurations are missing before sending the resume message in step 2 and 12A |
| Summary of change | In the function f_TC_8_1_5_9_3_NR5GC_TestBody, - DRB configuration and associated security configurations are provided before sending the resume message in step 2 and 12A |
| TTCN module | ttcn/develop/NR5GC/8_1_5/RRC_Others_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change:

function f_TC_8_1_5_9_3_NR5GC_TestBody() runs on NR5GC_PTC { var PDUSessionInfoList_Type v_PDUSessionInfo; var integer v_DRBId; var template (value) DL_DCCH_Message v_RRCResume; var octetstring v_RRCResumeEncoded; var integer v_PaddingLength; var octetstring v_LateNonCriticalExt; var integer v_ModificationPeriodFrames; var float v_ModificationPeriod; //@siclog "Step 1" siclog@ //The SS transmits a Paging message including a matched identity (correct fullI-RNTI). f_NR_Paging_Def(nr_Cell1, RRC_INACTIVE); //@siclog "Step 2" siclog@ //The UE transmit an RRCResumeRequest message by setting resumeIdentity to the stored shortI-RNTI value. f_NR_RRC_ResumeRequest_Def(nr_Cell1, tsc_NR_ShortI_RNTI_Value1, mt_Access); //@siclog "Step 3" siclog@ //The SS transmits a DLDedicatedMessageSegment containing the first segment of the 3 segment RRCResume message. //Get RRCResume message to segment and config SS v_RRCResume := f_NR_GetRRCResume_Def(nr_Cell1); //Add padding to get size of encoded RRCResume message < 3*maximum PDCP SDU size, so that it will fit in three segments -> e.g. total size = 2.5 x tsc_Max_NR_PDCP_SDU_size v_RRCResumeEncoded := bit2oct(encvalue(v_RRCResume)); v_PaddingLength := (tsc_Max_NR_PDCP_SDU_size * 25 / 10) - lengthof(v_RRCResumeEncoded); v_LateNonCriticalExt := ''O; while (v_PaddingLength >= 1000) { //the message is expanded by 1000 octets until less than 1000 octets are remaining v_LateNonCriticalExt := v_LateNonCriticalExt & tsc_Oct1000_0; v_PaddingLength := v_PaddingLength - 1000; } v_LateNonCriticalExt := v_LateNonCriticalExt & substr(tsc_Oct1000_0, 0, v_PaddingLength); //The remaining octets are concatenated v_RRCResume.message_.c1.rrcResume.criticalExtensions.rrcResume.lateNonCriticalExtension := v_LateNonCriticalExt; v_RRCResumeEncoded := bit2oct(encvalue(v_RRCResume)); // Send first segment f_NR_TransmitDLSegment(nr_Cell1, v_RRCResumeEncoded, 0, notLastSegment); //@siclog "Step 4" siclog@ //The SS transmits a DLDedicatedMessageSegment containing the second segment of the 3 segment RRCResume message. f_NR_TransmitDLSegment(nr_Cell1, v_RRCResumeEncoded, 1, notLastSegment); //@siclog "Step 5" siclog@ //The SS transmits a DLDedicatedMessageSegment containing the third (last) segment of the 3 segment RRCResume message. f_NR_TransmitDLSegment(nr_Cell1, v_RRCResumeEncoded, 2, lastSegment); //@siclog "Step 6" siclog@ //The UE transmits an RRCResumeComplete message. f_NR_RRCResumeComplete_Def(nr_Cell1); //@siclog "Step 7" siclog@ //Check: Does the test result of generic test procedure in TS 38.508-1 subclause 4.9.1 indicate that the UE is capable of exchanging IP data on DRB#n associated with the first PDU session? v_DRBId := f_NR_GetDefaultDRB_ForFirstPDUSession(); v_PDUSessionInfo := f_NR5GC_MobileInfo_GetSessionInfoList(); if (isvalue(v_PDUSessionInfo[0])) { f_NR5GC_CheckDataPath(nr_Cell1, f_NR5GC_GetPdnIndex(valueof(v_PDUSessionInfo[0])), dedicatedBearer, v_DRBId, "Step 7"); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "No Internet PDU Session"); } //@siclog "Step 8" siclog@ //The SS transmits an RRCRelease message including both fullI-RNTI and shortI-RNTI in suspendConfig. f_NR_RRC_InactiveDef(nr_Cell1); //@siclog "Step 9" siclog@ //The SS changes the SIB1 of NR Cell 1 to set the useFullResumeID to True. f_NR_CellInfo_SetSIB1_UseFullResumeID(nr_Cell1, true_); //@siclog "Step 10" siclog@ //The SS transmits a Short message on PDCCH using P-RNTI indicating a systemInfoModification. f_NR_ModifySysinfo(nr_Cell1, true, RRC_INACTIVE); //@siclog "Step 11" siclog@ //Wait for 2.1* modification period second for the UE to receive new system information. v_ModificationPeriodFrames := f_NR_CalculateModificationPeriod(nr_Cell1); v_ModificationPeriod := int2float(v_ModificationPeriodFrames)/100.0; //Modification period in seconds f_Delay(2.1 * v_ModificationPeriod); //@siclog "Step 12" siclog@ //The SS transmits a Paging message including a matched identity (correct fullI-RNTI). f_NR_Paging_Def(nr_Cell1, RRC_INACTIVE); //@siclog "Step 12A" siclog@ //The UE transmit an RRCResumeRequest1 message by setting resumeIdentity to the stored fullI-RNTI value? SRB.receive(car_NR_SRB0_RrcPdu_IND1(nr_Cell1, cr_38508_RRCResumeRequest1(tsc_NR_I_RNTI_Value1))); //@siclog "Step 13" siclog@ //The SS transmits a DLDedicatedMessageSegment containing the first segment of the 2 segment RRCResume message. << SKIPPED CODE >> }

After Change:

function f_TC_8_1_5_9_3_NR5GC_TestBody() runs on NR5GC_PTC { var PDUSessionInfoList_Type v_PDUSessionInfo; var integer v_DRBId; var template (value) DL_DCCH_Message v_RRCResume; var octetstring v_RRCResumeEncoded; var integer v_PaddingLength; var octetstring v_LateNonCriticalExt; var integer v_ModificationPeriodFrames; var float v_ModificationPeriod; var NR_DRBInfoList_Type v_ExistingDRBs := f_NR5GC_MobileInfo_GetDRBInfoList(); //WA#WI=1262902 var template (value) NR_RadioBearerList_Type v_SS_Config := f_NR5GC_GetRadioBearerList(v_ExistingDRBs); //WA#WI=1262902 //@siclog "Step 1" siclog@ //The SS transmits a Paging message including a matched identity (correct fullI-RNTI). f_NR_Paging_Def(nr_Cell1, RRC_INACTIVE); //@siclog "Step 2" siclog@ //The UE transmit an RRCResumeRequest message by setting resumeIdentity to the stored shortI-RNTI value. f_NR_RRC_ResumeRequest_Def(nr_Cell1, tsc_NR_ShortI_RNTI_Value1, mt_Access); f_NR_SS_CommonRadioBearerConfig(nr_Cell1, v_SS_Config); //WA#WI=1262902 //Activate security on newly configured DRB @sic R5s200102 sic@ f_NR_SS_AS_ActivateSecurity_NoRefresh(nr_Cell1); //WA#WI=1262902 //@siclog "Step 3" siclog@ //The SS transmits a DLDedicatedMessageSegment containing the first segment of the 3 segment RRCResume message. //Get RRCResume message to segment and config SS v_RRCResume := f_NR_GetRRCResume_Def(nr_Cell1); //Add padding to get size of encoded RRCResume message < 3*maximum PDCP SDU size, so that it will fit in three segments -> e.g. total size = 2.5 x tsc_Max_NR_PDCP_SDU_size v_RRCResumeEncoded := bit2oct(encvalue(v_RRCResume)); v_PaddingLength := (tsc_Max_NR_PDCP_SDU_size * 25 / 10) - lengthof(v_RRCResumeEncoded); v_LateNonCriticalExt := ''O; while (v_PaddingLength >= 1000) { //the message is expanded by 1000 octets until less than 1000 octets are remaining v_LateNonCriticalExt := v_LateNonCriticalExt & tsc_Oct1000_0; v_PaddingLength := v_PaddingLength - 1000; } v_LateNonCriticalExt := v_LateNonCriticalExt & substr(tsc_Oct1000_0, 0, v_PaddingLength); //The remaining octets are concatenated v_RRCResume.message_.c1.rrcResume.criticalExtensions.rrcResume.lateNonCriticalExtension := v_LateNonCriticalExt; v_RRCResumeEncoded := bit2oct(encvalue(v_RRCResume)); // Send first segment f_NR_TransmitDLSegment(nr_Cell1, v_RRCResumeEncoded, 0, notLastSegment); //@siclog "Step 4" siclog@ //The SS transmits a DLDedicatedMessageSegment containing the second segment of the 3 segment RRCResume message. f_NR_TransmitDLSegment(nr_Cell1, v_RRCResumeEncoded, 1, notLastSegment); //@siclog "Step 5" siclog@ //The SS transmits a DLDedicatedMessageSegment containing the third (last) segment of the 3 segment RRCResume message. f_NR_TransmitDLSegment(nr_Cell1, v_RRCResumeEncoded, 2, lastSegment); //@siclog "Step 6" siclog@ //The UE transmits an RRCResumeComplete message. f_NR_RRCResumeComplete_Def(nr_Cell1); //@siclog "Step 7" siclog@ //Check: Does the test result of generic test procedure in TS 38.508-1 subclause 4.9.1 indicate that the UE is capable of exchanging IP data on DRB#n associated with the first PDU session? v_DRBId := f_NR_GetDefaultDRB_ForFirstPDUSession(); v_PDUSessionInfo := f_NR5GC_MobileInfo_GetSessionInfoList(); if (isvalue(v_PDUSessionInfo[0])) { f_NR5GC_CheckDataPath(nr_Cell1, f_NR5GC_GetPdnIndex(valueof(v_PDUSessionInfo[0])), dedicatedBearer, v_DRBId, "Step 7"); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "No Internet PDU Session"); } //@siclog "Step 8" siclog@ //The SS transmits an RRCRelease message including both fullI-RNTI and shortI-RNTI in suspendConfig. f_NR_RRC_InactiveDef(nr_Cell1); //@siclog "Step 9" siclog@ //The SS changes the SIB1 of NR Cell 1 to set the useFullResumeID to True. f_NR_CellInfo_SetSIB1_UseFullResumeID(nr_Cell1, true_); //@siclog "Step 10" siclog@ //The SS transmits a Short message on PDCCH using P-RNTI indicating a systemInfoModification. f_NR_ModifySysinfo(nr_Cell1, true, RRC_INACTIVE); //@siclog "Step 11" siclog@ //Wait for 2.1* modification period second for the UE to receive new system information. v_ModificationPeriodFrames := f_NR_CalculateModificationPeriod(nr_Cell1); v_ModificationPeriod := int2float(v_ModificationPeriodFrames)/100.0; //Modification period in seconds f_Delay(2.1 * v_ModificationPeriod); //@siclog "Step 12" siclog@ //The SS transmits a Paging message including a matched identity (correct fullI-RNTI). f_NR_Paging_Def(nr_Cell1, RRC_INACTIVE); //@siclog "Step 12A" siclog@ //The UE transmit an RRCResumeRequest1 message by setting resumeIdentity to the stored fullI-RNTI value? SRB.receive(car_NR_SRB0_RrcPdu_IND1(nr_Cell1, cr_38508_RRCResumeRequest1(tsc_NR_I_RNTI_Value1))); f_NR_SS_CommonRadioBearerConfig(nr_Cell1, v_SS_Config); //WA#WI=1262902 //Activate security on newly configured DRB @sic R5s200102 sic@ f_NR_SS_AS_ActivateSecurity_NoRefresh(nr_Cell1); //WA#WI=1262902 //@siclog "Step 13" siclog@ //The SS transmits a DLDedicatedMessageSegment containing the first segment of the 2 segment RRCResume message. << SKIPPED CODE >> }

# Branches executed

## This NR5GC TC was executed in 5GS Stand Alone mode (Option 2) on NR Primary Band “n78” using NIA1 Integrity and NEA1 ciphering algorithms.

# Execution Log Files

## MediaTek MT6983

The MediaTek MT6983 passed this test case on R&S® 5G Protocol Conformance Test platform. The documentation below is enclosed as evidence of the successful test case run [1]:

Test case execution log file:
tc_8_1_5_9_3_MTK.log
(Note: PICS/PIXIT settings are captured at the beginning of the TLI log)

In the log file the complete test case execution as well as the PICS/PIXIT parameter settings can be seen. All TLI events are presented and message contents are fully decoded and can be verified. Preliminary verdicts and the final test case verdict can be seen in the log file.

# References

| [1] | R5s240359: Supporting information for Addition of NR5GC testcase 8.1.5.9.3 in FR1 |
| --- | --- |