3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240360

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-5 | CR | 0112 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction CSK upload |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160, UPV/EHU |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test, MCImp-UEConTest |  |  |  |  |  | Date: |  |  | 2024-05-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - R5-240565 has corrected the MIKEY message for CSK upload for the case of the SIP REGISTER but breaking the case of the CSK upload to CMS and GMS with SIP SUBCRIBE - According to 33.180 F.2.1 the Current Key Period No (P5) is calculated by P5 = Floor ( ( TIME - P4 ) / P3 ) but in f_MCX_SecurityParameters_Init the Key Period offset (P4) is missing in the calculation |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Parameter p_RequestUri := omit added to f_MCX_SIP_RegistrationSubscription_DecodeMatchAndCheckMikeyMessage_SetCSK to allow a_MCX_SIP_ServiceAuthorization_ReceiveSubscribeSend200OK to hand over the correct URI in case of CSK upload to GMS/CMS - Calculation of the Current Key Period No corrected |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail |  |  |  |  |  |  |  |  |
| Clauses affected: |  | f_MCX_SIP_RegistrationSubscription_DecodeMatchAndCheckMikeyMessage_SetCSK, a_MCX_SIP_ServiceAuthorization_ReceiveSubscribeSend200OK, f_MCX_SecurityParameters_Init |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 36.579-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | A prose CR will be raised for change 1 at RAN5#103 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2022-09_D24wk12 related to the title of this CR.

Contact:	Olivier Genoud
	olivier.genoud@etsi.org

# Corrections required

## Change 1

| Function names | f_MCX_SIP_RegistrationSubscription_DecodeMatchAndCheckMikeyMessage_SetCSK, a_MCX_SIP_ServiceAuthorization_ReceiveSubscribeSend200OK |
| --- | --- |
| Reason for change | R5-240565 has corrected the MIKEY message for CSK upload for the case of the SIP REGISTER but breaking the case of the CSK upload to CMS and GMS with SIP SUBCRIBE |
| Summary of change | Parameter p_RequestUri := omit added to f_MCX_SIP_RegistrationSubscription_DecodeMatchAndCheckMikeyMessage_SetCSK to allow a_MCX_SIP_ServiceAuthorization_ReceiveSubscribeSend200OK to hand over the correct URI in case of CSK upload to GMS/CMS |
| TTCN module | MCX_SIP_Registration |

function f_MCX_SIP_RegistrationSubscription_DecodeMatchAndCheckMikeyMessage_SetCSK(MCX_ServiceType_Type p_ServiceType, MIME_Message p_MIME_Message, MCX_KeyIndex_Type p_KeyIndex, template (omit) SipUrl p_RequestUri := omit) runs on MCX_IMS_PTC { var charstring v_IDRr; var template (present) MIKEY_Message v_MIKEY_MessageTemplate; var MIKEY_Message v_MIKEY_Message; if (isvalue(p_RequestUri)) { v_IDRr := f_SIP_SipUrl2Str(valueof(p_RequestUri)); } else { v_IDRr := f_MCX_MCServer_PublicServiceId(p_ServiceType); } v_MIKEY_MessageTemplate := f_MIKEY_SAKKE_MessageRX_CSK(p_ServiceType, v_IDRr); v_MIKEY_Message := f_MCX_MIME_DecodeMatchAndCheckMikeyMessage(p_MIME_Message, v_MIKEY_MessageTemplate); fl_MCX_IMS_MIKEY_Message_ExtractAndSetCSK(v_MIKEY_Message, p_KeyIndex); // p_KeyIndex may be CSK, CSK_CMS, CSK_GMS1, CSK_GMS2 }

altstep a_MCX_SIP_ServiceAuthorization_ReceiveSubscribeSend200OK(out MCX_DataRequestWithXmlBodies_Type p_DataRequestWithXmlBodies, MCX_ServiceType_Type p_ServiceType, MCX_Subscription_Type p_Subscription := INITIAL_SUBSCRIBE, IMS_PTC_DialogIndex_Type p_DialogIndex, SipUrl p_RequestURI, SipUrl p_ContactURI, template (present) ResourceLists_Type p_ResourceListsTemplate) runs on MCX_IMS_PTC { var MCX_DataRequestWithXmlBodies_Type v_DataRequestWithXmlBodies; var IMS_DATA_REQ v_IMS_SUBSCRIBE_REQ_ByRef; var SUBSCRIBE_Request v_SubscribeRequest; var MIME_Message v_MIME_Message; var MCX_KeyIndex_Type v_KeyIndex; var template (present) MCX_ServiceInfo_Type v_MCS_Info := cs_MCX_Service_Info_Init; [] a_MCX_SIP_ReceiveSubscribeSend200OK(v_IMS_SUBSCRIBE_REQ_ByRef, p_ServiceType, p_Subscription, p_DialogIndex, p_RequestURI, p_ContactURI) { v_DataRequestWithXmlBodies.Request := v_IMS_SUBSCRIBE_REQ_ByRef.Request; v_SubscribeRequest := v_DataRequestWithXmlBodies.Request.Subscribe; v_MIME_Message := v_SubscribeRequest.messageBody.mimeMessageBody; v_KeyIndex := fl_MCX_XcapDiffDialog_GetKeyIndex(p_DialogIndex); if (p_Subscription == INITIAL_SUBSCRIBE) { // Check MIKEY message according to 36.579-1 Table 5.5.9.1-1: MIKEY message from the UE f_MCX_SIP_RegistrationSubscription_DecodeMatchAndCheckMikeyMessage_SetCSK(p_ServiceType, v_MIME_Message, v_KeyIndex, v_SubscribeRequest.requestLine.requestUri); v_MCS_Info := fl_MCX_SIP_Registration_McsInfo(p_ServiceType, v_DataRequestWithXmlBodies.Request); } v_DataRequestWithXmlBodies.XmlBodies := f_MCX_MIME_DecodeAndMatchXML(v_MIME_Message, p_ServiceType, v_KeyIndex, v_MCS_Info.Mcptt, -, v_MCS_Info.Mcvideo, -, v_MCS_Info.Mcdata, -, -, -, -, p_ResourceListsTemplate); // Store client-id (if any: e.g. there is no MCPTT info in re-SUBSCRIBE) fl_MCX_SIP_Registration_ClientId_Set(v_DataRequestWithXmlBodies.XmlBodies); p_DataRequestWithXmlBodies := v_DataRequestWithXmlBodies; } }

## Change 2

| Function names | f_MCX_SecurityParameters_Init |
| --- | --- |
| Reason for change | According to 33.180 F.2.1 the Current Key Period No (P5) is calculated by P5 = Floor ( ( TIME - P4 ) / P3 ) but in f_MCX_SecurityParameters_Init the Key Period offset (P4) is missing in the calculation |
| Summary of change | Calculation of the Current Key Period No corrected |
| TTCN module | MCX_SecurityParameters |

function f_MCX_SecurityParameters_Init(charstring p_KSM_Id) return MCX_SecurityParameters_Type { var MCX_SecurityParameters_Type v_SecurityParameters := valueof(cs_MCX_SecurityParameters_Init); var integer v_TimestampSeconds := f_UTC_TimestampSeconds(); var MCX_SecurityKey_Type v_GMK := f_MCX_SecurityParameters_CreateKey(GMK); var MCX_SecurityKey_Type v_PCK := f_MCX_SecurityParameters_CreateKey(PCK); v_SecurityParameters := f_MCX_SecurityParameters_SetKey(v_SecurityParameters, GMK, v_GMK); v_SecurityParameters := f_MCX_SecurityParameters_SetKey(v_SecurityParameters, PCK, v_PCK); v_SecurityParameters := f_MCX_SecurityParameters_SetKey(v_SecurityParameters, CSK); // initialised as omit v_SecurityParameters := f_MCX_SecurityParameters_SetKey(v_SecurityParameters, CSK_CMS); // initialised as omit v_SecurityParameters := f_MCX_SecurityParameters_SetKey(v_SecurityParameters, CSK_GMS1); // initialised as omit v_SecurityParameters := f_MCX_SecurityParameters_SetKey(v_SecurityParameters, CSK_GMS2); // initialised as omit v_SecurityParameters := f_MCX_SecurityParameters_SetKey(v_SecurityParameters, CSK_GMS3); // initialised as omit @sic R5s210398 sic@ v_SecurityParameters := f_MCX_SecurityParameters_SetKey(v_SecurityParameters, CSK_GMS4); // initialised as omit @sic R5s210398 sic@ v_SecurityParameters := f_MCX_SecurityParameters_SetKey(v_SecurityParameters, TrK, cs_MCX_SecurityKey(px_MCX_TrK_ID, bit2oct(px_MCX_TrK))); v_SecurityParameters := f_MCX_SecurityParameters_SetKey(v_SecurityParameters, InK, cs_MCX_SecurityKey(px_MCX_InK_ID, bit2oct(px_MCX_InK))); v_SecurityParameters.KSM_Id := p_KSM_Id; v_SecurityParameters.ECCSI.KPAK := fx_ECCSI_GenerateKPAK(v_SecurityParameters.ECCSI.KSAK); v_SecurityParameters.SAKKE.PublicKey := fx_SAKKE_GeneratePublicKey(v_SecurityParameters.SAKKE.MasterSecret); v_SecurityParameters.KeyPeriod.Offset := v_TimestampSeconds mod v_SecurityParameters.KeyPeriod.Length; v_SecurityParameters.KeyPeriod.No := (v_TimestampSeconds - v_SecurityParameters.KeyPeriod.Offset) / v_SecurityParameters.KeyPeriod.Length; return v_SecurityParameters; }