3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240366

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3440 | rev | - | Current version: | 17.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for NR MAC TC 7.1.1.1.17 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test, NR_redcap_plus_ARCH-UEConTest |  |  |  |  |  | Date: |  |  | 2024-05-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | For network to correctly apply all changes made for modified SIB1 of NR Cell 1 to configure the specific PRACH resource for RedCap in initialUplinkBWP. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In current implementation of test 7.1.1.1.17, there is no SYSTEM_CTRL_REQ sent to signal network for the changes to configure the specific PRACH resource for RedCap in initialUplinkBWP. Therefore, the test procedure inside f_TC_7_1_1_1_17_NR5GC is modified to send this for updating network configuration prior to sending modified sib to UE at Step 1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.1.17.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR… |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2022-09_D24wk12 related to the title of this CR.

Contact:	Kalahasti, Narendra

Narendra.Kalahasti@anritsu.com

# Corrections required

## Change 1

| Function name | function f_TC_7_1_1_1_17_NR5GC() |
| --- | --- |
| Reason for change | For network to correctly apply all changes made for modified SIB1 of NR Cell 1 to configure the specific PRACH resource for RedCap in initialUplinkBWP |
| Summary of change | Modified the body of f_TC_7_1_1_1_17_NR5GC to generate System_Ctrl_Req for necessary updates to NW prior to SIB1 Modification on Step 1. |
| TTCN module | MAC_R16_R17_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change

function f_TC_7_1_1_1_17_NR5GC() runs on NR5GC_PTC { /* Random access procedure / RedCap UE identification */ var template (value) NR_UplinkBWP_Type v_NR_UplinkBWP ; //Initial BWP var template (value) RACH_ConfigCommon v_RACH_ConfigCommon; var template (value) RACH_ConfigCommon v_RACH_ConfigCommon_Back; var template (omit) BWP_UplinkCommon.pucch_ConfigCommon v_Pucch_ConfigCommon_Back := omit; f_NR5GC_Init(NR_1); // Init Cell parameters //@sic R5s230656 sic@ f_NR_CellInfo_SetSIB1_FeaturePriority(nr_Cell1, cs_FeaturePriorities_711117); f_NR_CellConfig_Def(nr_Cell1); // Create and configure NR cell 1 f_NR5GC_Preamble(nr_Cell1, STATE_IDLE_1A); f_NR_CellInfo_SetMAC_CellGroupConfigL2(nr_Cell1); f_NR_TestBody_Set(true); v_NR_UplinkBWP := f_NR_CellInfo_GetUplinkBWP (nr_Cell1, tsc_NR_BWP_Id); //Initial BWP v_Pucch_ConfigCommon_Back:= f_NR_CellInfo_GetSIB1_PUCCHConfigCommon( nr_Cell1); v_RACH_ConfigCommon := v_NR_UplinkBWP.Common.R15.rach_ConfigCommon.setup ; v_RACH_ConfigCommon_Back:= v_RACH_ConfigCommon; //@siclog "Step 1" siclog@ //PUCCH_ConfigCommon & FeaturePriorities configured in preamble //@sic R5-231529 sic@ v_RACH_ConfigCommon.featureCombinationPreamblesList_r17 := {cs_FeatureCombinationPreambles_711117(8)}; f_NR_CellInfo_SetSIB1_InitialUplinkBWP_Rach_ConfigCommon(nr_Cell1,cs_Rach_ConfigCommon_Config_Setup(v_RACH_ConfigCommon) ); //@siclog "Step 2-10" siclog@ f_TC_7_1_1_1_17_NR_TestBody(8,3); //@siclog "Step 11" siclog@ //The SS transmits RRCRelease f_NR_RRCRelease(nr_Cell1); //@siclog "Step 12" siclog@ f_NR_CellInfo_SetSIB1_PUCCHConfigCommon(nr_Cell1, v_Pucch_ConfigCommon_Back); f_NR_CellInfo_SetSIB1_InitialUplinkBWP_Rach_ConfigCommon(nr_Cell1,cs_Rach_ConfigCommon_Config_Setup(v_RACH_ConfigCommon_Back) ); v_RACH_ConfigCommon.featureCombinationPreamblesList_r17 := {cs_FeatureCombinationPreambles_711117(12)}; f_NR_ConfigRedCapUL_BWP(nr_Cell1,cds_PUCCH_ConfigCommon_ResourceCommonRedCap ,v_RACH_ConfigCommon ); //@siclog "Step 13-21" siclog@ f_TC_7_1_1_1_17_NR_TestBody(12,3); f_NR_TestBody_Set(false); f_NR_Postamble(nr_Cell1, STATE_CONNECTED_3A); };

After Change

function f_TC_7_1_1_1_17_NR5GC() runs on NR5GC_PTC { /* Random access procedure / RedCap UE identification */ var template (value) NR_UplinkBWP_Type v_NR_UplinkBWP ; //Initial BWP var template (value) RACH_ConfigCommon v_RACH_ConfigCommon; var template (value) RACH_ConfigCommon v_RACH_ConfigCommon_Back; var template (omit) BWP_UplinkCommon.pucch_ConfigCommon v_Pucch_ConfigCommon_Back := omit; var NR_PhysicalParameters_Type v_NR_PhysicalParameters := omit; f_NR5GC_Init(NR_1); // Init Cell parameters //@sic R5s230656 sic@ f_NR_CellInfo_SetSIB1_FeaturePriority(nr_Cell1, cs_FeaturePriorities_711117); f_NR_CellConfig_Def(nr_Cell1); // Create and configure NR cell 1 f_NR5GC_Preamble(nr_Cell1, STATE_IDLE_1A); f_NR_CellInfo_SetMAC_CellGroupConfigL2(nr_Cell1); f_NR_TestBody_Set(true); v_NR_UplinkBWP := f_NR_CellInfo_GetUplinkBWP (nr_Cell1, tsc_NR_BWP_Id); //Initial BWP v_Pucch_ConfigCommon_Back:= f_NR_CellInfo_GetSIB1_PUCCHConfigCommon( nr_Cell1); v_RACH_ConfigCommon := v_NR_UplinkBWP.Common.R15.rach_ConfigCommon.setup ; v_RACH_ConfigCommon_Back:= v_RACH_ConfigCommon; //@siclog "Step 1" siclog@ //PUCCH_ConfigCommon & FeaturePriorities configured in preamble //@sic R5-231529 sic@ v_RACH_ConfigCommon.featureCombinationPreamblesList_r17 := {cs_FeatureCombinationPreambles_711117(8)}; f_NR_CellInfo_SetSIB1_InitialUplinkBWP_Rach_ConfigCommon(nr_Cell1,cs_Rach_ConfigCommon_Config_Setup(v_RACH_ConfigCommon) ); v_NR_UplinkBWP.Common.R15.rach_ConfigCommon.setup := v_RACH_ConfigCommon; //Updated RACH Config Common f_NR_CellInfo_SetUplinkBWP(nr_Cell1, v_NR_UplinkBWP, tsc_NR_BWP_Id); v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(nr_Cell1); f_NR_SS_CommonCellConfig(nr_Cell1, cads_NR_CellConfigPhysicalLayerUplink(nr_Cell1, -, cs_NR_CellConfigPhysicalLayerUplink_Def(-,-,cs_NR_UplinkBWPs(v_NR_PhysicalParameters.UL_BWPs)))); //@siclog "Step 2-10" siclog@ f_TC_7_1_1_1_17_NR_TestBody(8,3); //@siclog "Step 11" siclog@ //The SS transmits RRCRelease f_NR_RRCRelease(nr_Cell1); //@siclog "Step 12" siclog@ f_NR_CellInfo_SetSIB1_PUCCHConfigCommon(nr_Cell1, v_Pucch_ConfigCommon_Back); f_NR_CellInfo_SetSIB1_InitialUplinkBWP_Rach_ConfigCommon(nr_Cell1,cs_Rach_ConfigCommon_Config_Setup(v_RACH_ConfigCommon_Back) ); v_RACH_ConfigCommon.featureCombinationPreamblesList_r17 := {cs_FeatureCombinationPreambles_711117(12)}; f_NR_ConfigRedCapUL_BWP(nr_Cell1,cds_PUCCH_ConfigCommon_ResourceCommonRedCap ,v_RACH_ConfigCommon ); //@siclog "Step 13-21" siclog@ f_TC_7_1_1_1_17_NR_TestBody(12,3); f_NR_TestBody_Set(false); f_NR_Postamble(nr_Cell1, STATE_CONNECTED_3A); };