3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240367

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4824 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to LTE testcase 7.1.3.9 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI8_Test |  |  |  |  |  | Date: |  |  | 2024-05-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | At the end of the test case Status PDU is transmitted with immediate activation time and postamble procedures are also triggered with immediate activation time. This could lead to the situation in CAT-M cases where the DRB is release before the data is transmitted, in order to improve the stability of the test case it is proposed to add a delay ( similar to other MAC test cases) |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added f_DelayForDLPDU_Tx before entering the postamble procedures |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.3.9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections required to test case 7.1.3.9	2

2.1	f_TC_7_1_3_9_EUTRA	2

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D24wk12 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required to test case 7.1.3.9

## f_TC_7_1_3_9_EUTRA

| Template name | f_TC_7_1_3_9_EUTRA |
| --- | --- |
| Reason for change | At the end of the test case Status PDU is transmitted with immediate activation time and postamble procedures are also triggered with immediate activation time. This could lead to the situation in CAT-M cases where the DRB is release before the data is transmitted, in order to improve the stability of the test case it is proposed to add a delay ( similar to other MAC test cases) |
| Summary of change | Added f_DelayForDLPDU_Tx before entering the postamble procedures |
| TTCN module | MAC_713.ttcn |
| MCC160 Comment |  |

Before Change:

function f_TC_7_1_3_9_EUTRA ( ) runs on EUTRA_PTC { /* L2/MAC * 7.1.3.9 : MAC reset-DL */ … … … //@siclog "Step 10" siclog@ // Automatically taken care of //@siclog "Step 11" siclog@ t_Watchdog.start; //@sic R5s120629 sic@ DRB.receive (car_DRB_COMMON_IND_RLC_PDUList (eutra_Cell2, tsc_RbId_DRB1, cr_TimingInfo_Any, {cr_RLC_AMD_1PDU(?, int2bit(0, 10), v_EncodedPdcpPdu)})); f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 11"); t_Watchdog.stop; f_RLC_Status_PDU_Tx (tsc_LchId_DRB1, 1, eutra_Cell2);//@sic R5s120629 sic@ f_EUTRA_TestBody_Set(false); //test body ends /////////////////////////////////////////////////////////////////////////////////// // Post Amble ////////////////////////////////////////////////////////////////////////////////// // to configure SS to stop report reception of HARQ ACK or NACK f_SS_ConfigUL_HARQ_IndMode (eutra_Cell1,tsc_L1Mac_IndicationMode_Disable ); // to disable reception of SR in cell 2 f_SS_ConfigSchedulingRequestIndMode(eutra_Cell2, tsc_L1Mac_IndicationMode_Disable); f_SS_Config1TM_DRB_NoMACHeader(eutra_Cell2, tsc_RbId_DRB1,cs_MAC_TestMode_NormalMode);

After Change:

function f_TC_7_1_3_9_EUTRA ( ) runs on EUTRA_PTC { /* L2/MAC * 7.1.3.9 : MAC reset-DL */ … … … //@siclog "Step 10" siclog@ // Automatically taken care of //@siclog "Step 11" siclog@ t_Watchdog.start; //@sic R5s120629 sic@ DRB.receive (car_DRB_COMMON_IND_RLC_PDUList (eutra_Cell2, tsc_RbId_DRB1, cr_TimingInfo_Any, {cr_RLC_AMD_1PDU(?, int2bit(0, 10), v_EncodedPdcpPdu)})); f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 11"); t_Watchdog.stop; f_RLC_Status_PDU_Tx (tsc_LchId_DRB1, 1, eutra_Cell2);//@sic R5s120629 sic@ f_EUTRA_TestBody_Set(false); //test body ends /////////////////////////////////////////////////////////////////////////////////// // Post Amble ////////////////////////////////////////////////////////////////////////////////// // to configure SS to stop report reception of HARQ ACK or NACK f_DelayForDLPDU_Tx(); // delay of 100 ms for status PDU transmission f_SS_ConfigUL_HARQ_IndMode (eutra_Cell1,tsc_L1Mac_IndicationMode_Disable ); // to disable reception of SR in cell 2 f_SS_ConfigSchedulingRequestIndMode(eutra_Cell2, tsc_L1Mac_IndicationMode_Disable); f_SS_Config1TM_DRB_NoMACHeader(eutra_Cell2, tsc_RbId_DRB1,cs_MAC_TestMode_NormalMode);