3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240379

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3447 | rev | - | Current version: | 17.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for NR5GC IRAT TC 9.1.9.7 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, RACS-UEConTest |  |  |  |  |  | Date: |  |  | 2024-05-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In current TTCN at the step 4 is UE radio capability information update needed IE is checking like mandatory parameter in TAU. Based on 38.508(Table 4.9.7.2.3-1: TRACKING AREA UPDATE ) This IE is mandatory only for condition First-N1-to-S1 which is not case at the step 4. This IE should be optional. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct TTCN that UE radio capability information update needed IE in TAU at the step 4 will be optional. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.9.7.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR… |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2022-09_D24wk12 related to the title of this CR.

Contact:	Kalahasti, Narendra

Narendra.Kalahasti@anritsu.com

# Corrections required

## Change 1

| Function name | function fl_TC_9_1_9_7_NR5GC_EUTRA_TestBody |
| --- | --- |
| Reason for change | In current TTCN at the step 4 is UE radio capability information update needed IE is checking like mandatory parameter in TAU. Based on 38.508(Table 4.9.7.2.3-1: TRACKING AREA UPDATE ) This IE is mandatory only for condition First-N1-to-S1 which is not case at the step 4. This IE should be optional. |
| Summary of change | Correct TTCN that UE radio capability information update needed IE in TAU at the step 4 will be optional. |
| TTCN module | RACS_IRAT_NR5GC_EUTRA.ttcn |
| MCC160 Comment |  |

Before Change

function fl_TC_9_1_9_7_NR5GC_EUTRA_TestBody() runs on EUTRA_Multi5GS_PTC { var NAS_MSG_Indication_Type v_NasInd; // @siclog "Step 1" siclog@ //The SS configures:- NGC Cell A as "Serving cell"- E-UTRA Cell A as "Non-suitable "off" cell". f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger); f_EUTRA38_SetCellPower(eutra_CellA, tsc_NonSuitableOffCellRS_EPRE, tsc_NonSuitableOffCellRS_EPRE); f_EUTRA_NR_WaitForCoOrd_Trigger(NR);// @sic R5s221235 sic@ // @siclog "Step 2" siclog@ //Check: Does the UE perform test procedure for UE Tracking area updating for Inter-system change from S1 mode to N1 mode in 5GMM/EMM-IDLE mode as described in TS 38.508-1 [4], Table 4.9.9.2.2-1 on NGC Cell A? f_EUTRA38_NR5GC_S1ToN1_IdleMode(eutra_CellA); f_EUTRA_NR_WaitForCoOrd_Trigger(NR);// @sic R5s221235 sic@ // @siclog "Step 3" siclog@ //The SS configures:- E-UTRA Cell A as "Serving cell"- NGC Cell A as "Non-suitable "off" cell". f_EUTRA38_SetCellPower(eutra_CellA, tsc_ServingCellRS_EPRE, tsc_ServingCellRS_EPRE); f_EUTRA_NR_WaitForCoOrd_Trigger(NR);// @sic R5s221235 sic@ // @siclog "Step 4" siclog@ //Check: Does the UE perform Test procedure for UE Tracking area updating for Inter-system change from N1 mode to S1 mode in 5GMM/EMM-IDLE mode as described in TS 38.508-1 [4], Table 4.9.7.2.2-1, 'connected without release' on E-UTRA Cell A? v_NasInd := f_EUTRA38_TAUReqFromN1(eutra_CellA,tsc_RRC_TI_Def); if (not match (v_NasInd.Pdu.Msg.tRACKING_AREA_UPDATE_REQUEST.ueRadioCapIdAvailability.availability, '001'B)) {// @sic R5-241158 sic@ f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "TAU req failed"); } f_EUTRA38_TrackingAreaUpdateFromN1_WithoutReq(eutra_CellA, v_NasInd, -, noRrcConnectionRelease); f_EUTRA_PreliminaryPass (__FILE__, __LINE__, "Step 4"); }

After Change

function fl_TC_9_1_9_7_NR5GC_EUTRA_TestBody() runs on EUTRA_Multi5GS_PTC { var NAS_MSG_Indication_Type v_NasInd; // @siclog "Step 1" siclog@ //The SS configures:- NGC Cell A as "Serving cell"- E-UTRA Cell A as "Non-suitable "off" cell". f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger); f_EUTRA38_SetCellPower(eutra_CellA, tsc_NonSuitableOffCellRS_EPRE, tsc_NonSuitableOffCellRS_EPRE); f_EUTRA_NR_WaitForCoOrd_Trigger(NR);// @sic R5s221235 sic@ // @siclog "Step 2" siclog@ //Check: Does the UE perform test procedure for UE Tracking area updating for Inter-system change from S1 mode to N1 mode in 5GMM/EMM-IDLE mode as described in TS 38.508-1 [4], Table 4.9.9.2.2-1 on NGC Cell A? f_EUTRA38_NR5GC_S1ToN1_IdleMode(eutra_CellA); f_EUTRA_NR_WaitForCoOrd_Trigger(NR);// @sic R5s221235 sic@ // @siclog "Step 3" siclog@ //The SS configures:- E-UTRA Cell A as "Serving cell"- NGC Cell A as "Non-suitable "off" cell". f_EUTRA38_SetCellPower(eutra_CellA, tsc_ServingCellRS_EPRE, tsc_ServingCellRS_EPRE); f_EUTRA_NR_WaitForCoOrd_Trigger(NR);// @sic R5s221235 sic@ // @siclog "Step 4" siclog@ //Check: Does the UE perform Test procedure for UE Tracking area updating for Inter-system change from N1 mode to S1 mode in 5GMM/EMM-IDLE mode as described in TS 38.508-1 [4], Table 4.9.7.2.2-1, 'connected without release' on E-UTRA Cell A? v_NasInd := f_EUTRA38_TAUReqFromN1(eutra_CellA,tsc_RRC_TI_Def, -, -, -, -, false); if (not match (v_NasInd.Pdu.Msg.tRACKING_AREA_UPDATE_REQUEST.ueRadioCapIdAvailability.availability, '001'B)) {// @sic R5-241158 sic@ f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "TAU req failed"); } f_EUTRA38_TrackingAreaUpdateFromN1_WithoutReq(eutra_CellA, v_NasInd, -, noRrcConnectionRelease); f_EUTRA_PreliminaryPass (__FILE__, __LINE__, "Step 4"); }