3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240385

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3451 | rev | - | Current version: | 17.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC common functions initialising Delta values |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-05-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The function fl_GetNR_Band_SsbForDelta returns the Band_SsbInfo_Type to be sent to SS to trigger SS to return the delta values associated to the band and SSB frequencies under test. In case of SUL, the function fl_GetNR_Band_SsbForDelta returns ‘omit’ because there is no SSB frequencies for these SUL bands. But this type in the ASP is not optional, so this will cause a run time error. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated common functions fl_GetNR_Band_SsbForDelta and f_NR_RetrieveDeltaValues. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE will fail the SUL test case 7.1.1.1.5.NR5GC. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.1.5.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Corrections required:

## Change to function fl_GetNR_Band_SsbForDelta

| Function name | fl_GetNR_Band_SsbForDelta |
| --- | --- |
| Reason for change | The function fl_GetNR_Band_SsbForDelta returns the Band_SsbInfo_Type to be sent to SS to trigger SS to return the delta values associated to the band and SSB frequencies under test. In case of SUL, the function fl_GetNR_Band_SsbForDelta currently returns ‘omit’ because there is no SSB frequencies for these SUL bands. The type in the ASP is not optional, so this will cause a run time error. |
| Summary of change | Update the returned type from template(omit) to template (value). When an SSB frequency is not available for a band, the SSB frequency field is set to omit. It is proposed to apply the same for SUL: return the template(value) initialised with the band and all SSB frequencies set to ‘omit’. |
| TTCN module | NR_CellInfoInit.ttcn |
| MCC160 Comment |  |

Before Change:

/*

* @desc      Retrieve NR_Band_SsbForDelta_Type per band - to be used for delta value intialisation

* @param     p_NR_Frequencyf1tof4

* @param     p_FreqBandIndicatorNR

* @return    template (omit) Band_SsbInfo_Type

* @status    APPROVED (ENDC, IMS, NR5GC, NR5GC_IRAT, POS)

*/

function fl_GetNR_Band_SsbForDelta(template (value) NR_Frequencyf1tof4_Type p_NR_Frequencyf1tof4,

FreqBandIndicatorNR  p_FreqBandIndicatorNR

) return template (omit) Band_SsbInfo_Type

{ var template (omit) Band_SsbInfo_Type v_Band_SsbInfo := omit;

if ( not(f_NR_IsBandSUL(p_FreqBandIndicatorNR))) {

v_Band_SsbInfo := cs_Band_SsbInfo( p_FreqBandIndicatorNR,

fl_GetNR_ASN1_ARFCN_ValueNR(p_NR_Frequencyf1tof4.FrequencyInfoDL_f1),

fl_GetNR_ASN1_ARFCN_ValueNR(p_NR_Frequencyf1tof4.FrequencyInfoDL_f2),

fl_GetNR_ASN1_ARFCN_ValueNR(p_NR_Frequencyf1tof4.FrequencyInfoDL_f3),

fl_GetNR_ASN1_ARFCN_ValueNR(p_NR_Frequencyf1tof4.FrequencyInfoDL_f4)

);

}

return v_Band_SsbInfo;

}

After Change:

/*

* @desc      Retrieve NR_Band_SsbForDelta_Type per band - to be used for delta value intialisation

* @param     p_NR_Frequencyf1tof4

* @param     p_FreqBandIndicatorNR

* @return    template (omit) Band_SsbInfo_Type

* @status    APPROVED (ENDC, IMS, NR5GC, NR5GC_IRAT, POS)

*/

function fl_GetNR_Band_SsbForDelta(template (value) NR_Frequencyf1tof4_Type p_NR_Frequencyf1tof4,

FreqBandIndicatorNR  p_FreqBandIndicatorNR

) return template (value) Band_SsbInfo_Type

{ var template (value) Band_SsbInfo_Type v_Band_SsbInfo;

if ( not(f_NR_IsBandSUL(p_FreqBandIndicatorNR))) {

v_Band_SsbInfo := cs_Band_SsbInfo( p_FreqBandIndicatorNR,

fl_GetNR_ASN1_ARFCN_ValueNR(p_NR_Frequencyf1tof4.FrequencyInfoDL_f1),

fl_GetNR_ASN1_ARFCN_ValueNR(p_NR_Frequencyf1tof4.FrequencyInfoDL_f2),

fl_GetNR_ASN1_ARFCN_ValueNR(p_NR_Frequencyf1tof4.FrequencyInfoDL_f3),

fl_GetNR_ASN1_ARFCN_ValueNR(p_NR_Frequencyf1tof4.FrequencyInfoDL_f4)

);

} else {

v_Band_SsbInfo := cs_Band_SsbInfo( p_FreqBandIndicatorNR);

}

return v_Band_SsbInfo;

}

## Change to function f_NR_RetrieveDeltaValues

| Function name | f_NR_RetrieveDeltaValues |
| --- | --- |
| Reason for change | In case of SUL, the function fl_GetNR_Band_SsbForDelta returns ‘omit’ because there is no SSB frequencies for these SUL bands. The type in the ASP is not optional, so this will cause a run time error. |
| Summary of change | Following change 2.1, the variables initialised with fl_GetNR_Band_SsbForDelta, shall be declared as template(value). |
| TTCN module | NR_CellInfoInit.ttcn |
| MCC160 Comment |  |

Before Change:

function f_NR_RetrieveDeltaValues(template (value) NR_Frequencyf1tof4_Type p_NR_PrimaryFrequencyf1tof4,

FreqBandIndicatorNR  p_PrimaryFreqBandIndicatorNR,

template (value) NR_Frequencyf1tof4_Type p_NR_SecondaryFrequencyf1tof4,

FreqBandIndicatorNR  p_SecondaryFreqBandIndicatorNR

) runs on NR_BASE_PTC return UE_NR_DeltaValues_Type

{ //@sic R5w190312: ASP for delta values sic@

var template (omit) Band_SsbInfo_Type v_Band_SsbInfoPrimary := fl_GetNR_Band_SsbForDelta(p_NR_PrimaryFrequencyf1tof4, p_PrimaryFreqBandIndicatorNR);

var template (omit) Band_SsbInfo_Type v_Band_SsbInfoSecondary := fl_GetNR_Band_SsbForDelta(p_NR_SecondaryFrequencyf1tof4, p_SecondaryFreqBandIndicatorNR);

var NR_SYSTEM_CTRL_CNF  v_ReceivedAsp;

SYS.send(cas_NR_DeltaValuesTrigger_REQ(v_Band_SsbInfoPrimary, v_Band_SsbInfoSecondary));

SYS.receive(car_NR_DeltaValuesTrigger_CNF) -> value v_ReceivedAsp;

return v_ReceivedAsp.Confirm.DeltaValues;

}

After Change:

function f_NR_RetrieveDeltaValues(template (value) NR_Frequencyf1tof4_Type p_NR_PrimaryFrequencyf1tof4,

FreqBandIndicatorNR  p_PrimaryFreqBandIndicatorNR,

template (value) NR_Frequencyf1tof4_Type p_NR_SecondaryFrequencyf1tof4,

FreqBandIndicatorNR  p_SecondaryFreqBandIndicatorNR

) runs on NR_BASE_PTC return UE_NR_DeltaValues_Type

{ //@sic R5w190312: ASP for delta values sic@

var template (value) Band_SsbInfo_Type v_Band_SsbInfoPrimary := fl_GetNR_Band_SsbForDelta(p_NR_PrimaryFrequencyf1tof4, p_PrimaryFreqBandIndicatorNR);

var template (value) Band_SsbInfo_Type v_Band_SsbInfoSecondary := fl_GetNR_Band_SsbForDelta(p_NR_SecondaryFrequencyf1tof4, p_SecondaryFreqBandIndicatorNR);

var NR_SYSTEM_CTRL_CNF  v_ReceivedAsp;

SYS.send(cas_NR_DeltaValuesTrigger_REQ(v_Band_SsbInfoPrimary, v_Band_SsbInfoSecondary));

SYS.receive(car_NR_DeltaValuesTrigger_CNF) -> value v_ReceivedAsp;

return v_ReceivedAsp.Confirm.DeltaValues;

}