3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240387

# Online,  , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-5 | CR | 0113 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to MCX common functions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test, MCImp-UEConTest |  |  |  |  |  | Date: |  |  | 2024-06-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | 1, In current implementation of f_XPATH_ExpressionForAnyNamespaces(), Escape Backslash (\) is needed to escape special regex characters “[“ and “]” 2, As RFC 3830 clause 5.2: "Calculate the MAC/signature over the entire MIKEY message, except the MAC/Signature field, and add the MAC/signature in the field In current TTCN implementation of f_MIKEY_SAKKE_Signature_Validate, the code ONLY removes the signature string field (MIKEY_Message.sign.s), where it should actually remove the whole sign (including sType, and sLen fields) |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1, Added Escape Backslash for special regex characters 2, Added code to remove the extra 2bytes (16 bits), i.e sType (B4_Type) and sLen (B12_Type) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE will fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | f_XPATH_ExpressionForAnyNamespaces, f_MIKEY_SAKKE_Signature_Validate |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	4

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2022-09_D24wk12 related to the title of this CR.

Contact:	Mohit Kanchan
	mohit.kanchan@keysight.com

# Corrections required

## Change 1

| Function name | f_XPATH_ExpressionForAnyNamespaces() |
| --- | --- |
| Reason for change | Escape Backslash (\) is needed to escape special regex characters “[“ and “]” |
| Summary of change | Added Escape Backslash for special regex characters |
| TTCN module | MCX_XML_Encryption.ttcn |
| MCC160 Comment |  |

Before Change:

function f_XPATH_ExpressionForAnyNamespaces(charstring p_Expression) return charstring { var charstring v_String := regexp(p_Expression, "//(*)", 0); var CharStringList_Type v_ElementList; var charstring v_Element; var charstring v_Expression; var charstring v_IndexExpr; var charstring v_RegExp; var integer i; if (v_String == "") { v_Expression := p_Expression; } else { v_ElementList := f_StringSplit(v_String, {"/"}); for (i := 0; i < lengthof(v_ElementList); i := i + 1) { v_Element := v_ElementList[i]; v_RegExp := "(*)([[][0-9]+[]])"; v_IndexExpr := regexp(v_Element, v_RegExp, 1); if (v_IndexExpr != "") { v_Element := regexp(v_Element, v_RegExp, 0); } v_ElementList[i] := "*[local-name()='" & v_Element & "']" & v_IndexExpr; } v_Expression := "//" & f_StringJoin(v_ElementList, "/"); } return v_Expression; }

After Change:

function f_XPATH_ExpressionForAnyNamespaces(charstring p_Expression) return charstring { var charstring v_String := regexp(p_Expression, "//(*)", 0); var CharStringList_Type v_ElementList; var charstring v_Element; var charstring v_Expression; var charstring v_IndexExpr; var charstring v_RegExp; var integer i; if (v_String == "") { v_Expression := p_Expression; } else { v_ElementList := f_StringSplit(v_String, {"/"}); for (i := 0; i < lengthof(v_ElementList); i := i + 1) { v_Element := v_ElementList[i]; v_RegExp := "(*)([\[][0-9]+[\]])"; v_IndexExpr := regexp(v_Element, v_RegExp, 1); if (v_IndexExpr != "") { v_Element := regexp(v_Element, v_RegExp, 0); } v_ElementList[i] := "*[local-name()='" & v_Element & "']" & v_IndexExpr; } v_Expression := "//" & f_StringJoin(v_ElementList, "/"); } return v_Expression; }

## Change 2

| - Function name | f_MIKEY_SAKKE_Signature_Validate |
| --- | --- |
| Reason for change | As RFC 3830 clause 5.2: "Calculate the MAC/signature over the entire MIKEY message, except the MAC/Signature field, and add the MAC/signature in the field In current TTCN implementation, the code ONLY removes the signature string field (MIKEY_Message.sign.s), where it should actually remove the whole sign (including sType, and sLen fields) |
| Summary of change | Added code to remove the extra 2bytes (16 bits), i.e sType (B4_Type) and sLen (B12_Type) from the octetstring v_Message |
| TTCN module | MCX_MIKEY_Messages.ttcn |
| MCC160 Comment |  |

Before Change:

function f_MIKEY_SAKKE_Signature_Validate(MCX_SecurityParameters_Type p_SecurityParameters, MIKEY_Message p_MIKEY_Message) return boolean { var MIKEY_Message v_MIKEY_Message := p_MIKEY_Message; var charstring v_IDRiURI := f_MIKEY_Message_GetIDRIdentity(v_MIKEY_Message, tsc_MIKEY_IDR_Role_IDRi); // Initiator's URI var octetstring v_UID := f_MCX_SecurityParameters_GetUID(p_SecurityParameters, v_IDRiURI); var octetstring v_Signature := p_MIKEY_Message.sign.s; var octetstring v_Message; v_MIKEY_Message.sign.s := ''O; /* RFC 3830 clause 5.2: "Calculate the MAC/signature over the entire MIKEY message, except the MAC/Signature field, and add the MAC/signature in the field" According to clause 6.5 the MAC/Signature field is the field of the Signature payload. => generation of the signature includes the Signature payload's type and length field */ v_Message := bit2oct(encvalue(v_MIKEY_Message)); return fx_ECCSI_VerifySignature(v_Message, v_Signature, p_SecurityParameters.ECCSI.KPAK, v_UID); }

After Change:

function f_MIKEY_SAKKE_Signature_Validate(MCX_SecurityParameters_Type p_SecurityParameters, MIKEY_Message p_MIKEY_Message) return boolean { var MIKEY_Message v_MIKEY_Message := p_MIKEY_Message; var charstring v_IDRiURI := f_MIKEY_Message_GetIDRIdentity(v_MIKEY_Message, tsc_MIKEY_IDR_Role_IDRi); // Initiator's URI var octetstring v_UID := f_MCX_SecurityParameters_GetUID(p_SecurityParameters, v_IDRiURI); var octetstring v_Signature := p_MIKEY_Message.sign.s; var octetstring v_Message; v_MIKEY_Message.sign.s := ''O; /* RFC 3830 clause 5.2: "Calculate the MAC/signature over the entire MIKEY message, except the MAC/Signature field, and add the MAC/signature in the field" According to clause 6.5 the MAC/Signature field is the field of the Signature payload. => generation of the signature includes the Signature payload's type and length field */ v_Message := bit2oct(encvalue(v_MIKEY_Message)); if (ispresent(v_MIKEY_Message.sign)) { v_Message := substr(v_Message, 0, lengthof(v_Message)-2); } return fx_ECCSI_VerifySignature(v_Message, v_Signature, p_SecurityParameters.ECCSI.KPAK, v_UID); }