3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240400

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4792 | rev | 1 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of LTE NAS RACS testcase 9.2.5.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, RACS-UEConTest |  |  |  |  |  | Date: |  |  | 2024-02-27 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To add test case 9.2.5.1 to the LTE_A_PRO ATS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to test case 9.2.5.1 required for approval. See detailed change description for further information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.5.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS36.523-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1	Change 1	3

3	Branches executed	7

4	Execution Log Files	7

4.1	Samsung Shannon S5E9945ERD UE	7

5	References	8

# Overview

This document lists all the essential changes needed to correct problems in the TTCN implementation of test case 9.2.5.1 which is part of the LTE_A_PRO test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:	Kalahasti, Narendra

Narendra.Kalahasti@anritsu.com

## Verification Test Summary

Test Case: 	9.2.5.1

ATS Version:	iwd-TTCN3-B2022-09_D23wk49

System Simulator used:	Anritsu Protocol Conformance Test System ME7834NR

UE used:	Samsung Shannon S5E9945ERD UE

Verification Status:	PASS

# Corrections required

## Change 1

| Testcase name | fl_TC_9_2_5_1_Body |
| --- | --- |
| Reason for change | - Incorrect EPS update type is expected in TAU request at Step18 and Step25 and incorrect EPS update type is provided in TAU complete at Step22 and Step27. - At Step19, SS transmits a NAS SECURITY MODE COMMAND message for only requesting UE radio capability ID, hence at Step20, resetting of downlink NAS COUNT counter is not needed. - According to cl. 5.3.20 od TS 24.301 “A UE radio capability ID can be either manufacturer-assigned or network-assigned.” UE with manufacturer-assigned UE Radio Capability ID can include UE Radio Capability availability inidication in the TAU request at Step25, hence it is proposed to request actual ID using NAS Security Mode command and validate if UE reports Network-assigned ID. |
| Summary of change | - Update EPS update type from 000’B [TA updating] to 001’B [Combined TA/LA updating] - Do not reset downlink NAS COUNT counter - Validate the UE radio capability ID received in the NAS Security mode complete message instead of checking UE radio capability availability indication in the TAU request message. |
| TTCN module | LTE_A_PRO\9_2\NAS_RACS.ttcn |
| MCC160 Comment |  |

Before change:

function fl_TC_9_2_5_1_Body() runs on EUTRA_PTC

{

var NAS_MSG_Indication_Type v_NAS_Ind;

var template (omit) EPS_BearerContextStatus v_EPS_ContextStatus;

var NasCount_Type v_NasCountUL;

var EUTRA_SecurityParams_Type v_AuthParams;

var template (omit) UERadioCapId v_UERadioCapId;

//@siclog "Step 1" siclog@

//The UE is switched on.

f_EUTRA_SwitchOnUEandStartIP(eutra_CellA);

//@siclog "Step 2" siclog@

//Check: Does the UE include UE radio capability availability indication in ATTACH REQUEST message on Cell A?

v_NAS_Ind:= f_EUTRA_RRC_ConnEst_DefWithNas(eutra_CellA,

tsc_RRC_TI_Def,

cr_EstablishmentCause_Any,

cr_NAS_IndicationWithPiggybacking(tsc_SHT_IntegrityProtected,

cr_508_ATTACH_REQUEST(f_GetEAttachType(NORMAL), f_GetAdditionalUpdateType(NORMAL)),

cr_508_PDN_CONNECTIVITY_REQUEST(tsc_PdnRequest_InitialAttach,

f_GetPdnType())));

if (match (v_NAS_Ind.Pdu.Msg.aTTACH_REQUEST.ueRadioCapIdAvailability.availability, '001'B)) { //prose CR needed.

f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 2");

}

//@siclog "Step 3-4" siclog@

//Steps 5-6 of the generic procedure for UE registration specified in TS 36.508 subclause 4.5.2.3 are performed.

f_EUTRA_InitialRegistration_Step5_6(eutra_CellA, v_NAS_Ind);

v_AuthParams := f_EUTRA_Security_Get();

//@siclog "Step 5" siclog@

//The SS transmits a NAS SECURITY MODE COMMAND message requesting UE radio capability ID.

//siclog "Step 6" siclog@

//Check: Does the UE transmit a NAS SECURITY MODE COMPLETE message including UE radio capability ID 1?

if (ispresent(v_NAS_Ind.Pdu.Msg.aTTACH_REQUEST.ueAddSecurityCap)) {

v_AuthParams.NAS_SecurityCap5G := v_NAS_Ind.Pdu.Msg.aTTACH_REQUEST.ueAddSecurityCap;

}

v_NasCountUL:= f_EUTRA_NAS_ActivateSecurity(eutra_CellA,v_AuthParams,-,true,true, v_NAS_Ind.Pdu.Msg.aTTACH_REQUEST.ueRadioCapIdAvailability);

v_UERadioCapId := f_EUTRA_MobileInfo_GetRxdRACS_Id();

if (isvalue(v_UERadioCapId)) {

if (match(valueof(v_UERadioCapId), cs_UERadioCapId('66'O, '01000000000010'O))) {

f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 6");

} else {

f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 6");

}

} else {

f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 6");

}

//@siclog "Steps 7-11" siclog@

//Steps 9-13 of the generic procedure for UE registration specified in TS 36.508 subclause 4.5.2.3 are performed.

//@siclog "Step 12" siclog@

//SS transmits a ATTACH ACCEPT message with a new UE radio capability ID 2.

//@siclog "Steps 13-16" siclog@

//Steps 15-18 of the generic procedure for UE registration specified in TS 36.508 subclause 4.5.2.3 are performed.//step15 is done before this.

f_EUTRA_MobileInfo_SetRACS_IdToSend(valueof(cs_UERadioCapId('66'O, '01000000000020'O)));

f_EUTRA_InitialRegistration_Step9_18b1(eutra_CellA,v_NAS_Ind,v_NasCountUL);

//@siclog "Step 17" siclog@

//The SS configures:- Cell A as the "Non-Suitable cell".- Cell B as a "Serving cell".

f_EUTRA_SetCellPower(eutra_CellA, tsc_NonSuitableCellRS_EPRE);

f_EUTRA_SetCellPower(eutra_CellB, tsc_ServingCellRS_EPRE);

//@siclog "Step 18" siclog@

//Check: Does the UE include UE radio capability availability indication in TRACKING AREA UPDATE REQUEST message?

v_NAS_Ind := f_EUTRA_TrackingAreaUpdate_Step1_3(eutra_CellB,

tsc_RRC_TI_Def,

NORMAL,

?,

'000'B);

if (match (v_NAS_Ind.Pdu.Msg.tRACKING_AREA_UPDATE_REQUEST.ueRadioCapIdAvailability.availability, '001'B)) { //prose CR needed.

f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 18");

}

v_AuthParams := f_EUTRA_Security_Get();

//@siclog "Step 19" siclog@

//The SS transmits a NAS SECURITY MODE COMMAND message requesting UE radio capability ID.

//@siclog "Step 20" siclog@

//Check: Does the UE transmit a NAS SECURITY MODE COMPLETE message including UE radio capability ID 2?

v_NasCountUL:= f_EUTRA_NAS_ActivateSecurity(eutra_CellB,v_AuthParams,-,true,false, v_NAS_Ind.Pdu.Msg.tRACKING_AREA_UPDATE_REQUEST.ueRadioCapIdAvailability);

v_UERadioCapId := f_EUTRA_MobileInfo_GetRxdRACS_Id();

if (isvalue(v_UERadioCapId)) {

if (match(valueof(v_UERadioCapId), cs_UERadioCapId('66'O, '01000000000020'O))) {

f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 20");

} else {

f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 20");

}

} else {

f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 20");

}

//@siclog "Step 21" siclog@

//The SS transmits a TRACKING AREA UPDATE ACCEPT message.

//@siclog "Step 22" siclog@

//The UE transmits a TRACKING AREA UPDATE COMPLETE message.

v_EPS_ContextStatus := v_NAS_Ind.Pdu.Msg.tRACKING_AREA_UPDATE_REQUEST.epsBearerContextStatus;

f_EUTRA_TrackingAreaUpdate_Accept_Complete(eutra_CellB, f_EUTRA_CellInfo_GetGuti(eutra_CellB), '000'B, v_EPS_ContextStatus, v_NAS_Ind.Pdu.Msg.tRACKING_AREA_UPDATE_REQUEST.addUpdateType, NORMAL);

//@siclog "Step 23" siclog@

//The SS releases the RRC connection.

f_EUTRA_RRC_ConnectionRelease(eutra_CellB);

//@siclog "Step 24" siclog@

//The SS configures:- Cell B as the "Non-Suitable cell".- Cell G as a "Serving cell".

f_EUTRA_SetCellPower(eutra_CellB, tsc_NonSuitableCellRS_EPRE);

f_EUTRA_SetCellPower(eutra_CellG, tsc_ServingCellRS_EPRE);

//@siclog "Step 25" siclog@

//Check: Does the UE include UE radio capability availability indication in TRACKING AREA UPDATE REQUEST message?

v_NAS_Ind := f_EUTRA_TrackingAreaUpdate_Step1_3(eutra_CellG,

tsc_RRC_TI_Def,

NORMAL,

?,

'000'B);

if (match (v_NAS_Ind.Pdu.Msg.tRACKING_AREA_UPDATE_REQUEST.ueRadioCapIdAvailability.availability, '000'B)) { //prose CR needed.

f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 25");

}

//@siclog "Step 26" siclog@

//The SS transmits a TRACKING AREA UPDATE ACCEPT message.

//@siclog "Step 27" siclog@

//The UE transmits a TRACKING AREA UPDATE COMPLETE message.

v_EPS_ContextStatus := v_NAS_Ind.Pdu.Msg.tRACKING_AREA_UPDATE_REQUEST.epsBearerContextStatus;

f_EUTRA_TrackingAreaUpdate_Accept_Complete(eutra_CellG, f_EUTRA_CellInfo_GetGuti(eutra_CellG), '000'B, v_EPS_ContextStatus, v_NAS_Ind.Pdu.Msg.tRACKING_AREA_UPDATE_REQUEST.addUpdateType, NORMAL);

//@siclog "Step 28" siclog@

//The SS releases the RRC connection.

f_EUTRA_RRC_ConnectionRelease(eutra_CellG);

} //End of fl_TC_9_2_5_1_Body

After change:

function fl_TC_9_2_5_1_Body() runs on EUTRA_PTC

{

var NAS_MSG_Indication_Type v_NAS_Ind;

var template (omit) EPS_BearerContextStatus v_EPS_ContextStatus;

var NasCount_Type v_NasCountUL;

var EUTRA_SecurityParams_Type v_AuthParams;

var template (omit) UERadioCapId v_UERadioCapId;

//@siclog "Step 1" siclog@

//The UE is switched on.

f_EUTRA_SwitchOnUEandStartIP(eutra_CellA);

//@siclog "Step 2" siclog@

//Check: Does the UE include UE radio capability availability indication in ATTACH REQUEST message on Cell A?

v_NAS_Ind:= f_EUTRA_RRC_ConnEst_DefWithNas(eutra_CellA,

tsc_RRC_TI_Def,

cr_EstablishmentCause_Any,

cr_NAS_IndicationWithPiggybacking(tsc_SHT_IntegrityProtected,

cr_508_ATTACH_REQUEST(f_GetEAttachType(NORMAL), f_GetAdditionalUpdateType(NORMAL)),

cr_508_PDN_CONNECTIVITY_REQUEST(tsc_PdnRequest_InitialAttach,

f_GetPdnType())));

if (match (v_NAS_Ind.Pdu.Msg.aTTACH_REQUEST.ueRadioCapIdAvailability.availability, '001'B)) { //prose CR needed.

f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 2");

}

//@siclog "Step 3-4" siclog@

//Steps 5-6 of the generic procedure for UE registration specified in TS 36.508 subclause 4.5.2.3 are performed.

f_EUTRA_InitialRegistration_Step5_6(eutra_CellA, v_NAS_Ind);

v_AuthParams := f_EUTRA_Security_Get();

//@siclog "Step 5" siclog@

//The SS transmits a NAS SECURITY MODE COMMAND message requesting UE radio capability ID.

//siclog "Step 6" siclog@

//Check: Does the UE transmit a NAS SECURITY MODE COMPLETE message including UE radio capability ID 1?

if (ispresent(v_NAS_Ind.Pdu.Msg.aTTACH_REQUEST.ueAddSecurityCap)) {

v_AuthParams.NAS_SecurityCap5G := v_NAS_Ind.Pdu.Msg.aTTACH_REQUEST.ueAddSecurityCap;

}

v_NasCountUL:= f_EUTRA_NAS_ActivateSecurity(eutra_CellA,v_AuthParams,-,true,true, v_NAS_Ind.Pdu.Msg.aTTACH_REQUEST.ueRadioCapIdAvailability);

v_UERadioCapId := f_EUTRA_MobileInfo_GetRxdRACS_Id();

if (isvalue(v_UERadioCapId)) {

if (match(valueof(v_UERadioCapId), cs_UERadioCapId('66'O, '01000000000010'O))) {

f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 6");

} else {

f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 6");

}

} else {

f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 6");

}

//@siclog "Steps 7-11" siclog@

//Steps 9-13 of the generic procedure for UE registration specified in TS 36.508 subclause 4.5.2.3 are performed.

//@siclog "Step 12" siclog@

//SS transmits a ATTACH ACCEPT message with a new UE radio capability ID 2.

//@siclog "Steps 13-16" siclog@

//Steps 15-18 of the generic procedure for UE registration specified in TS 36.508 subclause 4.5.2.3 are performed.//step15 is done before this.

f_EUTRA_MobileInfo_SetRACS_IdToSend(valueof(cs_UERadioCapId('66'O, '01000000000020'O)));

f_EUTRA_InitialRegistration_Step9_18b1(eutra_CellA,v_NAS_Ind,v_NasCountUL);

//@siclog "Step 17" siclog@

//The SS configures:- Cell A as the "Non-Suitable cell".- Cell B as a "Serving cell".

f_EUTRA_SetCellPower(eutra_CellA, tsc_NonSuitableCellRS_EPRE);

f_EUTRA_SetCellPower(eutra_CellB, tsc_ServingCellRS_EPRE);

//@siclog "Step 18" siclog@

//Check: Does the UE include UE radio capability availability indication in TRACKING AREA UPDATE REQUEST message?

v_NAS_Ind := f_EUTRA_TrackingAreaUpdate_Step1_3(eutra_CellB,

tsc_RRC_TI_Def,

NORMAL,

?,

'001'B);

if (match (v_NAS_Ind.Pdu.Msg.tRACKING_AREA_UPDATE_REQUEST.ueRadioCapIdAvailability.availability, '001'B)) { //prose CR needed.

f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 18");

}

v_AuthParams := f_EUTRA_Security_Get();

//@siclog "Step 19" siclog@

//The SS transmits a NAS SECURITY MODE COMMAND message requesting UE radio capability ID.

//@siclog "Step 20" siclog@

//Check: Does the UE transmit a NAS SECURITY MODE COMPLETE message including UE radio capability ID 2?

v_NasCountUL:= f_EUTRA_NAS_ActivateSecurity(eutra_CellB,v_AuthParams,-,omit,false, v_NAS_Ind.Pdu.Msg.tRACKING_AREA_UPDATE_REQUEST.ueRadioCapIdAvailability);

v_UERadioCapId := f_EUTRA_MobileInfo_GetRxdRACS_Id();

if (isvalue(v_UERadioCapId)) {

if (match(valueof(v_UERadioCapId), cs_UERadioCapId('66'O, '01000000000020'O))) {

f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 20");

} else {

f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 20");

}

} else {

f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 20");

}

//@siclog "Step 21" siclog@

//The SS transmits a TRACKING AREA UPDATE ACCEPT message.

//@siclog "Step 22" siclog@

//The UE transmits a TRACKING AREA UPDATE COMPLETE message.

v_EPS_ContextStatus := v_NAS_Ind.Pdu.Msg.tRACKING_AREA_UPDATE_REQUEST.epsBearerContextStatus;

f_EUTRA_TrackingAreaUpdate_Accept_Complete(eutra_CellB, f_EUTRA_CellInfo_GetGuti(eutra_CellB), '001'B, v_EPS_ContextStatus, v_NAS_Ind.Pdu.Msg.tRACKING_AREA_UPDATE_REQUEST.addUpdateType, NORMAL);

//@siclog "Step 23" siclog@

//The SS releases the RRC connection.

f_EUTRA_RRC_ConnectionRelease(eutra_CellB);

//@siclog "Step 24" siclog@

//The SS configures:- Cell B as the "Non-Suitable cell".- Cell G as a "Serving cell".

f_EUTRA_SetCellPower(eutra_CellB, tsc_NonSuitableCellRS_EPRE);

f_EUTRA_SetCellPower(eutra_CellG, tsc_ServingCellRS_EPRE);

//@siclog "Step 25" siclog@

//Check: Does the UE include UE radio capability availability indication in TRACKING AREA UPDATE REQUEST message?

v_NAS_Ind := f_EUTRA_TrackingAreaUpdate_Step1_3(eutra_CellG,

tsc_RRC_TI_Def,

NORMAL,

?,

'001'B);

v_AuthParams := f_EUTRA_Security_Get();

//@siclog "Step 25A" siclog@

//The SS transmits a NAS SECURITY MODE COMMAND message requesting UE radio capability ID.

//@siclog "Step 25B" siclog@

//Check: Does the UE transmit a NAS SECURITY MODE COMPLETE message including UE radio capability ID 2?

v_NasCountUL:= f_EUTRA_NAS_ActivateSecurity(eutra_CellG,v_AuthParams,-,omit,false, v_NAS_Ind.Pdu.Msg.tRACKING_AREA_UPDATE_REQUEST.ueRadioCapIdAvailability);

v_UERadioCapId := f_EUTRA_MobileInfo_GetRxdRACS_Id();

if (isvalue(v_UERadioCapId)) {

if (match(valueof(v_UERadioCapId), cs_UERadioCapId('66'O, '00?????????'O))) {

f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 25B");

} else {

f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step25B: UE reported Network-assigned UE radio capability ID");

}

} else {

f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 25B");

}

//@siclog "Step 26" siclog@

//The SS transmits a TRACKING AREA UPDATE ACCEPT message.

//@siclog "Step 27" siclog@

//The UE transmits a TRACKING AREA UPDATE COMPLETE message.

v_EPS_ContextStatus := v_NAS_Ind.Pdu.Msg.tRACKING_AREA_UPDATE_REQUEST.epsBearerContextStatus;

f_EUTRA_TrackingAreaUpdate_Accept_Complete(eutra_CellG, f_EUTRA_CellInfo_GetGuti(eutra_CellG), '001'B, v_EPS_ContextStatus, v_NAS_Ind.Pdu.Msg.tRACKING_AREA_UPDATE_REQUEST.addUpdateType, NORMAL);

//@siclog "Step 28" siclog@

//The SS releases the RRC connection.

f_EUTRA_RRC_ConnectionRelease(eutra_CellG);

} //End of fl_TC_9_2_5_1_Body

# Branches executed

The test case implementation was executed with Integrity and ciphering enabled using LTE FDD band 4.

# Execution Log Files

## Samsung Shannon S5E9945ERD UE

The Samsung Shannon S5E9945ERD UE passed this test case on Anritsu Protocol Conformance Test System ME7834NR. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

TC_9_2_5_1_LOG.xml

- PICS/PIXIT parameter file:

TC_9_2_5_1 _PIXIT.txt

# References

| [1] | R5s240216: Supporting information for addition of LTE NAS RACS testcase 9.2.5.1 |
| --- | --- |