3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240403

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3399 | rev | 1 | Current version: | 17.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NR5GC FR1 : Addition of Rel-17 eCPSOR test case 6.3.2.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test, eCPSOR_CON-UEConTest |  |  |  |  |  | Date: |  |  | 2024-03-20 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To add test case 6.3.2.2 to the 5G/NR5GC ATS for FR1 path. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to test case 6.3.2.2 required for approval. See detailed change description for further information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.2.2.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR ... CR…38.523-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1	Change 1	5

2.2          Change 2	6

2.3          Change 3	7

3	Branches executed	10

4	Execution Log Files	11

4.1	Mediatek MT6815	11

5	References	11

# Overview

This document lists all the essential changes needed to correct problems in the TTCN implementation of test case 6.3.2.2 which is part of the 5G NR5GC test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:	Kalahasti, Narendra

Narendra.Kalahasti@anritsu.com

## Verification Test Summary

Test Case: 	6.3.2.2

ATS Version:	iwd-TTCN3-B2022-09_D23wk49

System Simulator used:	Anritsu Protocol Conformance Test System ME7834NR

UE used:	Mediatek MT6815

Verification Status:	PASS

# Corrections required

## Change 1

| Function name | function f_TC_6_3_2_2_NR5GC () |
| --- | --- |
| Reason for change | - As per prose 38.523-1 cl. 6.3.2.2.3.1 expecting 2 cells, on that one is cell11 and second is cell12. But on current TTCN, cell13 is implemented, so in this TTCN CR cell13 is replaced by NR Cell12. Same applied on Change 2.2 as well. - Sequence of test functions in not correct in test body of f_TC_6_3_2_2_NR5GC(), so need to correct that. - Cell12 was not powered up after initialization. |
| Summary of change | - Replaced cell13 from NR Cell 12. Same applied on Change 2.2 as well - Sequence of functions f_UT_USIM_Insert, f_NR5GC_Preamble, updated. - Added function f_NR_SetCellPower for cell 12. |
| TTCN module | SOR_CMCI_NR5GC |
| MCC160 Comment |  |

Before Change

function f_TC_6_3_2_2_NR5GC() runs on NR5GC_PTC { //Steering of UE in roaming after registration / SOR-CMCI rule / MMTEL voice call / DL NAS transport f_NR5GC_Init(NR_4); // Set Sys info and cell info for cell 11 and 13 to be used f_NR_CellInfo_SetPLMN(nr_Cell11, tsc_NR_HPLMN_002_11);//PLMN2 f_NR_CellInfo_SetPLMN(nr_Cell13, tsc_NR_HPLMN_002_91);//PLMN14 //Create and configure cell f_NR_CellConfig_Def(nr_Cell11); f_NR_CellConfig_Def(nr_Cell13); f_UT_SwitchOnUE(UT, false, CNF_REQUIRED); f_UT_AutomaticPLMN_Select (UT); f_UT_SwitchOffUE(UT, false, CNF_REQUIRED); // Equip UE with the USIM configuration 21 specified in table 6.4.1-21 TS 38.508-1 f_UT_USIM_Insert(UT, "USIM configuration 21 specified in table 6.4.1-21 TS 38.508-1"); f_NR_SetCellPower (nr_Cell1, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); f_NR5GC_Preamble (nr_Cell13, STATE_IDLE_1A); //Prose CR needed f_NR_TestBody_Set(true); fl_TC_6_3_2_2_TestBody(); f_NR_TestBody_Set(false); // Finish f_NR_Postamble(nr_Cell11, STATE_IDLE_1A); }

After Change

function f_TC_6_3_2_2_NR5GC() runs on NR5GC_PTC { //Steering of UE in roaming after registration / SOR-CMCI rule / MMTEL voice call / DL NAS transport f_NR5GC_Init(NR_4); // Set Sys info and cell info for cell 11 and 13 to be used f_NR_CellInfo_SetPLMN(nr_Cell11, tsc_NR_HPLMN_002_11);//PLMN2 f_NR_CellInfo_SetPLMN(nr_Cell12, tsc_NR_HPLMN_002_81);//PLMN13 //Create and configure cell f_NR_CellConfig_Def(nr_Cell11); f_NR_CellConfig_Def(nr_Cell12); // Equip UE with the USIM configuration 21 specified in table 6.4.1-21 TS 38.508-1 f_UT_USIM_Insert(UT, "USIM configuration 21 specified in table 6.4.1-21 TS 38.508-1"); f_UT_SwitchOnUE(UT, false, CNF_REQUIRED); f_UT_AutomaticPLMN_Select (UT); f_UT_SwitchOffUE(UT, false, CNF_REQUIRED); f_NR5GC_Preamble (nr_Cell12, STATE_IDLE_1A); //Prose CR needed f_NR_SetCellPower (nr_Cell11, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); f_NR_SetCellPower (nr_Cell12, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); f_NR_TestBody_Set(true); fl_TC_6_3_2_2_TestBody(); f_NR_TestBody_Set(false); // Finish f_NR_Postamble(nr_Cell11, STATE_IDLE_1A); }

## Change 2

| Function name | function fl_TC_6_3_2_2_TestBody () |
| --- | --- |
| Reason for change | - As per Table 6.3.2.2.5-1, SOR header has to include Additional parameters (AP) value, so SOR header should include it. - On current TTCN variable v_Data definition is not as per Table 6.3.2.2.5-1, And accordingly v_data should have v_PlmnID, v_Length_SOR_CMCI_contents and v_Length_SOR_CMCI_rule_contents and v_TsorcmTimer and v_CriterionType. - On Current TTCN after triggering IMS call there is no reception for RRCsetupRequest function, So need to add at step 22-34. - After step38, UE is in 5GMM-CONNECTED mode, and on after T-SorCM timer expiry the UE shall perform the deregistration procedure (see clause 4.2.2.3 of 3GPP TS 23.502 [63]) that releases all the established PDU sessions and services. #draft proseCR would be raised for RAN5#103 meeting |
| Summary of change | - Modified v_Header definition for additional parameters. - Updated v_data, please see beow for details. - Added function f_NR_RRC_SetupRequest_Def for cell 12. - Added IMS call release co-ordination msgs at step 38A. |
| TTCN module | SOR_CMCI_NR5GC |
| MCC160 Comment |  |

Before Change

function fl_TC_6_3_2_2_TestBody() runs on NR5GC_PTC { var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var boolean v_ULNASTransportReceived := false; var template (omit) GSM_MobilityInfo_Type v_GSM_MobilityInfo; var GSM_MobilityInfo_Type v_GSM_MobilityInfo_1; var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var NG_NAS_MSG_Indication_Type v_DetachReq; var NR_SRB_COMMON_IND v_ReceivedAsp; var template (value) NG_NAS_DL_Pdu_Type v_PduToSend; var octetstring v_NAS_TxMsgContainer_SOR; var TypeOfRegistration_Type v_TypeOfRegistration; var O16_Type v_Mac; var B128_Type v_Mac_A17; var NG_NAS_SecurityParams_Type v_NAS_SecurityParams; var octetstring v_Data; var NAS_PlmnId v_PlmnID := f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (nr_Cell11)); var O2_Type v_AccessTechId :='0800'O; var O2_Type v_Counter :='0001'O; var O1_Type v_Octet_O :='03'O; // 24.501 Table 9.11.3.51.2A Octet O (Bits 8-4 are spare, SSSI = '0'B, SSCMI = '1'B, SI = '1'B) var O5_Type v_SOR_CMCI_Rule1 :='0003450500'O; //Length of Rule1 = 5 octets var octetstring v_SOR_CMCI := '0005'O & v_SOR_CMCI_Rule1; var template (value) SOR_Header v_Header := cs_NG_SOR_Header('1'B,'1'B,'1'B,'0'B); timer t_Tmax; timer t_Tsor_cm := 60.0; //@siclog "Step 22 -34" siclog@ //FFS : Prose CR needed // Steps 1 to step 13 from test procedure for IMS MO speech call establishment as described in TS 38.508-1 [4] Table 4.9.15.2.2-1 take place. f_UT_RequestIMSCall(UT); v_ReceivedMsg := f_NR_RRCSetupComplete_Def(nr_Cell13, cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def), tsc_SHT_IntegrityProtected ); if (not f_Check_NG_ServiceReqMsg (v_ReceivedMsg.Pdu, cs_NAS_KeySetIdentifier_lv(f_NR_SecurityKSIamf_Get (), tsc_NasKsi_NativeSecurityContext), f_NG_S_TMSI2MobileIdentity(omit, f_NR5GC_CellInfo_GetGutiParameters (nr_Cell13)), '0001'B)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Service Request Message Failed"); } f_NR5GC_RRC_ConnectedState3N_Steps5_8(nr_Cell13, v_ReceivedMsg); v_GSM_MobilityInfo:= f_Get_PDUSessionForDNNType(f_NR5GC_MobileInfo_GetSessionInfoList(), IMS_DNN); //Now wait for trigger from IMS PTC to add voice f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS1], "Add Voice Bearer"); // Now Modify the PDU Session to add voice QoS f_NR5GC_ModifyPDUSession_AddVoice(nr_Cell13, valueof(v_GSM_MobilityInfo)); if (not pc_Preconditions) { f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS1], cms_IMS_IPCAN_Trigger("Voice Bearer Added")); } f_NR_PreliminaryPass(__FILE__, __LINE__, "Voice Call Established"); f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS1], "Voice Call Established"); //@siclog "Step 35" siclog@ /*The SS transmits an DLInformationTransfer message containing steering of roaming information indicating list of preferred PLMN/access technology combination provided with acknowledgment requested from the UE for successful reception*/ v_Data := v_PlmnID & v_AccessTechId & v_Octet_O & v_SOR_CMCI; v_NAS_SecurityParams := f_NR5GC_Security_Get(); v_Mac_A17 :=f_NG_Authentication_A17(bit2oct(encvalue(v_Header)), v_NAS_SecurityParams.KAUSF, v_Counter, v_Data, v_NAS_SecurityParams.KDF); v_Mac := bit2oct(v_Mac_A17); v_NAS_TxMsgContainer_SOR := bit2oct(encvalue(cs_NG_SORTransparentContainerValue(v_Header, v_Mac, v_Counter, v_Data, omit, omit))); v_PduToSend := f_Get_NG_DLNASTransport (cs_PayloadContainerType(tsc_PayloadContainerSORMsg), omit, cs_PayloadContainer(omit, v_NAS_TxMsgContainer_SOR)); SRB.send(cas_NR_SRB_NasPdu_REQ(nr_Cell13, tsc_NR_RbId_SRB2, cs_TimingInfo_Now, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered,v_PduToSend.Msg))); //@siclog "Step 36 - 37" siclog@ //SS starts timer of tmax =(6 minutes + cell selection time) (Note 1, 2 and 3) and Tsor-cm t_Tmax.start(f_NR_SetTimerToleranceMax(nr_Cell13, nonProtocolTimer, 660.0)); t_Tsor_cm.start; alt{ //@siclog "Step 38" siclog@ //The UE transmits an ULInformationTransfer message carrying acknowledgement of successful reception of the steering of roaming information [] SRB.receive(car_NR_SRB_NasPdu_IND(nr_Cell13, tsc_NR_RbId_SRB2, cr_NG_NAS_Ind(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp { if (not f_Check_NG_ULNASTransport(v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg, cr_PayloadContainerType(tsc_PayloadContainerSORMsg), omit)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "UL NAS Transport Message Failed"); }else{ if(not v_ULNASTransportReceived) { f_NR_PreliminaryPass (__FILE__, __LINE__, "Step 38"); v_ULNASTransportReceived := true; } } repeat; } [] t_Tsor_cm.timeout { //@siclog "Step 38A" siclog@ f_NR5GC_CallRelease(nr_Cell13, IPCAN_SpeechCall); // @sic R5-233457 sic@ //@siclog "Step 39" siclog@ //Check: Does the UE transmit a DEREGISTRATION REQUEST after Tsor-cm timer expires? SRB.receive(car_NR_SRB_NasPdu_IND(nr_Cell13, tsc_NR_RbId_SRB2, cr_NG_NAS_IndWithPiggybacking((tsc_SHT_IntegrityProtected_Ciphered)))) -> value v_ReceivedAsp; if (ischosen(v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg.deregistration_RequestMO)) { v_DetachReq := v_ReceivedAsp.Signalling.Nas[0]; } else { // Not a Detach Req if (not f_Check_NG_PDUSessionReleaseReq (v_GSM_MobilityInfo_1, f_NR5GC_MobileInfo_GetSessionInfoList(), v_ReceivedAsp.Signalling.Nas[0].Pdu, ?)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "PDU Session Release Request Message Failed"); } f_NR5GC_PDUSessionRelease(nr_Cell13, v_GSM_MobilityInfo_1.SessionId); // Now wait for deregister msg SRB.receive(car_NR_SRB_NasPdu_IND(nr_Cell13, tsc_NR_RbId_SRB2, cr_NG_NAS_Ind(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp; v_DetachReq := v_ReceivedAsp.Signalling.Nas[0]; } if (not f_Check_NG_DeregistrationReqMOMsg (v_DetachReq.Pdu.Msg, crs_DeregisterType('0'B, '0'B, '01'B))) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Deregistration Request Message Failed"); }else { f_NR_PreliminaryPass (__FILE__, __LINE__, "Step 39"); } //@siclog "Step 40" siclog@ //The SS transmits a DEREGISTRATION ACCEPT message. SRB.send(cas_NR_SRB_NasPdu_REQ(nr_Cell13,-, -,cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, cs_NG_DEREGISTRATION_ACCEPT))); //@siclog "Step XX" siclog@ //Prose CR needed f_NR_RRCRelease(nr_Cell13); //Check: Does the UE transmit an RRCSetupRequest on NR Cell 11 before tmax expires? (Note 1, 2 and 3) //@siclog "Step 41" siclog@ alt { [] SRB.check(receive(car_NR_SRB0_RrcPdu_IND(nr_Cell11, cr_38508_RRCSetupRequest(cr_NR_EstablishmentCause_Any)))) { t_Tmax.stop; f_NR_PreliminaryPass (__FILE__, __LINE__, "Step 41"); } [] t_Tmax.timeout { f_NR_SetVerdictFailOrInconc (__FILE__, __LINE__, "Step 41"); } } } } f_NR5GC_MobileInfo_Init(); //Reset everything and get ready for next Registration cycle v_TypeOfRegistration := f_NR5GC_InitialOrMobilityRegistrationRequest (v_GMM_MobilityInfo, v_ReceivedMsg, nr_Cell11); //@siclog "44a1-44a16a1" siclog@ //Void // @sic R5-233457 sic@ //@siclog "44b1-44b3a1" siclog@ //Void // @sic R5-233457 sic@ //@siclog "45 - 60a1" siclog@ //Steps 5 to 20a1 of Table 4.5.2.2-2 of the generic procedure in TS 38.508-1 [4] are performed on NR Cell 11. f_NR5GC_RRC_Idle_Steps5_20(v_GMM_MobilityInfo, nr_Cell11, v_ReceivedMsg, TESTMode_OFF,-, v_TypeOfRegistration); // @sic R5-233457 sic@ }//End of fl_TC_6_3_2_2_TestBody

After Change

function fl_TC_6_3_2_2_TestBody() runs on NR5GC_PTC { var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var boolean v_ULNASTransportReceived := false; var template (omit) GSM_MobilityInfo_Type v_GSM_MobilityInfo; var GSM_MobilityInfo_Type v_GSM_MobilityInfo_1; var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var NG_NAS_MSG_Indication_Type v_DetachReq; var NR_SRB_COMMON_IND v_ReceivedAsp; var template (value) NG_NAS_DL_Pdu_Type v_PduToSend; var octetstring v_NAS_TxMsgContainer_SOR; var TypeOfRegistration_Type v_TypeOfRegistration; var O16_Type v_Mac; var B128_Type v_Mac_A17; var NG_NAS_SecurityParams_Type v_NAS_SecurityParams; var octetstring v_Data; var NAS_PlmnId v_PlmnID := f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (nr_Cell11)); var O2_Type v_AccessTechId :='0800'O; var O2_Type v_Counter :='0001'O; var O1_Type v_Octet_O :='03'O; // 24.501 Table 9.11.3.51.2A Octet O (Bits 8-4 are spare, SSSI = '0'B, SSCMI = '1'B, SI = '1'B) var O5_Type v_SOR_CMCI_Rule1 :='0003450500'O; //Length of Rule1 = 5 octets var octetstring v_SOR_CMCI := '0005'O & v_SOR_CMCI_Rule1; var template (value) SOR_Header v_Header := cr_NG_SOR_Header('1'B,'1'B,'1'B,'0'B,'1'B); timer t_Tmax; timer t_Tsor_cm := 60.0; var octetstring v_CriterionType:= '05'O; var O1_Type v_Length_SOR_CMCI_rule_contents :=int2oct(2,2); var O1_Type v_Length_SOR_CMCI_contents :=int2oct(4,2); var octetstring v_TsorcmTimer:= '21'O; var O1_Type v_Length_PlmnID_AccessTechId := int2oct(lengthof(v_PlmnID & v_AccessTechId),1); //@siclog "Step 22 -34" siclog@ //FFS : Prose CR needed // Steps 1 to step 13 from test procedure for IMS MO speech call establishment as described in TS 38.508-1 [4] Table 4.9.15.2.2-1 take place. f_UT_RequestIMSCall(UT); f_NR_RRC_SetupRequest_Def(nr_Cell12, mo_VoiceCall); v_ReceivedMsg := f_NR_RRCSetupComplete_Def(nr_Cell12, cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def), tsc_SHT_IntegrityProtected ); if (not f_Check_NG_ServiceReqMsg (v_ReceivedMsg.Pdu, cs_NAS_KeySetIdentifier_lv(f_NR_SecurityKSIamf_Get (), tsc_NasKsi_NativeSecurityContext), f_NG_S_TMSI2MobileIdentity(omit, f_NR5GC_CellInfo_GetGutiParameters (nr_Cell12)), '0001'B)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Service Request Message Failed"); } f_NR5GC_RRC_ConnectedState3N_Steps5_8(nr_Cell12, v_ReceivedMsg); v_GSM_MobilityInfo:= f_Get_PDUSessionForDNNType(f_NR5GC_MobileInfo_GetSessionInfoList(), IMS_DNN); //Now wait for trigger from IMS PTC to add voice f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS1], "Add Voice Bearer"); // Now Modify the PDU Session to add voice QoS f_NR5GC_ModifyPDUSession_AddVoice(nr_Cell12, valueof(v_GSM_MobilityInfo)); if (not pc_Preconditions) { f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS1], cms_IMS_IPCAN_Trigger("Voice Bearer Added")); } f_NR_PreliminaryPass(__FILE__, __LINE__, "Voice Call Established"); f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS1], "Voice Call Established"); //@siclog "Step 35" siclog@ /*The SS transmits an DLInformationTransfer message containing steering of roaming information indicating list of preferred PLMN/access technology combination provided with acknowledgment requested from the UE for successful reception*/ v_Data := v_Length_PlmnID_AccessTechId & v_PlmnID & v_AccessTechId & v_Octet_O & v_Length_SOR_CMCI_contents & v_Length_SOR_CMCI_rule_contents & v_TsorcmTimer & v_CriterionType; v_NAS_SecurityParams := f_NR5GC_Security_Get(); v_Mac_A17 :=f_NG_Authentication_A17(bit2oct(encvalue(v_Header)), v_NAS_SecurityParams.KAUSF, v_Counter, v_Data, v_NAS_SecurityParams.KDF); v_Mac := bit2oct(v_Mac_A17); v_NAS_TxMsgContainer_SOR := bit2oct(encvalue(cs_NG_SORTransparentContainerValue(v_Header, v_Mac, v_Counter, v_Data, omit, omit))); v_PduToSend := f_Get_NG_DLNASTransport (cs_PayloadContainerType(tsc_PayloadContainerSORMsg), omit, cs_PayloadContainer(omit, v_NAS_TxMsgContainer_SOR)); SRB.send(cas_NR_SRB_NasPdu_REQ(nr_Cell12, tsc_NR_RbId_SRB2, cs_TimingInfo_Now, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered,v_PduToSend.Msg))); //@siclog "Step 36 - 37" siclog@ //SS starts timer of tmax =(6 minutes + cell selection time) (Note 1, 2 and 3) and Tsor-cm t_Tmax.start(f_NR_SetTimerToleranceMax(nr_Cell12, nonProtocolTimer, 660.0)); t_Tsor_cm.start; alt{ //@siclog "Step 38" siclog@ //The UE transmits an ULInformationTransfer message carrying acknowledgement of successful reception of the steering of roaming information [] SRB.receive(car_NR_SRB_NasPdu_IND(nr_Cell12, tsc_NR_RbId_SRB2, cr_NG_NAS_Ind(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp { if (not f_Check_NG_ULNASTransport(v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg, cr_PayloadContainerType(tsc_PayloadContainerSORMsg), omit)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "UL NAS Transport Message Failed"); }else{ if(not v_ULNASTransportReceived) { f_NR_PreliminaryPass (__FILE__, __LINE__, "Step 38"); v_ULNASTransportReceived := true; } } repeat; } [] t_Tsor_cm.timeout { f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS1], cms_IMS_IPCAN_Trigger("Trigger MO Call Release")); f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS1], "IMS Call Release Msgs Sent"); f_NR_PreliminaryPass(__FILE__, __LINE__, "IMS Call Release"); //@siclog "Step 38A" siclog@ //f_NR5GC_CallRelease(nr_Cell12, IPCAN_SpeechCall); // @sic R5-233457 sic@ //@siclog "Step 39" siclog@ //Check: Does the UE transmit a DEREGISTRATION REQUEST after Tsor-cm timer expires? SRB.receive(car_NR_SRB_NasPdu_IND(nr_Cell12, tsc_NR_RbId_SRB2, cr_NG_NAS_IndWithPiggybacking((tsc_SHT_IntegrityProtected_Ciphered)))) -> value v_ReceivedAsp; if (ischosen(v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg.deregistration_RequestMO)) { v_DetachReq := v_ReceivedAsp.Signalling.Nas[0]; } else { // Not a Detach Req if (not f_Check_NG_PDUSessionReleaseReq (v_GSM_MobilityInfo_1, f_NR5GC_MobileInfo_GetSessionInfoList(), v_ReceivedAsp.Signalling.Nas[0].Pdu, ?)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "PDU Session Release Request Message Failed"); } f_NR5GC_PDUSessionRelease(nr_Cell12, v_GSM_MobilityInfo_1.SessionId); // Now wait for deregister msg SRB.receive(car_NR_SRB_NasPdu_IND(nr_Cell12, tsc_NR_RbId_SRB2, cr_NG_NAS_Ind(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp; v_DetachReq := v_ReceivedAsp.Signalling.Nas[0]; } if (not f_Check_NG_DeregistrationReqMOMsg (v_DetachReq.Pdu.Msg, crs_DeregisterType('0'B, '0'B, '01'B))) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Deregistration Request Message Failed"); }else { f_NR_PreliminaryPass (__FILE__, __LINE__, "Step 39"); } //@siclog "Step 40" siclog@ //The SS transmits a DEREGISTRATION ACCEPT message. SRB.send(cas_NR_SRB_NasPdu_REQ(nr_Cell12,-, -,cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, cs_NG_DEREGISTRATION_ACCEPT))); //@siclog "Step XX" siclog@ //Prose CR needed f_NR_RRCRelease(nr_Cell12); //Check: Does the UE transmit an RRCSetupRequest on NR Cell 11 before tmax expires? (Note 1, 2 and 3) //@siclog "Step 41" siclog@ alt { [] SRB.check(receive(car_NR_SRB0_RrcPdu_IND(nr_Cell11, cr_38508_RRCSetupRequest(cr_NR_EstablishmentCause_Any)))) { t_Tmax.stop; f_NR_PreliminaryPass (__FILE__, __LINE__, "Step 41"); } [] t_Tmax.timeout { f_NR_SetVerdictFailOrInconc (__FILE__, __LINE__, "Step 41"); } } } } f_NR5GC_MobileInfo_Init(); //Reset everything and get ready for next Registration cycle v_TypeOfRegistration := f_NR5GC_InitialOrMobilityRegistrationRequest (v_GMM_MobilityInfo, v_ReceivedMsg, nr_Cell11); //@siclog "44a1-44a16a1" siclog@ //Void // @sic R5-233457 sic@ //@siclog "44b1-44b3a1" siclog@ //Void // @sic R5-233457 sic@ //@siclog "45 - 60a1" siclog@ //Steps 5 to 20a1 of Table 4.5.2.2-2 of the generic procedure in TS 38.508-1 [4] are performed on NR Cell 11. f_NR5GC_RRC_Idle_Steps5_20(v_GMM_MobilityInfo, nr_Cell11, v_ReceivedMsg, TESTMode_OFF,-, v_TypeOfRegistration); // @sic R5-233457 sic@ }//End of fl_TC_6_3_2_2_TestBody

## Change 3

| Function name | function f_TC_6_3_2_2_NR5GC_IMS1 () |
| --- | --- |
| Reason for change | - Added wait trigger for IMS call release. - Added Co-ordination msg for confirmation of call release, after IMS call released successful. - After T_SorCOM expiry UE will not triggere deregister request. As UE will trigger deregistration Request MO. |
| Summary of change | - Added f_IMS_IPCAN_WaitForTrigger at step301-30a4. - Added f_IMS_IPCAN_SendCoOrdMsg at step 38A. - Commented out function f_IMS_IpcanReleaseWithOptionalDeregistration. |
| TTCN module | SOR_CMCI_NR5GC_IMS |
| MCC160 Comment |  |

Before Change

function f_TC_6_3_2_2_NR5GC_IMS1()runs on IMS_PTC { var IMS_InviteRequestWithSdp_Type v_InviteRequestWithSdp; f_IMS_PTC_Init(PDN_1); f_IMS_IPCAN_SendCoOrdMsg(IPCAN); f_IMS_Registration(); f_IMS_IPCAN_SendCoOrdMsg(IPCAN); //@siclog "Steps 30a1 - 30a4" siclog@ //FFS : Prose CR needed v_InviteRequestWithSdp := f_IMS_NR5GC_MOSpeechCall(); //added co-ordination step for call release //@siclog "Step 38A" siclog@ //Before the UE De-registers on NR PTC, it will release the voice call abd perform IMS deregistration f_IMS_CallReleaseMO(v_InviteRequestWithSdp.Invite); //added co-ordination step for call release f_IMS_IpcanReleaseWithOptionalDeregistration(); f_IMS_GlobalRelease(); //Since the UE has De-registered, the IMS needs to be reset //FFS : Prose CR needed f_IMS_IPCAN_SendCoOrdMsg(IPCAN); f_IMS_Registration(); f_IMS_IPCAN_SendCoOrdMsg(IPCAN); f_IMS_IpcanReleaseWithOptionalDeregistration(); }

After Change

function f_TC_6_3_2_2_NR5GC_IMS1()runs on IMS_PTC { var IMS_InviteRequestWithSdp_Type v_InviteRequestWithSdp; f_IMS_PTC_Init(PDN_1); f_IMS_IPCAN_SendCoOrdMsg(IPCAN); f_IMS_Registration(); f_IMS_IPCAN_SendCoOrdMsg(IPCAN); //@siclog "Steps 30a1 - 30a4" siclog@ //FFS : Prose CR needed v_InviteRequestWithSdp := f_IMS_NR5GC_MOSpeechCall(); f_IMS_IPCAN_WaitForTrigger(IPCAN, "Trigger MO Call Release"); //@siclog "Step 38A" siclog@ //Before the UE De-registers on NR PTC, it will release the voice call abd perform IMS deregistration f_IMS_CallReleaseMO(v_InviteRequestWithSdp.Invite); f_IMS_IPCAN_SendCoOrdMsg(IPCAN, cms_IMS_IPCAN_Trigger("IMS Call Release Msgs Sent")); //f_IMS_IpcanReleaseWithOptionalDeregistration(); f_IMS_GlobalRelease(); //Since the UE has De-registered, the IMS needs to be reset //FFS : Prose CR needed f_IMS_IPCAN_SendCoOrdMsg(IPCAN); f_IMS_Registration(); f_IMS_IPCAN_SendCoOrdMsg(IPCAN); f_IMS_IpcanReleaseWithOptionalDeregistration(); }

# Branches executed

The test case was executed on NR band n78 using NR ciphering nea2 and integrity algorithm nia2

# Execution Log Files

## Mediatek MT6815

The Mediatek MT6815 UE passed this test case on Anritsu Protocol Conformance Test System ME7834NR. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

TC_6_3_2_2_LOG.html

- PICS/PIXIT parameter file:

TC_6_3_2_2_PIXIT.xml

# References

| [1] | R5s230260: NR5GC FR1: Supporting information for addition of Rel-17 eCPSOR case 6.3.2.2 |
| --- | --- |