3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240405

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3401 | rev | 1 | Current version: | 17.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC testcase 11.3.12 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test |  |  |  |  |  | Date: |  |  | 2024-03-24 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the current TTCN implementation, at Step 8b16, the UE is being paged. This is unnecessary. At Step 8b14, the SS transmits an IP PDU, and releases the RRC connection at Step 8b15. The UE will initiate an RRC connection to loop back this IP PDU. Thus, there is no need to page the UE at Step 8b16. A Prose CR will be raised for this. Also, the UE will set the IE Service Type to data in the Service REQ it transmits at Step 8b19. This also needs to be addressed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Removed the call to function f_NR_Paging_Def Updated the TTCN to expect the Service Type set to data at Step 8b19. Please see below. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformant UE may failed the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.3.12.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  |  | 38.523-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2022-09_D24wk12 related to the title of this CR.

Contact:  Bhise Parikshit
parikshit.bhise@rohde-schwarz.com

# Corrections required

## Change 1

| Function name | fl_TC_11_3_12_TestBody |
| --- | --- |
| Reason for change | In the current TTCN implementation, at Step 8b16, the UE is being paged. This is unnecessary. At Step 8b14, the SS transmits an IP PDU, and releases the RRC connection at Step 8b15. The UE will initiate an RRC connection to loop back this IP PDU. Thus, there is no need to page the UE at Step 8b16. A Prose CR will be raised for this. Also, the UE will set the IE Service Type to data in the Service REQ it transmits at Step 8b19. This also needs to be addressed. |
| Summary of change | Removed the call to function f_NR_Paging_Def Updated the TTCN to expect the Service Type set to data at Step 8b19. Please see below. |
| TTCN module | UAC_NR5GC.ttcn |

Before Change:

<<SKIPPED CODE>> //@siclog "Step 8b14" siclog@ //The SS transmits one IP PDU. DRB.send(cas_NR_DRB_COMMON_REQ_SDAP_SDUList(nr_Cell1, v_QosFlowId, v_SDAP_Data)); //@siclog "Step 8b15" siclog@ // SS waits for 1s and then transmits a RRCRelease message f_Delay(1.0); f_NR_RRCRelease(nr_Cell1); //@siclog "Step 8b16 - 8b23" siclog@ //Steps 1-7 of procedure specified in 38.508-1 [4] Table 4.9.4.2.2-1 are perfromed //@sic R5s230765 R5-236448 sic@ f_NR_Paging_Def(nr_Cell1, RRC_IDLE); v_ReceivedMsg := f_NR5GC_RRCIdleToConnected_Steps2_4(nr_Cell1); f_NR5GC_RRC_ConnectedState3N_Steps5_8(nr_Cell1, v_ReceivedMsg, -, -, -, -, -, -, false); <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> //@siclog "Step 8b14" siclog@ //The SS transmits one IP PDU. DRB.send(cas_NR_DRB_COMMON_REQ_SDAP_SDUList(nr_Cell1, v_QosFlowId, v_SDAP_Data)); //@siclog "Step 8b15" siclog@ // SS waits for 1s and then transmits a RRCRelease message f_Delay(1.0); f_NR_RRCRelease(nr_Cell1); //@siclog "Step 8b16 - 8b23" siclog@ //Steps 1-7 of procedure specified in 38.508-1 [4] Table 4.9.4.2.2-1 are perfromed //@sic R5s230765 R5-236448 sic@ //WA#11_3_12 COMMENTED OUT f_NR_Paging_Def(nr_Cell1, RRC_IDLE); v_ReceivedMsg := f_NR5GC_RRCIdleToConnected_Steps2_4(nr_Cell1, '0010'B); //WA#11_3_12 f_NR5GC_RRC_ConnectedState3N_Steps5_8(nr_Cell1, v_ReceivedMsg, -, -, -, -, -, -, false); <<SKIPPED CODE>>