3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240406

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3402 | rev | 1 | Current version: | 17.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC testcase 6.3.1.10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-03-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Before R5-236305 was agreed, if pc_noOf_PDUsNewConnection >0 then the user was prompted for an AT command to establish a PDU session. With this understanding, the current sequence of Steps 16Aa1 and Steps 17Aa1-17Aa2 was working fine. However, with R5-236305 implemented, if pc_noOf_PDUsNewConnection > 0, then the user wont get an AT command prompt anymore. In this scenario, the Steps 17Aa1-17Aa2 wont be executed properly. If a user sets pc_noOf_PDUsSameConnection and pc_noOf_PDUsNewConnection to 0, then neither the Step 16Aa1 will be executed because pc_noOf_PDUsSameConnection = 0 , nor Step 17Aa1-17Aa2 will be executed. This needs to be corrected. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Instead of using condition (pc_noOf_PDUsNewConnection > 0) at Step 17Aa1, used (pc_noOf_PDUsNewConnection > 0 or pc_noOf_PDUsSameConnection == 0). A Prose CR will be raised for this. Please see below. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformant UE may failed the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.1.10.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  |  | 38.523-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Overview	4

2	Corrections required	4

2.1	Change 1	4

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2022-09_D24wk12 related to the title of this CR.

Contact:  Bhise Parikshit
parikshit.bhise@rohde-schwarz.com

# Corrections required

## Change 1

| Function name | fl_TC_6_3_1_10_TestBody |
| --- | --- |
| Reason for change | Before R5-236305 was agreed, if pc_noOf_PDUsNewConnection >0 then the user was prompted for an AT command to establish a PDU session. With this understanding, the current sequence of Steps 16Aa1 and Steps 17Aa1-17Aa2 was working fine. However, with R5-236305 implemented, if pc_noOf_PDUsNewConnection > 0, then the user wont get an AT command prompt anymore. In this scenario, the Steps 17Aa1-17Aa2 wont be executed properly. If a user sets pc_noOf_PDUsSameConnection and pc_noOf_PDUsNewConnection to 0, then neither the Step 16Aa1 will be executed because pc_noOf_PDUsSameConnection = 0 , nor Step 17Aa1-17Aa2 will be executed. This needs to be corrected. |
| Summary of change | Instead of using condition (pc_noOf_PDUsNewConnection > 0) at Step 17Aa1, used (pc_noOf_PDUsNewConnection > 0 or pc_noOf_PDUsSameConnection == 0). A Prose CR will be raised for this. Please see below. |
| TTCN module | SOR_NR5GC.ttcn |

Before Change:

<<SKIPPED CODE>> //EXCEPTION: Step 17A1 is performed if pc_noOf_PDUsNewConnection > 0 //@siclog "Step 18Ba1" siclog@//@sic R5-223418sic@ //The generic procedure for UE-requested PDU session establishment, specified in subclause 4.5.2.2-4, takes place performing establishment of UE-requested PDU session(s) with ExpectedNumberOfNewPDUSessions = pc_noOf_PDUsNewConnection and then release the RRC Connection. if (pc_noOf_PDUsNewConnection > 0 ){ f_NR5GC_RRC_Idle_Extension(nr_Cell1); f_NR_RRCRelease(nr_Cell1); //@sic R5-237333 sic@ } //@siclog "Step 18" siclog@ <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> //EXCEPTION: Step 17A1 is performed if pc_noOf_PDUsNewConnection > 0 //@siclog "Step 18Ba1" siclog@//@sic R5-223418sic@ //The generic procedure for UE-requested PDU session establishment, specified in subclause 4.5.2.2-4, takes place performing establishment of UE-requested PDU session(s) with ExpectedNumberOfNewPDUSessions = pc_noOf_PDUsNewConnection and then release the RRC Connection. if (pc_noOf_PDUsNewConnection > 0 or pc_noOf_PDUsSameConnection == 0 ){ //WA#6_3_1_10 f_NR5GC_RRC_Idle_Extension(nr_Cell1); f_NR_RRCRelease(nr_Cell1); //@sic R5-237333 sic@ } //@siclog "Step 18" siclog@ <<SKIPPED CODE>>