3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240411

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4827 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NB-IoT NTN test case 22.3.1.5a |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test, LTE_NBIOT_eMTC_NTN_plus_EPS-UEConTest |  |  |  |  |  | Date: |  |  | 2024-06-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | At step 4 of the test case, UL Grant transmission need to be started. However function call to f_NBIOT_ULGrantTransmission() is missing at step 4. Also NPdcchperiod need to be initialised with current NPdcchPeriod before adding Delta parameters to it. Otherwise NPdcchPeriod shall be based on Step 2 and not Step 3, which would be wrong as per test procedure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Reinitialised v_NpdcchPeriod with current NpdcchPeriod and added call to f_NBIOT_ULGrantTransmission() to start the UL Grant transmission. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 22.3.1.5a |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2022-09_D24wk12 related to the title of this CR.

Contact:	Mohit Kanchan
	mohit.kanchan@keysight.com

# Corrections required

## Change 1

| Function name | f_TC_22_3_1_5a_NBIOT () |
| --- | --- |
| Reason for change | At step 4 of the test case, UL Grant transmission need to be started. However function call to f_NBIOT_ULGrantTransmission() is missing at step 4. Also NPdcchperiod need to be initialised with current NPdcchPeriod before adding Delta parameters to it. Otherwise NPdcchPeriod shall be based on Step 2 and not Step 3, which would be wrong as per test procedure. |
| Summary of change | Reinitialised v_NpdcchPeriod with current NpdcchPeriod and added call to f_NBIOT_ULGrantTransmission() to start the UL Grant transmission. |
| TTCN module | NBIOT_MAC_NTN.ttcn |
| MCC160 Comment |  |

Before Change:

function f_TC_22_3_1_5a_NBIOT() runs on NBIOT_PTC { var template (value) NPDCCH_ConfigDedicated_NB_r13 v_NPDCCH_ConfigDedicated; var NPdcchSearchSpaceCandidateIndex_Type v_SearchSpaceCandidateForULGrant; var template (value) NPdcchUESpecificSearchSpace_Type v_NPdcchUESpecificSearchSpace; var template (value) MAC_MainConfig_NB_r13 v_MAC_MainConfig := cs_MAC_MainConfig_NB_SRB; var template (value) DL_CCCH_Message_NB v_RrcConnSetup; var NB_RLC_CountsInfoList_Type v_RLC_CountsInfoListByRef; // RLC status to be maintained by Send/Receive functions var template (value) TimingInfo_Type v_TimingInfo; var template (value) MAC_PDU_Type v_MAC_PDU; var template (value) RLC_SDU_Type v_RlcSdu; var MAC_SDUList_Type v_MAC_SDUList; var SubFrameTiming_Type v_NpdcchPeriod; var NBIOT_SearchSpaceParameters_Type v_SearchSpaceParameters := tsc_NBIOT_SearchSpaceParameters_Def; var template (present) MAC_PDU_Type v_MAC_PDU_RX; var template (value) NB_UL_TransRetransmissionList_Type v_TransRetransmissionList; var template (value) NB_DciUlInfo_Type v_DciUlInfo; var integer v_IncrementVRR; var NB_SYSTEM_IND v_SYSTEM_IND; //System information combination 8 as defined in TS 36.508 [18] clause 8.1.4.3.1.1 is used. f_NBIOT_Init(c8); //Create and configure all cells f_NBIOT_CellConfig_Def(nbiot_Cell1, CONTROL_PLANE); // configure RRC Connection Setup being used during RACH procedure: Message modified according to Table 22.3.1.5a.3.3-1 v_MAC_MainConfig := cs_MAC_MainConfig_NB_SRB; v_MAC_MainConfig.ul_SCH_Config_r13.periodicBSR_Timer_r13 := infinity_; v_MAC_MainConfig.drx_Config_r13 := cs_DRX_Config_NB_Setup; v_NPDCCH_ConfigDedicated := cs_NPDCCH_ConfigDedicated_NB_DEFAULT; v_NPDCCH_ConfigDedicated.npdcch_NumRepetitions_r13 := r64; v_SearchSpaceParameters.Rmax := 64; // to be used for timing calculations v_RrcConnSetup := f_NBIOT_508_RRCConnectionSetup(-, -, v_MAC_MainConfig, v_NPDCCH_ConfigDedicated); f_NBIOT_SS_ConfigRachProcedureMsg4(nbiot_Cell1, v_RrcConnSetup); // Modify cell configuration for UE specific search space according to Table 22.3.1.5a.3.3-1 and UL grants to be scheduled in the last search space candidate as needed for step 2: v_SearchSpaceCandidateForULGrant := tsc_LastSearchSpaceCandidate; /* as we have a NPDCCH period of 256ms (instead of 64ms), there is no issue always to use the last search space candidate during the test case */ v_NPdcchUESpecificSearchSpace := cs_NPdcchUESpecificSearchSpace(v_NPDCCH_ConfigDedicated.npdcch_NumRepetitions_r13, v_NPDCCH_ConfigDedicated.npdcch_StartSF_USS_r13, v_NPDCCH_ConfigDedicated.npdcch_Offset_USS_r13, -, v_SearchSpaceCandidateForULGrant); f_NBIOT_SS_CommonCellConfig(nbiot_Cell1, cas_NB_CellConfig_REQ(nbiot_Cell1, -, -, cds_NB_BasicCellConfig_NPDCCH(cs_NPdcchConfig(v_NPdcchUESpecificSearchSpace)))); f_NBIOT_SS_DrxCtrlConfig(nbiot_Cell1, cs_NB_DrxCtrl(cs_DRX_Config_NB_Setup)); //@sic R5s230746 Ch2.1.2 sic@ //The UE is in state 2B-NB according to TS 36.508 [18] clause 8.1.5.2B with test loop mode G closed f_NBIOT_SetCellPower(nbiot_Cell1, tsc_ServingCellRS_EPRE); // Switch on UE f_UT_SwitchOnUE(UT, false); //The UE is in Automatic PLMN selection mode //f_UT_AutomaticPLMN_Select(UT); //@sic R5s2390746 Ch.2.1.1 sic@ //The preconfigured UE location is defined in TS 36.508 [18] Clause 4.13. f_UT_ConfigureNTNPosition(UT,-,NO_CNF_REQUIRED); // Now continue with the procedure f_NBIOT_InitialRegistration(nbiot_Cell1, CONTROL_PLANE, PREAMBLE, STATE2A_NB_TESTLOOP_ModeG); f_NBIOT_CloseUE_TestLoopModeGH(nbiot_Cell1, // Bring UE to state 2B-NB f_NBIOT_UE_TestLoopMode_Type(STATE2A_NB_TESTLOOP_ModeG), tsc_TestLoopModeGH_UplinkMode_RLC); v_RLC_CountsInfoListByRef := f_NBIOT_L2_CPMode_StopULGrant_EnableSrbTestMode(nbiot_Cell1, TransparentMode_RLC_MAC); //@sic R5s2390746 Ch.2.1.3 sic@ f_NBIOT_TestBody_Set(true, layer2); f_NBIOT_SS_DrxCtrlConfig(nbiot_Cell1, cs_NB_DrxCtrl_None); //@sic R5s2390746 Ch.2.1.4 sic@ f_Delay(1.0); //@sic R5s2390746 Ch.2.1.5 sic@ f_NBIOT_SS_ConfigUL_HARQ_IndMode(nbiot_Cell1, enable); //@siclog "Step 1" siclog@ //In the first NPDCCH period of the next DRX On Duration the SS schedules the DL transmission of a MAC PDU in the first search space candidate of the search space. v_NpdcchPeriod := fl_NBIOT_DrxCycle_NpdcchPeriod(nbiot_Cell1, tsc_FirstNpdcchPeriod, v_SearchSpaceParameters); v_TimingInfo := cs_TimingInfo_SubFrameFull(v_NpdcchPeriod); v_RlcSdu := crs_PDCP_SDU_16B; //v_RlcSduList := { v_RlcSdu }; v_MAC_SDUList := { f_NBIOT_MAC_CPMode_EncodeMacSdu(STATE2A_NB_TESTLOOP_ModeG, v_RlcSdu, v_RLC_CountsInfoListByRef.AM_VTS) }; // NAS/RRC PDU gets encoded by NAS emulator and packed into RLC PDU v_MAC_PDU := f_NBIOT_MAC_PDU_WithSDUs_TX(tsc_LchId_SIB1bis, v_MAC_SDUList); f_NBIOT_MAC_CPMode_SendMacPdu(nbiot_Cell1, v_TimingInfo, v_MAC_PDU, tsc_FirstSearchSpaceCandidate); v_RLC_CountsInfoListByRef.AM_VTS := v_RLC_CountsInfoListByRef.AM_VTS + 1; //@siclog "Step 1A" siclog@ //The UE transmit a HARQ ACK for the DL MAC PDU in Step 1. SYSIND.receive(car_NB_UL_HARQ_IND(nbiot_Cell1, ack)); //@siclog "Step 2" siclog@ //In the last NPDCCH period of the next DRX On Duration the SS allocates an UL grant for UE in the last search space candidate of the search space. v_NpdcchPeriod := fl_NBIOT_DrxCycle_NpdcchPeriod(nbiot_Cell1, tsc_LastNpdcchPeriod, v_SearchSpaceParameters); v_TimingInfo := cs_TimingInfo_SubFrameFull(v_NpdcchPeriod); v_TransRetransmissionList := {cs_NB_UL_TransRetransmission_NewTransmission, cs_NB_UL_TransRetransmission_ReTransmission(1)}; //@sic R5s230746 Ch2.1.6 sic@ v_DciUlInfo := cs_NB_DciInfo_CcchDcchDtchUL(2, 4, v_TransRetransmissionList); f_NBIOT_ULGrantTransmission(nbiot_Cell1, cs_NB_UL_GrantScheduling_Start(cs_NB_SingleGrant(v_DciUlInfo)), v_TimingInfo); //@siclog "Step 3" siclog@ //The UE loops back the MAC PDU. v_MAC_PDU_RX := f_NBIOT_MacPdu_SingleRlcAmdPdu_RX(-, v_RlcSdu); //same as in Step 1 f_NBIOT_MAC_CPMode_MacPdu_Receive(v_RLC_CountsInfoListByRef, nbiot_Cell1, v_MAC_PDU_RX); //@siclog "Step 4" siclog@ //In the NPDCCH sub frame when the drx-uLRetransmissionTimer-r13 for MAC PDU in Step 3 is still running, the SS transmits UL grant for the retransmission of the MAC PDU in step 3, and NDI is the same with which in step 2, and redundancy version = 1 //i.e. UL HARQ RTT + drx-uLRetransmissionTimer-r13 - x subframes after step 3 (x is between 0 and 20 the difference between On duration and UL Retransmission timer) (Note 5) [Changes Here] v_NpdcchPeriod := f_SubFrameTiming_AddMilliSeconds(v_NpdcchPeriod, 10); //x=10 v_NpdcchPeriod := f_SubFrameTiming_CalculateCycleStartTime(v_NpdcchPeriod, f_NBIOT_SearchSpace_GetPdcchPeriod(v_SearchSpaceParameters), v_SearchSpaceParameters.Alpha); v_TimingInfo := cs_TimingInfo_SubFrameFull(v_NpdcchPeriod); v_TransRetransmissionList := {cs_NB_UL_TransRetransmission_ReTransmission(1)}; //RV=1 v_DciUlInfo := cs_NB_DciInfo_CcchDcchDtchUL(2, 4, v_TransRetransmissionList); [Changes Here] //@siclog "Step 5" siclog@ //Void. //@siclog "Step 6" siclog@ //Check: Does the UE loop back the MAC PDU? v_IncrementVRR := 0; f_NBIOT_MAC_CPMode_MacPdu_Receive(v_RLC_CountsInfoListByRef, nbiot_Cell1, v_MAC_PDU_RX, v_IncrementVRR); //@sic R5s230746 Ch2.1.7 sic@ f_NBIOT_PreliminaryPass(__FILE__, __LINE__, "Step 6"); //@siclog "Step 6A" siclog@ //The SS transmits an RLC STATUS PDU to acknowledge received data. f_NBIOT_MAC_CPMode_SendRlcStatus(v_RLC_CountsInfoListByRef, nbiot_Cell1); SYSIND.receive(car_NB_UL_HARQ_IND(nbiot_Cell1, ack)); //@sic R5s230746 Ch2.1.8 sic@ //The SS stops allocating UL Grant. f_NBIOT_ULGrantTransmission(nbiot_Cell1, cs_NB_UL_GrantScheduling_Stop); //@siclog "Steps 7-10" siclog@ //Void. //@siclog "Step 11" siclog@ //In the last NPDCCH period when onDurationTimer-r13 is running, SS schedules the DL transmission of a MAC PDU in the last search space candidate and generates a CRC error causing the UE to send a NACK, but not perform any retransmissions. v_NpdcchPeriod := fl_NBIOT_DrxCycle_NpdcchPeriod(nbiot_Cell1, tsc_LastNpdcchPeriod, v_SearchSpaceParameters); fl_TC_22_3_1_5_SendInvalidMacPdu(nbiot_Cell1, v_NpdcchPeriod, tsc_LastSearchSpaceCandidate, v_SearchSpaceParameters); //@siclog "Step 12" siclog@ //The UE transmits a HARQ NACK for the DL MAC PDU in Step 11. SYSIND.receive(car_NB_UL_HARQ_IND(nbiot_Cell1, nack)); //@siclog "Step 13" siclog@ //In the next NPDCCH period the SS schedules the DL transmission of a (new) MAC PDU in the first search space candidate. //Note: The NDI of the corresponding DCI indicates a new transmission. (Note 5) v_NpdcchPeriod := fl_NBIOT_DrxCycle_NpdcchPeriod(nbiot_Cell1, tsc_FirstNpdcchPeriod, v_SearchSpaceParameters); v_TimingInfo := cs_TimingInfo_SubFrameFull(v_NpdcchPeriod); v_RlcSdu := crs_PDCP_SDU_16B; v_MAC_SDUList := { f_NBIOT_MAC_CPMode_EncodeMacSdu(STATE2A_NB_TESTLOOP_ModeG, v_RlcSdu, v_RLC_CountsInfoListByRef.AM_VTS) }; // NAS/RRC PDU gets encoded by NAS emulator and packed into RLC PDU v_MAC_PDU := f_NBIOT_MAC_PDU_WithSDUs_TX(tsc_LchId_SIB1bis, v_MAC_SDUList); f_NBIOT_MAC_CPMode_SendMacPdu(nbiot_Cell1, v_TimingInfo, v_MAC_PDU, tsc_FirstSearchSpaceCandidate); v_RLC_CountsInfoListByRef.AM_VTS := v_RLC_CountsInfoListByRef.AM_VTS + 1; //@siclog "Step 14" siclog@ //Check: Does the UE transmit a HARQ ACK for the DL MAC PDU in Step 13? SYSIND.receive(car_NB_UL_HARQ_IND(nbiot_Cell1, ack)); f_NBIOT_PreliminaryPass(__FILE__, __LINE__, "Step 14"); //@siclog "Step 15" siclog@ //In the last NPDCCH period when onDurationTimer-r13 is running, SS schedules the DL transmission of a MAC PDU in the last search space candidate and generates a CRC error causing the UE to send a NACK, but not perform any retransmissions. v_NpdcchPeriod := fl_NBIOT_DrxCycle_NpdcchPeriod(nbiot_Cell1, tsc_LastNpdcchPeriod, v_SearchSpaceParameters); fl_TC_22_3_1_5_SendInvalidMacPdu(nbiot_Cell1, v_NpdcchPeriod, tsc_LastSearchSpaceCandidate, v_SearchSpaceParameters); //@siclog "Step 16" siclog@ //The UE transmits a HARQ NACK for the DL MAC PDU in Step 15. SYSIND.receive(car_NB_UL_HARQ_IND(nbiot_Cell1, nack)) -> value v_SYSTEM_IND; v_TimingInfo := v_SYSTEM_IND.Common.TimingInfo; //@sic R5-237435 sic@ //@siclog "Step 17" siclog@ //Six NPDCCH periods after the step 16 the SS schedules the DL re-transmission of the MAC PDU in the last search space candidate. //Note 1: Six NPDCCH periods is the duration of the drx-RetransmissionTimer-r13. //(Note 5) v_NpdcchPeriod := f_NBIOT_IncrementSearchSpace(valueof(v_TimingInfo.SubFrame), int2float(5), v_SearchSpaceParameters); //@sic R5s230746 Ch2.1.9, R5-237435, R5s240006 sic@ v_TimingInfo := cs_TimingInfo_SubFrameFull(v_NpdcchPeriod); f_NBIOT_MAC_CPMode_SendMacPdu(nbiot_Cell1, v_TimingInfo, v_MAC_PDU, tsc_LastSearchSpaceCandidate); //@siclog "Step 18" siclog@ //Check: Does the UE transmit a HARQ ACK for the DL MAC PDU in Step 17? SYSIND.receive(car_NB_UL_HARQ_IND(nbiot_Cell1, ack)); f_NBIOT_PreliminaryPass(__FILE__, __LINE__, "Step 18"); //@siclog "Step 19" siclog@ //The SS starts allocating UL Grant f_NBIOT_SS_ConfigUL_HARQ_IndMode(nbiot_Cell1, disable); //@sic R5s230746 Ch.2.1.10 R5-237435 sic@ f_NBIOT_ULGrantTransmission(nbiot_Cell1, cs_NB_UL_GrantScheduling_Start); //@siclog "Step 20" siclog@ //The UE loops back the MAC PDU. f_NBIOT_MAC_CPMode_MacPdu_Receive(v_RLC_CountsInfoListByRef, nbiot_Cell1, v_MAC_PDU_RX); //@siclog "Step 21" siclog@ //The SS transmits an RLC STATUS PDU to acknowledge received data. f_NBIOT_MAC_CPMode_SendRlcStatus(v_RLC_CountsInfoListByRef, nbiot_Cell1); f_NBIOT_SS_DrxCtrlConfig(nbiot_Cell1, cs_NB_DrxCtrl(cs_DRX_Config_NB_Setup)); //@sic R5s2390746 Ch.2.1.4 sic@ f_NBIOT_TestBody_Set(false); //------------------------------------ f_NBIOT_L2_Postamble_State2B_NB(nbiot_Cell1, v_RLC_CountsInfoListByRef.AM_VTS, v_RLC_CountsInfoListByRef.AM_VRR); } //f_TC_22_3_1_5a_NTN_NBIOT

After Change:

function f_TC_22_3_1_5a_NBIOT() runs on NBIOT_PTC { var template (value) NPDCCH_ConfigDedicated_NB_r13 v_NPDCCH_ConfigDedicated; var NPdcchSearchSpaceCandidateIndex_Type v_SearchSpaceCandidateForULGrant; var template (value) NPdcchUESpecificSearchSpace_Type v_NPdcchUESpecificSearchSpace; var template (value) MAC_MainConfig_NB_r13 v_MAC_MainConfig := cs_MAC_MainConfig_NB_SRB; var template (value) DL_CCCH_Message_NB v_RrcConnSetup; var NB_RLC_CountsInfoList_Type v_RLC_CountsInfoListByRef; // RLC status to be maintained by Send/Receive functions var template (value) TimingInfo_Type v_TimingInfo; var template (value) MAC_PDU_Type v_MAC_PDU; var template (value) RLC_SDU_Type v_RlcSdu; var MAC_SDUList_Type v_MAC_SDUList; var SubFrameTiming_Type v_NpdcchPeriod; var NBIOT_SearchSpaceParameters_Type v_SearchSpaceParameters := tsc_NBIOT_SearchSpaceParameters_Def; var template (present) MAC_PDU_Type v_MAC_PDU_RX; var template (value) NB_UL_TransRetransmissionList_Type v_TransRetransmissionList; var template (value) NB_DciUlInfo_Type v_DciUlInfo; var integer v_IncrementVRR; var NB_SYSTEM_IND v_SYSTEM_IND; //System information combination 8 as defined in TS 36.508 [18] clause 8.1.4.3.1.1 is used. f_NBIOT_Init(c8); //Create and configure all cells f_NBIOT_CellConfig_Def(nbiot_Cell1, CONTROL_PLANE); // configure RRC Connection Setup being used during RACH procedure: Message modified according to Table 22.3.1.5a.3.3-1 v_MAC_MainConfig := cs_MAC_MainConfig_NB_SRB; v_MAC_MainConfig.ul_SCH_Config_r13.periodicBSR_Timer_r13 := infinity_; v_MAC_MainConfig.drx_Config_r13 := cs_DRX_Config_NB_Setup; v_NPDCCH_ConfigDedicated := cs_NPDCCH_ConfigDedicated_NB_DEFAULT; v_NPDCCH_ConfigDedicated.npdcch_NumRepetitions_r13 := r64; v_SearchSpaceParameters.Rmax := 64; // to be used for timing calculations v_RrcConnSetup := f_NBIOT_508_RRCConnectionSetup(-, -, v_MAC_MainConfig, v_NPDCCH_ConfigDedicated); f_NBIOT_SS_ConfigRachProcedureMsg4(nbiot_Cell1, v_RrcConnSetup); // Modify cell configuration for UE specific search space according to Table 22.3.1.5a.3.3-1 and UL grants to be scheduled in the last search space candidate as needed for step 2: v_SearchSpaceCandidateForULGrant := tsc_LastSearchSpaceCandidate; /* as we have a NPDCCH period of 256ms (instead of 64ms), there is no issue always to use the last search space candidate during the test case */ v_NPdcchUESpecificSearchSpace := cs_NPdcchUESpecificSearchSpace(v_NPDCCH_ConfigDedicated.npdcch_NumRepetitions_r13, v_NPDCCH_ConfigDedicated.npdcch_StartSF_USS_r13, v_NPDCCH_ConfigDedicated.npdcch_Offset_USS_r13, -, v_SearchSpaceCandidateForULGrant); f_NBIOT_SS_CommonCellConfig(nbiot_Cell1, cas_NB_CellConfig_REQ(nbiot_Cell1, -, -, cds_NB_BasicCellConfig_NPDCCH(cs_NPdcchConfig(v_NPdcchUESpecificSearchSpace)))); f_NBIOT_SS_DrxCtrlConfig(nbiot_Cell1, cs_NB_DrxCtrl(cs_DRX_Config_NB_Setup)); //@sic R5s230746 Ch2.1.2 sic@ //The UE is in state 2B-NB according to TS 36.508 [18] clause 8.1.5.2B with test loop mode G closed f_NBIOT_SetCellPower(nbiot_Cell1, tsc_ServingCellRS_EPRE); // Switch on UE f_UT_SwitchOnUE(UT, false); //The UE is in Automatic PLMN selection mode //f_UT_AutomaticPLMN_Select(UT); //@sic R5s2390746 Ch.2.1.1 sic@ //The preconfigured UE location is defined in TS 36.508 [18] Clause 4.13. f_UT_ConfigureNTNPosition(UT,-,NO_CNF_REQUIRED); // Now continue with the procedure f_NBIOT_InitialRegistration(nbiot_Cell1, CONTROL_PLANE, PREAMBLE, STATE2A_NB_TESTLOOP_ModeG); f_NBIOT_CloseUE_TestLoopModeGH(nbiot_Cell1, // Bring UE to state 2B-NB f_NBIOT_UE_TestLoopMode_Type(STATE2A_NB_TESTLOOP_ModeG), tsc_TestLoopModeGH_UplinkMode_RLC); v_RLC_CountsInfoListByRef := f_NBIOT_L2_CPMode_StopULGrant_EnableSrbTestMode(nbiot_Cell1, TransparentMode_RLC_MAC); //@sic R5s2390746 Ch.2.1.3 sic@ f_NBIOT_TestBody_Set(true, layer2); f_NBIOT_SS_DrxCtrlConfig(nbiot_Cell1, cs_NB_DrxCtrl_None); //@sic R5s2390746 Ch.2.1.4 sic@ f_Delay(1.0); //@sic R5s2390746 Ch.2.1.5 sic@ f_NBIOT_SS_ConfigUL_HARQ_IndMode(nbiot_Cell1, enable); //@siclog "Step 1" siclog@ //In the first NPDCCH period of the next DRX On Duration the SS schedules the DL transmission of a MAC PDU in the first search space candidate of the search space. v_NpdcchPeriod := fl_NBIOT_DrxCycle_NpdcchPeriod(nbiot_Cell1, tsc_FirstNpdcchPeriod, v_SearchSpaceParameters); v_TimingInfo := cs_TimingInfo_SubFrameFull(v_NpdcchPeriod); v_RlcSdu := crs_PDCP_SDU_16B; //v_RlcSduList := { v_RlcSdu }; v_MAC_SDUList := { f_NBIOT_MAC_CPMode_EncodeMacSdu(STATE2A_NB_TESTLOOP_ModeG, v_RlcSdu, v_RLC_CountsInfoListByRef.AM_VTS) }; // NAS/RRC PDU gets encoded by NAS emulator and packed into RLC PDU v_MAC_PDU := f_NBIOT_MAC_PDU_WithSDUs_TX(tsc_LchId_SIB1bis, v_MAC_SDUList); f_NBIOT_MAC_CPMode_SendMacPdu(nbiot_Cell1, v_TimingInfo, v_MAC_PDU, tsc_FirstSearchSpaceCandidate); v_RLC_CountsInfoListByRef.AM_VTS := v_RLC_CountsInfoListByRef.AM_VTS + 1; //@siclog "Step 1A" siclog@ //The UE transmit a HARQ ACK for the DL MAC PDU in Step 1. SYSIND.receive(car_NB_UL_HARQ_IND(nbiot_Cell1, ack)); //@siclog "Step 2" siclog@ //In the last NPDCCH period of the next DRX On Duration the SS allocates an UL grant for UE in the last search space candidate of the search space. v_NpdcchPeriod := fl_NBIOT_DrxCycle_NpdcchPeriod(nbiot_Cell1, tsc_LastNpdcchPeriod, v_SearchSpaceParameters); v_TimingInfo := cs_TimingInfo_SubFrameFull(v_NpdcchPeriod); v_TransRetransmissionList := {cs_NB_UL_TransRetransmission_NewTransmission, cs_NB_UL_TransRetransmission_ReTransmission(1)}; //@sic R5s230746 Ch2.1.6 sic@ v_DciUlInfo := cs_NB_DciInfo_CcchDcchDtchUL(2, 4, v_TransRetransmissionList); f_NBIOT_ULGrantTransmission(nbiot_Cell1, cs_NB_UL_GrantScheduling_Start(cs_NB_SingleGrant(v_DciUlInfo)), v_TimingInfo); //@siclog "Step 3" siclog@ //The UE loops back the MAC PDU. v_MAC_PDU_RX := f_NBIOT_MacPdu_SingleRlcAmdPdu_RX(-, v_RlcSdu); //same as in Step 1 f_NBIOT_MAC_CPMode_MacPdu_Receive(v_RLC_CountsInfoListByRef, nbiot_Cell1, v_MAC_PDU_RX); //@siclog "Step 4" siclog@ //In the NPDCCH sub frame when the drx-uLRetransmissionTimer-r13 for MAC PDU in Step 3 is still running, the SS transmits UL grant for the retransmission of the MAC PDU in step 3, and NDI is the same with which in step 2, and redundancy version = 1 //i.e. UL HARQ RTT + drx-uLRetransmissionTimer-r13 - x subframes after step 3 (x is between 0 and 20 the difference between On duration and UL Retransmission timer) (Note 5) v_NpdcchPeriod := fl_NBIOT_DrxCycle_NpdcchPeriod(nbiot_Cell1, tsc_LastNpdcchPeriod, v_SearchSpaceParameters); v_NpdcchPeriod := f_SubFrameTiming_AddMilliSeconds(v_NpdcchPeriod, 10); //x=10 v_NpdcchPeriod := f_SubFrameTiming_CalculateCycleStartTime(v_NpdcchPeriod, f_NBIOT_SearchSpace_GetPdcchPeriod(v_SearchSpaceParameters), v_SearchSpaceParameters.Alpha); v_TimingInfo := cs_TimingInfo_SubFrameFull(v_NpdcchPeriod); v_TransRetransmissionList := {cs_NB_UL_TransRetransmission_ReTransmission(1)}; //RV=1 v_DciUlInfo := cs_NB_DciInfo_CcchDcchDtchUL(2, 4, v_TransRetransmissionList); f_NBIOT_ULGrantTransmission(nbiot_Cell1, cs_NB_UL_GrantScheduling_Start(cs_NB_SingleGrant(v_DciUlInfo)), v_TimingInfo); //@siclog "Step 5" siclog@ //Void. //@siclog "Step 6" siclog@ //Check: Does the UE loop back the MAC PDU? v_IncrementVRR := 0; f_NBIOT_MAC_CPMode_MacPdu_Receive(v_RLC_CountsInfoListByRef, nbiot_Cell1, v_MAC_PDU_RX, v_IncrementVRR); //@sic R5s230746 Ch2.1.7 sic@ f_NBIOT_PreliminaryPass(__FILE__, __LINE__, "Step 6"); //@siclog "Step 6A" siclog@ //The SS transmits an RLC STATUS PDU to acknowledge received data. f_NBIOT_MAC_CPMode_SendRlcStatus(v_RLC_CountsInfoListByRef, nbiot_Cell1); SYSIND.receive(car_NB_UL_HARQ_IND(nbiot_Cell1, ack)); //@sic R5s230746 Ch2.1.8 sic@ //The SS stops allocating UL Grant. f_NBIOT_ULGrantTransmission(nbiot_Cell1, cs_NB_UL_GrantScheduling_Stop); //@siclog "Steps 7-10" siclog@ //Void. //@siclog "Step 11" siclog@ //In the last NPDCCH period when onDurationTimer-r13 is running, SS schedules the DL transmission of a MAC PDU in the last search space candidate and generates a CRC error causing the UE to send a NACK, but not perform any retransmissions. v_NpdcchPeriod := fl_NBIOT_DrxCycle_NpdcchPeriod(nbiot_Cell1, tsc_LastNpdcchPeriod, v_SearchSpaceParameters); fl_TC_22_3_1_5_SendInvalidMacPdu(nbiot_Cell1, v_NpdcchPeriod, tsc_LastSearchSpaceCandidate, v_SearchSpaceParameters); //@siclog "Step 12" siclog@ //The UE transmits a HARQ NACK for the DL MAC PDU in Step 11. SYSIND.receive(car_NB_UL_HARQ_IND(nbiot_Cell1, nack)); //@siclog "Step 13" siclog@ //In the next NPDCCH period the SS schedules the DL transmission of a (new) MAC PDU in the first search space candidate. //Note: The NDI of the corresponding DCI indicates a new transmission. (Note 5) v_NpdcchPeriod := fl_NBIOT_DrxCycle_NpdcchPeriod(nbiot_Cell1, tsc_FirstNpdcchPeriod, v_SearchSpaceParameters); v_TimingInfo := cs_TimingInfo_SubFrameFull(v_NpdcchPeriod); v_RlcSdu := crs_PDCP_SDU_16B; v_MAC_SDUList := { f_NBIOT_MAC_CPMode_EncodeMacSdu(STATE2A_NB_TESTLOOP_ModeG, v_RlcSdu, v_RLC_CountsInfoListByRef.AM_VTS) }; // NAS/RRC PDU gets encoded by NAS emulator and packed into RLC PDU v_MAC_PDU := f_NBIOT_MAC_PDU_WithSDUs_TX(tsc_LchId_SIB1bis, v_MAC_SDUList); f_NBIOT_MAC_CPMode_SendMacPdu(nbiot_Cell1, v_TimingInfo, v_MAC_PDU, tsc_FirstSearchSpaceCandidate); v_RLC_CountsInfoListByRef.AM_VTS := v_RLC_CountsInfoListByRef.AM_VTS + 1; //@siclog "Step 14" siclog@ //Check: Does the UE transmit a HARQ ACK for the DL MAC PDU in Step 13? SYSIND.receive(car_NB_UL_HARQ_IND(nbiot_Cell1, ack)); f_NBIOT_PreliminaryPass(__FILE__, __LINE__, "Step 14"); //@siclog "Step 15" siclog@ //In the last NPDCCH period when onDurationTimer-r13 is running, SS schedules the DL transmission of a MAC PDU in the last search space candidate and generates a CRC error causing the UE to send a NACK, but not perform any retransmissions. v_NpdcchPeriod := fl_NBIOT_DrxCycle_NpdcchPeriod(nbiot_Cell1, tsc_LastNpdcchPeriod, v_SearchSpaceParameters); fl_TC_22_3_1_5_SendInvalidMacPdu(nbiot_Cell1, v_NpdcchPeriod, tsc_LastSearchSpaceCandidate, v_SearchSpaceParameters); //@siclog "Step 16" siclog@ //The UE transmits a HARQ NACK for the DL MAC PDU in Step 15. SYSIND.receive(car_NB_UL_HARQ_IND(nbiot_Cell1, nack)) -> value v_SYSTEM_IND; v_TimingInfo := v_SYSTEM_IND.Common.TimingInfo; //@sic R5-237435 sic@ //@siclog "Step 17" siclog@ //Six NPDCCH periods after the step 16 the SS schedules the DL re-transmission of the MAC PDU in the last search space candidate. //Note 1: Six NPDCCH periods is the duration of the drx-RetransmissionTimer-r13. //(Note 5) v_NpdcchPeriod := f_NBIOT_IncrementSearchSpace(valueof(v_TimingInfo.SubFrame), int2float(5), v_SearchSpaceParameters); //@sic R5s230746 Ch2.1.9, R5-237435, R5s240006 sic@ v_TimingInfo := cs_TimingInfo_SubFrameFull(v_NpdcchPeriod); f_NBIOT_MAC_CPMode_SendMacPdu(nbiot_Cell1, v_TimingInfo, v_MAC_PDU, tsc_LastSearchSpaceCandidate); //@siclog "Step 18" siclog@ //Check: Does the UE transmit a HARQ ACK for the DL MAC PDU in Step 17? SYSIND.receive(car_NB_UL_HARQ_IND(nbiot_Cell1, ack)); f_NBIOT_PreliminaryPass(__FILE__, __LINE__, "Step 18"); //@siclog "Step 19" siclog@ //The SS starts allocating UL Grant f_NBIOT_SS_ConfigUL_HARQ_IndMode(nbiot_Cell1, disable); //@sic R5s230746 Ch.2.1.10 R5-237435 sic@ f_NBIOT_ULGrantTransmission(nbiot_Cell1, cs_NB_UL_GrantScheduling_Start); //@siclog "Step 20" siclog@ //The UE loops back the MAC PDU. f_NBIOT_MAC_CPMode_MacPdu_Receive(v_RLC_CountsInfoListByRef, nbiot_Cell1, v_MAC_PDU_RX); //@siclog "Step 21" siclog@ //The SS transmits an RLC STATUS PDU to acknowledge received data. f_NBIOT_MAC_CPMode_SendRlcStatus(v_RLC_CountsInfoListByRef, nbiot_Cell1); f_NBIOT_SS_DrxCtrlConfig(nbiot_Cell1, cs_NB_DrxCtrl(cs_DRX_Config_NB_Setup)); //@sic R5s2390746 Ch.2.1.4 sic@ f_NBIOT_TestBody_Set(false); //------------------------------------ f_NBIOT_L2_Postamble_State2B_NB(nbiot_Cell1, v_RLC_CountsInfoListByRef.AM_VTS, v_RLC_CountsInfoListByRef.AM_VRR); } //f_TC_22_3_1_5a_NTN_NBIOT