3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240412

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4828 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NB-IoT NTN test case 22.3.1.13 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test, LTE_NBIOT_eMTC_NTN_plus_EPS-UEConTest |  |  |  |  |  | Date: |  |  | 2024-06-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - At Step 4 and Step 16H of the test case, rrcconnectionRequest is expected from device. However, UE is in detached state device could use UE ID as randomevalue. However, the template handling this message expects it to be TMSI. - At step 21 after Successful reception of the UECapabilityInformation message from device, TTCN implementation is assigning a FAIL Verdict |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Modified the function to accept any UE ID in the RRCConnectionRequest message at step 4 & 6H - Modified the function to set a Priliminary PASS verdict at step 21 after handling the UECapabilityInformation message from device |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 22.3.1.13 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2022-09_D24wk12 related to the title of this CR.

Contact:	Mohit Kanchan
	mohit.kanchan@keysight.com

# Corrections required

## Change 1

| Function name | f_TC_22_3_1_13_NBIOT () |
| --- | --- |
| Reason for change | At Step 4 and Step 16H of the test case, rrcconnectionRequest is expected from device. However, UE is in detached state device could use UE ID as randomevalue. However, the template handling this message expects it to be TMSI. At step 21 after Successful reception of the UECapabilityInformation message from device, TTCN implementation is assigning a FAIL Verdict |
| Summary of change | Modified the function to accept any UE ID in the RRCConnectionRequest message at step 4 & 6H Modified the function to set a Priliminary PASS verdict at step 21 after handling the UECapabilityInformation message from device |
| TTCN module | NBIOT_MAC_NTN.ttcn |
| MCC160 Comment |  |

Before Change:

function f_TC_22_3_1_13_NBIOT() runs on NBIOT_PTC {//NB-IoT / NTN / UE specific TA report / UE specific Koffset //@sic R5-241563, R5-234494 sic@ var template (value) SystemInformationBlockType2_NB_r13 v_SIB2; var NAS_MSG_Indication_Type v_NAS_Ind; var NB_RLC_CountsInfoList_Type v_RLC_CountsInfoListByRef; // RLC status to be maintained by Send/Receive functions var template (value) TimingInfo_Type v_TimingInfo; var SubFrameTiming_Type v_TimingStart; var NB_SYSTEM_IND v_NB_SYSTEM_IND; var NB_SRB_COMMON_IND v_NB_SRB_COMMON_IND; var template (value) DL_CCCH_Message_NB v_RrcConnectionSetup; var float v_ModificationPeriod_s; timer t_Timer; //System information combination 8 as defined in TS 36.508 [18] clause 8.1.4.3.1.1 is used. f_NBIOT_Init(c8); //ta-Report-r17 is enabled by default //Create and configure all cells f_NBIOT_CellConfig_Def(nbiot_Cell1, CONTROL_PLANE); f_NBIOT_SetCellPower(nbiot_Cell1, tsc_ServingCellRS_EPRE); //The UE is in state 1-NB according to TS 36.508 [18] clause 8.1.5.1 // Switch on UE f_UT_SwitchOnUE(UT, false); //The UE is in Automatic PLMN selection mode //f_UT_AutomaticPLMN_Select (UT); //The preconfigured UE location is defined in TS 36.508 [18] Clause 4.13. f_UT_ConfigureNTNPosition(UT,-,NO_CNF_REQUIRED); // Now continue with the procedure f_NBIOT_InitialRegistration(nbiot_Cell1, CONTROL_PLANE, PREAMBLE); f_NBIOT_UE_Detach_SwitchOffUe(nbiot_Cell1, CONTROL_PLANE, RRC_CONNECTED); f_NBIOT_SS_ConfigRachPreambleIndMode(nbiot_Cell1, enable); f_NBIOT_SS_ConfigTimingAdvanceReportMode(nbiot_Cell1, enable);// @sic R5s240246 ch1 sic@ f_NBIOT_TestBody_Set(true); //@siclog "Step 1" siclog@ //Power on UE. f_UT_SwitchOnUE(UT, false); //@siclog "Step 2" siclog@ //The UE transmits preamble on PRACH SYSIND.receive(car_NB_RachPreamble_IND(nbiot_Cell1)); f_NBIOT_SS_ConfigRachPreambleIndMode(nbiot_Cell1, disable); //@siclog "Step 3" siclog@ //The SS transmits Random Access Response with RAPID corresponding to the transmitted Preamble in step 2 //@siclog "Step 4" siclog@ //The UE transmits an RRCConnectionRequest-NB message SRB.receive(car_NB_SRB0_RrcPdu_IND(nbiot_Cell1, cr_RRCConnectionRequest_NB(?))); //MB FFS cr_RRCConnectionRequest_NB(?,?) //@siclog "Step 4a" siclog@ //The SS transmits an RRCConnectionSetup-NB message //@sic R5240264 sic@ RRC Connection setup is already configured and provided to the SS as part of the RACH procedure //@siclog "Step 4b" siclog@ //Check: Does the UE transmit an RRCConnectionSetupComplete-NB message with one Timing Advance Report MAC CE? interleave { [] SRB.receive(car_NB_SRB_RrcNasPdu_IND(nbiot_Cell1, tsc_SRB1bis, cr_508_RRCConnectionSetupComplete_NB(tsc_RRC_TI_Def),?)) -> value v_NB_SRB_COMMON_IND {} //MB FFS cr_NAS_IndicationWithPiggybacking(v_AttReqSecurityHeader,v_AttachReq,v_ESMMessage) [] SYSIND.receive(car_NB_TARreport_IND(nbiot_Cell1, ?)) -> value v_NB_SYSTEM_IND {} } if (v_NB_SRB_COMMON_IND.Common.TimingInfo != v_NB_SYSTEM_IND.Common.TimingInfo) { f_NBIOT_SetVerdictFailOrInconc(__FILE__, __LINE__, "TA report is not received in the same MAC PDU as RRCConnectionSetupComplete-NB"); } else { f_NBIOT_PreliminaryPass(__FILE__, __LINE__, "Step 4b") } //@siclog "Steps 5-6" siclog@ //Void //@siclog "Steps 7-16" siclog@ // Steps 5 to 14b1 of the Attach, connected mode procedure described in TS 36.508 [18] subclause 8.1.5.2.3 are performed on Ncell 1. v_NAS_Ind := v_NB_SRB_COMMON_IND.Signalling.Nas[0]; f_NBIOT_InitialRegistration_Step5_14 (nbiot_Cell1, CONTROL_PLANE, STATE2_NB_CONNECTED_UPDATED, v_NAS_Ind); //@siclog "Step 16A" siclog@ //Power off UE. f_UT_SwitchOffUE(UT, false); //@siclog "Step 16B" siclog@ //The UE transmits a DETACH REQUEST message with the Detach type IE indicating "switch off". f_NBIOT_UE_DetachMO(nbiot_Cell1, RRC_CONNECTED, cr_DetachTypeOnSwitchOff); //@siclog "Step 16C" siclog@ //The SS transmits an RRCConnectionRelease-NB message to release RRC connection. f_NBIOT_RRC_ConnectionRelease(nbiot_Cell1, CONTROL_PLANE); //@siclog "Step 16D" siclog@ //SS broadcasts SystemInformationBlockType2-NB with ta-Report-r17 not present. v_SIB2 := f_NBIOT_CellInfo_GetSIB2(nbiot_Cell1); v_SIB2.radioResourceConfigCommon_r13.ntn_ConfigCommon_r17.ta_Report_r17 := omit; f_NBIOT_CellInfo_SetSIB2(nbiot_Cell1,v_SIB2); f_NBIOT_ModifySysinfo(nbiot_Cell1, false); v_ModificationPeriod_s := int2float(f_NB_CalculateModificationPeriod (nbiot_Cell1))/100.0; f_Delay(2.1 * v_ModificationPeriod_s); //@sic R5s240246 sic@ f_NBIOT_SS_ConfigRachPreambleIndMode(nbiot_Cell1, enable); // Prepare Msg4 at the SS for step 17 v_RrcConnectionSetup := f_NBIOT_508_RRCConnectionSetup(); v_RrcConnectionSetup.message_.c1.rrcConnectionSetup_r13.criticalExtensions.c1.rrcConnectionSetup_r13.radioResourceConfigDedicated_r13.mac_MainConfig_r13.explicitValue_r13.offsetThresholdTA_r17.setup := ms3; f_NBIOT_SS_ConfigRachProcedureMsg4(nbiot_Cell1, v_RrcConnectionSetup); //@sic R5s240246 sic@ //@siclog "Step 16E" siclog@ //Power on UE. f_UT_SwitchOnUE(UT, false); //@siclog "Step 16F" siclog@ //The UE transmits preamble on NPRACHSteps SYSIND.receive(car_NB_RachPreamble_IND(nbiot_Cell1)); f_NBIOT_SS_ConfigRachPreambleIndMode(nbiot_Cell1, disable); //@siclog "Step 16G" siclog@ //The SS transmits Random Access Response with RAPID corresponding to the transmitted Preamble in step 16F //@siclog "Step 16H" siclog@ //The UE transmits an RRCConnectionRequest-NB message with no Timing Advance Report MAC CE alt{ [] SRB.receive(car_NB_SRB0_RrcPdu_IND(nbiot_Cell1, cr_RRCConnectionRequest_NB(?))) { //MB FFS prose CR, implementation postponed t_Timer.start(0.2); alt { [] SYSIND.receive(car_NB_TARreport_IND(nbiot_Cell1, ?)){ f_NBIOT_SetVerdictFailOrInconc(__FILE__, __LINE__, "TA report should not be received"); } []t_Timer.timeout{} } } [] SYSIND.receive(car_NB_TARreport_IND(nbiot_Cell1, ?)){ f_NBIOT_SetVerdictFailOrInconc(__FILE__, __LINE__, "TA report should not be received"); } } //@siclog "Step 17" siclog@ //The SS transmits an RRCConnectionSetup-NB message to configure the offsetThresholdTA, Table 22.3.1.13.3.3-2 //@siclog "Step 18" siclog@ //Check: Does the UE transmit an RRCConnectionSetupComplete-NB message with one Timing Advance Report MAC CE? interleave { [] SRB.receive(car_NB_SRB_RrcNasPdu_IND(nbiot_Cell1, tsc_SRB1bis, cr_508_RRCConnectionSetupComplete_NB(tsc_RRC_TI_Def,?),?)) -> value v_NB_SRB_COMMON_IND {} //@sic R5s240246 sic@ [] SYSIND.receive(car_NB_TARreport_IND(nbiot_Cell1, ?)) -> value v_NB_SYSTEM_IND {} } if (v_NB_SRB_COMMON_IND.Common.TimingInfo != v_NB_SYSTEM_IND.Common.TimingInfo) { f_NBIOT_SetVerdictFailOrInconc(__FILE__, __LINE__, "TA report is not received in the same MAC PDU as RRCConnectionSetupComplete-NB"); } else { f_NBIOT_PreliminaryPass(__FILE__, __LINE__, "Step 18") } v_NAS_Ind := v_NB_SRB_COMMON_IND.Signalling.Nas[0]; //@siclog "Step 18A-18J" siclog@ //Steps 5 to 14b1 of the Attach, connected mode procedure described in TS 36.508 [18] subclause 8.1.5.2.3 are performed on Ncell 1. f_NBIOT_InitialRegistration_Step5_14 (nbiot_Cell1, CONTROL_PLANE, STATE2_NB_CONNECTED_UPDATED, v_NAS_Ind); //switch to L2 test mode v_RLC_CountsInfoListByRef := f_NBIOT_L2_CPMode_StopULGrant_EnableSrbTestMode(nbiot_Cell1, TransparentMode_RLC_MAC); //@siclog "Step 19" siclog@ //The SS transmits a MAC PDU containing a Differential Koffset MAC CE with differential Koffset set to 5 ms. f_NBIOT_MAC_CPMode_SendMacPdu(nbiot_Cell1, -, cs_MAC_PDU_DifferentialKoffset(int2bit(5,6))); //@sic R5s240246, ch 2.1.9 sic@ v_TimingStart := f_NBIOT_GetNextSearchSpace(nbiot_Cell1); v_TimingInfo := cs_TimingInfo_SubFrameFull(v_TimingStart); f_NBIOT_SS_CommonCellConfig(nbiot_Cell1, cas_NB_CellConfig_Update_NTNConfig(nbiot_Cell1, v_TimingInfo, -, cs_NB_NTN_Config(cs_NB_NTN_TimingOffset (545)))); //@sic R5s240246, ch 2.1.9 sic@ //leave L2 test mode v_TimingInfo := cs_TimingInfo_SubFrameFull(f_NBIOT_IncrementSearchSpace(v_TimingStart, 2.0)); //@sic R5s240246, ch 2.1.10 sic@ f_NBIOT_L2_CPMode_RestoreSRBs(nbiot_Cell1, v_RLC_CountsInfoListByRef.AM_VTS, v_RLC_CountsInfoListByRef.AM_VRR, v_TimingInfo); //@sic R5s240246, ch 2.1.10 sic@ f_NBIOT_ULGrantTransmission(nbiot_Cell1, cs_NB_UL_GrantScheduling_Start); //MB FFS postponed //@siclog "Steps 20-21" siclog@ //The SS sends UECapabilityEnquiry-NB message to the UE //Check: Does UE send a UECapabilityInformation-NB message after applying differential Koffset of 5ms received at step 19? f_NBIOT_Capability(nbiot_Cell1, CONTROL_PLANE); //Note: Successful reception of this message at SS confirms that differential Koffset is applied at UE. f_NBIOT_SetVerdictFailOrInconc (__FILE__, __LINE__, "Step 21"); f_NBIOT_TestBody_Set(false); //------------------------------------ f_NBIOT_Postamble(nbiot_Cell1, CONTROL_PLANE, N2_CONNECTED); }

After Change:

function f_TC_22_3_1_13_NBIOT() runs on NBIOT_PTC {//NB-IoT / NTN / UE specific TA report / UE specific Koffset //@sic R5-241563, R5-234494 sic@ var template (value) SystemInformationBlockType2_NB_r13 v_SIB2; var NAS_MSG_Indication_Type v_NAS_Ind; var NB_RLC_CountsInfoList_Type v_RLC_CountsInfoListByRef; // RLC status to be maintained by Send/Receive functions var template (value) TimingInfo_Type v_TimingInfo; var SubFrameTiming_Type v_TimingStart; var NB_SYSTEM_IND v_NB_SYSTEM_IND; var NB_SRB_COMMON_IND v_NB_SRB_COMMON_IND; var template (value) DL_CCCH_Message_NB v_RrcConnectionSetup; var float v_ModificationPeriod_s; timer t_Timer; //System information combination 8 as defined in TS 36.508 [18] clause 8.1.4.3.1.1 is used. f_NBIOT_Init(c8); //ta-Report-r17 is enabled by default //Create and configure all cells f_NBIOT_CellConfig_Def(nbiot_Cell1, CONTROL_PLANE); f_NBIOT_SetCellPower(nbiot_Cell1, tsc_ServingCellRS_EPRE); //The UE is in state 1-NB according to TS 36.508 [18] clause 8.1.5.1 // Switch on UE f_UT_SwitchOnUE(UT, false); //The UE is in Automatic PLMN selection mode //f_UT_AutomaticPLMN_Select (UT); //The preconfigured UE location is defined in TS 36.508 [18] Clause 4.13. f_UT_ConfigureNTNPosition(UT,-,NO_CNF_REQUIRED); // Now continue with the procedure f_NBIOT_InitialRegistration(nbiot_Cell1, CONTROL_PLANE, PREAMBLE); f_NBIOT_UE_Detach_SwitchOffUe(nbiot_Cell1, CONTROL_PLANE, RRC_CONNECTED); f_NBIOT_SS_ConfigRachPreambleIndMode(nbiot_Cell1, enable); f_NBIOT_SS_ConfigTimingAdvanceReportMode(nbiot_Cell1, enable);// @sic R5s240246 ch1 sic@ f_NBIOT_TestBody_Set(true); //@siclog "Step 1" siclog@ //Power on UE. f_UT_SwitchOnUE(UT, false); //@siclog "Step 2" siclog@ //The UE transmits preamble on PRACH SYSIND.receive(car_NB_RachPreamble_IND(nbiot_Cell1)); f_NBIOT_SS_ConfigRachPreambleIndMode(nbiot_Cell1, disable); //@siclog "Step 3" siclog@ //The SS transmits Random Access Response with RAPID corresponding to the transmitted Preamble in step 2 //@siclog "Step 4" siclog@ //The UE transmits an RRCConnectionRequest-NB message SRB.receive(car_NB_SRB0_RrcPdu_IND(nbiot_Cell1, cr_RRCConnectionRequest_NB(?, ?))); //MB FFS cr_RRCConnectionRequest_NB(?,?) //@siclog "Step 4a" siclog@ //The SS transmits an RRCConnectionSetup-NB message //@sic R5240264 sic@ RRC Connection setup is already configured and provided to the SS as part of the RACH procedure //@siclog "Step 4b" siclog@ //Check: Does the UE transmit an RRCConnectionSetupComplete-NB message with one Timing Advance Report MAC CE? interleave { [] SRB.receive(car_NB_SRB_RrcNasPdu_IND(nbiot_Cell1, tsc_SRB1bis, cr_508_RRCConnectionSetupComplete_NB(tsc_RRC_TI_Def),?)) -> value v_NB_SRB_COMMON_IND {} //MB FFS cr_NAS_IndicationWithPiggybacking(v_AttReqSecurityHeader,v_AttachReq,v_ESMMessage) [] SYSIND.receive(car_NB_TARreport_IND(nbiot_Cell1, ?)) -> value v_NB_SYSTEM_IND {} } if (v_NB_SRB_COMMON_IND.Common.TimingInfo != v_NB_SYSTEM_IND.Common.TimingInfo) { f_NBIOT_SetVerdictFailOrInconc(__FILE__, __LINE__, "TA report is not received in the same MAC PDU as RRCConnectionSetupComplete-NB"); } else { f_NBIOT_PreliminaryPass(__FILE__, __LINE__, "Step 4b") } //@siclog "Steps 5-6" siclog@ //Void //@siclog "Steps 7-16" siclog@ // Steps 5 to 14b1 of the Attach, connected mode procedure described in TS 36.508 [18] subclause 8.1.5.2.3 are performed on Ncell 1. v_NAS_Ind := v_NB_SRB_COMMON_IND.Signalling.Nas[0]; f_NBIOT_InitialRegistration_Step5_14 (nbiot_Cell1, CONTROL_PLANE, STATE2_NB_CONNECTED_UPDATED, v_NAS_Ind); //@siclog "Step 16A" siclog@ //Power off UE. f_UT_SwitchOffUE(UT, false); //@siclog "Step 16B" siclog@ //The UE transmits a DETACH REQUEST message with the Detach type IE indicating "switch off". f_NBIOT_UE_DetachMO(nbiot_Cell1, RRC_CONNECTED, cr_DetachTypeOnSwitchOff); //@siclog "Step 16C" siclog@ //The SS transmits an RRCConnectionRelease-NB message to release RRC connection. f_NBIOT_RRC_ConnectionRelease(nbiot_Cell1, CONTROL_PLANE); //@siclog "Step 16D" siclog@ //SS broadcasts SystemInformationBlockType2-NB with ta-Report-r17 not present. v_SIB2 := f_NBIOT_CellInfo_GetSIB2(nbiot_Cell1); v_SIB2.radioResourceConfigCommon_r13.ntn_ConfigCommon_r17.ta_Report_r17 := omit; f_NBIOT_CellInfo_SetSIB2(nbiot_Cell1,v_SIB2); f_NBIOT_ModifySysinfo(nbiot_Cell1, false); v_ModificationPeriod_s := int2float(f_NB_CalculateModificationPeriod (nbiot_Cell1))/100.0; f_Delay(2.1 * v_ModificationPeriod_s); //@sic R5s240246 sic@ f_NBIOT_SS_ConfigRachPreambleIndMode(nbiot_Cell1, enable); // Prepare Msg4 at the SS for step 17 v_RrcConnectionSetup := f_NBIOT_508_RRCConnectionSetup(); v_RrcConnectionSetup.message_.c1.rrcConnectionSetup_r13.criticalExtensions.c1.rrcConnectionSetup_r13.radioResourceConfigDedicated_r13.mac_MainConfig_r13.explicitValue_r13.offsetThresholdTA_r17.setup := ms3; f_NBIOT_SS_ConfigRachProcedureMsg4(nbiot_Cell1, v_RrcConnectionSetup); //@sic R5s240246 sic@ //@siclog "Step 16E" siclog@ //Power on UE. f_UT_SwitchOnUE(UT, false); //@siclog "Step 16F" siclog@ //The UE transmits preamble on NPRACHSteps SYSIND.receive(car_NB_RachPreamble_IND(nbiot_Cell1)); f_NBIOT_SS_ConfigRachPreambleIndMode(nbiot_Cell1, disable); //@siclog "Step 16G" siclog@ //The SS transmits Random Access Response with RAPID corresponding to the transmitted Preamble in step 16F //@siclog "Step 16H" siclog@ //The UE transmits an RRCConnectionRequest-NB message with no Timing Advance Report MAC CE alt{ [] SRB.receive(car_NB_SRB0_RrcPdu_IND(nbiot_Cell1, cr_RRCConnectionRequest_NB(?,?))) { //MB FFS prose CR, implementation postponed t_Timer.start(0.2); alt { [] SYSIND.receive(car_NB_TARreport_IND(nbiot_Cell1, ?)){ f_NBIOT_SetVerdictFailOrInconc(__FILE__, __LINE__, "TA report should not be received"); } []t_Timer.timeout{} } } [] SYSIND.receive(car_NB_TARreport_IND(nbiot_Cell1, ?)){ f_NBIOT_SetVerdictFailOrInconc(__FILE__, __LINE__, "TA report should not be received"); } } //@siclog "Step 17" siclog@ //The SS transmits an RRCConnectionSetup-NB message to configure the offsetThresholdTA, Table 22.3.1.13.3.3-2 //@siclog "Step 18" siclog@ //Check: Does the UE transmit an RRCConnectionSetupComplete-NB message with one Timing Advance Report MAC CE? interleave { [] SRB.receive(car_NB_SRB_RrcNasPdu_IND(nbiot_Cell1, tsc_SRB1bis, cr_508_RRCConnectionSetupComplete_NB(tsc_RRC_TI_Def,?),?)) -> value v_NB_SRB_COMMON_IND {} //@sic R5s240246 sic@ [] SYSIND.receive(car_NB_TARreport_IND(nbiot_Cell1, ?)) -> value v_NB_SYSTEM_IND {} } if (v_NB_SRB_COMMON_IND.Common.TimingInfo != v_NB_SYSTEM_IND.Common.TimingInfo) { f_NBIOT_SetVerdictFailOrInconc(__FILE__, __LINE__, "TA report is not received in the same MAC PDU as RRCConnectionSetupComplete-NB"); } else { f_NBIOT_PreliminaryPass(__FILE__, __LINE__, "Step 18") } v_NAS_Ind := v_NB_SRB_COMMON_IND.Signalling.Nas[0]; //@siclog "Step 18A-18J" siclog@ //Steps 5 to 14b1 of the Attach, connected mode procedure described in TS 36.508 [18] subclause 8.1.5.2.3 are performed on Ncell 1. f_NBIOT_InitialRegistration_Step5_14 (nbiot_Cell1, CONTROL_PLANE, STATE2_NB_CONNECTED_UPDATED, v_NAS_Ind); //switch to L2 test mode v_RLC_CountsInfoListByRef := f_NBIOT_L2_CPMode_StopULGrant_EnableSrbTestMode(nbiot_Cell1, TransparentMode_RLC_MAC); //@siclog "Step 19" siclog@ //The SS transmits a MAC PDU containing a Differential Koffset MAC CE with differential Koffset set to 5 ms. f_NBIOT_MAC_CPMode_SendMacPdu(nbiot_Cell1, -, cs_MAC_PDU_DifferentialKoffset(int2bit(5,6))); //@sic R5s240246, ch 2.1.9 sic@ v_TimingStart := f_NBIOT_GetNextSearchSpace(nbiot_Cell1); v_TimingInfo := cs_TimingInfo_SubFrameFull(v_TimingStart); f_NBIOT_SS_CommonCellConfig(nbiot_Cell1, cas_NB_CellConfig_Update_NTNConfig(nbiot_Cell1, v_TimingInfo, -, cs_NB_NTN_Config(cs_NB_NTN_TimingOffset (545)))); //@sic R5s240246, ch 2.1.9 sic@ //leave L2 test mode v_TimingInfo := cs_TimingInfo_SubFrameFull(f_NBIOT_IncrementSearchSpace(v_TimingStart, 2.0)); //@sic R5s240246, ch 2.1.10 sic@ f_NBIOT_L2_CPMode_RestoreSRBs(nbiot_Cell1, v_RLC_CountsInfoListByRef.AM_VTS, v_RLC_CountsInfoListByRef.AM_VRR, v_TimingInfo); //@sic R5s240246, ch 2.1.10 sic@ f_NBIOT_ULGrantTransmission(nbiot_Cell1, cs_NB_UL_GrantScheduling_Start); //MB FFS postponed //@siclog "Steps 20-21" siclog@ //The SS sends UECapabilityEnquiry-NB message to the UE //Check: Does UE send a UECapabilityInformation-NB message after applying differential Koffset of 5ms received at step 19? f_NBIOT_Capability(nbiot_Cell1, CONTROL_PLANE); //Note: Successful reception of this message at SS confirms that differential Koffset is applied at UE. f_NBIOT_PreliminaryPass(__FILE__, __LINE__, "Step 21"); f_NBIOT_TestBody_Set(false); //------------------------------------ f_NBIOT_Postamble(nbiot_Cell1, CONTROL_PLANE, N2_CONNECTED); }