3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240420

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-5 | CR | 0114 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of TTCN type definitions, Annex E |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test, MCImp-UEConTest |  |  |  |  |  | Date: |  |  | 2024-06-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Annex E for the TTCN type definitions should be updated to document the changes in the TTCN delivery iwd-24wk24 (Friday 14th June 2024). |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated Annex E. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The ASP changes will not be correctly documented. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex E |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Annex E (normative):
TTCN-3 Definitions for the MCX IPCAN test model

# E.0	Introduction

The present Annex E specifies the TTCN-3 type definitions used at the system interface to configure and control the SS. The type definitions are specific for the MCX IPCAN test model.

In case of discrepancy between the content of the present Annex E and the equivalent TTCN-3 definitions / semantic requirements found in the TTCN modules provided as attachments to the present specification, the latter shall take precedence.

NOTE:	This annex is automatically generated from the TTCN-3 modules provided as attachment to the present specification and containing the listed TTCN-3 type definitions.

# E.1	MCX_IPCAN_ASP_TypeDefs

MCX_IPCAN_ASP_TypeDefs: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| MCX_IPCAN_AbsoluteCellPower_Type | integer (-150..0) | absolute cell power (dBm); same as AbsoluteCellPower_Type |
| MCX_IPCAN_DeactivateDedicatedEpsBearerMCX_Type | MCX_IPCAN_EpsBearerIdList_Type | Deactivate dedicated EPS bearer(s) for the MCX PDN CellId : cell id of the cell Reference implementation: f_EUTRA_DeactivateEPS_BearerContext |

MCX_IPCAN_EpsBearerIdList_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | MCX_IPCAN_EpsBearerIdList_Type |
| Comment |  |
| record of MCX_IPCAN_EpsBearerId_Type |  |

MCX_IPCAN_PacketFilterDirection_Type

| TTCN-3 Enumerated Type |  |
| --- | --- |
| Name | MCX_IPCAN_PacketFilterDirection_Type |
| Comment | packet filter direction according to 24.008 Table 10.5.162 |
| downlinkOnly |  |
| uplinkOnly |  |
| bidirectional |  |

MCX_IPCAN_PacketFilterComponentPortRange_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | MCX_IPCAN_PacketFilterComponentPortRange_Type |  |  |
| Comment |  |  |  |
| Low | integer |  |  |
| High | integer |  |  |

MCX_IPCAN_PacketFilterComponent_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | MCX_IPCAN_PacketFilterComponent_Type |  |
| Comment | 24.008 Table 10.5.162 |  |
| Protocol | integer | Protocol identifier/Next header type: 17 for UDP |
| SingleRemotePort | integer |  |
| RemotePortRange | MCX_IPCAN_PacketFilterComponentPortRange_Type |  |

MCX_IPCAN_PacketFilterComponentList_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | MCX_IPCAN_PacketFilterComponentList_Type |
| Comment |  |
| record of MCX_IPCAN_PacketFilterComponent_Type |  |

MCX_IPCAN_PacketFilter_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | MCX_IPCAN_PacketFilter_Type |  |  |
| Comment |  |  |  |
| Direction | MCX_IPCAN_PacketFilterDirection_Type |  |  |
| ComponentList | MCX_IPCAN_PacketFilterComponentList_Type |  |  |

MCX_IPCAN_PacketFilterList_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | MCX_IPCAN_PacketFilterList_Type |
| Comment |  |
| record of MCX_IPCAN_PacketFilter_Type |  |

MCX_IPCAN_ASP_DedicatedEpsBearerInfo_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | MCX_IPCAN_ASP_DedicatedEpsBearerInfo_Type |  |  |
| Comment |  |  |  |
| EpsBearerId | MCX_IPCAN_EpsBearerId_Type |  | EPS bearer id of the dedicated EPS bearer |
| PacketFilterList | MCX_IPCAN_PacketFilterList_Type |  |  |

MCX_IPCAN_IPv4IPv6Address_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | MCX_IPCAN_IPv4IPv6Address_Type |  |  |
| Comment |  |  |  |
| IPv4 | charstring |  |  |
| IPv6 | charstring |  |  |

MCX_IPCAN_PDN_Info_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | MCX_IPCAN_PDN_Info_Type |  |  |
| Comment |  |  |  |
| PdnType | MCX_Registration_PDN_Type |  | MCX, IMS or INTERNET |
| EpsBearerId | MCX_IPCAN_EpsBearerId_Type |  | EPS bearer id of the PDN's default EPS bearer |
| IPAddressNW | MCX_IPCAN_IPv4IPv6Address_Type | opt | not present for MCX PDN as the IP signalling is handled by TTCN |
| IPAddressUE | MCX_IPCAN_IPv4IPv6Address_Type | opt | not present for MCX PDN as the IP signalling is handled by TTCN |

MCX_IPCAN_PDN_InfoList_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | MCX_IPCAN_PDN_InfoList_Type |
| Comment | list of PDNs to be connected to during Idle Update procedure |
| record length (1..3) of MCX_IPCAN_PDN_Info_Type |  |

MCX_IPCAN_ASP_CommonReqPart_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | MCX_IPCAN_ASP_CommonReqPart_Type |  |  |
| Comment |  |  |  |
| CellId | EUTRA_CellId_Type |  |  |
| CnfFlag | boolean |  |  |

MCX_IPCAN_Init_Type

| TTCN-3 Record Type |  |
| --- | --- |
| Name | MCX_IPCAN_Init_Type |
| Comment | Initialisation of the system simulator. CellId : eutra_Cell_NonSpecific Reference implementation: f_EUTRA_Init with p_SysinfoCombination:=c1, p_HandleULDataUM:=HANDLE_UM_DATA, p_UseBigGrants:=USE_BIG_GRAN |

MCX_IPCAN_CellConfig_Type

| TTCN-3 Record Type |  |
| --- | --- |
| Name | MCX_IPCAN_CellConfig_Type |
| Comment | Create a cell and send out system information. CellId : cell id of the cell to be created Reference implementation: f_EUTRA_CellConfig_Def |

MCX_IPCAN_CellPower_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | MCX_IPCAN_CellPower_Type |  |  |
| Comment | Set the cell power of a cell. CellId : cell id of the cell Reference implementation: f_EUTRA_SetCellPower |  |  |
| AbsoluteCellPower | MCX_IPCAN_AbsoluteCellPower_Type |  |  |

MCX_IPCAN_InitialRegistration_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | MCX_IPCAN_InitialRegistration_Type |  |  |
| Comment | Perform initial registration procedure according to 36.579-1 clause 5.4.1A and Table 5.4.2.2.3-1 steps 2 - 16A to bring the UE into state 3 according to Figure 4.5.1-1 in 36.508. As described in clause 5.4.1A of 36.579-1 after switch on the UE may register to an internet PDN, an IMS PDN and the MCX PDN. NOTE: The UE does not need to register to MCX during initial registration after switch-on. CellId : cell id of the cell |  |  |
| PdnList | MCX_IPCAN_PDN_InfoList_Type |  | list of PDNs the UE is expected to connect to during Idle Update procedure; the list may or may not contain MCX |
| DefaultAPN | octetstring |  | APN to be used for the PDN (if any) for which the UE does not provide an APN; APN encoded according to 23.003 clause 9.1 and RFC 1035 clause 4.1.2 |

MCX_IPCAN_AdditionalRegistrationMCX_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | MCX_IPCAN_AdditionalRegistrationMCX_Type |  |  |
| Comment | Procedure used, if the UE is configured not to connect automatically to the MCX PDN after switch-on: In this case after initial registration the UE gets triggered from TTCN by an MMI command to start MCX registration and this procedure is started to perform steps 19 - 26 of 36.579-1 Table 5.4.2.2.3-1. CellId : cell id of the cell |  |  |
| EpsBearerId | MCX_IPCAN_EpsBearerId_Type |  |  |

MCX_IPCAN_RRCConnectionSetupMO_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | MCX_IPCAN_RRCConnectionSetupMO_Type |  |  |
| Comment | MO RRC connection establishment according to 36.579-1 table 5.4.3.2.3-1 CellId : cell id of the cell Reference implementation: f_MCX_EUTRA_IPCAN_RRCConnectionEstablishment_MO |  |  |
| DedicatedEpsBearers | MCX_IPCAN_EpsBearerIdList_Type |  | In case of a pre-established session: list of dedicated EPS bearers for which the DRBs need to be established; otherwise {} |

MCX_IPCAN_RRCConnectionSetupMT_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | MCX_IPCAN_RRCConnectionSetupMT_Type |  |  |
| Comment | MT RRC connection establishment according to 36.579-1 table 5.4.4.2.3-1 CellId : cell id of the cell Reference implementation: f_MCX_EUTRA_IPCAN_RRCConnectionEstablishment_MT |  |  |
| DedicatedEpsBearers | MCX_IPCAN_EpsBearerIdList_Type |  | In case of a pre-established session: list of dedicated EPS bearers for which the DRBs need to be established; otherwise {} |

MCX_IPCAN_ActivateDedicatedEpsBearerMCX_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | MCX_IPCAN_ActivateDedicatedEpsBearerMCX_Type |
| Comment | Establish dedicated EPS bearer(s) for the MCX PDN CellId : cell id of the cell Reference implementation: f_MCX_EUTRA_IPCAN_ActivateDedicatedEpsBearer |
| record of MCX_IPCAN_ASP_DedicatedEpsBearerInfo_Typee |  |

MCX_IPCAN_RRCConnectionRelease_Type

| TTCN-3 Record Type |  |
| --- | --- |
| Name | MCX_IPCAN_RRCConnectionRelease_Type |
| Comment | Relase RRC connection CellId : cell id of the cell Reference implementation: f_EUTRA_RRC_ConnectionRelease |

MCX_IPCAN_Release_Type

| TTCN-3 Record Type |  |
| --- | --- |
| Name | MCX_IPCAN_Release_Type |
| Comment | Release all configuration at the system simulator; CellId : eutra_Cell_NonSpecific |

MCX_IPCAN_SystemRequest_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | MCX_IPCAN_SystemRequest_Type |  |
| Comment |  |  |
| Init | MCX_IPCAN_Init_Type |  |
| CellConfig | MCX_IPCAN_CellConfig_Type |  |
| CellPower | MCX_IPCAN_CellPower_Type |  |
| InitialRegistration | MCX_IPCAN_InitialRegistration_Type |  |
| AdditionalRegistrationMCX | MCX_IPCAN_AdditionalRegistrationMCX_Type |  |
| RRCConnectionSetupMO | MCX_IPCAN_RRCConnectionSetupMO_Type |  |
| RRCConnectionSetupMT | MCX_IPCAN_RRCConnectionSetupMT_Type |  |
| ActivateDedicatedEpsBearerMCX | MCX_IPCAN_ActivateDedicatedEpsBearerMCX_Type |  |
| DeactivateDedicatedEpsBearerMCX | MCX_IPCAN_DeactivateDedicatedEpsBearerMCX_Type |  |
| RRCConnectionRelease | MCX_IPCAN_RRCConnectionRelease_Type |  |
| Release | MCX_IPCAN_Release_Type |  |

MCX_IPCAN_SYSTEM_CTRL_REQ

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | MCX_IPCAN_SYSTEM_CTRL_REQ |  |  |
| Comment |  |  |  |
| Common | MCX_IPCAN_ASP_CommonReqPart_Type |  |  |
| Request | MCX_IPCAN_SystemRequest_Type |  |  |

MCX_IPCAN_InitConfirmation_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | MCX_IPCAN_InitConfirmation_Type |  |  |
| Comment |  |  |  |
| RanType | IPCAN_RAN_Type |  |  |

MCX_IPCAN_SystemConfirmation_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | MCX_IPCAN_SystemConfirmation_Type |  |
| Comment |  |  |
| Init | MCX_IPCAN_InitConfirmation_Type | Confirmation for Init request: carries the RAN Type |
| Other | Null_Type | Confirmation for any other request |
| Error | charstring | Error indication when the corresponding request causes error; the charstring may be used to provide information about the error |

MCX_IPCAN_SYSTEM_CTRL_CNF

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | MCX_IPCAN_SYSTEM_CTRL_CNF |  |  |
| Comment |  |  |  |
| Confirmation | MCX_IPCAN_SystemConfirmation_Type |  |  |

MCX_IPCAN_SYSTEM_PORT

| TTCN-3 Port Type |  |  |
| --- | --- | --- |
| Name | MCX_IPCAN_SYSTEM_PORT |  |
| Comment |  |  |
| out | MCX_IPCAN_SYSTEM_CTRL_REQ |  |
| in | MCX_IPCAN_SYSTEM_CTRL_CNF |  |

# E.2	MCX_CommonIPCAN

MCX_CommonIPCAN: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| MCX_IPCAN_EpsBearerId_Type | integer (5..13) | EPS bearer id: the associated DRB Id shall always be EpsBearerId - 4 |

MCX_Registration_PDN_Type

| TTCN-3 Enumerated Type |  |
| --- | --- |
| Name | MCX_Registration_PDN_Type |
| Comment | Type definition for PIXITs px_MCX_InitialRegistration_TypeOfPDN1, px_MCX_InitialRegistration_TypeOfPDN2, px_MCX_InitialRegistration_TypeOfPDN3; (see 36.579-1 clause 5.4.1A) NOTE: 'ims', 'internet', 'mcx' and 'none' cannot be used as 'none' is a keyword; 'IMS', 'INTERNET', 'MCX' and 'NONE' cannot be used as 'IMS' is a port name of various PTCs |
| imsPDN |  |
| internetPDN |  |
| mcxPDN |  |
| noPDN |  |

# E.3	CommonDefs

CommonDefs: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| Null_Type | boolean (true) | dummy type for 'typeless' fields in unions |

EUTRA_CellId_Type

| TTCN-3 Enumerated Type |  |
| --- | --- |
| Name | EUTRA_CellId_Type |
| Comment |  |
| eutra_Cell_NonSpecific |  |
| eutra_Cell1 |  |
| eutra_Cell2 |  |
| eutra_Cell3 |  |
| eutra_Cell4 |  |
| eutra_Cell6 |  |
| eutra_Cell10 |  |
| eutra_Cell11 |  |
| eutra_Cell12 |  |
| eutra_Cell13 |  |
| eutra_Cell14 |  |
| eutra_Cell23 |  |
| eutra_Cell28 |  |
| eutra_Cell29 |  |
| eutra_Cell30 |  |
| eutra_Cell31 |  |
| eutra_CellA |  |
| eutra_CellB |  |
| eutra_CellC |  |
| eutra_CellD |  |
| eutra_CellE |  |
| eutra_CellG |  |
| eutra_CellH |  |
| eutra_CellI |  |
| eutra_CellJ |  |
| eutra_CellK |  |
| eutra_CellL |  |
| eutra_CellM |  |

IPCAN_RAN_Type

| TTCN-3 Enumerated Type |  |
| --- | --- |
| Name | IPCAN_RAN_Type |
| Comment | radio access network technology used by the IPCAN PTC |
| GERAN |  |
| UTRAN_FDD |  |
| UTRAN_TDD |  |
| EUTRA_FDD |  |
| EUTRA_TDD |  |
| WLAN |  |
| NR |  |

# E.4	References to TTCN-3

| References to TTCN-3 |  |  |
| --- | --- | --- |
| MCX_IPCAN_ASP_TypeDefs | IPCAN_IpcanModel/MCX_IPCAN_ASP_TypeDefs.ttcn | Rev 38472 |
| MCX_CommonIPCAN | IPCAN_Common/MCX_CommonIPCAN.ttcn | Rev 37866 |
| CommonDefs | Common/CommonDefs.ttcn | Rev 38144 |